3GPP TSG-RAN WG4 Meeting #116                                                                                             R4-2511733

Bengaluru, IN, 25th –29th Aug, 2025

Source: 	CATT

Title: 	TP for TS 38.191 Clause 4 on General

Agenda item:	7.22.3.2

Document for:	Approval

- Introduction

This contribution provides the text proposal to the general parts of TS 38.191.

- References

- 3GPP TS 38.191 V0.0.1

- 3GPP TS 38.5xx

- Text proposal to TS 38.191

The following TP to TS 38.191 is provided for approval.

---------------------------------- Start of text proposal ----------------------------------

4	General

4.1	Relationship between minimum requirements and test requirements

The present document is a Single-RAT specification for NR Ambient IoT device, covering RF characteristics and minimum performance requirements. Conformance to the present specification is demonstrated by fulfilling the test requirements specified in the conformance specification 3GPP TS 5xx [2].

The Minimum Requirements given in this specification make no allowance for measurement uncertainty. The test specification TS 38.5xx [2] defines test tolerances. These test tolerances are individually calculated for each test. The test tolerances are used to relax the minimum requirements in this specification to create test requirements. For some requirements, including regulatory requirements, the test tolerance is set to zero.

The measurement results returned by the test system are compared - without any modification - against the test requirements as defined in 3GPP TS 38.5xx [2].

4.2	Applicability of minimum requirements

a)	In this specification the Minimum Requirements are specified as general requirements and additional requirements. Where the Requirement is specified as a general requirement, the requirement is mandated to be met in all scenarios

b)	For specific scenarios for which an additional requirement is specified, in addition to meeting the general requirement, the Ambient IoT device is mandated to meet the additional requirements.

c)	The spurious emissions power requirements are for the long-term average of the power. For the purpose of reducing measurement uncertainty it is acceptable to average the measured power over a period of time sufficient to reduce the uncertainty due to the statistical nature of the signal

----------------------------------- End of text proposal ----------------------------------