3GPP TSG-RAN4 Meeting #116	R4-2511745

Bengaluru, India, 25 – 29, August 2025

Agenda item:			7.22.5

Source:	Huawei, HiSilicon

Title:	Text proposal for Annex A and B of 39.191

Document for:	approval

<<< START OF CHANGES >>>

### Annex A (Normative)

### Device Coordinate System

Figure A.1: Reference coordinate system

The reference coordinate system is shown in figure A.1. If device has a rectangular shape, its orientation in the coordinate system is shown in figure A.2 with front and back side declared by device manufacturers. Otherwise, device positioning is based on device declaration.

Figure A.2: Reference coordinate system

<<< END OF CHANGES >>>

<<< START OF CHANGES >>>

### Annex B (Normative)

### Estimation of Measurement uncertainty

Based on Measurement Uncertainty (MU) values in [x, 38.870], the MU values of ambient IoT measurements for backscatter power and sensitivity are given below.

Table 1 MU for backscatter power

| UID | Description of uncertainty contribution for backscatter power | Uncertainty value |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Mismatch of receiver chain | 0.22 |
| 2 | Insertion loss of receiver chain | 0 |
| 3 | Influence of the measurement antenna cable | 0 |
| 4 | CW for backscatter measurement uncertainty of the absolute output level | -0.65 |
| 5 | Sensitivity measurement: output level step resolution | 0 |
| 6 | Measurement distance | 0 |
| 7 | Quality of quiet zone | 0.5 |
| 8 | Coarse sampling grid | 0.1 |
| 9 | Random uncertainty | 0.4 |
| 10 | Frequency Response | 0 |
| Stage 1: Calibration measurement, network analyzer method |  |  |
| 11 | Uncertainty of network analyzer | 0.2 |
| 12 | Mismatch of receiver chain | 0 |
| 13 | Insertion loss of receiver chain | 0 |
| 14 | Mismatch in the connection of calibration antenna | 0 |
| 15 | Influence of the calibration antenna feed cable | 0.3 |
| 16 | Influence of the measurement antenna cable | 0 |
| 17 | Uncertainty of the absolute gain/radiation efficiency of the calibration antenna | 0.58 |
| 18 | Measurement distance | 0 |
| 19 | Quality of quiet zone | 0.5 |
| Systematic Errors |  |  |
| 20 | Systematic Error related to grids | 0 |

Table 2 MU for sensitivity

| UID | Description of uncertainty contribution for sensitivity | Uncertainty value |
| --- | --- | --- |
| Stage 2: DUT measurement |  |  |
| 1 | Mismatch of receiver chain | 0.22 |
| 2 | Insertion loss of receiver chain | 0 |
| 3 | Influence of the measurement antenna cable | 0 |
| 4 | R2D for sensitivity measurement: uncertainty of the absolute output level | 0.27 |
| 5 | Sensitivity measurement: output level step resolution | 0 |
| 6 | Measurement distance | 0 |
| 7 | Quality of quiet zone | 0.5 |
| 8 | DUT sensitivity drift | 0.12 |
| 9 | Coarse sampling grid | 0.1 |
| 10 | Random uncertainty | 0.4 |
| 11 | Frequency Response | 0 |
| Stage 1: Calibration measurement, network analyzer method (Figure 7.3-1) |  |  |
| 12 | Uncertainty of network analyzer | 0.2 |
| 13 | Mismatch of receiver chain | 0 |
| 14 | Insertion loss of receiver chain | 0 |
| 15 | Mismatch in the connection of calibration antenna | 0 |
| 16 | Influence of the calibration antenna feed cable | 0.3 |
| 17 | Influence of the measurement antenna cable | 0 |
| 18 | Uncertainty of the absolute gain/radiation efficiency of the calibration antenna | 0.58 |
| 19 | Measurement distance | 0 |
| 20 | Quality of quiet zone | 0.5 |
| Systematic Errors |  |  |
| 21 | Systematic Error related to grids | 0 |

<<< END OF CHANGES >>>