3GPP TSG-RAN4 Meeting #116	R4-2511757

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | 1413 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big formal CR for TS38.101-3 for HPUE objective |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Huawei, Hisilicon, Skyworks Solutions Inc., Nokia, Apple, Qualcomm, Murata Manufacturing Co Ltd., ZTE, vivo, Mediatek, T-Mobile USA, CHTTL, AT&T, Ericsson, xiaomi, OPPO, LG Electronics, Verizon, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_Ph4-Core |  |  |  |  |  | Date: |  |  | 2025-08-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Merge all endorsed dCRs for HPUE objectives |  |  |  |  |  |  |  |  |
| Summary of change: |  | Merge following endorsed Rel-19 dCRs Tdoc number; Title. R4-2505142; Draft Rel-19 CR for 38.101-3 on increasing higher power limit for EN-DC with 2Tx/3Tx. R4-2511864; draft CR for the support of Inter-band EN-DC with 2Tx and 3Tx (ver.3). R4-2502866; draft CR on duty cycle solution for ENDC. R4-2505210; draft CR on duty cycle SAR solution for inter-band EN-DC with 3Tx. |
| Consequences if not approved: |  | Requirements missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2, 6.2B.1.3, 6.2B.4.1.3, 6.2H.1.3, 6.2H.4.1.3, 6.2L.1.3, 6.2L.4.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... TS 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## << Start of changes >>

## 4.2	Applicability of minimum requirements

a)	In this specification the Minimum Requirements are specified as general requirements and additional requirements. Where the Requirement is specified as a general requirement, the requirement is mandated to be met in all scenarios

b)	For specific scenarios for which an additional requirement is specified, in addition to meeting the general requirement, the UE is mandated to meet the additional requirements.

c)	The spurious emissions power requirements are for the long-term average of the power. For the purpose of reducing measurement uncertainty it is acceptable to average the measured power over a period of time sufficient to reduce the uncertainty due to the statistical nature of the signal

d)	Terminal that supports EN-DC or NE-DC configuration shall meet E-UTRA requirements as specified in TS 36.101 [4] and NR requirements as in TS 38.101-1 [2] and TS 38.101-2 [3] unless otherwise specified in this specification

e)	All the requirements for intra-band contiguous and non-contiguous EN-DC or NE-DC apply under the assumption of the same uplink-downlink and special subframe configurations in the E-UTRA and slot format indicated by UL-DL-configurationCommon and UL-DL-configurationDedicated in the NR for the EN-DC or NE-DC, a time offset between the two RATs configurations may be required.

f)	For EN-DC or NE-DC combinations with CA configurations for E-UTRA and/or NR, all the requirements for E-UTRA and/or NR all the requirements for E-UTRA and/or NR intra-band contiguous and non-contiguous CA apply under the assumption of the same slot format indicated by UL-DL-configurationCommon and UL-DL-configurationDedicated in the PSCell and SCells for NR and the same uplink-downlink and special subframe configurations in Pcell and SCells for E-UTRA.

A terminal which supports an EN-DC or NE-DC configuration shall support:

If any subsets of the EN-DC or NE-DC configuration do not specify its own bandwidth combination sets in 5.3B, then the terminal shall support the same E-UTRA bandwidth combination sets it signals the support for in E-UTRA CA configuration part of E-UTRA – NR DC and shall support the same NR bandwidth combination sets it signals the support for in NR CA configuration part of E-UTRA – NR DC.

Else if one of the subsets of the EN-DC or NE-DC configuration specify its own bandwidth combination sets in 5.3B, then the terminal shall support a product set of channel bandwidth for each band specified by E-UTRA bandwidth combination sets, NR bandwidth combination sets, and EN-DC or NE-DC bandwidth combination sets it singnals the support.A terminal which supports an inter-band EN-DC or NE-DC configuration with a certain UL configuration shall support the all lower order DL configurations of the lower order EN-DC or NE-DC combinations, which have this certain UL configuration and the fallbacks of this UL configuration.

A terminal which supports NE-DC configurations shall meet the minimum requirements for corresponding EN-DC configuration, unless otherwise specified.

For CA or DC configurations, which include FR2 intra-band CA combinations with multiple FR2 sub-blocks, where at least one of the sub-blocks is a contiguous CA combination :

-	if the field partialFR2-FallbackRX-Req is not present, the UE shall meet all applicable UE RF requirements for the highest order CA configuration and all associated fallback CA configurations;

-	if the field partialFR2-FallbackRX-Req is present, for each FR2 intra-band CA configuration with multiple sub-blocks that the UE indicates support for explicitly in UE capability signalling: the in-gap UE RF requirements in clauses 7.5A, 7.5B, 7.6A, 7.6B apply as the equivalent requirements for the associated fallback FR2 intra-band CA configurations with the same number of sub-blocks, where at least one of the sub-blocks consists of a contiguous CA configuration. The UE shall meet all applicable UE RF requirements for fallback CA configurations with a lesser number of sub-blocks;

-	regardless of the field partialFR2-FallbackRX-Req, the UE shall meet all DL out-of-gap requirements for all lower order fallback CA configurations.

Terminal that supports inter-band NR-DC between FR1 and FR2 configuration shall meet the requirements for corresponding CA configuration (suffix A), unless otherwise specified.

## << Next changes >>

#### 6.2B.1.3	Inter-band EN-DC within FR1

For inter-band EN-DC of E-UTRA and NR in FR1, the following UE Power Classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

The maximum output power for inter-band EN-DC with one Tx per band is specified in Table 6.2B.1.3-1. These configurations are subject to the applicable power class of each E-UTRA band and NR band specified in Table 6.2.2-1 of TS 36.101 and Table 6.2.1-1 of TS 38.101-1 respectively. The power classes referenced are according to the reported ue-PowerClass-N-r13 for the E-UTRA band or ue-CA-PowerClass-N for the E-UTRA intra-band UL CA of the EN-DC combination, and reported powerClassNRPart-r16 for the NR band and for NR intra-band UL CA of the EN-DC combination if indicated or ue-PowerClass otherwise.

If higherPowerLimitMRDC-r17 is indicated for an UL inter-band EN-DC configuration as specified in Table 6.2B.1.3-1  and with uplink bands of different power class capabilities, the UE maximum output power specified in Table 6.2B.1.3-1 for this UL EN-DC configuration is modified in accordance with sub-clause 6.2B.4.1.3.

Table 6.2B.1.3-1: Maximum output power for inter-band EN-DC (two bands)

| EN-DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n3A |  |  |  |  | 23 | +2/-3 |
| DC_1A_n5A |  |  |  |  | 23 | +2/-3 |
| DC_1A_n7A |  |  |  |  | 23 | +2/-3 |
| DC_1A_n8A |  |  |  |  | 23 | +2/-3 |
| DC_1A_n20A |  |  |  |  | 23 | +2/-3 |
| DC_1A_n26A |  |  |  |  | 23 | +2/-3 |
| DC_1A_n28A |  |  |  |  | 23 | +2/-3 |
| DC_1A_n38A |  |  |  |  | 23 | +2/-3 |
| DC_1A_n40A |  |  |  |  | 23 | +2/-3 |
| DC_1A_n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_1A_n50A |  |  |  |  | 23 | +2/-3 |
| DC_1A_n51A |  |  |  |  | 23 | +2/-3 |
| DC_1A_n71A |  |  |  |  | 23 | +2/-3 |
| DC_1A_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_1A_n84A_ULSUP-TDM_n77A |  |  | [266] | [+2/-3] | 23 | +2/-3 |
| DC_1A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_1A_n84A_ULSUP-TDM_n78A |  |  | [266] | [+2/-3] | 23 | +2/-3 |
| DC_1A_n79A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_1A_n84A_ULSUP-TDM_n79A |  |  |  |  | 23 | +2/-3 |
| DC_1A_n80A |  |  |  |  | 23 | +2/-3 |
| DC_1A_n105A |  |  |  |  | 23 | +2/-3 |
| DC_2A_n5A |  |  |  |  | 23 | +2/-3 |
| DC_2A_n7A |  |  |  |  | 23 | +2/-3 |
| DC_2A_n12A |  |  |  |  | 23 | +2/-3 |
| DC_2A_n25A |  |  |  |  | N/A | N/A |
| DC_2A_n28A |  |  |  |  | 23 | +2/-3 |
| DC_2A_n30A |  |  |  |  | 23 | +2/-3 |
| DC_2A_n38A |  |  |  |  | 23 | +2/-3 |
| DC_2A_n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_2A_n46A |  |  |  |  | 23 | +2/-3 |
| DC_2A_n48A |  |  |  |  | 23 | +2/-3 |
| DC_2A_n66A |  |  |  |  | 23 | +2/-3 |
| DC_2A_n71A |  |  |  |  | 23 | +2/-3 |
| DC_2A_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_2A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_3A_n1A |  |  |  |  | 23 | +2/-3 |
| DC_3C_n1A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n7A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n7B |  |  |  |  | 23 | +2/-3 |
| DC_3C_n7A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n8A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n20A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n26A |  |  |  |  | 23 | +2/-3 |
| DC_3C_n28A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n28A |  |  |  |  | 23 | +2/-3 |
| DC_3C_n26A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n38A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n40A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_3C_n41A, |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_3A_n50A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n51A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n71A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_3C_n77A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_3C_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_3A_n79A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_3C_n79A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n41A |  |  |  |  | 23 | +2/-3 |
| DC_3C_n80A_ULSUP-TDM_n41A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n77AA |  |  |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n78A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n80A_ULSUP-TDM_n79A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n82A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n84A |  |  |  |  | 23 | +2/-3 |
| DC_3A_n105A |  |  |  |  | 23 | +2/-3 |
| DC_4A_n2A |  |  |  |  | 23 | +2/-3 |
| DC_4A_n5A |  |  |  |  | 23 | +2/-3 |
| DC_4A_n7A |  |  |  |  | 23 | +2/-3 |
| DC_4A_n28A |  |  |  |  | 23 | +2/-3 |
| DC_4A_n38A |  |  |  |  | 23 | +2/-3 |
| DC_4A_n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_4A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_5A_n2A |  |  |  |  | 23 | +2/-3 |
| DC_5A_n7A |  |  |  |  | 23 | +2/-3 |
| DC_5A_n12A |  |  |  |  | 23 | +2/-3 |
| DC_5A_n25A |  |  |  |  | 23 | +2/-3 |
| DC_5A_n28A |  |  |  |  | 23 | +2/-3 |
| DC_5A_n30A |  |  |  |  | 23 | +2/-3 |
| DC_5A_n38A |  |  |  |  | 23 | +2/-3 |
| DC_5A_n40A |  |  |  |  | 23 | +2/-3 |
| DC_5A_n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_5A_n48A |  |  |  |  | 23 | +2/-3 |
| DC_5A_n66A |  |  |  |  | 23 | +2/-3 |
| DC_5A_n71A |  |  |  |  | 23 | +2/-3 |
| DC_5A_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_5A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_5A_n79A |  |  |  |  | 23 | +2/-3 |
| DC_5A_n89A_ULSUP-TDM_n78A |  |  |  |  | 23 | +2/-3 |
| DC_7A_n1A |  |  |  |  | 23 | +2/-3 |
| DC_7A_n2A |  |  |  |  | 23 | +2/-3 |
| DC_7A_n3A |  |  |  |  | 23 | +2/-3 |
| DC_7A_n5A |  |  |  |  | 23 | +2/-3 |
| DC_7C_n5A |  |  |  |  | 23 | +2/-3 |
| DC_7A_n8A |  |  |  |  | 23 | +2/-3 |
| DC_7A_n12A |  |  |  |  | 23 | +2/-3 |
| DC_7A_n20A |  |  |  |  | 23 | +2/-3 |
| DC_7A_n25A |  |  |  |  | 23 | +2/-3 |
| DC_7A_n26A |  |  |  |  | 23 | +2/-3 |
| DC_7C_n26A |  |  |  |  | 23 | +2/-3 |
| DC_7A_n28A |  |  |  |  | 23 | +2/-3 |
| DC_7A_n40A |  |  |  |  | 23 | +2/-3 |
| DC_7A_n51A |  |  |  |  | 23 | +2/-3 |
| DC_7A_n66A |  |  |  |  | 23 | +2/-3 |
| DC_7A_n71A |  |  |  |  | 23 | +2/-3 |
| DC_7A_n77A |  |  |  |  | 23 | +2/-3 |
| DC_7A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_7C_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_7A_n79A |  |  |  |  | 23 | +2/-3 |
| DC_7A_n80A |  |  |  |  | 23 | +2/-3 |
| DC_7A_n105A |  |  |  |  | 23 | +2/-3 |
| DC_8A_n1A |  |  |  |  | 23 | +2/-3 |
| DC_8B_n1A |  |  |  |  | 23 | +2/-3 |
| DC_8A_n2A |  |  |  |  | 23 | +2/-3 |
| DC_8A_n3A |  |  |  |  | 23 | +2/-3 |
| DC_8A_n7A |  |  |  |  | 23 | +2/-3 |
| DC_8A_n20A |  |  |  |  | 23 | +2/-3 |
| DC_8A_n28A |  |  |  |  | 23 | +2/-3 |
| DC_8A_n34A |  |  |  |  | 23 | +2/-3 |
| DC_8A_n38A |  |  |  |  | 23 | +2/-3 |
| DC_8A_n39A |  |  |  |  | 23 | +2/-3 |
| DC_8A_n40A |  |  |  |  | 23 | +2/-3 |
| DC_8A_n41A, |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_8A_n71A |  |  |  |  | 23 | +2/-3 |
| DC_8A_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_8A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_8B_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_8A_n79A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_8A_n79C |  |  |  |  | 23 | +2/-3 |
| DC_8A_n80A |  |  |  |  | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n41A |  |  |  |  | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n78A |  |  |  |  | 23 | +2/-3 |
| DC_8A_n81A_ULSUP-TDM_n79A |  |  |  |  | 23 | +2/-3 |
| DC_11A_n1A |  |  |  |  | 23 | +2/-3 |
| DC_11A_n3A |  |  |  |  | 23 | +2/-3 |
| DC_11A_n28A |  |  |  |  | 23 | +2/-3 |
| DC_11A_n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_11A_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_11A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_11A_n79A |  |  |  |  | 23 | +2/-3 |
| DC_12A_n2A |  |  |  |  | 23 | +2/-3 |
| DC_12A_n5A |  |  |  |  | 23 | +2/-3 |
| DC_12A_n7A |  |  |  |  | 23 | +2/-3 |
| DC_12A_n25A |  |  |  |  | 23 | +2/-3 |
| DC_12A_n30A |  |  |  |  | 23 | +2/-3 |
| DC_12A_n38A |  |  |  |  | 23 | +2/-3 |
| DC_12A_n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_12A_n66A |  |  |  |  | 23 | +2/-3 |
| DC_12A_n71A7 |  |  |  |  | 23 | +2/-3 |
| DC_12A_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_12A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_13A_n2A |  |  |  |  | 23 | +2/-3 |
| DC_13A_n5A |  |  |  |  | 23 | +2/-3 |
| DC_13A_n7A |  |  |  |  | 23 | +2/-3 |
| DC_13A_n25A |  |  |  |  | 23 | +2/-3 |
| DC_13A_n48A |  |  |  |  | 23 | +2/-3 |
| DC_13A_n66A |  |  |  |  | 23 | +2/-3 |
| DC_13A_n71A |  |  |  |  | 23 | +2/-3 |
| DC_13A_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_13A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_14A_n2A |  |  |  |  | 23 | +2/-3 |
| DC_14A_n5A |  |  |  |  | 23 | +2/-3 |
| DC_14A_n30A |  |  |  |  | 23 | +2/-3 |
| DC_14A_n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_14A_n66A |  |  |  |  | 23 | +2/-3 |
| DC_14A_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_18A_n3A |  |  |  |  | 23 | +2/-3 |
| DC_18A_n28A |  |  |  |  | 23 | +2/-3 |
| DC_18A_n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_18A_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_18A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_18A_n79A |  |  |  |  | 23 | +2/-3 |
| DC_19A_n1A |  |  |  |  | 23 | +2/-3 |
| DC_19A_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_19A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_19A_n79A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_20A_n1A |  |  |  |  | 23 | +2/-3 |
| DC_20A_n3A |  |  |  |  | 23 | +2/-3 |
| DC_20A_n7A |  |  |  |  | 23 | +2/-3 |
| DC_20A_n8A |  |  |  |  | 23 | +2/-3 |
| DC_20A_n38A |  |  |  |  | 23 | +2/-3 |
| DC_20A_n28A |  |  |  |  | 23 | +2/-3 |
| DC_20A_n40A |  |  |  |  | 23 | +2/-3 |
| DC_20A_n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_20A_n50A |  |  |  |  | 23 | +2/-3 |
| DC_20A_n51A |  |  |  |  | 23 | +2/-3 |
| DC_20A_n77A |  |  |  |  | 23 | +2/-3 |
| DC_20A_n80A |  |  |  |  | 23 | +2/-3 |
| DC_20A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_20A_n82A_ULSUP-TDM_n78A |  |  |  |  | 23 | +2/-3 |
| DC_20A_n83A |  |  |  |  | 23 | +2/-3 |
| DC_21A_n1A |  |  |  |  | 23 | +2/-3 |
| DC_21A_n28A |  |  |  |  | 23 | +2/-3 |
| DC_21A_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_21A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_21A_n79A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_25A_n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_25A_n77A |  |  |  |  | 23 | +2/-3 |
| DC_25A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_26A_n25A |  |  |  |  | 23 | +2/-3 |
| DC_26A_n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_26A_n77A |  |  |  |  | 23 | +2/-3 |
| DC_26A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_26A_n79A |  |  |  |  | 23 | +2/-3 |
| DC_28A_n1A |  |  |  |  | 23 | +2/-3 |
| DC_28A_n2A |  |  |  |  | 23 | +2/-3 |
| DC_28A_n3A |  |  |  |  | 23 | +2/-3 |
| DC_28A_n5A |  |  |  |  | 23 | +2/-3 |
| DC_28A_n7A |  |  |  |  | 23 | +2/-3 |
| DC_28A_n7B |  |  |  |  | 23 | +2/-3 |
| DC_28A_n8A |  |  |  |  | 23 | +2/-3 |
| DC_28A_n20A |  |  |  |  | 23 | +2/-3 |
| DC_28A_n38A |  |  |  |  | 23 | +2/-3 |
| DC_28A_n40A |  |  |  |  | 23 | +2/-3 |
| DC_28A_n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_28A_n50A |  |  |  |  | 23 | +2/-3 |
| DC_28A_n51A |  |  |  |  | 23 | +2/-3 |
| DC_28A_n66A |  |  |  |  | 23 | +2/-3 |
| DC_28A_n71A7 |  |  |  |  | 23 | +2/-3 |
| DC_28A_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_28A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_28A_n79A |  |  |  |  | 23 | +2/-3 |
| DC_28A_n105A7 |  |  |  |  | 23 | +2/-3 |
| DC_28A_n83A_ULSUP-TDM_n41A |  |  |  |  | 23 | +2/-3 |
| DC_28A_n83A_ULSUP-TDM_n78A |  |  |  |  | 23 | +2/-3 |
| DC_30A_n2A |  |  |  |  | 23 | +2/-3 |
| DC_30A_n5A |  |  |  |  | 23 | +2/-3 |
| DC_30A_n66A |  |  |  |  | 23 | +2/-3 |
| DC_30A_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_38A_n1A |  |  |  |  | 23 | +2/-3 |
| DC_38A_n3A |  |  |  |  | 23 | +2/-3 |
| DC_38A_n8A |  |  |  |  | 23 | +2/-3 |
| DC_38A_n28A |  |  |  |  | 23 | +2/-3 |
| DC_38A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_38A_n79A |  |  |  |  | 23 | +2/-3 |
| DC_39A_n40A |  |  |  |  | 23 | +2/-3 |
| DC_39A_n41A |  |  | 265 | +2/-3 | 23 | +2/-3 |
| DC_39C_n41A |  |  | 265 | +2/-3 | 23 | +2/-3 |
| DC_39A_n78A |  |  |  |  | 23 | +2/-3 |
| DC_39A_n79A |  |  | 265 | +2/-3 | 23 | +2/-3 |
| DC_40A_n1A |  |  |  |  | 23 | +2/-3 |
| DC_40A_n3A |  |  |  |  | 23 | +2/-3 |
| DC_40A_n7A |  |  |  |  | 23 | +2/-3 |
| DC_40A_n28A |  |  |  |  | 23 | +2/-3 |
| DC_40A_n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_40A_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_40C_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_40A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_40C_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_40A_n79A |  |  |  |  | 23 | +2/-3 |
| DC_41A_n1A |  |  |  |  | 23 | +2/-3 |
| DC_41C_n1A |  |  |  |  | 23 | +2/-3 |
| DC_41A_n3A |  |  |  |  | 23 | +2/-3 |
| DC_41C_n3A |  |  |  |  | 23 | +2/-3 |
| DC_41A_n28A |  |  |  |  | 23 | +2/-3 |
| DC_41C_n28A |  |  |  |  | 23 | +2/-3 |
| DC_41A_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_41C_n77A |  |  | [266] | [+2/-3] | 23 | +2/-3 |
| DC_41A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_41C_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_41A_n79A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_41A_n79C |  |  |  |  | 23 | +2/-3 |
| DC_41C_n79A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_42A_n1A |  |  |  |  | 23 | +2/-3 |
| DC_42C_n1A |  |  |  |  | 23 | +2/-3 |
| DC_42A_n3A |  |  |  |  | 23 | +2/-3 |
| DC_42C_n3A |  |  |  |  | 23 | +2/-3 |
| DC_42A_n28A |  |  |  |  | 23 | +2/-3 |
| DC_42C_n28A |  |  |  |  | 23 | +2/-3 |
| DC_42A_n51A |  |  |  |  | 23 | +2/-3 |
| DC_42A_n77A |  |  |  |  | N/A | N/A |
| DC_42A_n78A |  |  |  |  | N/A | N/A |
| DC_42A_n79A |  |  |  |  | N/A | N/A |
| DC_48A_n2A |  |  |  |  | 23 | +2/-3 |
| DC_48A_n5A |  |  |  |  | 23 | +2/-3 |
| DC_48A_n12A |  |  |  |  | 23 | +2/-3 |
| DC_48A_n25A |  |  |  |  | 23 | +2/-3 |
| DC_48A_n46A |  |  |  |  | 23 | +2/-3 |
| DC_48A_n66A |  |  |  |  | 23 | +2/-3 |
| DC_48A_n71A |  |  |  |  | 23 | +2/-3 |
| DC_66A_n2A |  |  |  |  | 23 | +2/-3 |
| DC_66A_n5A |  |  |  |  | 23 | +2/-3 |
| DC_66A_n7A |  |  |  |  | 23 | +2/-3 |
| DC_66A_n12A |  |  |  |  | 23 | +2/-3 |
| DC_66A_n25A |  |  |  |  | 23 | +2/-3 |
| DC_66A_n28A |  |  |  |  | 23 | +2/-3 |
| DC_66A_n30A |  |  |  |  | 23 | +2/-3 |
| DC_66A_n38A |  |  |  |  | 23 | +2/-3 |
| DC_66A_n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_66A_n46A |  |  |  |  | 23 | +2/-3 |
| DC_66A_n48A |  |  |  |  | 23 | +2/-3 |
| DC_66A_n71A |  |  |  |  | 23 | +2/-3 |
| DC_66A_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_66A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_66A_n86A_ULSUP-TDM_n78A |  |  |  |  | 23 | +2/-3 |
| DC_68A_n1A |  |  |  |  | 23 | +2/-3 |
| DC_68A_n3A |  |  |  |  | 23 | +2/-3 |
| DC_68A_n7A |  |  |  |  | 23 | +2/-3 |
| DC_68A_n8A |  |  |  |  | 23 | +2/-3 |
| DC_68A_n20A |  |  |  |  | 23 | +2/-3 |
| DC_68A_n38A |  |  |  |  | 23 | +2/-3 |
| DC_68A_n40A |  |  |  |  | 23 | +2/-3 |
| DC_68A_n41A |  |  |  |  | 23 | +2/-3 |
| DC_68A_n77A |  |  |  |  | 23 | +2/-3 |
| DC_68A_n78A |  |  |  |  | 23 | +2/-3 |
| DC_68A_n79A |  |  |  |  | 23 | +2/-3 |
| DC_71A_n2A |  |  |  |  | 23 | +2/-3 |
| DC_71A_n5A |  |  |  |  | 23 | +2/-3 |
| DC_71A_n7A |  |  |  |  | 23 | +2/-3 |
| DC_71A_n12A7 |  |  |  |  | 23 | +2/-3 |
| DC_71A_n25A |  |  |  |  | 23 | +2/-3 |
| DC_71A_n38A |  |  |  |  | 23 | +2/-3 |
| DC_71A_n41A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_71A_n48A |  |  |  |  | 23 | +2/-3 |
| DC_71A_n66A |  |  |  |  | 23 | +2/-3 |
| DC_71A_n77A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_71A_n78A |  |  | 266 | +2/-3 | 23 | +2/-3 |
| DC_106A_n41A |  |  |  |  | 23 | +2/-3 |
| DC_106A_n48A |  |  |  |  | 23 | +2/-3 |
| NOTE 1: An uplink DC configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 in TS 38.101-1 or NOTE 2 in Table 6.2.2-1 in TS 36.101 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power Class 3 is the default power class unless otherwise stated. NOTE 5: The UE is not required to support PC2 within each individual cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 6: The UE supports PC3 within E-UTRA cell group, and supports either PC3 or PC2 within NR cell group. Power class support within each individual cell group is signaled separately by the UE. NOTE 7: Only single switched UL is supported. NOTE 8: Void. NOTE 9: Void. |  |  |  |  |  |  |

Table 6.2B.1.3-1a: Per band power class applicable to REFSENS exceptions (two band UL EN-DC)

| Inter-band UL power class (NOTE 1) | Uplink bands of same power class |  | Uplink bands of different power class |  |
| --- | --- | --- | --- | --- |
|  | E-UTRA band2 | NR band | E-UTRA band2 | NR band |
| Class 3 | Class 3 | Class 3 | Class 3 | Class 5 |
| Class 2 | Class 3 | Class 3 | Class 3 | Class 2 |
|  | Class 2 | Class 2 | Class 2 | Class 3 |
| Class 1.5 | Class 2 | Class 2 |  |  |
| NOTE 1: Indicated by powerClass/powerClass-v1610. NOTE 2: For the FDD E-UTRA band, only Class 3 is applicable. |  |  |  |  |

If a UE supports a different power class than the default UE power class for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

–	if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30% (The exact evaluation period is no less than one radio frame); or

–	if the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is not absent and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower;

–	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4;

–	Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 and larger than or equal to 0.5*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331; or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 30% and larger than or equal to 15% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame):

–	shall apply all requirements for power class 2 and set the configured transmitted power class as specified in sub-clause 6.2B.4.

Else shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

If a UE supports a different power class than the default UE power class for an E-UTRA FDD and NR TDD EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

If UE indicating the two capabilities maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2:

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal tomaxUplinkDutyCycle-FDD-TDD-EN-DC1as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame)

–	shall apply all requirements for the supported power class and set the configured transmitted power class as specified in sub-clause 6.2B.4.

–	else

–	shall apply all requirements for the default power class and set the configured transmitted power as specified sub-clause 6.2B.4;

else

–	shall apply all requirements for the supported power class and set the configured transmitted power as specified sub-clause 6.2B.4;

## << Next changes >>

##### 6.2B.4.1.3	Inter-band EN-DC within FR1

For inter-band dual connectivity with one uplink serving cell or more than one uplink serving cells configured for intra-band UL CA on the E-UTRA CG and one uplink serving cell on the NR CG or more than one uplink serving cells configured for intra-band UL CA, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum transmission power for EN-DC operation, PTotalEN-DC= 10log10(PtotalEN-DC) with PtotalEN-DC as specified in clause 7.6 of TS 38.213 [10]. For EN-DC with more than one uplink serving cells configured for intra-band UL CA on the E-UTRA CG, the PCMAX applies to the entire E-UTRA CG. For EN-DC with more than one uplink serving cells configured for intra-band UL CA on the NR CG, the PCMAX applies to the entire NR CG.

For a UE configured with EN-DC and serving cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the serving cell and E-UTRA Pcell is FDD, the UE is not expected to be configured with more than one serving cells in the uplink.

The configured maximum output power PCMAX_ E-UTRA,c (p) in sub-frame p for the configured E-UTRA uplink carrier(s) shall be set within the bounds:

PCMAX_L_ E-UTRA,c (p) ≤  PCMAX_ E-UTRA,c (p) ≤  PCMAX H _ E-UTRA,c (p)

where PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for a serving cell c as specified in TS 36.101 [4] clause 6.2.5 modified by PLTE as follows:

PCMAX_L_ E-UTRA,c = MIN { PEMAX, EN-DC , (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), MIN(PEMAX,c , PLTE) – tC_ E-UTRA, c,  (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MPRc + A-MPRc + ΔTIB,c  + tC_ E-UTRA, c + TProSe, P-MPRc)}

PCMAX H _ E-UTRA,c = MIN {PEMAX,c,  PEMAX, EN-DC  , (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), PLTE, PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA}

For EN-DC with more than one uplink serving cells configured for intra-band UL CA on the E-UTRA CG, PCMAX_L_ E-UTRA,c and PCMAX H _ E-UTRA,c are the limits for the E-UTRA CG as specified in TS 36.101 [4] clause 6.2.5A modified by PLTE as follows:

PCMAX_L_ E-UTRA,c  = MIN{10 log10 ∑ pEMAX,c  - TC , (PPowerClass,E-UTRA – ΔPPowerClass,E-UTRA) – MAX(MPR + A-MPR + ΔTIB,c + TC + TProSe, P-MPR ), PLTE, PPowerClass,EN-DC }

PCMAX H _ E-UTRA,c  = MIN{10 log10 ∑ pEMAX,c , PPowerClass,E-UTRA, PLTE, PPowerClass,EN-DC}

The configured maximum output power PCMAX,f,c,NR (q) in physical-channel q for the configured NR carrier shall be set within the bounds:

PCMAX_L,f,c,NR (q) ≤  PCMAX,f,c,NR (q) ≤  PCMAX_H,f,c,NR (q)

where PCMAX_L,f,c,NR and PCMAX_H,f,c,NR are the limits for a serving cell c as specified in clause 6.2.4 of TS 38.101-1 [2] modified as follows:

PCMAX_L,f,c,NR = MIN { PEMAX, EN-DC  , (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), MIN(PEMAX,c , PNR ) - TC_NR, c,  (PPowerClass,NR – ΔPPowerClass,NR) – MAX(MAX(MPRc, A-MPRc)+ ΔTIB,c + TC_NR, c + ∆TRxSRS,  P-MPRc) }

PCMAX_H,f,c,NR = MIN {PEMAX,c, PEMAX, EN-DC  , (PPowerClass, EN-DC – ΔPPowerClass,EN-DC ), PNR , PPowerClass,NR – ΔPPowerClass,NR }

For EN-DC with more than one uplink serving cells configured for intra-band UL CA on the NR CG, PCMAX_L,f,c, NR and PCMAX_H,f,c, NR are the limits for the NR CG as specified in [2] subclause 6.2A.4 modified by PNR as follows:

PCMAX_L,f,c,NR  = MIN{10 log10 ∑ pEMAX,c  - TC , PEMAX,CA, PPowerClass,NR – MAX(MPR + A-MPR + ΔTIB,c + T_NR ,C + TRxSRS, P-MPR ), PNR, PPowerClass,EN-DC }

PCMAX_H,f,c,NR  = MIN{10 log10 ∑ pEMAX,c , PEMAX,CA, PPowerClass,NR, PNR, PPowerClass,EN-DC}

where

-	PEMAX,EN-DC is the value given by the field p-maxUE-FR1 of the RRCConnectionReconfiguration-v1530 IE as defined in TS 36.331 [8];

-	If more than one E-UTRA uplink serving cell is configured as intra-band UL CA in the E-UTRA CG, PPowerClass refers to the maximum output power of the E-UTRA intra-band CA power class given in Table 6.2.2A-1 of TS 36.101 [4],

-	If more than one NR uplink serving cell is configured as intra-band UL CA in the NR CG, PPowerClass refers to the maximum output power of the NR intra-band CA power class given in sub clause 6.2A.1 of [2],

-	PLTE is the value given by the field p-maxEUTRA-r15 of the RRCConnectionReconfiguration-v1510 IE as defined in TS 36.331 [8];

-	If more than one E-UTRA uplink serving cell is configured as intra-band UL CA in the E-UTRA CG, MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR specified in clause 6.2.3A and clause 6.2.4A of TS 36.101 [4] respectively. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR. PCMAX_ E-UTRA,c is calculated under the assumption that the transmit power is increased by the same amount in dB on all component carriers within the E-UTRA CG.

-	If more than one NR uplink serving cell is configured as intra-band UL CA in the NR CG, MPRc and A-MPRc are determined by subclause 6.2.2 of [2]. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR.

-	PNR is the value given by the field p-NR-FR1 of the PhysicalCellGroupConfig IE as defined in TS 38.331 [9];

-	Δtc_E-UTRA, c = 1.5 dB when NOTE 2 in Table 6.2.2-1 in TS 36.101 [4] applies for a serving cell c, otherwise TC_ E-UTRA,c = 0 dB;

-	TC_NR,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in TS 38.101-1 [2] applies for a serving cell c, otherwise TC_NR,c = 0 dB;TC_NR,C is the highest value TC_NR,C among all serving cells c if more than one NR uplink serving cell is configured as intra-band UL CA in the NR CG;

-	PPowerClass, EN-DC is the nominal UE power class indicated by PowerClass defined in clause 6.2B.1.3 for inter-band EN-DC; if the UE indicates higherPowerLimitMRDC-r17 for an UL inter-band EN-DC configuration with uplink bands of different power class capabilities specified in Table 6.2B.1.3-1 and ΔPPowerClass,EN-DC = 0, PPowerClass,EN-DC is replaced by the sum of the linear powers of PPowerClass,NR and PPowerClass,E-UTRA converted to dB;

-	∆PPowerClass,EN-DC = 3 dB for a power class 2 capable EN-DC UE when requirements of default power class had been applied as specified in sub-clause 6.2B.1; otherwise ∆PPowerClass,EN-DC = 0 dB;

-	NOTE: UE reports ∆PPowerClass,EN-DC when deltaPowerClassReporting-r18 is present, dpc-Reporting-FR1 [7] is configured and the reporting is triggered only by uplink duty cycle exceedance or by return to the powerClass after the duty cycle exceedance.

-	PPowerClass,NR is the nominal UE power of the power class indicated by ue-PowerClass that the UE supports for the NR band and for NR intra-band UL CA of the EN-DC combination as defined in clause 6.2.1 or 6.2A1.1 of 38.101-1 [2]; in case IE powerClassNRPart-r16 as defined in TS 38.331 [9] is indicated, PPowerClass,NR should use that value instead;

-	ΔPPowerClass,NR is 3 dB or 0 dB according to clause 6.2.4 of TS 38.101-1 [2] for a UE that supports power class 2 in the NR band of the EN-DC combination as defined in clause 6.2.1 of TS 38.101-1 [2];

-	PPowerClass,E-UTRA is the nominal UE power of the power class indicated by ue-PowerClass-N-r13 that the UE supports for the E-UTRA band or indicated by ue-CA-PowerClass-N E-UTRA intra-band UL CA of the EN-DC combination as defined in clause 6.2.2 or 6.2.2A of 36.101 [4];

-	ΔPPowerClass,E-UTRA is 3 dB or 0 dB according to clause 6.2.5 of TS 36.101 [4] for a UE that supports power class 2 in the E-UTRA band of the EN-DC combination as defined in clause 6.2.2 of TS 36.101 [4];

-	ΔTIB,c specified in clause 6.2B.4.2.3 for EN-DC, the individual Power Class defined in table 6.2B.1.3 and any other additional power reductions parameters specified in clauses 6.2B.2 and 6.2B.3for EN-DC are applicable to PCMAX_ E-UTRA,c and PCMAX,f,c,NR evaluations.

-	∆TRxSRS is the highest value among all serving cells c.

If the transmissions from NR and E-UTRA do not overlap, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications apply with the modifications specified above. The lower value between PPowerClass, EN-DC or PEMAX, EN-DC shall not be exceeded at any time by UE.

PTotalEN-DC = 10log10(PtotalEN-DC) with PTotalEN-DC the configured maximum transmission power for EN-DC operation as specified in clause 7.6 of TS 38.213 [10].

The total configured maximum transmission power for both synchronous and non-synchronous operation is

PTotalEN-DC= MIN { PEMAX, EN-DC ,PPowerClass, EN-DC – ΔPPowerClass, EN-DC }

If the UE does not support dynamic power sharing,

PTotalEN-DC= MIN { PEMAX, EN-DC ,PPowerClass, EN-DC  – ΔPPowerClass, EN-DC } + 0.3 dB

If the EN-DC UE does not support dynamic power sharing, then the complete clauses for configured transmitted power for E-UTRA and NR respectively from their own specifications TS 36.101 [4] and TS 38.101-1 [2] respectively apply with the modifications specified above and PTotalEN-DC applies.

When a UE supporting dynamic sharing is configured for overlapping E-UTRA uplink and NR uplink transmissions, the UE can set its configured maximum output power PCMAX_ E-UTRA,c and PCMAX,f,c,NR for the configured E-UTRA and NR uplink carriers, respectively, and its configured maximum transmission power for EN-DC operation, PTotalEN-DC, as specified above.

The measured total maximum output power PUMAX over both CGs/RATs, measured over the transmission reference time duration is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,c,NR],

where pUMAX,c,E-UTRA and pUMAX,c,NR denotes the measured output power of serving cell c for E-UTRA and NR respectively, expressed in linear scale.

The measured total configured maximum output power PUMAX shall be within the following bounds:

PCMAX_L -TLOW (PCMAX_L)  ≤  PUMAX  ≤  PCMAX_H + THIGH (PCMAX_H)

with the tolerances TLOW(PCMAX_H) and THIGH(PCMAX_H) for applicable values of PCMAX specified in Table 6.2B.4.1.3-2.

When an UL subframe transmission p from E-UTRA overlap with a physical-channel q from the NR, then for PUMAX evaluation, the E-UTRA subframe p is taken as reference period TREF and always considered as the reference measurement duration and the following rules are applicable.

TREF and Teval are specified in Table 6.2B.4.1.3-1 when same or different subframe and physical-channel durations are used in aggregated carriers. The lesser of PPowerClass ,EN-DC and PEMAX,EN-DC shall not be exceeded by the UE during any evaluation period of time where PPowerClass ,EN-DC is replaced by the sum of the linear powers of PPowerClass,NR and PPowerClass,E-UTRA converted to dB if the UE indicates higherPowerLimitMRDC-r17.

Table 6.2B.4.1.3-1: PCMAX evaluation window

| transmission duration | TREF | Teval |
| --- | --- | --- |
| Different transmission duration in different RAT carriers | E-UTRA Subframe on all aggregated cells of E-UTRA | Min(Tno_hopping, Physical Channel Length) on all aggregated cells of NR |

For each TREF, the PCMAX_H is evaluated per Teval and given by the maximum value over the transmission(s) within the Teval as follows:

PCMAX_H  = MAX { PCMAX_ EN-DC _H (p,q) , PCMAX_ EN-DC _H (p,q+1), … , PCMAX_ EN-DC _H (p,q+n) }

where PCMAX_ EN-DC _H are the applicable upper limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with E-UTRA subframe p.

While PCMAX_L is computed as follows:

PCMAX_L = MIN { PCMAX_ EN-DC _L (p,q) , PCMAX_ EN-DC _L (p,q+1), … , PCMAX_ EN-DC _L (p,q+n)}

where PCMAX_EN-DC_L are the applicable lower limits for each overlapping scheduling unit pairs (p,q) , (p, q+1) , up to (p, q+n) for each applicable Teval duration, where q+n is the last NR UL physical-channel overlapping with E-UTRA subframe p,

With

PCMAX_ EN-DC _H(p,q) = MIN {10 log10 [pCMAX H _ E-UTRA,c (p) + pCMAX H,f,c,NR (q)], PEMAX, EN-DC ,PPowerClass, EN-DC}

And:

a= 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) ] > PTotalEN-DC

b= 10 log10 [pCMAX_ E-UTRA,c (p) +pCMAX,f,c,NR (q) /X_scale] > PTotalEN-DC

If a= FALSE

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,NR (q)], PEMAX, EN-DC ,PPowerClass, EN-DC}

ELSE If (a=TRUE) AND (b=FALSE)

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) + pCMAX L,f,c,NR (q) /X_scale ], PEMAX, EN-DC ,PPowerClass, EN-DC}

ELSE If b= TRUE

PCMAX_ EN-DC _L(p,q) = MIN {10 log10 [pCMAX L _ E-UTRA,c (p) ], PEMAX, EN-DC ,PPowerClass, EN-DC}

where

-	pCMAX H _ E-UTRA,c (p) is the E-UTRA higher limit of the maximum configured power expressed in linear scale;

-	pCMAX L,f,c,NR (q) is the NR higher limit of the maximum configured power expressed in linear scale;

-	pCMAX L _ E-UTRA,c (p) is the E-UTRA lower limit of the maximum configured power expressed in linear scale;

-	pCMAX L,f,c,NR (q) is the NR lower limit of the maximum configured power expressed in linear scale;

-	PPowerClass, EN-DC is defined in clause 6.2B.1.3-1 for inter-band EN-DC; if the UE indicates higherPowerLimitMRDC-r17, PPowerClass,EN-DC is replaced by the sum of the linear powers of PPowerClass,NR and PPowerClass,E-UTRA converted to dB;

-	X_scale is the linear value of X dB which is configured by RRC and can only take values [0 , 6]

-	pCMAX_ E-UTRA,c (p) is the linear value of PCMAX_ E-UTRA,c (p), the configured max power for E-UTRA. If more than one E-UTRA uplink serving cell is configured as intra-band UL CA in the E-UTRA CG, PCMAX_ E-UTRA,c (p) will be replaced by PCMAX(p) which is the configured maximum power for the entire E-UTRA CG.

-	pCMAX,f,c,NR (q) is the linear value of PCMAX,f,c,NR (q), the configured max power of NR, If more than one NR uplink serving cell is configured as intra-band UL CA in the NR CG,  PCMAX_ NR,c (q) will be replaced by PCMAX(q) which is the configured maximum power for the entire NR CG.

Table 6.2B.4.1.3-2: PCMAX tolerance for Dual Connectivity E-UTRA-NR

| PCMAX(dBm) | Tolerance TLOW (PCMAX_L) (dB) | Tolerance THIGH (PCMAX_H) (dB) |
| --- | --- | --- |
| 23 ≤ PCMAX ≤ 33 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX< 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |
| NOTE 1: For UEs not indicating support of dynamic power sharing, the upper tolerance Thigh shall be reduced by 0.3 dB for P ≥ 20 dBm. |  |  |

When E-UTRA and NR transmissions overlap and the condition (If (a=TRUE) AND (b=FALSE)) is met, SCG shall be transmitted and the following supplementary minimum requirement apply for the measured SCG power, PUMAX,f,c,NR (q), under nominal conditions.

10log(pCMAX L,f,c,NR (q)/X_scale)  –  TLOW (10log(pCMAX L,f,c,NR (q)/X_scale) )}  ≤  PUMAX,f,c,NR (q) ≤  10log(pCMAX H, f,c,NR (q)) + THIGH (10log(pCMAX H, f,c,NR (q))).

with the tolerances TLOW and THIGH for applicable values of PCMAX specified in Table 6.2B.4.1.3-2.

## << Next changes >>

#### 6.2H.1.3	Inter-band EN-DC with UL MIMO within FR1

For inter-band EN-DC of E-UTRA and NR in FR1, the following UE Power Classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

The maximum output power for inter-band EN-DC with one Tx in E-UTRA band and 2Tx in NR band is specified in Table 6.2H.1.3-1. These configurations are subject to the applicable power class of each E-UTRA band and NR band specified in Table 6.2.2-1 of TS 36.101 and Table 6.2D.1-1 of TS 38.101-1 respectively.The power classes referenced are according to the reported ue-PowerClass-N-r13 for the E-UTRA band or ue-CA-PowerClass-N for the E-UTRA intra-band UL CA of the EN-DC combination, and reported powerClassNRPart-r16 for the NR band and for NR intra-band UL CA of the EN-DC combination if indicated or ue-PowerClass otherwise.

If higherPowerLimitMRDC-r17 is indicated for an UL inter-band EN-DC configuration as specified in Table 6.2H.1.3-1  and with uplink bands of different power class capabilities, the UE maximum output power specified in Table 6.2H.1.3-1 for this UL EN-DC configuration is modified in accordance with sub-clause 6.2H.4.1.3.

Table 6.2H.1.3-1: Maximum output power for inter-band EN-DC with UL MIMO and/or TxD (two bands)

| EN-DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_1A_n78A |  |  | 265 | +2/-3 | 23 | +2/-3 |
| DC_3A_n78A |  |  | 265 | +2/-3 | 23 | +2/-3 |
| DC_5A_n78A |  |  | 265 | +2/-3 | 23 | +2/-3 |
| DC_7A_n78A |  |  | 265 | +2/-3 | 23 | +2/-3 |
| DC_8A_n78A |  |  | 265 | +2/-3 | 23 | +2/-3 |
| DC_20A_n78A |  |  | 265 | +2/-3 | 23 | +2/-3 |
| DC_28A_n78A |  |  | 265 | +2/-3 | 23 | +2/-3 |
| DC_40A_n78A |  |  | 265 | +2/-3 | 23 | +2/-3 |
| DC_41A_n78A |  |  | 265 | +2/-3 | 23 | +2/-3 |
| NOTE 1: An uplink DC configuration in which at least one of the bands has NOTE 3 in Table 6.2.1-1 in TS 38.101-1 or NOTE 2 in Table 6.2.2-1 in TS 36.101 is allowed to reduce the lower tolerance limit by 1.5 dB when the transmission bandwidths of at least one of the bands is confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high. NOTE 2: PPowerClass, EN-DC is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band EN-DC the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 4: Power Class 3 is the default power class unless otherwise stated. NOTE 5: The UE supports PC3 in E-UTRA band, and supports PC3 or PC2 with UL MIMO and/or TxD in NR band NOTE 6: Void NOTE 7: Void |  |  |  |  |  |  |

Table 6.2H.1.3-1a: Per band power class applicable to REFSENS exceptions for inter-band UL EN-DC with 1Tx in E-UTRA band and 2Tx UL MIMO and/or TxD in NR band.

| Inter-band Power class (NOTE 1) | Uplink bands of same power class |  | Uplink bands of different power class |  |
| --- | --- | --- | --- | --- |
|  | E-UTRA band2 | NR band | E-UTRA band2 | NR band |
| Class 2 | Class 3 | Class 3 | Class 3 | Class 2 |
|  | Class 2 | Class 2 | Class 2 | Class 3 |
| Class 1.5 | Class 2 | Class 2 | Class 3 | Class 1.5 |
|  |  |  | Class 2 | Class 1.5 |
| NOTE 1: Indicated by powerClass/powerClass-v1610. NOTE 2: For the FDD E-UTRA band, only Class 3 is applicable. |  |  |  |  |

If a UE supports power class 2 for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30% when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is present (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower;

–	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified sub-clause 6.2H.4;

–	Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331 when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is present; or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 30% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame):

–	shall apply all requirements for the power class 2 and set the configured transmitted power class as specified in sub-clause 6.2H.4.

If a UE supports power class 1.5 for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 15% when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is present (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower;

–	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified sub-clause 6.2H.4;

–	Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 0.25*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 but less than or equal to 0.5*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331 when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is present; or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 7.5% but less than or equal to 15% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power between 23dBm and 26dBm;

–	shall apply all requirements for the power class 2 and set the configured transmitted power class as specified in sub-clause 6.2H.4.

–	Else

–	shall apply all requirements for the power class 1.5 and set the configured transmitted power class as specified in sub-clause 6.2H.4.

If a UE supports power class 2 for an E-UTRA FDD and NR TDD EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

If UE indicating the two capabilities maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2:

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal tomaxUplinkDutyCycle-FDD-TDD-EN-DC1as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame)

–	shall apply all requirements for the power class 2 and set the configured transmitted power class as specified in sub-clause 6.2H.4.

–	else

–	shall apply all requirements for the default power class and set the configured transmitted power as specified sub-clause 6.2H.4;

else

–	shall apply all requirements for the power class 2 and set the configured transmitted power as specified sub-clause 6.2H.4;

If a UE supports power class 1.5 for an E-UTRA FDD and NR TDD EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

If UE indicating the two capabilities maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2:

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC1as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame)

–	shall apply all requirements for the power class 1.5 and set the configured transmitted power class as specified in sub-clause 6.2H.4.

–	else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC1 but less than or equal to 1.0*maxUplinkDutyCycle-FDD-TDD-EN-DC1 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC2 but less than or equal to 1.0*maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame)

–	shall apply all requirements for the power class 2 and set the configured transmitted power class as specified in sub-clause 6.2H.4.

–	else

–	shall apply all requirements for the default power class and set the configured transmitted power as specified sub-clause 6.2H.4;

else

–	shall apply all requirements for the power class 1.5 and set the configured transmitted power as specified sub-clause 6.2H.4;

## << Next changes >>

##### 6.2H.4.1.3	Inter-band EN-DC with UL MIMO within FR1

For inter-band EN-DC with UL MIMO in one NR band, the requirements in clause 6.2B.4.1.3 apply except that:

-	PPowerClass,EN-DC is the maximum UE power specified in Table 6.2H.1.3-1 without taking into account the tolerance; if the UE indicates higherPowerLimitMRDC-r17 for an UL inter-band EN-DC configuration with uplink bands of different power class capabilities specified in Table 6.2H.1.3-1 and ΔPPowerClass,EN-DC = 0, PPowerClass,EN-DC is replaced by the sum of the linear powers of PPowerClass,NR and PPowerClass,E-UTRA converted to dB;

-	If the NR component carrier is configured with UL MIMO, the MPRc and A-MPRc are specified in clause 6.2D.2 and clause 6.2D.3 of [2] respectively.

-	∆PPowerClass,EN-DC:

–	For a power class 2 capable UE, it is 3dB when the requirements of default power class are applied as specified in sub-clause 6.2H.1.3, otherwise ΔPPowerClass,EN-DC = 0 dB;

## << Next changes >>

#### 6.2L.1.3	Inter-band EN-DC with Tx Diversity within FR1

For inter-band EN-DC of E-UTRA and NR in FR1, the UE Power Classes in table 6.2H.1.3 define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms). UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector.

If higherPowerLimitMRDC-r17 is indicated for an UL inter-band EN-DC configuration with Tx Diversity as specified in Table 6.2H.1.3-1 and with uplink bands of different power class capabilities, the UE maximum output power specified in Table 6.2H.1.3-1 for EN-DC configuration is increased in accordance with sub-clause 6.2L.4.1.3.

If a UE supports power class 2 for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 30% when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is present (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower;

–	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified sub-clause 6.2L.4;

–	Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331 when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is present; or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 30% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame):

–	shall apply all requirements for the power class 2 and set the configured transmitted power class as specified in sub-clause 6.2L.4.

If a UE supports power class 1.5 for an E-UTRA TDD and NR TDD Inter-band EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 15% when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS38.331 when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is present (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power of the default power class or lower;

–	shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified sub-clause 6.2L.4;

–	Else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 0.25*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 but less than or equal to 0.5*maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 as defined in TS 38.331 when the field of UE capability maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is present; or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 7.5% but less than or equal to 15% when maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 is absent. (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is provided and set to the maximum output power between 23dBm and 26dBm;

–	shall apply all requirements for the power class 2 and set the configured transmitted power class as specified in sub-clause 6.2L.4.

–	Else

–	shall apply all requirements for the power class 1.5 and set the configured transmitted power class as specified in sub-clause 6.2L.4.

If a UE supports power class 2 for an E-UTRA FDD and NR TDD EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

If UE indicating the two capabilities maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2:

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal tomaxUplinkDutyCycle-FDD-TDD-EN-DC1as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame)

–	shall apply all requirements for the power class 2 and set the configured transmitted power class as specified in sub-clause 6.2L.4.

–	else

–	shall apply all requirements for the default power class and set the configured transmitted power as specified sub-clause 6.2L.4;

else

–	shall apply all requirements for the power class2  and set the configured transmitted power as specified sub-clause 6.2L.4;

If a UE supports power class 1.5 for an E-UTRA FDD and NR TDD EN-DC band combination and the supported power class enables higher maximum output power than that of the default power class:

If UE indicating the two capabilities maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2:

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC1as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is less than or equal to 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame)

–	shall apply all requirements for the power class 1.5 and set the configured transmitted power class as specified in sub-clause 6.2L.4.

–	else if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is between 40% and 70%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC1 but less than or equal to 1.0*maxUplinkDutyCycle-FDD-TDD-EN-DC1 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame); or

–	if the IE p-maxUE-FR1 as defined in TS 38.331 is not provided or set to the higher value than the maximum output power of the default power class, and the percentage of EUTRA uplink symbols transmitted in a certain evaluation period is no larger than 40%, and the percentage of NR uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-FDD-TDD-EN-DC2 but less than or equal to 1.0*maxUplinkDutyCycle-FDD-TDD-EN-DC2 as defined in TS 38.331 (The exact evaluation period is no less than one radio frame)

–	shall apply all requirements for the power class 2 and set the configured transmitted power class as specified in sub-clause 6.2L.4.

–	else

–	shall apply all requirements for the default power class and set the configured transmitted power as specified sub-clause 6.2L.4;

else

–	shall apply all requirements for the power class 1.5 and set the configured transmitted power as specified sub-clause 6.2L.4;

## << Next changes >>

##### 6.2L.4.1.3	Inter-band EN-DC with Tx Diversity within FR1

For inter-band EN-DC with Tx Diversity in one NR band, the requirements in clause 6.2B.4.1.3 apply except that:

-	PPowerClass,EN-DC is the maximum UE power specified in Table 6.2H.1.3-1 without taking into account the tolerance; if the UE indicates higherPowerLimitMRDC-r17 for an UL inter-band EN-DC configuration with uplink bands of different power class capabilities specified in Table 6.2H.1.3-1 and ΔPPowerClass,EN-DC = 0, PPowerClass,EN-DC is replaced by the sum of the linear powers of PPowerClass,NR and PPowerClass,E-UTRA converted to dB;

-	If the NR component carrier is configured with Tx Diversity, the MPRc and A-MPRc are specified in clause 6.2G.2 and clause 6.2G.3 of [2] respectively.

-	∆PPowerClass,EN-DC:

–	For a power class 2 capable UE, it is 3dB when the requirements of default power class are applied as specified in sub-clause 6.2L.1.3, otherwise ΔPPowerClass,EN-DC = 0 dB;

## << End of changes >>