3GPP TSG-RAN WG4 Meeting #116	R4-2511792

# Bangalore, India, 25th – 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | draft | rev | 1 | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.101-5: Refsens for NR NTN HPUE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | PC2 and PC1 requirements are defined for NR NTN. This CR introduces the agreed reference sensitivity requirements to TS 38.101-5 |  |  |  |  |  |  |  |  |
| Summary of change: |  | RSD tables and description how RSD impacts reference sensitivity are added for PC2 and PC1 UEs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC2 and PC1 requirements are incomplete and the feature cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

### 7.3.2	Reference sensitivity power level

The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annex A3.2.2 of 3GPP TS 38.101-1 [5], with parameters specified in Table 7.3.2-1.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n256 | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n252 | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |

For power class 2 and power class 1 UEs, certain degradation of the reference sensitivity in Table 7.3.2-1 is allowed. For a power class 2 UE the maximum amount of degradation is specified in Table 7.3.2-1a, and in Table 7.3.2-1b for a UE that indicates txDiversity-r16 or txDiversity2Tx-r18. For a power class 1 UE the maximum amount of degradation is specified in Table 7.3.2-1c, and in Table 7.3.2-1d for a UE that indicates txDiversity-4Tx-r18.

Table 7.3.2-1a Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE with 1Tx

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| n255 | 0.8 | 0.8 | 0.9 | 0.9 |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-1b Reference Sensitivity Degradation from PC3 to PC2 for
FDD bands for UE with 2Tx

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0 | 0 |  |  |  |  |  |  |
| n255 | 1.6 | 1.6 | 1.7 | 1.7 |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-1c Reference Sensitivity Degradation from PC3 to PC1 for
FDD bands for UE with 1Tx

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n256 | 1.5 | 1.5 | 1.5 | 1.5 |  |  |  |  |  |  |
| n255 | 2 | 2 | 2 | 2 |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-1d Reference Sensitivity Degradation from PC3 to PC1 for
FDD bands for UE with 4Tx

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n256 | 2.1 | 2.1 | 2.1 | 2.1 |  |  |  |  |  |  |
| n255 | 3.3 | 3.3 | 3.3 | 3.3 |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |

The reference receiver sensitivity (REFSENS) requirement specified in Table 7.3.2-1 shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-2 and with default Tx-Rx carrier center frequency separation except for cases specified in Table 7.3.2-3.

Table 7.3.2-2: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | Duplex Mode |
|  | 15 | 25 | 50 | 75 | 100 |  |
| n256 | 30 |  | 24 | 36 | 50 | FDD |
|  | 60 |  | 10 | 18 | 24 |  |
|  | 15 | 25 | 50 | 75 | 752 503 |  |
| n255 | 30 |  | 24 | 36 | 362 243 | FDD |
|  | 60 |  | 10 | 18 | 182 103 |  |
|  | 15 | 25 | 50 | 75 |  |  |
| n254 | 30 |  | 24 | 36 |  | FDD |
|  | 60 |  | 10 | 18 |  |  |
|  | 15 | 25 | 50 | 75 | 100 |  |
| n252 | 30 |  | 24 | 36 | 50 | FDD |
|  | 60 |  | 10 | 18 | 24 |  |
| NOTE 1: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth in Table 5.3.2-1. NOTE 2: Applicable for Tx-Rx frequency separation of -101.5 MHz and -115.5 MHz. NOTE 3: Applicable for Tx-Rx frequency separation of -87.5 MHz. |  |  |  |  |  |  |

Table 7.3.2-3: TX – RX carrier centre frequency separation for REFSENS verification

| Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| n256 | 5 MHz | 165 MHz, 215 MHz |
|  | 20 MHz | 180 MHz, 200 MHz |
| n255 | 5 MHz | -72.5 MHz, -130.5 MHz |
|  | 20 MHz | -87.5 MHz, -115.5 MHz |
| n252 | 5 MHz | 165 MHz, 195 MHz |
|  | 15 MHz | 175 MHz, 185 MHz |

The minimum requirements specified in Table 7.3.2-1 shall be verified with the network signalling value NS_01 configured in Table 6.2.3.1-1.

## 7.4	Maximum input level

<End of Change 1>