3GPP TSG-RAN4 Meeting #116	R4-2511808

Bengaluru,  25th – 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2869 | rev | 1 | Current version: | 16.23.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.101-1 - Correction to A-MPR for CA_NS_04 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1_enh-Core |  |  |  |  |  | Date: |  |  | 2025-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Correct A-MPR algorithm for non-contiguous RB allocations |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify A-MPR algoritm on how A-MPR is assigned based on location of IM3 terms. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A-MPR algorithm would not be correct |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3.1.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

6.2A.3.1.1.1	A-MPR for CA_NS_04

<Start of change>

6.2A.3.1.1.1.2	Non-contiguous allocations

For intra-band contiguous CA_n41B and CA_n41C and it receives IE CA_ NS_04, the UE determines the allowed Additional Maximum Power Reduction (AMPR) for the maximum output power as specified in this clause. The AMPR is specified by AMPRIM3 to meet -25dBm/MHz when IM3 falls in -25dBm/MHz region of Table 6.5A.2.3.1.1-1 or Table 6.5A.3.3.1.1-1. And uses MPR for all other cases.

The UE determines the A-MPR for all waveforms, modulations, and SCSs as follows:

If FIM3,low_block,low 2490.5 MHz  AND  FIM3,low_block,high > Ffilter,low,

- A-MPR = A-MPRIM3 defined in Clause 6.2A.3.1.1.1.3.

else,

- if RB allocation is an inner or outer 1 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR.

if RB allocation is an outer 2 allocation as defined in clause 6.2A.2.1 then A-MPR = MPR + 1 dB.

where

- MPR is the MPR as defined in Table 6.2A.2.1-2, Table 6.2A.2.1-3 and Table 6.2A.2.1-4 for PC3 and PC2 respectively and the respective CA bandwidth class

-     FIM3,low_block,low = (2 * Flow_alloc,low_edge ) – Fhigh_alloc,high_edge

-	FIM3,low_block,high = (2 * Flow_alloc,high_edge ) – Fhigh_alloc,low_edge

-

-	Flow_alloc,low_edge is the lowermost frequency of lower transmission bandwidth allocation.

-	Flow_alloc,high_edge is the uppermost frequency of lower transmission bandwidth allocation.

-	Fhigh_alloc,low_edge is the lowermost frequency of upper transmission bandwidth allocation.

-	Fhigh_alloc,high_edge is the uppermost frequency of upper transmission bandwidth allocation.

-	Ffilter,low = 2480 MHz

-

-

<End of change>