3GPP TSG-RAN WG4 Meeting #116	R4-2511816

# Bangalore, India,  25th - 29th August 2025								(revision of R4-2509528)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.764 | CR | 0008 | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (IoT_NTN_FDD_LS_band-Core) Additional power back-off results for the IOT NTN band 254 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_FDD_LS_band-Core |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TR 36.764 captures a lot of aspects of the NTN IOT bands including power back-off simulations from different proponents. This CR adds additional power back-off simulations for the NTN IOT band 254. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In sub-clause 6.2.3.2, additional power back-off simulations are added to the existing results in the TR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some power back-off simulation results will not be captured in the TR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 6.2.3.2	Evaluation and simulation on Cat. NB1/NB2

For A-MPR for category NB1/NB2, simulations where a QPSK signal with different allocations including 3 tones, 6 tones and 12 tones is generated and passed to a typical PC3 power amplifier. And LO is set to the center of the carrier. The simulation results are shown in the following figures from R4-2317572 and R4-2318708.

a.	3-tone [0-2] allocation, FCC  No A-MPR is required

b.	3-tone [0-2] allocation, ETSI, Fc = 1610 – 1618.25 MHz  A-MPR = 0.7dB

c.	3-tone [0-2] allocation, ETSI, Fc = 1618.25 – 1626.5 MHz  A-MPR = 1.5 dB

Fig. 6.2.3.2-1, MPR simulation results with 3-tone allocation for FCC and ETSI requirements.

d.	6-tone [0-6] allocation, FCC  No A-MPR is required

e.	6-tone [0-5] allocation, ETSI, Fc = 1610 – 1618.25 MHz  A-MPR = 0

f.	6-tone [0-5] allocation, ETSI, Fc = 1618.25 – 1626.5 MHz  A-MPR = 0.7dB

Fig. 6.2.3.2-2, MPR simulation results with 6-tone allocation for FCC and ETSI requirements.

g.	FCC

h.	ETSI, Fc = 1618.25 MHz – 1626.5 MHz

i.	ETSI, Fc = 1610 MHz – 1618.25 MHz

Fig. 6.2.3.2-3, MPR simulation results with 12-tone allocation for FCC and ETSI requirements.

The additional MPR is not required for FCC regulations.

The additional MPR are summarized in the tables below for ETSI regulations.

Table 6.2.3.2-1: A-MPR for NS_04N power class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | ≤ 0.7 dB | 0.2 dB |  | ≤ 0.7 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | 0 dB |  | 0 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | 0 dB |  |  |  |

Table 6.2.3.2-2: A-MPR for NS_05N power class 3

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| A-MPR | ≤ 1.5 dB | 0.5 dB |  | ≤ 1.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| A-MPR | ≤ 0.7 dB |  | ≤ 0.7 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| A-MPR | 0 dB |  |  |  |

For 1-tone allocation, the below results are based on the measurements from R4-2501167.

Table 6.2.3.2-3: 1-Tone A-MPR for NS_04N power class 3

| Modulation | π/4 QPSK |  |  |
| --- | --- | --- | --- |
| Tone positions for 15kHz 1-Tone allocation | 0/11 | 1/10 | 2/3/4/5/6/7/8/9 |
| A-MPR | ≤ 0.5 dB | 0 dB | 0 dB |

Table 6.2.3.2-4: 1-Tone A-MPR for NS_05N power class 3

| Modulation | π/4 QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 15kHz 1-Tone allocation | 0/11 | 1/10 | 2/9 | 3/4/5/6/7/8 |
| A-MPR | ≤ 1.5 dB | ≤ 1.0 dB | 0 dB | 0 dB |

For 1/3/6/12 tones allocations, the below results are based on the measurements from R4-2501153.

Table6.2.3.2-5: A-MPR for NS_05N power class 3

| Modulation | QPSK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for single tone allocation | 0 | 1 | 2 | 3 |  | 4 | 5 |  | 6 | 7 |  | 8 | 9 | 10 | 11 |  |
| A-MPR | ≤ 11.5 dB |  | TBD |  |  |  |  |  |  |  |  |  |  | ≤ 11.5 dB |  |
| Tone positions for 3 Tones allocation | 0-2 |  |  |  | 3-5 and 6-8 |  |  |  |  |  | 9-11 |  |  |  |  |
| A-MPR | ≤ 9 dB |  |  |  | TBD |  |  |  |  |  | ≤ 9 dB |  |  |  |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR | ≤ 6.5 dB |  |  |  |  |  |  | ≤ 6.5 dB |  |  |  |  |  |  |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR | ≤ 3.5 dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.2-6: A-MPR for NS_04N power class 3

| Modulation | QPSK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for single tone allocation | 0 | 1 | 2 | 3 |  | 4 | 5 |  | 6 | 7 |  | 8 | 9 | 10 | 11 |
| A-MPR | ≤ 9.5 dB |  | TBD |  |  |  |  |  |  |  |  |  |  | ≤ 9.5 dB |  |
| Tone positions for 3 Tones allocation | 0-2 |  |  |  | 3-5 and 6-8 |  |  |  |  |  | 9-11 |  |  |  |  |
| A-MPR | ≤ 7 dB |  |  |  | TBD |  |  |  |  |  | ≤ 7 dB |  |  |  |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR | ≤ 4.5 dB |  |  |  |  |  |  | ≤ 4.5 dB |  |  |  |  |  |  |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A-MPR | ≤ 1.5 dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For 12-tone allocation, the below results are based on the measurements from R4-2321798.

Figure 6.2.3.2-4: Measurement results for the 12-tone allocation

Table 6.2.3.2-7 and 6.2.3.2-8 below provide a summary of required power back-off values for the NS_04N and NS_05N flags, more detailed spectrum plots for which can be found in the corresponding Figure 6.2.3.2-5. It should be noted that no power back-off is needed for NS_03N (FCC regulations).

Table 6.2.3.2-7: Power back-off for NS_04N power class 3

| Modulation | QPSK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for single tone allocation | 0 | 1 | 2 | 3 |  | 4 | 5 |  | 6 | 7 |  | 8 | 9 | 10 | 11 |
| Power back-off | 0.5 dB | 0 dB |  |  |  |  |  |  |  |  |  |  |  |  | 0.5 dB |
| Tone positions for 3 Tones allocation | 0-2 |  |  |  | 3-5 and 6-8 |  |  |  |  |  | 9-11 |  |  |  |  |
| Power back-off | 0.5 dB |  |  |  | 0 dB |  |  |  |  |  | 0.5 dB |  |  |  |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Power back-off | 0 dB |  |  |  |  |  |  | 0 dB |  |  |  |  |  |  |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Power back-off | 0 dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.2-8: Power back-off for NS_05N power class 3

| Modulation | QPSK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tone positions for single tone allocation | 0 | 1 | 2 | 3 |  | 4 | 5 |  | 6 | 7 |  | 8 | 9 | 10 | 11 |
| Power back-off | 1.5 dB | 0 dB |  |  |  |  |  |  |  |  |  |  |  |  | 1.5 dB |
| Tone positions for 3 Tones allocation | 0-2 |  |  |  | 3-5 and 6-8 |  |  |  |  |  | 9-11 |  |  |  |  |
| Power back-off | 1.5 dB |  |  |  | 0 dB |  |  |  |  |  | 1.5 dB |  |  |  |  |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Power back-off | 1 dB |  |  |  |  |  |  | 1 dB |  |  |  |  |  |  |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Power back-off | 0 dB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Figure 6.2.3.2-5: Spectrum plots for different numbers of tones (top to bottom 12- and 1-tones) and different regulations (left to right FCC, ETSI sub-range 1 and 2).