3GPP TSG-RAN WG4 Meeting #116	R4-2511828

Bengaluru, India, 25 – 29 August 2025

Source:	Ericsson, BT plc

Title:	TP for TR 38.750 to add PC2 2BUL for CA_n7-n28

Agenda item:			6.8.4

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for TR 38.750 to add PC2 2BUL for CA_n7-n28 as specified in WID [1].

# 2. Text Proposal

# ---Start of changes---

## 5.X	CA_n7-n28

### 5.X.1	UE maximum output power

Table 5.X.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (two bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n7A-n28A | n78 CA_n7A-n28A8 | n7 | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  |  | n28 | 5, 10, 15, 20 |  |
|  |  | n7 | n7 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n28 | n28 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 8: Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination |  |  |  |  |

Table 5.X.1-2: UE Power Class for uplink inter-band CA (two bands)

| Uplink CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n7A-n28A |  |  | 2610 | +2/-3 | 23 | +2/-3 |  |  |
| NOTE 10: The UE supports PC3 within each NR FDD band |  |  |  |  |  |  |  |  |

### 5.X.2	Reference sensitivity requirements

For PC3 DL CA_n7A-n28A with UL CA_n7A-n28A, there are no intermodulation, harmonics and harmonic mixing issues and no MSD is defined. Therefore, there is no need to define MSD values for CA_n7A-n28A with PC2 UL CA_n7A-n28A.

### 5.X.3	∆TIB and ∆RIB values (Void)

---End of changes---

# Reference

[1] 	RP-251450, Rel-19 High power UE (power class 1.5 or 2) for NR intra-band Carrier Aggregation (CA) or NR inter-band CA/Dual connectivity (DC) band combinations with/without NR Supplementary Uplink (UL); China Telecom