3GPP TSG-RAN WG4 Meeting # 116	R4-2511854

# Bangalore, India,  August 25 - 29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | <CR#> | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR for 38.101-1: Maximum input power for WUR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | LP-WUS is introduced in Rel-19 and WUR receiver requirement to be added, this CR is adding the maximum input power for WUR |  |  |  |  |  |  |  |  |
| Summary of change: |  | adding the maximum input power for WUR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No WUR requirment for Rel-19 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 7.4M |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/* start change Option 1 according to WF */

## 7.4M	Maximum input level for LP-WUR

Maximum input level is defined as the maximum mean power received at the UE antenna connector, at which the MDR criterion shall be met with the side conditions in clause 7.1M and with parameters specified in Table 7.4M-1, and in Tables 7.4M-2a and 7.4M-2b respectively.

The UE supporting LP-WUS shall fulfil the minimum requirements specified in Table 7.4M-1 for type 1 and type 2 LP-WUR.For this requirement, a non-LP-WUS DL RB is defined as any RB in the DL transmission bandwidth configuration of the UE channel that is not occupuied by the LP-WUS carrier as defined in sub-clause 5.3M.

Table 7.4M-1: Maximum input level for Type1 LR and Type2 LR

| Rx Parameter | Units | Maximum input power |
| --- | --- | --- |
| Case 1: Power in LP-WUS | dBm | -25.0 |
| Case 2: Total power including LP-WUS RBs and non-LPWUS RBs | dBm | -25.0 |
| NOTE: Reference measurement channel is specified in A.3M.1 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 with the same PSD as LP-WUS. |  |  |

Table 7.4M-2a: Test parameters for Maximum input level for Type1 and Type 2 LR , case1

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS | dBm | -25.0 |
| ASCS Guard RB1 | PRB | X |
| Power of each non-LP-WUS DL RB | mW | 0 |
| NOTE 1: ASCS Guard RBs are zero-power and exist when NRB,LP-WUS < NRB. X=1 for type 1 LR and X=0 for type 2 LR. The Guard RB is configured for type-1 LR at the both sides of LP-WUS RB as specified in section 7.5M.3. NOTE 2: The interfering NR RBs consists of CP-OFDM QPSK with the same SCS as LP-WUS, in all PRB in the DL transmission BW configuration not occupied by either LP-WUS or Guard RB. |  |  |

Table 7.4M-2b: Test parameters for Maximum input level for Type1 and type 2 LR, case 2

| RX parameter | Units | Value |
| --- | --- | --- |
| Total power including LP-WUS RBs and non-LPWUS RBs | dBm | -25.0 |
| ASCS Guard RB1 | PRB | X |
| LP-WUS power boosting | dB | 0 |
| NOTE 1: ASCS Guard RBs are zero-power and exist when NRB,LP-WUS < NRB. X=1 for type 1 LR and X=0 for type 2 LR. The Guard RB is configured for type-1 LR at the both sides of LP-WUS RB as specified in section 7.5M.3. NOTE 2: The interfering NR RBs consists of CP-OFDM QPSK with the same SCS as LP-WUS, in all PRB in the DL transmission BW configuration not occupied by either LP-WUS or Guard RB. NOTE 3: non-LPWUS DL RBs consists of CP-OFDM QPSK with the same SCS as LP-WUS. |  |  |

/* end change */