3GPP TSG-RAN WG4 Meeting #116	R4-2511855

Bangalore, India, 25th – 29th August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | CRNum | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR to TS 38.101-1 on Adjacent subcarrier selectivity for LP-WUR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, vivo, ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS-Core |  |  |  |  |  | Date: |  |  | 2025-08-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction ASCS requirements for LP-WUR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add ASCS requirements for LP-WUR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | LP-WUR ASCS requirements are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Start of change>>

7.5M.3	Adjacent subcarrier selectivity for LP-WUR

Adjacent subcarrier selectivity (ASCS) is a measure of a receiver’s ability to receive the LP-WUS on the assigned subcarriers in the presence of fully populated NR signal at all adjacent subcarriers. ASCS is the ratio of the receiver filter attenuation on the LP-WUS RBs to the receiver filter attenuation on the adjacent subcarriers.

The UE shall fulfil the minimum requirements specified in Table 7.5M.3-1 and 7.5M.3-2.

Table 7.5M.3-1: ASCS test parameters for type-1 LP-WUR

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS | dBm | Type-1 REFSENSLP-WUS + 0.5 |
| LP-WUS power boosting2 | dB | 0 |
| NOTE 1: LP-WUS is configured in NR carrier with 1 guard RB on each side of LP-WUS signal. NOTE 2: 0dB power boost means equal PSD between NR RBs and LP-WUS RBs. NOTE 3: The NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs and the guard RB between the NR and LP-WUS signals. |  |  |

Table 7.5M.3-2: ASCS test parameters for type-2 LP-WUR

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS | dBm | Type-2 REFSENSLP-WUS |
| LP-WUS power boosting2 | dB | 0 |
| NOTE 1: LP-WUS is configured in NR carrier with 0 guard RB on each side of LP-WUS signal. NOTE 2: 0dB power boost means equal PSD between NR RBs and LP-WUS RBs. NOTE 3: The NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs and the guard RB between the NR and LP-WUS signals. |  |  |

<<End of change>>