3GPP TSG-RAN WG4 Meeting #116	R4-2511862

Bengaluru, India, 25 – 29 August, 2025

Agenda item:			7.3.1

Source:	Apple

Title:	WF on UE RF requirements for LB CA via switching

Document for:	Approval

# Way Forward

Issue 1: Downlink reference measurement channels

Agreement:

-	RAN4 can continue to discuss how to specify the DL RMC for this feature as part of the Performance phase of the work item

-	The following options for the RMC have been proposed:

-	Option 1: 10ms periodicity with switching period placed in slots 4 and 8

Fixed reference channel for receiver requirements configured for low NR band inter-band carrier aggregation via switching [supportedLowBandSwitching-r19] (SCS 15 kHz, FDD, QPSK 1/3) Parameter; Unit; Value. Channel bandwidth; MHz; 3; 5, 10, 15, 20 (Note 5); 10; 15; 20; 25; 30; 40; 50. Subcarrier spacing; kHz; 15; 15; 15; 15; 15; 15; 15; 15; 15. Subcarrier spacing configuration; 0; 0; 0; 0; 0; 0; 0; 0; 0. Allocated resource blocks; 15; 25; 52; 79; 106; 133; 160; 216; 270. Subcarriers per resource block; 12; 12; 12; 12; 12; 12; 12; 12; 12. Allocated slots per Frame; 8; 8; 8; 8; 8; 8; 8; 8; 8. MCS Index; 4; 4; 4; 4; 4; 4; 4; 4; 4. MCS Table for TBS determination; 64QAM. Modulation; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK. Target Coding Rate; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3. Maximum number of HARQ transmissions; 1; 1; 1; 1; 1; 1; 1; 1; 1. Information Bit Payload per Slot. For Slots 0,1,5,6,7,8; Bits; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A. For Slots 2,3,9; Bits; 984; 1672; 3368; 5120; 6912; 8712; 10504; 14088; 17424. For Slot 4; Bits; 504; 1064; 2600; 4064; 5576; 7088; 8648; 11784; 14808. Transport block CRC; Bits; 16; 16; 16; 24; 24; 24; 24; 24; 24. LDPC base graph; 2; 2; 2; 1; 1; 1; 1; 1; 1. Number of Code Blocks per Slot. For Slots 0,1,5,6,7,8; CBs; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A. For Slots 2,3,9; CBs; 1; 1; 1; 1; 1; 2; 2; 2; 3. For Slot 4; CBs; 1; 1; 1; 1; 1; 2; 2; 2; 3. Binary Channel Bits per Slot. For Slots 0,1,5,6,7,8; Bits; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A. For Slots 2,3,9; Bits; 3240; 5400; 11232; 17064; 22896; 28728; 34560; 46656; 58320. For Slot 4; Bits; 2520; 4200; 8736; 13272; 17808; 22344; 26880; 36288; 45360. Max. Throughput averaged over 1 frame; Mbps; 0.3456; 0.6080; 1.2704; 1.9424; 2.6312; 3.3224; 3.8600; 5.4048; 6.7080. NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: Channel bandwidths 10, 15, and 20 MHz in this column only apply to UEs supporting IE supportOfERedCap-r18 but not eRedCapNotReducedBB-BW-r18.. Fixed reference channel for receiver requirements configured for low NR band inter-band carrier aggregation via switching [supportedLowBandSwitching-r19] (SCS 15 kHz, SDL, QPSK 1/3) Parameter; Unit; Value. Channel bandwidth; MHz; 3; 5, 10, 15, 20 (Note 5); 10; 15; 20; 25; 30; 40; 50. Subcarrier spacing; kHz; 15; 15; 15; 15; 15; 15; 15; 15; 15. Subcarrier spacing configuration; 0; 0; 0; 0; 0; 0; 0; 0; 0. Allocated resource blocks; 15; 25; 52; 79; 106; 133; 160; 216; 270. Subcarriers per resource block; 12; 12; 12; 12; 12; 12; 12; 12; 12. Allocated slots per Frame; 8; 8; 8; 8; 8; 8; 8; 8; 8. MCS Index; 4; 4; 4; 4; 4; 4; 4; 4; 4. MCS Table for TBS determination; 64QAM. Modulation; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK; QPSK. Target Coding Rate; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3; 1/3. Maximum number of HARQ transmissions; 1; 1; 1; 1; 1; 1; 1; 1; 1. Information Bit Payload per Slot. For Slots 0,1,2,3,4,9; Bits; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A. For Slots 5,6,7; Bits; 984; 1672; 3368; 5120; 6912; 8712; 10504; 14088; 17424. For Slot 8; 504; 1064; 2600; 4064; 5576; 7088; 8648; 11784; 14808. Transport block CRC; Bits; 16; 16; 16; 24; 24; 24; 24; 24; 24. LDPC base graph; 2; 2; 2; 1; 1; 1; 1; 1; 1. Number of Code Blocks per Slot. For Slots 0,1,2,3,4,9; CBs; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A. For Slots 5,6,7; CBs; 1; 1; 1; 1; 1; 2; 2; 2; 3. For Slot 8; CBs; 1; 1; 1; 1; 1; 2; 2; 2; 3. Binary Channel Bits per Slot. For Slots 0,1,2,3,4,9; Bits; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A. For Slots 5,6,7; Bits; 3240; 5400; 11232; 17064; 22896; 28728; 34560; 46656; 58320. For Slots 8; Bits; 2520; 4200; 8736; 13272; 17808; 22344; 26880; 36288; 45360. Max. Throughput averaged over 1 frame; Mbps; 0.3456; 0.6080; 1.2704; 1.9424; 2.6312; 3.3224; 3.8600; 5.4048; 6.7080. NOTE 1: Additional parameters are specified in Table A.3.1-1 and Table A.3.2.1-1. NOTE 2: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). NOTE 3: SS/PBCH block is transmitted in slot #0 of each frame NOTE 4: Slot i is slot index per frame NOTE 5: Channel bandwidths 10, 15, and 20 MHz in this column only apply to UEs supporting IE supportOfERedCap-r18 but not eRedCapNotReducedBB-BW-r18..

-	Option 2: Increase period of RMC to 40 slots and use the LBCA-SwitchingPattern: ‘00011 00110 01100 11001 11100 11001 10011 00110’

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Switching Pattern for Low-band CA | LBCA-SwitchingPattern | A bitmap of slots | 00011 00110 01100 11001 11100 11001 10011 00110 |
|  | Period of Switching Pattern | Slots | 40 |
|  | Slots on the PCell |  | A bit value of ‘0’ in LBCA-SwitchingPattern |
|  | Slots on the SCell |  | A bit value of ‘1’ in LBCA-SwitchingPattern |
| Switch-from DL slot on the PCell/SCell | Slot Location |  | Mod(i, 20) = 2, 4, 6, 8, 10, 12, 14, 16, 18 |
|  | The number of Switch-from Slot |  | 18 slots for each period of 40 slots. |
|  | CORESET frequency domain allocation |  | Full BW |
|  | CORESET time domain allocation |  | 1 OFDM symbol at the begin of each slot |
|  | PDSCH mapping type |  | Type B |
|  | PDSCH start symbol index (S) |  | 12 - LBCA-SwitchingGap-Duration-PCelltoSCell Or 12 - LBCA-SwitchingGap-SCelltoPCell |
|  | Number of consecutive PDSCH symbols (L) |  | 2 |
|  | PDSCH PRB bundling | PRBs | 1 |
|  | Dynamic PRB bundling |  | false |
|  | Overhead value for TBS determination |  | 0 |
|  | First DMRS position for Type B PDSCH mapping |  | PDSCH start symbol index (S) |
|  | DMRS type |  | Type 1 |
|  | Number of additional DMRS |  | 0 |
|  | FDM between DMRS and PDSCH |  | Disable |
|  | The number of slots between PDSCH and corresponding HARQ-ACK information (K1 Value) | Slots | {1, 2, 5} |
| Switch-to DL slot on the PCell/SCell | Slot Location |  | Mod(i, 20) = 3, 5, 7, 9, 11, 13, 15, 17, 19 |
|  | The number of Switch-from Slot |  | 18 slots for every period of 40 slots. |
|  | CORESET frequency domain allocation |  | Full BW |
|  | CORESET time domain allocation |  | 1 OFDM symbol at the begin of each slot |
|  | PDSCH mapping type |  | Type B |
|  | PDSCH start symbol index (S) |  | 1 |
|  | Number of consecutive PDSCH symbols (L) |  | 2 |
|  | PDSCH PRB bundling | PRBs | 1 |
|  | Dynamic PRB bundling |  | false |
|  | Overhead value for TBS determination |  | 0 |
|  | First DMRS position for Type B PDSCH mapping |  | PDSCH start symbol index (S) |
|  | DMRS type |  | Type 1 |
|  | Number of additional DMRS |  | 0 |
|  | FDM between DMRS and PDSCH |  | Disable |
|  | The number of slots between PDSCH and corresponding HARQ-ACK information (K1 Value) | Slots | {1, 2, 4} |
| HARQ process number |  |  | 4 |
| Subcarrier spacing |  | kHz | 15 |
| Subcarrier spacing configuration µ |  |  | 0 |
| Subcarriers per resource block |  |  | 12 |
| Allocated slots per period of 40 slots on the PCell |  | Slots | 18 |
| Allocated slots per period of 40 slots on the SCell |  | Slots | 18 |
| MCS Index |  |  | 4 |
| MCS Table for TBS determination |  |  | 64QAM / table 5.1.3.1-1: MCS index table 1 for PDSCH in TS 38.214 |
| Modulation |  |  | QPSK |
| Target Coding Rate R x [1024] |  |  | 308 |
| Maximum number of HARQ transmissions |  |  | 1 |
| Channel bandwidth |  |  | These parameters can refer to Table A.8.2-2 for differernt channel bandwidth. As PCell and SCell can be configured with different channel bandwidth, TBS per slot for PCell and SCell could be different. |
| Allocated resource blocks |  |  |  |
| Information Bit Payload (TBS) per Slot |  |  |  |
| Number of Code Blocks per Slot |  |  |  |
| Number of Code Blocks per Slot |  |  |  |
| Max. Throughput averaged over 4 frame |  | Mbps | (18*TBSPCell + 18*TBSSCell) / 0.04 / 10^6 |
| ∆TRTD, P2S |  | µs | 0 |
| ∆TRTD, S2P |  | µs | 0 |
| NTA |  | Tc | 0 |
| NTA,offset |  | Tc | 25600 |
| CSI-RS for tracking | First subcarrier index in the PRB used for CSI-RS (k0) |  | For PCell: 0 for CSI-RS resource 1,2,3,4,5,6,7,8 For SCell: 0 for CSI-RS resource 1,2,3,4,5,6,7,8 |
|  | OFDM symbols in the PRB used for CSI-RS |  | For both PCell and SCell: l0 = 6 for CSI-RS resource 1, 3, 5, 7 l0 = 10 for CSI-RS resource 2, 4, 6, 8 |
|  | Number of CSI-RS ports |  | For both PCell and SCell: 1 for CSI-RS resource 1,2,3,4,5,6,7,8 |
|  | CDM Type |  | For both PCell and SCell: 'No CDM' for CSI-RS resource 1,2,3,4,5,6,7,8 |
|  | Density (ρ) |  | For both PCell and SCell: 3 for CSI-RS resource 1,2,3,4,5,6,7,8 |
|  | CSI-RS periodicity | Slots | 15 kHz SCS: 40 for CSI-RS resource 1,2,3,4,5,6,7,8 |
|  | CSI-RS offset | Slots | 15 kHz SCS: (One swtich-to slot per frame) For PCell: Mod(i, 40) = 5 for CSI-RS resource 1 and 2 Mod(i, 40) = 13 for CSI-RS resource 3 and 4 Mod(i, 40) = 23 for CSI-RS resource 5 and 6 Mod(i, 40) = 35 for CSI-RS resource 7 and 8 For SCell: Mod(i, 40) = 3 for CSI-RS resource 1 and 2 Mod(i, 40) = 15 for CSI-RS resource 3 and 4 Mod(i, 40) = 25 for CSI-RS resource 5 and 6 Mod(i, 40) = 33 for CSI-RS resource 7 and 8 |
|  | Frequency Occupation |  | Start PRB 0 Number of PRB = BWP size |
|  | QCL info |  | TCI state #0 |
| PTRS configuration |  |  | PTRS is not configured |
| NOTE 1: Slot i is slot index per four frames. |  |  |  |

Table A.8.2-2 DL TBS for different channel bandwidth under the inter-band CA via switching

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | MHz | 5 | 10 | 15 | 20 | 25 |
| Allocated resource blocks |  | 25 | 52 | 79 | 106 | 133 |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots i in which PDSCH is not allocated, If Mod(i, 20) = 0 or 1 | Bits | NA | NA | NA | NA | NA |
| For Slots i in which PDSCH is allocated, if Mod(i, 20) = 2,3,4,5,6,7,8,9,10,11,12,13,14,15,16,17,18,19 | Bits | 176 | 368 | 576 | 768 | 984 |
| Transport block CRC |  | 16 | 16 | 16 | 16 | 16 |
| LDPC base graph |  | 2 | 2 | 2 | 2 | 2 |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots i in which PDSCH is not allocated, | CBs | NA | NA | NA | NA | NA |
| For Slots i in which PDSCH is allocated, | CBs | 1 | 1 | 1 | 1 | 1 |
| Binary Channel Bits per Slot |  |  |  |  |  |  |
| For Slots i in which PDSCH is not allocated, | Bits | NA | NA | NA | NA | NA |
| For Slots i in which PDSCH is allocated, | Bits | 600 | 1248 | 1896 | 2544 | 3192 |
| NOTE 1: Additional parameters are specified in Table A.8.2-1. NOTE 2: If TB size is larger than 3824, the CRC sequence of L = 24 Bits is attached to each Code Block. NOTE 3: SS/PBCH block is transmitted in slot 0 and 20 of each period of 40 slots NOTE 4: Slot i is slot index per four frames. |  |  |  |  |  |  |

-	Option 3: Bitmap (0000000000, 1111111111, 0000000000, 1111111111)

0 as FDD carrier, 1 as SDL carrier 1 switch within 10 slots UL TA assume as 0 us for RMC definition and test

-	Option 4: other options are not precluded

Issue 2: Uplink reference measurement channels

Agreement:

-	RAN4 can further discuss whether an UL RMC is needed for this feature

Issue 3: Reference measurement channels: general aspects

Agreement:

-	In RMC, NTA=0 is considered.

Issue 4: UE capability update

Agreement:

-	The following FG description is agreed:

Issue 5: Whether a liaison to RAN2 is useful to clarify the impact of low band switching on BandCombinationList

Agreement:

-	Send a liaison to RAN2 to clarify the impact of low band switching on BandCombinationList