3GPP TSG-RAN WG4 Meeting # 116		R4-2511905

Bengaluru , IN, August 25 – 29, 2025

Agenda item:	7.3.3

Source:	Apple, Samsung, LG Electronics, Huawei

Title:	TP to TR38.768 on low NR band aggregation via switching

WI/SI:	NR_LBCA_Sw-Core

Release:	Rel-19

Document for:	Approval

# 1	Introduction

In the previous RAN4 meeting (Meeting #115, Malta, 19th – 23rd May, 2025), several` aspects of the low NR band aggregation via switching feature were debated. As highlighted in the meeting WF [1], agreements were made on the time masks, switching period locations, as well as the addition of 70s to the list of switching period values, resulting in a set of 3 values (35s, 70s, and 140s) as possible values for the UE switching period capability when the UE supports the low NR band aggregation via switching feature. The TR38.768 [2] was updated to include the requirements for the time masks and switching period locations. However, the new value of the switching period (70s) was not added in the TR.

This contribution provides the following text proposals to TR38.768:

-	Change 1: Include 70s switching period value in all the clauses where the switching period values are captured.

-	Change 2: Add the applicability of the inter-band carrier aggregation aspects for a UE which supports the low NR band carrier aggregation via switching

-	Change 3: The configuration aspects related to low NR band carrier aggregation via switching

-	Change 4: Impact on MSD requirements

-	Change 5: Impact on ΔRIB,c and ΔTIB,c requirements

-	Change 6: Aspects related to the RMCs

# 2	References

[1]	R4-2508118, “WF on UE RF requirements for LB CA via switching,” by Apple, 3GPP RAN4 #115, Malta, 19th – 23rd May, 2025

[2]	R4-2508119, “TR 38.768-004” by Apple, 3GPP RAN4 #115, Malta, 19th – 23rd May, 2025

[3]	R4-2505439, “UE RF requirements for LB CA via switching,” Apple, 3GPP RAN4 #115, May 2025

# 3	Text Proposal

<< begin change 1 >>

## 5.1	Switching periods

To accommodate different UE implementations, RAN4 has reached the agreement in RAN4#114 on the applicable switching periods of 35μs, 70μs, and 140μs for switching between Case 1 and Case 2 as an optional UE capability.

## 5.2	Switching period location

### 5.2.1	General

For what follows we consider switching between an FDD and SDL band for the CA-n5A-29A configuration in Table 4-1 but without loss of generality: one UL is configured in a paired band with the DL Scell configured in either a DL band of another paired band or in an SDL band.

.

The timing reference for the switching pattern and switching gap at the UE is the DL timing of the Pcell (in the FDD band). The uplink radio frame transmission on the Pcell takes place TTA = (NTA + TAoffset)Tc before the reception of the first detected path (as defined in TS 38.133) in time on the corresponding DL radio frame. This means that the timing advance must be considered at switching events.

The duration of the switching gap is RRC configured. The network ensures that the switching gap is large enough to cover the switching period (35 , 70μs, and  140 𝜇s according to the UE capability), transient periods and the TA, if needed, in both switch directions.

### 5.2.2	FDD to SDL switch

For the FDD to SDL switch the gNB configures a gap long enough to accommodate timing advance such that no collisions should occur. Figure 5.2.2-1 shows the case with TTA > 0 µs. The figure illustrates the applicable transient periods, switching period (35, 70μs, and  140 𝜇s according to UE capability), and the applicability of ON and OFF power requirements.

Figure 5.2.2-1: FDD to SDL switch with the switching gap on the switched-from carrier.

The UL slot will always end before the DL slot, neglecting any timing errors in case TA = 0 µs. The worst case in terms of switching is that in which TTA = 0 µs, the DL and UL timing are the same (again neglecting the errors that are of the order of 1 µs) and the SDL starts RTD µs before the Pcell DL slot timing. Switching period is always after UL transient period. The switching gap with respect to the end of the last FDD DL symbol must therefore be at least

actual switching gap length > Tswitch + MRTD + FDD UL trailing end transient period + SDL leading transient period.

where the MRTD = [CP length/TAE] assuming that the FDD and SDL carriers are colocated

### 5.2.3	SDL to FDD switch

For the SDL to FDD switch, the switching gap ends at end of slot on “switch from” carrier (SDL). The timing advance must also be accounted for; Figure 5.2.3-1 illustrates this for all applicable switching periods Tswitch (35μs, 70μs, and  140 𝜇s according to UE capability),  by placing the switching period X . The applicability of ON and OFF power requirements is also shown.

Figure 5.2.3-1: SDL to FDD switch with the switch occurring before the start of the UL slot.

The time mask shall be devised such that the switching period occurs always before the  transient period of the UL slot of the switched-to FDD carrier, this to avoid dropping symbols on the UL that starts TTA before the first symbol of the FDD DL slot. Notwithstanding, the switching gap on the SDL carrier must be at least

Actual switching gap length > Tswitch + TTA + MRTD + SDL trailing transient period + FDD UL leading transient period

with account of the SDL received at maximum RTD of a [CPlength/TAE]. The worst case is when the SDL is traling the Pcell DL as shown in the figure.

Where RTD is the receiving timing difference with the range of negative MRTD to positive MRTD. And MRTD = 3us. assuming that the FDD and SDL carriers are colocated. UL transient period is assumed as 10us.

In the live network operation, timing advance for a UE in a cell is variable. It is the network task to adjust the length of the switching gap by [LBCA-SwitchingGap-Duration-SCelltoPCell] so that the UL transient period is not overlapping with switching period X.

<< end change 1  >>

<< begin change 2  >>

## 5.3	Time mask for switching

### 5.3.1	General

The switching gap is configured by RRC in terms of number of symbols blanked by the network, while the actual location of the switch is specified by time masks similarly to UL Tx switching.

### 5.3.2	UL time mask

Figure 5.3.2-1 below illustrates the ON/OFF time mask for low NR band carrier aggregation via switching.

Figure 5.3.2-1: ON/OFF time mask for NR UL transmission for DL CA via switching with non-CA in the UL

## 5.	Applicability of simultaneous Rx/Tx requirements

The following statement in section 5.2A.2 of TS38.101-1 captures the applicability of simultaneous Rx/Tx requirements when the UE supports the low NR band carrier aggregation via switching: “Concurrent operation between these bands is not applicable to UEs indicating support of low NR band aggregation via switching [supportedLowBandSwitching-r19] for this band combination.”

<< end of change 2  >>

<< begin of change 3  >>

## 5.5	CA configurations

Low NR band inter-band CA configurations in which the UE is allowed to indicate support of the configuration via switching [supportedLowBandSwitching-r19] are indicated with the corresponding note in the configuration tables in sub-clause 5.5A.3.1 of TS38.101-1.

The following notes to be added in Table  5.5A.3.1-1d of TS38.101-1 next to the band combinations that support low NR band aggregation via switching: “The UEs is allowed to indicate support of low NR band carrier aggregation via switching [supportedLowBandSwitching-r19] for this NR CA configuration”.

The following notes to be added in Table 5.5A.3.1-1f of TS38.101-1 next to the band combinations that support low NR band aggregation via switching: “Applicable only for UEs which indicate support of low NR band carrier aggregation via switching [supportedLowBandSwitching-r19] for this NR CA configuration”

<< end of change 3 >>

<< begin of change 4  >>

## 5.6	MSD requirements

The following note to be added in Table 7.3A.6-1 and Table 7.3A.6-1a-1 of TS38.101-1 to indicate that the specified MSD is not applicable to UEs indicating support of low NR band carrier aggregation via switching :.”Not applicable to UEs indicating support of low NR band aggregation via switching [supportedLowBandSwitching-r19] for this band combination”

<< end of change 4 >>

<< begin of change 5>>

## 5.7	ΔRIB,c  requirements

The following note to be added in Table 7.3A.3.2.1-1 of TS38.101-1 to indicate that the specified ΔRIB,c is not applicable to UEs indicating support of low NR band carrier aggregation via switching : “Not applicable to UEs indicating support of low NR band aggregation only via switching [supportedLowBandSwitching-r19] for this band combination”

Annex B:

The discussion is out of current scope. But this issue will need to be resolved when introducing higher order CA with the LB CA switching as one of fallback configuration and/or different device type of LB CA switching (e.g., con-current DL CA, or FDD+FDD CA switching with concurrent DL CA)