# 3GPP TSG-RAN WG4 Meeting #116                                                                R4-2512124

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:			7.14.1

Source:	Moderator (Apple)

Title:	WF on RRM requirements for NR_RRM_Ph5_Part1

Document for:	Information

# Topic #1: FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC (7.14.3.2)

### Issue 1-1: issues related with solution 1 (measure one serving CC per band)

Issue 1-1-1: LS reply to RAN2 (R2-2504728) on solution 1

Agreement:

Reply LS R4-2512161 has been agreed.

RAN4 would like to thank RAN2 for the LS on the CSSF optimization for NR RRM Phase 5. RAN4 had discussion on the alternatives from RAN2, and following conclusions have been made: - About the network indication of a specific SCC per-band for measurement, RAN4 agreed to adopt an alternative 3 as following: - If UE support R19 CSSF solution 1 (measurement one CC per FR2-1 band) and network enables this feature, - Follow the new RRC network indication (if present) to measure the indicated SCC in an FR2-1 band, if - Neither PCC nor PSCC is in the same FR2-1 band, and - The indicated SCC to be measured shall be one of the SCCs on which UE is configured to report SSB based measurements in the same FR2-1 band. - For case that only MO(s) are configured on SCC(s) in an FR2-1 band, when neither PCC nor PSCC is in the same FR2-1 band, and there is no SCCs on which UE is configured to report SSB based measurements in this FR2-1 band, UE will select and perform serving and intra-frequency neighbour cell measurements on one SCC in that FR2-1 band. SCC selection in this FR2-1 band to measure is up to UE implementation. - RAN4 kindly requests RAN2 to implement RRC signalling enabling network to indicate which SCC the UE shall measure in an FR2-1 band.

Issue 1-1-2: revision on solution 1 according to issue 1-1-1

- Discuss in draft CR

Issue 1-1-3: Expand solution 1 to FR1 case

Agreement:

Not extend the solution 1 to FR1.

### Issue 1-2: issues related with solution 3 (3 searchers)

Issue 1-2-1: applicability limitation for solution 3

Agreement:

For UE supporting [new 3-searcher capability] the Enhanced CSSFoutside_gap,i scaling factor does not apply for the E-UTRA inter-RAT MOs without MG (for the scenario of NR SA) that are being measured outside of MG.

FFS: For UE supporting [new 3-searcher capability], whether or not the Enhanced CSSFoutside_gap,i scaling factor applies for the inter-frequency MOs without MG that are being measured outside of MG. This part can be discussed in maintenance stage.

### Issue 1-3: issues related UE capability for CSSF enhancement

Issue 1-3-1: detailed UE capability for CSSF enhancement

Agreement:

Solution 1 (measure one CC per band):

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-3 | L3 serving cell and neighbor cells measurement and report on one serving carrier per-band for intra-frequency measurements without measurement gap | Support of serving cell and neighbor cells measurement and report on one serving carrier per-band for intra-frequency measurements without measurement gap if multiple serving carriers are configured in a same band. |  | Yes |  | UE does not support serving cell and neighbor cells measurement and report on one serving carrier per-band for intra-frequency measurements without measurement gap if multiple serving carriers are configured in a same band. | Per-UE | No | FR2-1 only |  | UE indicating this capability shall meet the corresponding enhanced requirements in TS38.133 Clause 9.2.3.2, 9.1.5.1.1, 9.1.5.1.2, 9.1.5.1.3, and 9.1.5.1.4 | Optional with capability signaling |

Solution 3 (3 searchers):

This capability has been agreed in last meeting, and following is just to remove the square bracket

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-1 | Simultaneous L3 measurement on three carriers for measurements without measurement gap under CA/DC operation. | Support of measuring serving cell and neighbor cells measurement on three carriers simultaneously for measurements without measurement gap. Support of separate indications for different cases: {FR1 only CA and FR1 only NR-DC}, {FR1 only EN-DC}, {FR1+FR2 CA PCell is FR1 only}, and {FR1+FR2 NR-DC PCell is FR1 only}. |  | Yes |  | UE does not support measuring serving cell and neighbor cells on three carriers simultaneously for measurements without measurement gap. | Per-UE | No | No |  | UE indicating this capability shall meet the corresponding enhanced requirements defined in TS38.133 Clause 9.1.5.1.1, 9.1.5.1.2, and 9.1.5.1.3 if the feature is supported. | Optional with capability signaling |

# Topic #2: RRM performance requirements of FR2-1 SSB based L3 measurement delay reduction for connected mode (7.14.5)

### Issue 2-1: Test case design for FR2-1 L3 measurement delay reduction by optimizing Rx BSF

Issue 2-1-1: baseline setup for BSF test case

- Proposal 1(Samsung):

  - For the AoA setup of measurement delay reduction for SSB based Inter-frequency measurement by optimizing Rx beam sweeping factor, RAN4 to discuss whether AoA Setup #5 can be considered.

  - To define the new test case of measurement delay reduction for SSB based Inter-frequency measurement by optimizing Rx beam sweeping factor, the test requirement should be defined by using serving cell’s quality based triggering condition

    - How to define the test parameters involving hysteresis and the TTT/evaluation period of the timer can wait for further solid conclusions.

- [Way forward]

  - Continue discussion in RAN4#116bis.

Issue 2-1-2: whether or not to test FBS quitting

- Option 1(CMCC): it is proposed to discuss whether to test FBS quiting when FBS conditions are met, e.g. whether to test that UE fallback to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition.

- [Way forward]

  - Continue discussion in RAN4#116bis.

### Issue 2-2: Test case design for FR2-1 L3 measurement delay reduction by optimizing CSSF

- TC for solution 1:

Issue 2-2-1: baseline TC setup for CSSF optimization solution 1

- Proposal 1(Nokia): A scenario with at least two carrier frequencies in one FR2 band is required to verify the measurement delay with enhanced CSSF.

- Proposal 2 (Nokia): The test cases for enhanced CSSF are designed by configuring event triggered measurement reporting on a neighbour cell on the same frequency with PSCell (in EN-DC) or PCell (in FR2 SA).

- [Way forward]

  - Continue discussion in RAN4#116bis.

Issue 2-2-2: sub-TC for CSSF optimization solution 1 with network indication

- Option 1(Apple): for CSSF enhancement solution 1, if the network indication for a specific SCC measurement is replaced by “SCC selection by UE implementation”, remove the TCs related with network indication in last meeting.

- Option 2 (Nokia): To define sub-test to verify the measurement delay with enhanced CSSF in case network indicates the carrier to be measured per FR2 band.

- [Way forward]

  - Continue discussion in RAN4#116bis.

Issue 2-2-3: TC for CSSF optimization solution 1 in FR1

- Proposal 1(Nokia, CMCC):

  - To define the test cases to verify the measurement delay by measuring one SCC per FR1 band, if this would be agreed in core requirement discussion.

- Agreement:

  - This issue is closed since issue 1-1-3 agreed to not extend the solution 1 to FR1

- TC for solution 3:

Issue 2-2-4: baseline TC setup for CSSF optimization solution 3

- Proposal 1(Huawei):

  - To unify the test set of Solution 3 test cases, the test set of Solution is able to refer to four cells: one PCell, one neighbour Cell of PCC or one inter-RAT E-UTRAN Cell, and two deactivated SCells.

- [Way forward]

  - Continue discussion in RAN4#116bis.