3GPP TSG-RAN WG4 Meeting #116	R4-2512125

Bengaluru, India, August 25th – August 29th, 2025

Title: 	WF on RRM requirements for NR_RRM_Ph5_Part2

Agenda Item:	7.14.1

Source: 	Moderator (CATT)

Document for:	Approval

- Introduction

This WF includes the agreements and open issues discussed in topic summary for [116][212] RRM requirements for NR_RRM_Ph5_Part2.

Topic #1: RRM core requirements for FR2-1 L3 measurement delay by optimizing Rx BSF

Sub-topic 1-1 RRM core requirements for FR2-1 L3 measurement delay by optimizing Rx BSF

Issue 1-1-1: Value for T1

Agreement:

- A fixed value of 90 seconds.

Issue 1-1-2: Re-evaluation mechanism

Agreement:

FFS following options in the maintenance stage:

- Option 1: Introduce a predefined time duration T2, and UE re-evaluates the activation condition after this predefined time duration expires, where this time duration starts after UE quits from Multi-Rx mode. Value of T2 can be:

    - Alt 1: Use the values for T1 as a starting point.

    - Alt 2: 5 s, which is predefined in RAN4 spec.

- Option 2: It is up to UE implementation when to re-enter the FBS after quitting the last FBS.

- Option 3: It is up to UE implementation when to re-evaluation of the activation condition for FBS after quitting the last FBS.

Agreement in Friday online:

- No consensus to introduce a predefined time duration T2 in the WI core part in RAN4.

Issue 1-1-3: UE capability for Rx BSF optimization

Agreement:

The following UE capability is defined:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 49. NR_RRM_Ph5 | 49-2 | Fast Rx beam sweeping factor for FR2-1 L3 measurement delay reduction | Supports fast Rx beam sweeping factor reduction for L3 measurement for FR2-1. |  | Yes | N/A | UE does not support fast Rx beam sweeping for FR2-1 L3 measurement delay reduction. | Per band | TDD only | FR2-1 only | N/A | Candidate values: {2, 4, 6} for FR2-1. Note: It is only supported for power class 3. | Optional with capability signalling |

Issue 1-1-4: Requirements when UE reports different BSF values for different bands

Way forward:

Further discuss this issue in the maintenance stage:

  - Option 1:

    - It is proposed to discuss how to specify the measurement requirements when multiple bands are involved and UE reports different BSF values for different bands. One simple option is to specify the requirement based on the largest BSF value reported over all the bands where an MO is configured.

Issue 1-1-5: Revision of the agreement on applicability requirement

Agreement in Tuesday ad-hoc:

- The FBS enhancement applies for the case when the target and serving carrier frequency are same.

- FFS in maintenance which CC to apply the FBS when the target and serving carrier frequency are on different.

Agreement in Thursday online:

When the target and serving carrier frequency are different,

- FBS requirement does not apply to inter-frequency measurement without measurement gap.

- FFS inter-frequency measurement with per-FR measurement gap.

- Note: inter-RAT is not in the scope.

Topic #2: RRM requirements of Fast SCell activation for UE supporting Rel-18 EMR

Sub-topic 2-1 Core requirements of Fast SCell activation for UE supporting Rel-18 EMR

Issue 2-1-1: UE capability for EMR-based fast SCell activation

Agreement in Friday online:

- Introduce one single UE capability for R19 SCell activation based on EMR, further discuss the detail.

Sub-topic 2-2 Performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Issue 2-2-1: Test case list

Further discuss this issue taking the options in the summary R4-2509055 into consideration.

Issue 2-2-2: Applicability rules of test case

Further discuss this issue taking the options in the summary R4-2509055 into consideration.

Issue 2-2-3: Test case design

Further discuss this issue taking the options in the summary R4-2509055 into consideration.

Issue 2-2-4: Report mechanism of EMR results in the test cases

Further discuss this issue taking the options in the summary R4-2509055 into consideration.

References

- R4-2509055, Topic summary for [116][212] NR_RRM_Ph5_Part2, CATT, RAN4#116.

- R4-2512123, Ad-hoc minutes for NR_RRM_Ph5, Apple, RAN4#116.