3GPP TSG-RAN WG4 Meeting #116				                             R4-2512128

Bengaluru, India, August 25th – 29th, 2025

Agenda item:			7.19.1

Source:	OPPO

Title:	WF on FS_NR_AIML_Mob_Part2

Document for:	Approval

Topic 1 Testability and interoperability

Sub-topic 1-1: Testability aspects for RRM measurement prediction

Issue 1-1-1: Testing of Generalization aspects

Agreement:

- Start discussion on the requirement and testing aspects related to the generalization in WI phase.

Issue 1-1-2: Interoperability

Agreement:

- RAN4 does not identify any interoperability issue which impacts the feasibility.

Issue 1-1-3: Testing aspects of intra-frequency prediction scenarios

- Proposals and Observations:

  - For case A:

    - Proposal 1:

      - TE must account for the delays in acquiring prediction and corresponding Ground Truth as some UEs might not be capable of doing both concurrently.

    - Proposal 2:

      - RAN4 to consider the delay parameter δ in Time Domain Case A for both Alt 1 (operates sequentially) and Alt 2 (operates concurrently), defined as the time gap between prediction and GT measurement references.

      - δ shall be measured or configured by the test equipment to align results for both sequential (larger δ) and concurrent (smaller δ with processing delays) UE modes, with tolerances to prevent false failures from timing offsets.

  - For case B:

    - Proposal 3:

      - For testing time domain prediction case B, RAN4 to follow RAN2 agreements regarding skipping pattern indication as part of the testcase configuration.

      - For testing time domain prediction case B, RAN4 to consider parallel processing of testing procedure to evaluate different configurations (mainly MRRT and related skipping patterns).

    - Proposal 4:

      - RAN4 to define a standardized set of representative skipping patterns for each MRRT in Time Domain Case B, including at least one uniformly spaced (e.g., measure–skip–measure–skip) and one bursty (e.g., measure–measure–skip–skip) pattern, to cover both easier and more challenging prediction scenarios.

Issue 1-1-4: Testing aspects of inter-frequency prediction scenarios

- Proposals and Observations:

  - Proposal 1:

    - Colocation can be reflected by a minor RSRP difference between two carriers.

  - Proposal 2:

    - For FR1-to-FR1 inter-frequency prediction scenario, it should be clarified whether synchronization between the timing of measurement and prediction is required for testing.

    - The test setup should closely reflect the deployment conditions and inter-frequency correlation characteristics.

  - Proposal 3:

    - For inter-frequency prediction, RAN4 to discuss whether and how to construct the relation of configurations between the two carries.

  - Proposal 4:

    - The testability discussion of sub-use-case 1 (L1-L1) and sub-use-case 3 (L1-L3) should use the progress of the testability study in AI-BM as baseline. And RAN4 can start the discussion on sub-use-case 2 (L3-L3) in AI mob.

Issue 1-1-5: Ground Truth derivation in FR1

<Way forward >

  - Consider the following options for ground truth derivation and refer to the discussion in [116][214] FS_NR_AIML_Mob_Part1

    - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power

    - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions

  - For Alt2, consider the solutions to identify and avoid UE cheating.

Issue 1-1-6: LCM related testability aspects

Agreement:

  - Leave the LCM related testing discussion to WI stage after the requirements are clear.

Issue 1-1-7: Testing goal

- Proposals and Observations:

  - Proposal 1:

    - Propose a dual-accuracy framework for Release-19 to differentiate between existing accuracy requirements for measured values and adjusted requirements for predicted values.

      - Measured RSRP should continue to meet the current absolute accuracy mask, ensuring alignment with legacy conformance.

      - Predicted RSRP should satisfy the same absolute mask with additional absolute slack Δ_abs applied only to predictions, allowing for the extra uncertainty introduced by prediction.

      - In addition, a relative-delta requirement |Predicted – Measured| ≤ Δ_rel(MRRT, PW) should be applied, where Δ_rel may widen under side conditions such as higher MRRT, longer PW, increased UE speed, or higher Doppler.

        - Both Δ_abs and Δ_rel should be defined in a standard parameter table, linked to side conditions, and verified against agreed ground-truth.

        - Any widening of Δ_abs or Δ_rel must be conditional on demonstrating that core mobility KPIs (e.g., handover success rate, RLF rate) are not degraded.

  - Proposal 2:

    - For FR1-to-FR1 inter-frequency prediction scenario, one of the testing goals should be handover delay.

  - Proposal 3:

    - The testing goal for measurement prediction is to verify:

      - the measurement reported in OW meets the new requirements if defined;

      - the predicted measurement in PW meets the accuracy requirements under certain side conditions.

  - Proposal 4:

    - It is proposed to follow previous agreements that the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

  - Proposal 5:

    - From the perspective of test, core part and performance part requirements should be considered, i.e. measurement delay and measurement accuracy.

  - Proposal 6:

    - For testing goal, the principle of defining the minimum performance can be discussed based on the following two options:

      - Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy requirement.

      - Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters.

        - FFS: The value of MRRT.

        - FFS: Other parameters and the values, e.g., the prediction window.

    - When defining measurement prediction requirements, the gain derived from power consumption reduction and measurement gap reduction shall also be taken into account.

  - Proposal 7:

    - Testing in intra-cell scenario should validate the measurements reported by the UE for the target cell in the test setup meet:

      - requirement for the measurements performed inside the OW,

      - requirement for the measurements predicted for the time instances inside the PW.

    - Testing in inter-cell scenario should validate the measurements reported by the UE:

      - meet the requirements for measured cell, and

      - meet the requirements for the predicted cell.

    - Relative accuracy of the reported measured L3-RSRP and reported predicted L3-RSRP is verified in the test.

  - Proposal 8:

    - For temporal domain case B, the testing goal is to verify if the reported predicted results satisfy the accuracy requirement based on the purpose of the measurement reduction (i.e. Option 1: The predicted measurement is a replacement for the actual measurement or Option 2: The predicted measurement is not a full replacement of the actual measurement).

    - For temporal domain case A, testing goal is to verify if the reported predicted results meet the new accuracy requirement which is yet to be defined.

Issue 1-1-8: Test environment mismatch

- Proposals and Observations:

  - Proposal 1:

    - RAN4 should align AI/ML mobility conformance testing with key real-world conditions to avoid training–testing mismatch.

      - Test scenarios must cover representative Doppler and SNR ranges, MRRT patterns, and beam/TA dynamics matching the model’s training data.

      - Control-plane coherence should be preserved by updating TCI and TA states at realistic cadences linked to the UE trajectory, keeping beam index changes in step with the channel.

      - Neighbor-cell effects should also be included to reproduce realistic interference.

  - Proposal 2:

    - Discuss in work item how to derive the dataset and how much alignment is needed for training and inference dataset.

  - Proposal 3:

    - RAN4 to study the minimum test configuration to be specified in order to ensure the consistency between training data set and test environment for the UE sided AIML models/ functionalities.

Sub-topic 1-2: Testing setup for RRM measurement prediction

Issue 1-2-1: Testing Setup for FR1 and FR2

- Proposals and Observations:

  - Proposal 1:

    - RAN4 configures one SSB per cell and the measured RSRP results are regarded as cell-level RSRPs.

    - AWGN or TDL channel can be used in FR1 conducted test.

  - Proposal 2:

    - RAN4 should follow the FR2 AI/ML mobility testing procedure outlined in the accompanying flowchart, using a single-probe OTA chamber configured with one active SSB per cell, a TDL-C fading model, and a common reference UE trajectory from TR 38.901. The setup shall embed OW, PW, and MRRT in each test script to ensure time alignment, with high-SNR “core” runs for baseline accuracy and “performance” runs adding controlled ±6 dB slow-fading variations to stress prediction robustness.

    - RAN4 is invited to incorporate test configurations that include both L3 filtering and multi-beam scenarios when developing AI/ML mobility use case test cases.

  - Proposal 3:

    - The other testing setup can be left to WI based on the requirements definition.

      - Received power pends on SINR side condition of requirements

      - Measurement prediction specific parameters pend on the sets of requirements

  - Proposal 4:

    - For testing setup in FR1 RRM measurement prediction, RAN4 to take antenna configuration of 1 Tx and SSB configuration of 1 SSB per SS-burst as baseline.

    - For FR2 measurement prediction, conclusions on testability that RAN4 made for AI/ML BM can be taken as a baseline.

  - Proposal 5:

    - For FR1 time domain and frequency domain RRM measurement prediction, at least TDL can be used for testing, FFS for AWGN.

    - For FR2 spatial domain RRM measurement prediction, it is proposed to take the outcome of AI/ML BM as baseline.

  - Proposal 6:

    - For testing on AI mob in FR1, prefer to use AWGN/TDL channel model as baseline.

  - Proposal 7:

    - Postpone the testability discussion for FR2-to-FR2 prediction to wait the progress of the testability study in AI-BM.

  - Proposal 8:

    - Dynamic range of received power under certain SNR

      - From signal can be detected to signal can be acquired

    - Emulation of noise and EPRE

      - How the emulated noise is generated/transmitted, and for the SNR/SINR in various evaluation scenarios, the test will define the exact noise/SNR level per TRxP at the reference point

  - Proposal 9:

    - RAN4 to consider multi-beam scenarios while defining the test cases for AIML mobility.

    - RAN4 to consider CDL channel model (agreed for AIML Beam management use case) as baseline for AIML Mobility test.

    - RAN4 to wait for AIML Beam management use case outcome for reference UE trajectory and consider it as baseline for AIML Mobility test.

    - RAN4 should not consider the measurement prediction parameters (i.e., OW/ PW, MRRT, skipping pattern for temporal domain case B etc.) for the test setup discussions.

Issue 1-2-2: Cell number for inter-frequency prediction

- Proposals and Observations:

  - Proposal 1:

    - 2 cells are enough if single-cell approach is adopted and more cells are needed if cluster approach is used. RAN4 can decide the exact number in WI phase.

  - Proposal 2:

    - RAN4 to adopt a cluster-based testing setup for inter-frequency inter-cell prediction scenarios, requiring at least two input cells (e.g., serving and neighbor) and at least one target cell as output.

  - Proposal 3:

    - For inter frequency measurement prediction, more than 3 cells are needed if cluster approach is assumed and the exact number can be decided in WI stage according to the requirements discussion.

  - Proposal 4:

    - For the scenario of FR1 to FR1 inter-frequency inter-cell frequency domain scenario, the legacy test procedure can be taken as baseline for single cell prediction. FFS for multi-cells prediction.

  - Proposal 5:

    - For inter-cell case Scenario 3 (FR1 to FR1 inter-frequency (frequency domain)), prefer to reuse the same cell layout adopted in RAN2 evaluations.  It is sufficient for TE to transmit the strongest 8 intra-f cells (serving cell included) of the 21 cells during the test.

  - Proposal 6:

    - For inter-frequency domain prediction, it is proposed that at least 2 cells are needed as model input, since RAN2 focus on cluster based approach.

  - Proposal 7:

    - For inter-frequency predictions, consider multi-cell setup in the test, including serving cell and neighbor cells.

  - Proposal 8:

    - 2-cell setup is used to validate requirements for inter-cell prediction.

      - Cell 1 in the test can be considered as the cell for measurement.

      - Cell 2 in the test can be considered as the target cell for measurement prediction.

  - Proposal 9:

    - RAN4 should consider more than two cells for inter-frequency inter-cell scenario (i.e., scenario 3) in the test setup based on the RAN2 agreements.

    - For the cluster approach, at least two input cells are required that should be intra-frequency, non co-located cells and the target cell should be inter-frequency cell and co-located with one of the input cells.

Issue 1-2-3: Prediction consistency in time domain

<Way forward >

  - To verify the measurement prediction accuracy, the following approaches can be considered to model the time-varying characteristics per cell/site:

    - Option 1: TE to transmit the signals of cells with time-variance following a certain fading channel model, i.e., TE to use BS simulator plus channel simulator to transmit the signals.

      - FFS the fading channel model:

        - Option A: TDL channel

        - Option B: Simplified CDL

      - FFS whether and how to add UE trajectory considering the following aspects:

        - The solutions provided by TE vendor in R4-2511598 can be considered.

        - The UE trajectory from TR 38.843 can be considered as baseline.

        - Follow TR 38.901 clause 7.6.3 on spatial consistency.

  - Consider the above options when discussing the simulation assumptions in the WI phase.

Sub-topic 1-3: Testability aspects for measurement event prediction

Issue 1-3-1: Testing aspects of measurement event prediction

- Proposals and Observations:

  - Proposal 1:

    - RAN4 should adopt a spec-based ground truth (GT) as the primary reference for event-prediction conformance.

      - This GT is computed entirely offline from the scripted fading profile using the exact signalled event parameters (threshold, L3 filter coefficient, TTT, hysteresis). UE-reported GT should be excluded from conformance testing to prevent trivial alignment cheats, though it may remain available for vendor debugging.

    - Calibration and Prediction Workflow

      - To ensure that the spec-based GT reflects real receiver behavior, each test bench should periodically perform a one-time high-SNR calibration replay using a golden reference receiver. Measured event times must fall within ±x slot of the spec-based GT before use in testing. For prediction runs, the identical fading script is replayed under the intended noisy conditions, and the DUT’s predicted event times are compared directly to the pre-computed GT using agreed Event-Time-Deviation (ETD) accuracy masks.

  - Proposal 2:

    - For the testing of event prediction, RAN4 to study how to know the real event occurs.

  - Proposal 3:

    - For the testing of event prediction, RAN4 may use UE measurement event reporting based on actual measurements, or ground truth derived from UE reports or known test equipment conditions in conducted tests, for analyzing the occurrence of the real event.

    - For the testing of event prediction, RAN4 may wait further progress on testing procedures for measurement prediction and consider reusing the agreed prediction evaluation approach for indirect event prediction.

Reference

- R4-2509058 Topic summary for [116][215] FS_NR_AIML_Mob_Part2, OPPO, RAN4#116