3GPP TSG-RAN WG4 Meeting # 116											R4-2512129

Bengaluru, India, August 25th – 29th, 2025

Agenda item:			7.20.1

Source:	Samsung

Title:	Ad-hoc minutes for NR_MIMO_Ph5

Document for:	Approval

Introduction

This Ad-hoc minutes is to cover the discussion in [116][216] NR_MIMO_Ph5_Part1 and [116][217] NR_MIMO_Ph5_Part2 for the following issues:

Topic #1: RRM requiements for MIMO_Ph5_Part2

Moderator’s note on the scenarios’ definition discussed in this section:

- Scenario#1: if UE is configured with PL offset in joint/UL TCI state(s), UE does not expect to receive SSB from UL TRP(s)

- Scenario#2: if UE is not configured with PL offset in joint/UL TCI state(s), UE may expect to receive SSB from UL TRP(s).

- Issue 2-1: For Scenario#1, whether to update RACH requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

- Proposals

  - Proposal 1 (QC, Samsung, CMCC, HW, MTK): reuse the legacy timing requirement for Scenario #1.

    - Proposal 1a (QC): For Scenario #1, the reference timing is the first detected path (in time) of the downlink reference signal(s) of the TRP that transmits downlink reference signals. The reference timing of the PRACH is the same for both TRPs.

      - No additional time offset to the PRACH timing should be introduced to mitigate impacts of asynchronous TRPs.

    - Proposal 1b (Samsung): For Scenario#1, no update for RACH requirements for 2TA enhancement. They are the same as legacy single TA solution.

    - Proposal 1c (CMCC, HW): For scenario 1, it is proposed that, for both TAGs, the uplink transmission(s) timing for PRACH take place (NTA + NTA offset) x Tc before the reception of the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with DL or Joint TCI state.

    - Proposal 1d (MTK): The RACH requirements for 2TA for Scenario#1 can reuse the legacy timing requirement where the reference point for PRACH transmission triggered by PDCCH order,

      - if “PRACH association indicator” is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

      - if “PRACH association indicator” is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG.

  - Proposal 2 (Nokia, E///): RAN4 to introduce a further timing offset for the PRACH transmission

    - Proposal 2a (Nokia): For an asymmetric DL sTRP/UL mTRP deployment with two TAs, RAN4 to introduce a further timing offset/delta in addition to n-TimingAdvanceOffset/n-TimingAdvanceOffset2 when PRACH is transmitted by the UE.

    - Proposal 2b (E///): UE to apply additional DL reference timing offset for the PRACH transmission. The DL reference timing offset can be [0.5*TAE].

- Recommended WF

  - Down select between P1 and P2

Adhoc Discussion:

Adhoc Chair: P1 is to define the DL reference timing. P2 it to introduce a further timing offset based on the DL reference timing. P1a and P1b are the same. Suggest to pick up from P1a and P1c.

Nokia: Question for P1d. For the “RACH association indicator”, from the RAN1 LS. It is only for inter-cell for two DL timings.

Samsung: for P1d, the same understanding as Nokia for RAN1 LS.

E///: For this scenario#1, there is only one DL timing. P1a doesn’t work. We propose option 2 to use additional timing offset for DL timing advanced for PRACH.

Adhoc Chair: but for P2, you also need the DL ref. timing point definition, then can apply the timing offset base on that. We need to define the DL reference timing firstly. Then discuss whether to have additional value of offset.

QC: We have the same understanding as Chair. We need the definition of DL reference timing.

Apple: We support P1.

Samsung: P1a are the P1c are technical the same. We are fine for either of them.

QC: P1a and P1c are both fine.

Tentative Agreement:

- For DL reference point, agree on the concept of proposal P1c. For scenario 1, it is proposed that, for both TAGs, the reference point for PRACH, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DL or Joint TCI state.

Then we can discuss whether to have the same timing offset for both TRPs.

Option 1: (Samsung, QC, Apple, Nokia- can compromise)

- The same offset for both of TRPs.

Option 2: (E///)

- For UL-TRP, add another offset for PRACH.

Adhoc Chair: Question to P2a, only n-TimingAdvanceOffset according to RAN1 agreement. So, we just discuss whether to add further margin as P2 here.

Nokia:

QC: Do not see the necessity to introduce the addition margin. Don’t need to add it.

Nokia: This is the last meeting. We are ok to compromise to no further optimization.

E///: send LS to RAN1 for P2a for further clarification.

QC: for P2a. have RAN1 and RAN2 impacts.

Adhoc Chair: For P2a, what is the purpose to send RAN1 for the LS. To convert the RAN1 agreement? I cannot see this necessity. If there is still no consensus, keep further offline discussion on the options below:

Option 1: (Samsung, QC, Apple, Nokia- can compromise)

- The same offset for both of TRPs.

Option 2: (E///)

- For UL-TRP, add another offset for PRACH.

- Issue 2-3: For Scenario#1, how to define MRTD and MTTD requirements?

- Proposals

  - Proposal 1 (Nokia, MTK): Reuse Rel 18 MTTD values for Scenario #1.

  - Proposal 2 (Samsung): For Scenario#1, the MRTD and MTTD requirements can reuse the requirements in Rel-18 “multi-TRPs with two TAs for UE not capable of rxTimingDiff-r18”.

  - Proposal 3 (HW): There is no need to define MRTD or MTTD for Scenario 1.

  - Proposal 4 (Nokia): Define a new optional UE capability to support a case scenario when Tx timing difference between two TAGs is larger than CP length.

a.	Level 1:

i.	MTTD < CP length + 1.6 us for FR1

ii.	MTTD < CP length + 0.5 us for FR2

b.	Level 2:

i.	MTTD < 34.6 us for FR1

ii.	MTTD < 8.5 us for FR2

- Recommended WF

  - This is for scenario#1, there is only one DL (i.e., time difference between 2 DL is not valid), yet time difference can be valid for UL only. Thus, check whether to define MTTD only for scenario#1.

Adhoc Discussion:

Samsung: For this scenario, it can only work for less than CP condition. So proposal:

i.	MTTD < CP length + 1.6 us for FR1

ii.	MTTD < CP length + 0.5 us for FR2

Nokia: less than CP limited the scenario. For the cell size, the CP can be large than CP. For FR1, it is fine. But for FR2, it is critical. Thus, we support both larger than CP and less than CP.

a.	Level 1:

i.	MTTD < CP length + 1.6 us for FR1

ii.	MTTD < CP length + 0.5 us for FR2

b.	Level 2:

i.	MTTD < 34.6 us for FR1

ii.	MTTD < 8.5 us for FR2

Samsung: There is no larger than CP support for this case in RAN1.

Agreement:

- For scenario#1, no MRTD requirements.

- For MTTD requirements:

i.	MTTD < CP length + 1.6 us for FR1

ii.	MTTD < CP length + 0.5 us for FR2

FFS on the larger than CP capability:

i.	MTTD < 34.6 us for FR1

ii.	MTTD < 8.5 us for FR2

Further check the FFS part offline and on Thursday.

- Issue 2-2: For Scenario#1, how to update gradual timing requirements for 2TA enhancement for asymmetric DL sTRP/ UL mTRP?

- Proposals

  - Proposal 1 (Apple, Samsung): For scenario 1, The gradual timing adjustment requirements are only applicable to TRP with both UL and DL.

  - Proposal 2 (QC, CMCC, HW, E///, MTK): For scenario 1, same gradual timing adjustment applies for each TAG and shall be met for each TAG.

  - Proposal 3 (Nokia):

    - specify conditions under which UE autonomous timing adjustment applies to just the DL/UL TRP or also to the UL-only TRP.

    - autonomous timing adjustment towards the UL-only TRP is disabled when RSRP variation measured at the UE is large.

- Recommended WF

  - FFS

Adhoc Discussion:

Samsung: We identify the issue in the scenario: when the UE moves for opposite directions for 2 TRPs with different propagation delay change. (like in the pic)

Nokia: We understand the issue in P1. So we want to add some condition to identify the difference for whether UE moves fast or not by RSRP condition.

QC: It is true the uplink timing shall follow the DL timing and adjust by Tq and Tp. If the Tq and Tp are not applied for the second TRP, the timing will be incorrect.

Apple: Understand the concern from QC. Even it is applied for the second TAG, it doesn’t help. It gets worse.

E///:  NW can send the MAC-CE based adjustment. Support P2. If it goes bad, then NW can use the TA adjustment by NW.

Nokia: The only concern for P1 is for UL-TRP, there is no requirements.

Apple: Cannot accept for P2.

Adhoc Chair: For both P1 and P2, if the UE failed for autonomous TA adjustment, the MAC-CE based adjustment works. Still no compromise for consensus. Try further offline discussion.

Agreement:

- Issue 1-1: How to define measurement period and accuracy requirements for delay and frequency offset reporting for CJT calibration?

Background:

RAN4#115: Agreement: • RAN4 to discuss the achievable accuracy in terms of delay and frequency offsets based on 1 sample and [3 or 5] samples in the next meeting. • RAN4 to conclude the delay requirement for delay and frequency offset reporting in the next meeting. The delay requirement for phase offset is deprioritized in Rel-19.

- Proposals

  - Proposal 1 (QC, Apple, Samsung, HW, Nokia [with condition]): Based on 1 sample.

    - Proposal 1a (QC): RAN4 should only define accuracy requirements for delay and frequency offset reporting for one sample, i.e., for one NZP-CSI-RS resource set.

    - Proposal 1b (Apple): Define core requirements for CJT calibration reporting enhancement for delay/frequency reporting based on single shot measurement.

    - Proposal 1c (Samsung): Do not define multiple sets of requirements with different samples. Define the measurement period based on 1 sample. The SNR condition and accuracy can be decided in performance phase.

    - Proposal 1d (HW): One optional accuracy requirement for delay and frequency offset estimation in different conditions can be defined as:

| Frequency offset |  |  |  |
| --- | --- | --- | --- |
| Accuracy | Conditions |  |  |
| Normal condition | SNR | SCS | Sample |
| level | dB | KHz |  |
| 42 | 0 | 15 | 1 |
| 35 | +3 | 15 | 1 |
| 108 | 0 | 30 | 1 |
| 70 | +3 | 30 | 1 |

| Delay offset |  |  |  |
| --- | --- | --- | --- |
| Accuracy | Conditions |  |  |
| Normal condition | SNR | SCS | Sample |
| level | dB | KHz |  |
| 20 | 0 | 15 | 1 |
| 15 | +3 | 15 | 1 |
| 18 | 0 | 30 | 1 |
| 16 | +3 | 30 | 1 |

These accuracy values are defined with a little tolerance, and is able to be further discussed

    - Proposal 1e (Nokia): RAN4 to consider no averaging as long as CJTC-D error is below 0.1 CP and CJTC-F error is below 100 Hz.

      - Use SNR=-3 dB for defining the requirements

  - Proposal 2 (MTK): Based on 3 samples.

    - Reuse the legacy L1-RSRP CSI-RS measurement period as baseline for delay and frequency offset measurement.

      - TReport = 0

      - M (number of samples) = 3

      - Reuse legacy sharing factor (P).

  - Proposal 3 (E///): Based on 3 and 5 samples.

    - RAN4 to agree on using 3 samples for the frequency offset requirements for SNR > -3dB.

    - RAN4 to agree on using 5 samples for the delay offset requirements for SNR > 0 dB

  - Proposal 4 (MTK): If RAN4 cannot reach consensus on the number of samples used for delay and frequency offset, RAN4 to decide not to define the corresponding RRM requirements for CJT.

- Recommended WF

  - The majority view support Proposal 1. Check whether P1 is agreeable. Also check whether the condition in Proposal 1e is achievable.

Adhoc Discussion:

E///: We are fine for MTK’s proposal for 3 samples for both of them. Question for P1 from UE side: what is the issue by multiple samples.

QC: From implementation, we just measurement for every one sample based on TRS for delay and frequency offset and report to NW side. And the average can be applied and performed at NW side.

QC: How can UE do the average at UE side. Offline discuss with NW how to do the average.

Nokia: The number of samples is what we need to finalize the core part on time. That’s why we propose some simulation results to find the condition.

E///: Want to check offline for the feasible accuracy requirements based on 1 sample.

Nokia: our simulation results show:

RAN4 to consider no averaging as long as CJTC-D error is below 0.1 CP and CJTC-F error is below 100 Hz. Use SNR=-3 dB for defining the requirements

Agreement:

- Issue 1-2: How to define scheduling restriction requirements for delay and frequency offset reporting for CJT calibration?

- Proposals

  - Proposal 1 (QC, Nokia, E///): No scheduling restrictions should be defined for CJT reporting.

    - Proposal 1a (E///): CJTC measurement requirements are applicable when the RTD is less than CP.

  - Proposal 2 (Apple): RAN4 to discuss the necessity of scheduling restriction for CJT calibration reporting.

  - Proposal 3 (Samsung): The scheduling restriction is specified as:

    - For the UE is configured with CJTC reporting, the UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for CQI on the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting, and on one data symbol before and one data symbol after the symbols corresponding to the CSI-RS resource configured for [“CJTC-Dd”] and [“CJTC-F”] reporting

  - Proposal 4 (MTK): Reuse the legacy L1-RSPR scheduling restriction in FR1 as baseline for delay and frequency offset measurement.

    - There are no scheduling restrictions due to delay and frequency offset measurement performed on CSI-RS (tracking RS) with the same SCS as PDSCH/PDCCH in FR1.

    - For UEs which support simultaneousRxDataSSB-DiffNumerology, there are no restrictions on scheduling availability due to delay and frequency offset measurement based on CSI-RS (tracking RS).

    - For UEs which do not support simultaneousRxDataSSB-DiffNumerology, the following restrictions apply due to delay and frequency offset measurement based on CSI-RS (tracking RS).

      - The UE is not expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/CSI-RS for tracking/CSIRS for CQI on symbols corresponding to the CSI-RS (tracking RS) indexes configured for delay and frequency offset measurement.

- Recommended WF

  - Further discuss the proposals.

  - Check whether scheduling restriction is needed when DO/FO measurements performed on CSI-RS with different SCS from PDSCH/PDCCH.

Adhoc Discussion:

Agreement:

- Issue 1-3: How to define measurement restriction requirements for delay and frequency offset reporting for CJT calibration?

- Proposals

  - Proposal 1 (Apple, Samsung, MTK): Define measurement restriction for CJT calibration reporting based on UE support of simultaneousRxDataSSB-DiffNumerology and SCS of SSB and CSI-RS.

    - Proposal 1a (Samsung): If CJTC measurement collides with other L1 measurement, specify the measurement restriction for FR1:

      - For FR1, when the CSI-RS for CJTC-Dd or CJTC-F measurement is in the same OFDM symbol as SSB for RLM, BFD, CBD or L1-RSRP measurement, UE is not required to receive CSI-RS for L1-RSRP measurement in the PRBs that overlap with an SSB.

      - For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement has same SCS than CSI-RS for CJTC-Dd or CJTC-F measurement, the UE shall be able to perform CSI-RS measurement without restrictions.

      - For FR1, when the SSB for RLM, BFD, CBD or L1-RSRP measurement has different SCS than CSI-RS for CJTC-Dd or CJTC-F measurement, the UE shall be able to perform CSI-RS measurement with restrictions according to its capabilities:

        - If the UE supports simultaneousRxDataSSB-DiffNumerology the UE shall be able to perform CSI-RS measurement without restrictions.

        - If the UE does not support simultaneousRxDataSSB-DiffNumerology, UE is required to measure one of but not both CSI-RS for CJTC-Dd or CJTC-F measurement and SSB. Longer measurement period for CSI-RS based CJTC-Dd or CJTC-F measurement is expected, and no requirements are defined.

      - For FR1, when the CSI-RS for CJTC-Dd or CJTC-F measurement is in the same OFDM symbol as another CSI-RS for RLM, BFD, CBD or L1-RSRP measurement, UE shall be able to measure the CSI-RS for L1-RSRP measurement without any restriction.

    - Proposal 1b (MTK): Reuse the legacy CSI-RS based L1 measurement restriction as baseline in FR1 for delay and frequency offset measurement.

      - Measurement restriction does apply,

        - If UE does not support simultaneousRxDataSSB-DiffNumerology and

        - SSB for RLM, BFD, CBD or L1-RSRP with different SCS as CSI-RS (TRS) for delay and frequency offset measurement and they are fully or partially overlapped on symbols in FR1.

      - Measurement restriction does not apply,

        - If UE support simultaneousRxDataSSB-DiffNumerology or

        - SSB for RLM, BFD, CBD or L1-RSRP with the same SCS as CSI-RS (TRS)for delay and frequency offset measurement in FR1.

  - Proposal 2 (E///): RAN4 not to define additional measurement restrictions for the CJTC reporting.

- Recommended WF

  - Check the need for measurement restrictions for the cases mentioned by P1a and P1b

Adhoc Discussion:

Nokia: For measurement restriction, we are fine to P1b.

Samsung: We are fine to reuse legacy principle

Agreement:

    - Reuse the legacy CSI-RS based L1 measurement restriction as baseline in FR1 for delay and frequency offset measurement.

      - Measurement restriction does apply,

        - If UE does not support simultaneousRxDataSSB-DiffNumerology and

        - SSB for RLM, BFD, CBD or L1-RSRP with different SCS as CSI-RS (TRS) for delay and frequency offset measurement and they are fully or partially overlapped on symbols in FR1.

      - Measurement restriction does not apply,

        - If UE support simultaneousRxDataSSB-DiffNumerology or

        - SSB for RLM, BFD, CBD or L1-RSRP with the same SCS as CSI-RS (TRS)for delay and frequency offset measurement in FR1.

Topic #2: RRM requirements for UE-initiated/event-driven beam management

- Issue 1-1: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19?

- Proposals

  - Option 1: (Qualcomm, Apple)

    - Do not define requirements for the window-based approach whether the event has occurred M times.

    - Option 1b: (Apple)

      - Add note longer delay is expected if eventInstanceCount-r19 >1 is configured

  - Option 2: (China Telecom, Nokia, LGE, Xiaomi, Samsung, CMCC, Huawei, ZTE, Ericsson, MediaTek)

    - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

- Recommended WF

  - First decide from two options:

    - Option 1: No requirements for with time window based with eventInstanceCount-r19>1

    - Option 2: Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

  - Suggest to agree on Option 2 based on majority views

Adhoc Discussion:

QC: How we can define such req. RAN1 defined different timer for the measurement beams. If RAN1 define a fixed window length but shorter than RAN4 measurement, how to define such requirements.

E///: it applied both FR1 and FR2. RAN1 decided multiple values for the window length.

Apple: The event count is counted by UE to check whether M times is met at UE side. What is the intention of this requirements?

Samsung: The purpose of this requirements is to verify xxx in the core requirement. Like the UE shall not measure the RS occasion very far from the time should be triggered. If no core requirements, the performance cannot be ensured

Apple: Tt is something we did for TTT. It still seems no difference from TTT.

Agreement:

- Issue 1-1-a: How to specify requirements for “with time window based with eventInstanceCount-r19” if option 2 in Issue 1-1-1 is supported?

- Proposals

  - Option 1: (Nokia)

    - For the window/timer based approach, when the timer is started for the first time and if the reporting condition for a beam is met before the expiry of the first timer, reporting delay is the time from when the condition is met over the air until the first PUCCH transmission.

    - If the reporting condition becomes met after restarting the timer for the beam to be reported, the reporting delay is the time from the start of a subsequent timer during which reporting condition is met until the first PUCCH transmission.

    - The reporting delay for the window-based approach shall not be larger than the evaluation delay + the time until the first PUCCH transmission after M event occasions have occurred within one timer.

  - Option 2: (LGE)

    - event triggered reporting delay can be reused

  - Option 3: (Samsung)

    - The measurement period is at least be larger than max{eventInstanceCount-r19 * measurement period (for basic), Ttime_window}

  - Option 4: (Huawei)

    - When the conditions is met over the air and it is kept fulfilled, UE shall be able to trigger the event no later than TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel, and no earlier than TL1-RSRP_event triggered_measurement_Period * (C-1) + Tfirst UL channel, where C is the configured counter.

    - For time window-based event triggering, one event instance shall be evaluated based on latest results from both new beam and current beam. i.e. one result shall not be used to evaluate whether event instance is met for multiple instance.

  - Option 5: (Ericsson)

    - After the first event entering condition is met, UE starts the timer with length T_window and after the T_window+ TFirst UL channel, UE should send the indication on the first UL channel if the event has been met M number of times in the T_window

  - Option 6: (MediaTek)

    - Alt1. Measurement reporting delay can re-use the same principle as L3 event triggered reporting defined in legacy.

    - Alt2. 	Measurement reporting delay is TL1_measurement period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where,

      - TL1_measurement period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS.

      - C: total number of measurement period used for event triggered condition is met (At least event instances is greater than or equal to an M (event instances threshold) configured by NW.

      - Ttime_window: time window is configured by NW.

      - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st  PUCCH.

- Recommended WF

  - If we can get consensus on Issue 1-1, to discuss:

    - Option 1&3&4&6 have similar idea but with different details,

    - Option 2 reuse the same as no window based.

    - Option 5 use fixed T_window length

- Issue 1-4: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)?

- Proposals

  - Option 1: (Qualcomm, China Telecom, Apple, Samsung, Huawei, Ericsson)

    - For event trigger L1 reporting, do not define such capability in parallel per category like L3.

  - Option 2: (Nokia)

    - Introduce the number of supported UEIBM intra- and inter-frequency L1 events for SSB and CSI-RS based reporting on top of the existing L3 events in section 9.1.4.

  - Option 3: (MediaTek)

    - For intra-frequency, numbers of event = X, where X is candidate value of [3 or 6 or 9].

  - Option 4: (CMCC)

    - if it is agreed to define capabilities for support of L1 event triggering and reporting criteria, it is proposed to define a separate capability for L1 measurement, not prefer to mix L1 measurement reporting capability with legacy L3 measurement reporting capability.

- Recommended WF

  - Choose one of option 1, 2, 3 in this meeting. Suggest to agree on Option 1 based on majority views.

Adhoc Discussion:

Agreement: