3GPP TSG-RAN WG4 Meeting # 116											R4-2512130

Bengaluru, India, August 25th – 29th, 2025

Title: 	WF on RRM requirements for NR_MIMO_Ph5_Part1

Agenda Item:	7.20.1

Source: 	Samsung

Document for:	Approval

<Topic 1: RRM core requirements for UE-initiated/event-driven beam management >

Issue 1-1-a: How to specify requirements for “with time window based with eventInstanceCount-r19” if option 2 in Issue 1-1-1 is supported?

Agreement:

For the window-based solution:

if eventDetectionTimeWindowLength-r19 is provided, the event triggered measurement reporting delay shall be no larger than (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel

- eventInstanceCount-r19 is configured by NW

- L is the number of measurement instances (TL1-meas_basic) that do not satisfy the event condition, counted after the event counter is started and before the required number of events (eventInstanceCount-r19) are detected within the specified time window (eventDetectionTimeWindowLength-r19).

- TL1-meas_basic is the L1 measurement period which agreed in without window-based soltuion, in details, it is the maximum of L1-RSRP measurement periods on the beams corresponding to the event

The above requriement is applicable when eventDetectionTimeWindowLength-r19 is not less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic , and no requirement when eventDetectionTimeWindowLength-r19 is less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic.

Send the LS capturing the RAN4 agreements to RAN1/2, and the action requested from RAN4 to RAN1/2 is to take RAN4 agreements into account.

Issue 1-2: Whether to specify new unified TCI switching delay requirements?

Agreement:

- This issue is closed.

Issue 1-3: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time?

Agreement:

- No need to capture this scenario in RAN4 requirements.

Issue 1-4: Capabilities for Support of Event Triggering and Reporting Criteria (similar Capabilities in 38.133 9.1.4)?

Agreement:

- For event trigger L1 reporting, do not define such capability in parallel per category like L3.

Issue 1-6 (New): Whether RAN4 to specify minimum interval requirement between two event instances?

Agreement:

- RAN4 not to specify minimum interval requirement between two event instances.

Issue 1-8: UE-initiated/event-driven beam management for FR1

Agreement:

- Agree for FR1 support and check the update in draft CRs R4-2509236 & R4-2509485 directly.

Issue 1-9: L1-RSRP measurement period for event-7

Agreement:

- Check the update in draft CRs R4-2509236 & R4-2509485 directly, further revision in dCRs.

< Topic #2: RRM performance requirements for Rel-19 MIMO >

Sub-topic 2-1: Performance requirements for UE-initiated/event-driven beam management

Issue 2-1-1: Test scope

<Way forward >

|  | QC | Apple | Samsung | CMCC | Huawei | ZTE | Nokia | Ericsson | MediaTek |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| High level: Define TCs to verify UEIBM | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Whether to cover Event-2/Event-1/Event-7? | Event-2: Y Event-1/Event-7: FFS | Event-2: Y Event-1/Event-7: No | Event-2: Y Event-1: limited TC Event-7: No | All | Event-2 FFS on others | All | Event-2 | Event-2&7 | Event-2 |
| Whether to cover Mode A and Mode B? |  | Either A or B | Both | Both |  | Both | Mode A&B as Test configurations. Can be skip depends on UE capability | Mode A&B as Test configurations. Can be skip depends on UE capability |  |
| Event-trigger for Intra-cell BM for SSB | Y | N | Y | Y | Y | Y | Y | Y | Y |
| Event-trigger for Intra-cell BM for CSI-RS | Y | Y | Y | Y | Y | Y | Y | Y | Y |
| Event-trigger for Inter-cell BM for SSB | Y | Y | Y | Y | FFS | Y | Y | ? | Y |
| Event-trigger for Inter-cell BM for CSI-RS | N | N | N | Y |  | Y | N |  | N |
| Whether to cover no time window and eventInstanceCount-r19 > 1? | N | N | Y | Y |  | Y | Y | Y |  |
| Frequency Range – FR2? |  | Y. No FR1 | Y. FR2 |  |  | Y. FR2 |  |  |  |
| Beam switching delay for TCI states switching | Y Both known and unknown TCI | N |  |  |  | FFS on core req. | FFS on core req. | No MAC CE-based TCI state switching FFS on DCI-based TCI state switching |  |
| L1 measurement reporting delay |  |  |  | Y |  |  |  |  |  |
| CMI |  |  |  |  |  |  | N |  |  |

Issue 2-1-3: Whether to add requirement to verify the performance of CMI?

<Way forward >

- Option 1: (Xiaomi)

  - RAN4 to define requirement to verify the performance of condition met indicator(CMI) in the report.

  - RAN4 to discuss how to define performance metric for event triggered reporting

- Option 2:

  - Others are not precluded

Sub-topic 2-2: Performance requirements for CJTC reporting

Issue 2-2-1: Whether to define reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P?

Agreement:

- RAN4 to add reporting mapping tables for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P.

Issue 2-2-2: Whether to define accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F, CJTC-P?

Agreement:

- RAN4 to define accuracy requirements for CJTC reporting including CJTC-Dd, CJTC-F, further discuss the accuracy and SNR in the performance part.

Issue 2-2-3: Test scope

<Way forward >

| High level: | QC | Apple | Samsung | CMCC | Nokia | Ericsson | MTK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Whether to define TCs to verify CJTC-Dd, measurement period | Y | FFS | Y | Y | Y | Y | Y |
| Whether to define TCs to verify CJTC-f，measurement period | Y | FFS | Y | Y | Y | Y | Y |
| Whether to define TCs to verify CJTC-P， (To be verify?) | N | N | N |  |  |  |  |
| Whether to define TCs to verify scheduling restriction |  |  |  | Y (if core req.) |  |  |  |
| Whether to define TCs to verify CJTC-Dd, CJTC-f，accuracy |  |  |  |  | Y | Y |  |

Sub-topic 2-3: Performance requirements for enhancement for Asymmetric DL sTRP / UL mTRP

Issue 2-3-1: Test scope

<Way forward >

| High level: | QC | Apple | Samsung | CMCC | Huawei | Nokia | MTK |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Whether to define TCs to verify timing requirements? |  | Y | Y | Y | Y | Y |  |
| Whether to define TCs to verify TCI state switching requirements? | Y |  | N |  |  |  |  |
| FR1 and/or FR2? | both |  |  |  |  |  |  |
| Whether to cover Scenario#1 and Scenario#2? | Both for FR1. Scenario#2 for FR2 | Both |  | Both | Scenarios#1 | both | Scenarios#1 |