3GPP TSG-RAN WG4 Meeting # 116											R4- 2512132

Bengaluru, India, August 25th – 29th, 2025

Agenda item:			7.21.1

Source:	Samsung

Title:	Ad-hoc minutes for NR_duplex_evo

Document for:	Approval

Introduction

This Ad-hoc minutes is to cover the discussion in [116][218] NR_duplex_evo for the following issues:

Issue 1-3-2: Measurement restriction (for L1-CLI-RSSI)

Issue 1-2-1: Collision between Method #1 and Method #2

Issue 1-3-1: Measurement period (for L1-CLI-RSSI)

Issue 2-1-1d: Lmax in NR-U approach

Issue 1-1-2: L1 CLI measurement in non-SBFD symbols

Topic #1: Core part of UE-to-UE CLI handling

Issue 1-3-2: Measurement restriction (L1-CLI-RSSI)

- Background: WF from RAN4#114-bis

- Collision between L1-CLI-RSSI measurements and L1/L3 RRM measurements - For Method #1& Method 3: - Measurement requirements do not apply for L1-CLI-RSSI when CLI resources and resources for L1/L3 RRM measurements overlap in time for FR2, FFS for FR1.

- Proposals for collision between L1-CLI-RSSI measurement and L1/L3 RRM measurements in FR1

  - Proposal 1 (CATT, Nokia, CTC, HW, ZTE, E///)

    - Measurement requirements for L1-CLI-RSSI apply by default.

    - Define exception cases when L1-CLI-RSSI would not apply:

      - L1-CLI-RSSI measurement resources overlap with MG (CATT, HW, ZTE, E///)

      - L1-CLI-RSSI measurement resources overlap with SSB for L1/L3 measurement and configured with a different SCS (CATT, HW, ZTE)

      - L1-CLI-RSSI measurement resources overlap with L3 measurement resources (Nokia)

  - Proposal 2 (MTK)

    - Leave it to the UE implementation which measurement to perform.

- Recommended WF for collision between L1 CLI measurements and L1/L3 RRM measurements in FR1

  - Suggest to agree on P1 based on majority view.

  - Regarding the detailed exception cases when L1-CLI-RSSI would not apply, use the following as starting point, and discuss whether there are other cases where L1-CLI-RSSI measurement cannot be measured in parallel with L1/L3 measurements.

    - L1-CLI-RSSI measurement resources overlap with MG

    - L1-CLI-RSSI measurement resources overlap with SSB for L1/L3 measurement and configured with a different SCS

Adhoc Discussion:

QC: We prefer P2 and to leave it to UE implementation. The reason is UE has the knowledge of which one shall be measure.

HW: We understanding this issue has already discussed for multiple meetings. We agreed for FR2. And for FR1 it is the similar approach as legacy.

HW: This L1 RSSI measurement is based on DL timing. It has already relax according to Rel-16 requirements.

Samsung: we support P1.

MTK: This is the L1 measurement. 
E///: the DL timing is used in RSSI. And for Rel-16 legacy. So we support the proposal from moderator’s suggestion here.

Nokia: we support the P1 to define the measurement restrictions for some cases.

Apple: What we should do it to add some condition in the CR?

MTK: We understand what we did in L3. If there is measurement restriction, it is also left to the UE implementation.

E///: ask from clarification from MTK: if they are colliding, whether UE drop one measurement, what is the exact of UE behaviour.

MTK: my point is just agreeing like UE implementation, UE can choice from L1 or L3 measurement.

E///: the understanding it UE will

CMCC: we support option 1. The FR1 cases, we don’t see the issue from UE side to use the same DL timing to handle different measurement.

ZTE: we support option 1. Same as CMCC. If go option 1, UE pick one of the measurements. It is unclear.

MTK: question for Option 1. For companies cannot see the issue in Option 1, if go option2. What is the issue by UE implantation?

E///: whether the measurement can be parallel done in some cases as normal, in such cases, UE can measure both of them.

Nokia: based on the majority view, UE can perform the measurement parallel in some cases.

Keep further offline discussion below

Option 1: (CATT, Nokia, CTC, HW, ZTE, E///, Samsung, CMCC, ZTE)

  - Regarding the detailed exception cases when L1-CLI-RSSI would not apply, use the following as starting point, and discuss whether there are other cases where L1-CLI-RSSI measurement cannot be measured in parallel with L1/L3 measurements.

    - L1-CLI-RSSI measurement resources overlap with MG

    - L1-CLI-RSSI measurement resources overlap with SSB for L1/L3 measurement and configured with a different SCS

Option 2: (MTK, QC)

  - Left to UE implementation

Agreement:

Issue 1-2-1: Collision between Method #1 and Method #2

- Background: WF from RAN4#115

Agreement - Take into account following RAN1 agreement in RAN1#121, and further discuss the measurement restriction between Method #1 and Method #2. Agreement A UE does not expect to be configured with L1-CLI-RSSI measurements resources within an UL subband and L1-SRS-RSRP measurement resources on the same symbol..

- Proposals

  - Proposal 1 (CATT):

    - Prioritize L1-CLI-RSSI measurement.

  - Proposal 2 (Nokia):

    - Prioritize L1-SRS-RSRP measurement.

  - Proposal 3 (Samsung, HW, ZTE, MTK):

    - UE does not expect to be configured with L1-CLI-RSSI measurements resources within DL subband(s) and L1-SRS-RSRP measurement resources on the same symbol.

  - Proposal 4 (CMCC):

    - Leave it to UE implementation whether to measure L1-SRS-RSRP or L1-CLI-RSSI.

  - Proposal 5 (E///):

    - Up to RAN1 discussion.

- Recommended WF

  - Suggest to agree on P3 based on slight majority view.

Adhoc Discussion:

Adhoc Chair: Any concern for P3?

Nokia: It is risky to agree here if RAN1 also discuss this one. So, we want to wait for RAN1 progress.

E///: RAN1 agreed the M#1 and M3 cannot be configured on the same symbol. RAN1 has released this issue and conclude for M2 and M3. So, we think RAN1 should continue to discuss it. So, we prefer to wait for RAN1 decision. Otherwise, it is strange RAN1 decided for some cases but no for other cases.

OPPO: we don’t observe the extension discussion in RAN1 for this. RAN4 can make the decision for the requirements.

CATT: We proposed UE should measure CLI firstly. but we are also fine for P3.

Apple. In RAN1, the M#2 and M3 cannot be configured.

QC: we don’t see the impact the of the CR. If go some proposal like P3, it will have no impact in the CR.

ZTE: Ran1 addressed the case, RAN1 made some decision for cases in last meeting. There is no technical issue. How about to try P3 and add a note if RAN1 have conflict agreement as P3 here in RAN4.

Agreement:

    - UE does not expect to be configured with L1-CLI-RSSI measurements resources within DL subband(s) and L1-SRS-RSRP measurement resources on the same symbol.

    - Note: this can be revised if RAN1 have different agreement for this Collision between Method#1 and Method#2.

Issue 1-3-1: Measurement period

Note: P4 from QC R4-2511638 is not captured for open issue discussion, as the L1-SRS-RSRP measurement period has been concluded in RAN4#114-bis.

- Proposals

  - Proposal 1 (Samsung, vivo, E///, MTK, QC):

    - Based on 1 sample regardless of whether timeRestrictionForChannelMeasurement is configured.

  - Proposal 2 (Huawei, ZTE):

    - Based on 1 sample if timeRestrictionForChannelMeasurement is configured;

    - Based on 3 samples if timeRestrictionForChannelMeasurement is not configured.

- Recommended WF

  - Suggest to agree on P1 based on majority view.

Adhoc Discussion:

Nokia: normal approach is to follow RAN1 design. We support P2.

E///: 2 samples in Proposal 2 is the new value for compromised value?

Adhoc Chair: It is just typo. 3 samples as legacy in P2.

CMCC: according to the RAN1 LS, we support the legacy requirements according to the timeRestrictionForChannelMeasurement

E///: for Rel-16, the legacy it is no timeRestrictionForChannelMeasurement. It is just one sample. No strong view for the measurement period.

Apple: We achieved the accuracy based on one sample.

Adhoc Chair: in legacy L1-RSRP, the accuracy is based on one sample no matter 1 sample or 3 samples in core. Try P2 to check any objection.

Agreement:

    - Based on 1 sample if timeRestrictionForChannelMeasurement is configured;

    - Based on 3 samples if timeRestrictionForChannelMeasurement is not configured.

Issue 1-1-2: L1 CLI measurement in non-SBFD symbols

- Background: agreements from RAN1#121

Agreement For a CSI report associated with periodic/semi-persistent SRS-RSRP measurement resources or CLI-RSSI measurement resources, the valid symbol type for CSI derivation configured in CSI-ReportConfig is applicable for L1 UE-to-UE CLI measurement and reporting if SBFD symbols are configured for the UE - UE shall perform L1 UE-to-UE CLI measurement based on the valid symbol type configured in CSI-ReportConfig The valid symbol type does not apply for a UE if SBFD symbols are not configured for the UE - UE shall perform L1 UE-to-UE CLI measurement based on the configuration provided in CSI-ResourceConfig indicated by CSI-ReportConfig FG 60-1 is not the prerequisite of FG 60-8 and FG60-9

- Proposals

  - Proposal 1 (CATT, Nokia, Samsung, vivo, HW, ZTE, MTK, QC):

    - Define requirements for L1 UE-to-UE CLI measurements based on valid symbol type.

      - RAN4 to assume the valid symbol type is configured in CSI-ReportConfig for L1 UE-to-UE CLI measurements, and UE only measures the L1 CLI measurement resources based on the valid symbol type (Nokia)

      - Measurement period should be extended due to resources on invalid symbol type, and NR-U approach is re-used (vivo, HW, QC)

  - Proposal 2 (E///):

    - RAN4 close this issue due to L1-CLI measurement only applicable when SBFD symbols are configured for the UE.

- Recommended WF

  - Suggest to agree on P1 based on majority view.

  - The 2 sub-bullets under P1 are not conflicting. Further discuss in RAN4#116 whether we can re-use NR-U approach for defining measurement period extension due to invalid symbol type.

From Monday GTW online discussion Agreement: - Define requirements for L1 UE-to-UE CLI measurements based on valid symbol type. Discussion: - Further discuss in RAN4#116 whether we can re-use NR-U approach for defining measurement period extension due to invalid symbol type. - Reuse: HW, Samsung, CATT, ZTE, QC, Apple, vivo, OPPO - RAN4 to assume the valid symbol type is configured in CSI-ReportConfig for L1 UE-to-UE CLI measurements, and UE only measures the L1 CLI measurement resources based on the valid symbol type (Nokia) - Measurement period should be extended due to resources on invalid symbol type, and NR-U approach is re-used (vivo, HW, QC) - Not Reuse: E/// E///: not agree to reuse the NR-U approach.

Topic #2: Core part of SBFD operation impacts

Issue 2-1-1d: Lmax in NR-U approach

From Monday GTW online discussion: Issue 2-1-1d: Lmax in NR-U approach - Proposals for L1-RSRP and L1-SINR - Proposal 1 (Nokia, Samsung, HW, vivo): - Do not define Lmax values in the extension of the L1-RSRP and L1-SINR measurement periods. - Proposal 2 (CTC): - Re-use Lmax related requirements defined in R16 NR-U. - Proposal 3 (E///): - For L1-RSRP, - RSRP_0 ‘not valid’ can be used for report when either L>Lmax or X≥M, where X is the number of consecutive invalid measurement occasion - Re-use Lmax value defined in R16 NR-U, i.e. {7 5 3} depending on max(TDRX,TCSI-RS) - For L1-SINR with CSI-RS based CMR and no dedicated IMR, - UE shall report “SINR not valid” when L>Lmax or X ≥ M, and new SINR report value shall be introduced from RAN4 to allow UE report SINR invalid - Lmax value is same as for L1-RSRP. - For L1-SINR with SSB based CMR and dedicated IMR, - Restrict the IMR configuration only applicable to CSI-IM resources. - No measurement period needs to be extended. - For L1-SINR with CSI-RS based CMR and dedicated IMR, - Restrict the IMR configuration only applicable to NZP-CSI-RS resources. - UE shall report “SINR not valid” when L>Lmax or X ≥ M, and new SINR report value shall be introduced from RAN4 to allow UE report SINR invalid - Lmax value is same as for L1-RSRP. - Proposals for RLM/BFD/CBD - Proposal 1 (Samsung): - If RAN4 to define Lmax for RLM/BFD/CBD, RAN4 to define one fixed value for regardless of DRX and SMTC. - Proposal 2 (CTC, HW): - Re-use Lmax related requirements defined in R16 NR-U. - Proposal 3 (E///): - For RLM In-sync, the layer 1 or the UE shall not send any in-sync indication for the cell to higher layers when L>Lmax or X ≥ M. Lmax = {10 8 5} depending on max(TDRX,TCSI-RS). - For RLM Out-of-sync, Lmax = {20 15 10} depending on max(TDRX,TCSI-RS). - For BFD, Lmax = {10 8 5} depending on max(TDRX,TCSI-RS). - For CBD, Lmax = {7 5 3} depending on max(TDRX,TCSI-RS), i.e. same as in R16 NR-U. - Recommended WF - For L1-RSRP and L1-SINR, suggest to discuss whether UE behaviour of reporting “invalid value” is needed when L>Lmax, and if so, agree to re-use the Lmax values from R16 NR-U. - For RLM and BFD, suggest to agree to re-use the UE behaviour from R16 NR-U, i.e. UE shall not send any in-sync or out-of-sync indication to higher layers when L>Lmax, and further discuss the value for Lmax. - For CBD, suggest to agree to re-use the UE behaviour from R16 NR-U, i.e. UE shall assume no new candidate beams are found for this evaluation period when L>Lmax, and further discuss the value for Lmax. Discussion: - For L1-RSRP and L1-SINR, suggest to discuss whether UE behaviour of reporting “invalid value” is needed when L>Lmax, and if so, agree to re-use the Lmax values from R16 NR-U. - Option 1: Do not define Lmax values in the extension of the L1-RSRP and L1-SINR measurement periods. (Samsung, QC, Nokia, MTK, Apple, OPPO) - Option 2: UE behaviour of reporting “invalid value” is needed when L>Lmax (E///) - Option 3: UE behavior of stopping and restarting the measurement is needed when L>Lmax (Compromise proposed by moderator, E///) Samsung: no need to have Lmax. Different scenarios for duplex and NR-U. Network knows better for duplex. QC: agree with Samsung. Nokia: can further discuss MTK: Support option 1. Network has the related knowledge. Apple: Support option 1. Avoid new option to complicate the feature. Discussion: - For RLM and BFD, - Option 1: Do not define Lmax values. (QC, Apple, MTK, Nokia, OPPO) - Option 2: agree to re-use the UE behaviour from R16 NR-U, i.e. UE shall not send any in-sync or out-of-sync indication to higher layers when L>Lmax, and further discuss the value for Lmax. (E///) QC: no need Lmax. Avoid to discuss the values as well. Apple: agree with QC. Everything is under network control. MTK: Agree with QC, Apple. Discussion: - For CBD, - Option 1: Do not define Lmax values. (Apple, Nokia, QC, MTK) - Option 2: agree to re-use the UE behaviour from R16 NR-U, i.e. UE shall assume no new candidate beams are found for this evaluation period when L>Lmax, and further discuss the value for Lmax. (E///)