# 3GPP TSG-RAN WG4 Meeting #116                                                      R4-2512134

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:			7.22.1

Source:	Moderator (CMCC)

Title:	WF on RRM for Ambient_IoT_Solutions

Document for:	Approval

Topic #1: RRM core requirements (7.22.4)

Sub-topic 1-1: A-IoT random access procedure

Issue 1-1-1: A-IoT random access procedure

WayForward:

- Directly discuss the CR

Sub-topic 1-2: timing

Issue 1-2-1: Device transmit timing

Agreement:

- For D2R transmission, the device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last transition edge of PRDCH of the corresponding R2D transmission to the starting time of D2R transmission. The reference point for the device transmission timing is TR→D after the end of R2D transmission intended for the device as define in 7.1.2 TS 38.291

- TR→D is

- For CBRA MSG1 X=1: TR→D is Toffset1 after the end of corresponding R2D transmission

- For CBRA MSG1 X=2: TR→D is Toffset1+ Toffset2 after the end of corresponding R2D transmission

- For CBRA MSG3: TR→D is Toffset3 after the end of corresponding R2D transmission

- For CFRA first D2R: TR→D is Toffset3 after the end of corresponding R2D transmission

- For D2R transmission except for Msg1 and Msg3: TR→D is Toffset4 after the end of corresponding R2D transmission

                - Note: considering that TR→D is in use in RAN1 spec (TS 38.291), above details about TR→Dwill not be captured in TS38.191

Issue 1-2-2: The value for TD2R_min

Agreement:

- The value of TD2R_min is max {3* TD2R_chip, 10 us}, where TD2R_chip is the chip length of D2R transmission

Issue 1-2-3: Whether to define accuracy requiements for TD2R_min

Agreement:

- Not define accuracy requirements on TD2R_min.

Topic #2: RRM performance requirements (7.22.7)

Issue 2-1-1: Test scenario

Agreement:

- The random access and D2R transmit timing requirements will be tested in RAN4 test cases, FFS whether dedicated RRM test or combined with RF/demod test.

Issue 2-1-2: Test configuration

- Proposals from companies:

- Option 1 (CATT):

    - RAN4 to discuss the following test parameters as a starting point:

- CW transmitting power: [33] dBm.

- CW2D distance: [5, 10, 20] m.

- R2D SCS = 15 kHz only.

- R2D transmission bandwidth configuration: [1 PRB, 2 PRB, 3 PRB, 4 PRB].

- Option 2 (OPPO):

    - Set X=1 for MSG 1 transmission in the test case.

    - Consider the multiple Toffset1 values as different configuration in the test case:

- Configuration 1: Toffset1 = 1333.4us, reference D2R chip length is 133.33us

- Configuration 2: Toffset1 = 666.6us, reference D2R chip length is 33.33us

- Configuration 3: Toffset1 = 132.32us, reference D2R chip length is 4.17us

- Configuration 4: Toffset1 = 33.33us, reference D2R chip length is 2.08us, R2D chip length is 11.11us

- Option 3 (Xiaomi):

    - The common testing configuration for Rel19 AIoT RRM test cases can be defined as:

- SCS = 15KHz

- BW =[180kHz]

- Device 1 only

- Option 4 (CMCC):

    - For RACH, the transmit timing is verified by the device D2R transmission, and the test equipment will verify that the timing of the device is within a period of the end the corresponding R2D transmission triggering random access, where the period is related with Toffset1 /Toffset2/Toffset3/Toffset4 and timing error due to SFO.