# 3GPP TSG-RAN WG4 Meeting #116	R4-2512136

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:			7.23.1

Source:	Ericsson

Title:	WF on RRM requirements for Netw_Energy_NR_enh_Part1 v1

Document for:	Approval

# Topic #1: On-demand SSB(OD-SSB) requirements (AI 7.23.3.1)

### Sub-topic 1-1: OD-SSB based deactivated SCell measurement

Issue 1-1-1: OD-SSB in FMW collided with MG

Agreement

  - UE is allowed to skip the OD-SSB measurement if OD-SSB in FMW collides with MG occasion.

    - ‘Kp’ should be introduced to extend the FMW length.

Issue 1-1-2: UE’s behaviour when OD-SSB further activation/reconfiguration within T2

Agreement

  - The UE should follow legacy deactivated SCell measurements after T2.

Issue 1-1-3: FMW clarification

Agreement

  - The actual FMW measurement duration equals min(Treport , Tidentify) from the end of uncertain time.

    - Treport is the first OD-SSB based L3 measurement reporting

    - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow.

      - A detected cell: OD-SSB based measurement period

      - A newly detectable cell: OD-SSB based cell identification period

    - Note: The UE can stop doing fast measurements after time Tidentify even if no measurement report has been sent to the network.

Issue 1-1-4: Requirement for RAN1 Alt Time-C2

Agreement

- RAN4 to define the requirement for Alt Time-C2 only based on the OD-SSB periodicity.

  - It applies to deactivated SCell L3 measurement

  - It applies to L1 measurement after the SCell activation [command].

  - No scheduling restriction is expected for AO-SSB outside the SMTC in Alt Time-C2.

Note: QC to check and confirm.

### Sub-topic 1-2: OD-SSB based L3 measurement

Issue 1-2-1: OD-SSB based L3 measurement when SCell is activated(scenario 3B)

Agreement (on Thursday)

- For SCell measurement, UE shall follow the OD-SSB specific SMTC when OD-SSB is activated; For neighbor cell measurement, UE follows legacy SMTC regardless of status of OD-SSB.

### Sub-topic 1-3: OD-SSB based SCell activation

Issue 1-3-1: The known cell condition in Case 2

Agreement

- The known cell condition agreement for Case 2 Alt Time-C1 can be applied to Alt Time-C2.

Issue 1-3-2: Multiple OD-SSBs based deactivated SCells measurement

Agreement

- In deactivated SCell stage, when the time periods from OD-SSB activation to deactivation among multiple OD-SSBs’ frequency layers are non-overlapping.

  - the requirement of deactivated SCells measurement with multiple OD-SSBs SCell shall be the same as the deactivated SCells measurement requirement for single OD-SSB.

- Otherwise, NO requirements

Issue 1-3-3: Multiple OD-SSBs based SCells activation

Agreement

- RAN4 to define the multiple OD-SSB based SCells activation delay requirements.

  - Reuse the principle of legacy multiple SCells activation requirements

  - For case1, at least one of the OD-SSB activation command comes together with SCell activation command

  - No requirements if two or more SCells’ OD-SSBs transmission are overlapped in the deactivated SCell stage.

### Sub-topic 1-4: General

Issue 1-4-1: Additional Processing time for OD-SSB parameter update

Agreement

- RAN4 should apply the same UE capability with 2ms and 5 ms for OD-SSB parameter updating.

  - Note 1: Based on the RAN1 spec, the additional processing time is only applied to L1 and L3 measurement transition period after the SCell is activated.

Issue 1-4-2: OD-SSB deactivation timeline

Agreement

- RAN4 not to introduce additional processing time for OD-SSB deactivation.

Issue 1-4-3: Additional Processing time for joint OD-SSB and SCell activation

Agreement

  - Not introduce an UE capability for OD-SSB processing with the same values as for MAC CE based OD-SSB activation for the deactivated SCell.

  - The issue of “additional Processing time for joint OD-SSB and SCell activation” is closed.

  - Not define test case for verifying OD-SSB and Scell activation command come together and the OD-SSB is transmitted immediately after T_HARQ+3ms when the Scell is a known cell.

Issue 1-4-4: OD-SSB based L3 reporting

Agreement

The discussion of the issue shall not impact the RAN4 core part completion of the WI.

Issue 1-4-5: OD-SSB interruption spec. impact

Agreement

- Discuss in the CR directly.

Issue 1-4-6: OD-SSB interruption requirements

Agreement

- Discuss in the CR directly.

Issue 1-4-7: Cell identification

Agreement

- If UE has already measured OD-SSB for Tidentify_intra_without_index_ODSSB or Tidentify_intra_with_index_ODSSB and the OD-SSB deactivated period is within 5s and the SCell is detectable when OD-SSB is transmitted, then the OD-SSB is activated again and the SCell remain detectable with the same spatial reception parameter and same frequency, UE shall skip the OD-SSB identification component provided the timing to that cell has not changed more than  3200/2µ Tc.

- Note: Option 2 can be discussed in maintenance stage.

Issue 1-4-8: L1 measurement – collision with CSI-RS

Agreement

- When OD-SSB colliding with CSI-RS, existing L1 measurement restriction can apply.

### Sub-topic 1-5: OD-SSB and AO-SSB with different frequencies(Alt scenario 3)

Issue 1-5-1: Requirement for Alt scenario 3 (OD-SSB and AO-SSB are within different frequencies)

Agreement

- RAN4 NOT to define the requirements when OD-SSB and AO-SSB are within different frequencies.

  - Issue 1-5-1(Requirement for Alt scenario 3), Issue 1-5-2(Neighbour cell measurement for scenario 3), Issue 1-5-3(Serving cell measurement for scenario 3), Issue 1-5-4(SCell activation for scenario 3), Issue 1-5-5(Measurement reporting for scenario 3), Issue 1-5-6(AO-SSB and OD-SSB CA combination) are closed

Issue 1-5-2: Neighbour cell measurement, if agreed to define, for Alt scenario 3

Agreement

- Close the issue.

Issue 1-5-3: Serving cell measurement, if agreed to define, for Alt scenario 3

Agreement

- Close the issue.

Issue 1-5-4: SCell activation, if agreed to define, for Alt scenario 3

Agreement

- Close the issue.

Issue 1-5-5: Measurement reporting, if agreed to define, for Alt scenario 3

Agreement

- Close the issue.

Issue 1-5-6: AO-SSB and OD-SSB CA combination

Agreement

- Close the issue.

# Topic #2: General (AI 7.23.2)

### Sub-topic 2-1: UE capability

Agreement

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 52-2 | Additional processing time for OD-SSB activation and parameter update | Indicates an additional processing time of 2ms in addition to T_min = m+3Nslotsubframe,μ+1 for reception of on-demand SSB bursts from the time when UE receives OD-SSB activation or parameter update MAC CE command. | At least one among RAN1 feature of 61-3, 61-4, 61-4a | Yes | n/a | Additional processing time of 5ms in addition to T_min = m+3Nslotsubframe,μ+1 for reception of on-demand SSB bursts from the time when UE receives OD-SSB activation or parameter update MAC CE command. | Per band | n/a | n/a | n/a | Note 1: Default value of additional processing time is 5 ms | Optional with capability signaling |

# Topic #3: NES enh. performance requirements (AI 7.23.4)

### Sub-topic 3-1: OD-SSB performance part

Sub-issue 1: Test scope

Issue 3-1-1: Scenario - General

- RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement.

- No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB.

- RAN4 should not define fast SCell activation test cases for OD-SSB.

- RAN4 only choose one test case for case 2 to justify the effective periodicity of OD-SSB and AO-SSB.

- RAN4 to further discuss the test case scope for scenario #2, scenario #2A and scenario #3B as follow.

  - Option 1:

    - In Case 1, RAN4 should define the test cases for scenario #2, scenario #2A.

    - In Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B.

    - The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell.

  - Option 2: RAN4 not to define test cases for scenario #3B.

  - Option 3: RAN4 to define the OD-SSB based test cases for

    - Case #1 and Scenario #2

    - Case #2 and Scenario #2A

- RAN4 to further discuss the test case scope for Case 2 as follow.

  - FFS: RAN4 to define the test cases to verify the UE behaviour in Alt. Time C1, Alt. Time C2 and scenario 3 if RAN4 agrees to define the RRM requirements for all these scenarios.

Issue 3-1-2: Scenario – Scell activation scope

- Proposals

  - Proposal 1: RAN4 should focus on SCell activation for case 1 and case 2 Alt Time-C1 and multiple SCells activation. The other SCell activation scenarios should have lower priority.

  - Proposal 2: RAN4 only define the test cases for single SCell activation.

- Recommended WF

  - Further discussion

Issue 3-1-3: Scenario – known/unknown Scell activation

- Proposal 1: RAN4 to define the OD-SSB based SCell activation test case for both known SCell and unknown SCell.

- Proposal 2: RAN4 to define the OD-SSB based SCell activation test case for known SCell only.

- Proposal 3: RAN4 to define the OD-SSB based test case for both Case 1 and Case 2. One in known scenario and the other in unknown scenario such as:

  - Case 1 in unknown scenario

  - Case 2 in known scenario

Sub-issue 2: Test case list

Issue 3-1-4: Test case list

Agreement

RAN4 to at least define the following OD-SSB test cases.

- SCell activation in Case 1

- Intra-frequency measurement(deactivated SCell) in Case 1

- L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

RAN4 to further discuss whether the following OD-SSB test cases can be agreed.

- SCell activation in Case 2 Alt Time C-1

- OD-SSB based multiple SCell activation

- OD-SSB based direct SCell Activation at SCell addition in Case 1

- OD-SSB based direct SCell Activation at SCell addition in Case 2

- OD-SSB based PUCCH SCell Activation in Case 1

- OD-SSB based PUCCH SCell Activation in Case 2

- Fast SCell Activation with OD-SSB;

- OD-SSB based SCell Activation with the L3 reporting during activation

- Intra-frequency measurement(deactivated SCell) in Case 2

- Intra-frequency measurement in Case 1

- Intra-frequency measurement in Case 2

- Interruption at OD-SSB activation/deactivation

- Interruptions during measurements on deactivated NR SCC

Sub-issue 3: Test configuration

Issue 3-1-5: General principle

RAN4 to discuss whether the following principles to design the OD-SSB test case can be agreed.

- Both FR1 and FR2

- Test OD-SSB periodicities with 5ms, 20ms. 10ms

- Only test non-DRX mode

- The test without SSB time index detection

- Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

- SMTC and measCycleSCell as 40ms/160ms

- Only NCD-SSB for OD-SSB test case.

Issue 3-1-6: OD-SSB configuration

- Proposals

  - In Case 1, RAN4 to define the OD-SSB based SCell activation test case with 5ms periodicity.

  - In Case 2, RAN4 to define the OD-SSB based SCell activation test case as follow.

    - OD-SSB(5ms) is overlapping with AO-SSB(20ms)

    - The union of OD-SSB(20ms) and AO-SSB(20ms) meets the legacy SSB periodicity(10ms)

  - RAN4 to introduce new NCD-SSB RMC configuration for 5ms, 10ms and 20ms periodicity.

### Sub-topic 3-2: OD-SIB1 performance part

Issue 3-2-1: Test case

- Background

In RAN4 #115 meeting, Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Agreement: - For core requirements: Clarify that the paging interruption could be longer when the target Cell is NES Cell - Define test case with longer TSI-NR with reasonable value in performance part.

- Proposals

  - Option 1: RAN4 not to define OD-SIB1 RRM performance test case.

  - Option 2: RAN4 to define OD-SIB1 RRM performance test case.

### Sub-topic 3-3: SSB adaptation performance part

Issue 3-3-1: Test case list

Agreement

RAN4 to at least define the following SSB adaptation test case.

  - L3 intra-frequency measurement

RAN4 to discuss whether the following SSB adaptation test case list can be defined.

- L1-RSRP

- L1-SINR

- CBD

Issue 3-3-2: General principle

RAN4 to discuss whether the following principles to design the SSB adaptation test case can be agreed.

- RAN4 to define the test cases for SSB adaptation in RRC_CONNECT mode only.

- RAN4 to define test case for SMTC and SSB periodicity change based on DCI 2-9.

- RAN4 to reuse legacy trigger events for measurement report after SSB adaptation.

- Both FR1 and FR2.

- RAN4 not to define TC for SSB adaptation for L1/L3 transition requirement.

- Only test non-DRX mode.

- without SSB time index detection.