3GPP TSG-RAN WG4 Meeting #116	                                                	         R4-2512140

# Bangalore, India, 25 – 29 August, 2025

Agenda item:			7.25.1

Source:	Apple

Title:		Ad-hoc minutes for NR_Mob_Ph4

Document for:	Information

# Topic #1: General aspects and work plan

#### Issue 1-1: RAN4 UE feature list for event triggered L1 reporting

Candidate options:

- Proposal 1: For FG 52-1, the number of CSI-RS resources for L1-RSRP measurement within a slot is not counted in FG 2-24 (vivo)

- Proposal 2: For FG 52-1 and FG 52-2, the number of CSI-RS resources is counted across all candidate cells. (vivo)

- Proposal 3: FG 52-3 is revised to Skip SSB based L1-RSRP measurement for neighboring cell CSI-RS-based L1-RSRP measurement. (vivo)

Recommended WF:

- Discuss above proposals.

Candidate options in RAN4#116:

- Option 1: vivo

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 52. NR_Mob_Ph4 | 52-1 | Number of CSI-RS resources for L1-RSRP measurement within a slot | - The max number of CSI-RS resources for L1-RSRP measurement that UE can measure within a slot [across candidate cells] for L1-RSRP measurement | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | No | There is no limitation on the number of CSI-RS resources for L1 measurement within a slot. | Per BC | No | No | N/A | Candidate value: {1,2,3,4,5,6,7,8,16,32, 48,64} - [Note: FFS It is also counted in FG 2-24] | Optional with capability signaling |
|  | 52-2 | Number of total CSI-RS resources to be measured | - The max number of total CSI-RS resources of serving cells and [candidate cells] [across all CCs] for L1 measurement. Both LTM candidate cell(s) and serving cells for CSI-RS based L1-RSRP measurement are counted, regardless of whether CSI-RS based L1-RSRP measurement on serving cell is configured in [LTM-CSI-ResourceConfig-r18] or not. | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | No | There is no limitation on the total number of CSI-RS resources of serving cells and neighbouring cells across all CCs for L1 measurement. | Per BC | No | No | N/A | Candidate values: [{2,4,8,12,16,32,64}] - Note: the value should be not smaller than UE capability of beamManagementSSB-CSI-RS (Component 2 of 2-24) | Optional with capability signaling |
|  | 52-3 | Skip SSB based L1-RSRP measurement for [neighboring or candidate] cell CSI-RS-based L1-RSRP measurement | - Indicates support for skipping SSB-based L1-RSRP during [neighboring or candidate] cell CSI-RS-based L1-RSRP measurement. | [RAN1 feature of CSI-RS based L1 RSRP measurement on LTM candidate cell] | Yes | N/A | SSB based L1-RSRP cannot skipped while performing CSI-RS based L1-RSRP | Per UE | no | FR2-1 only | N/A |  | Optional with capability signalling |

Recommended WF:

- Agree on the following updates.

Discussion:

- Proposal 1: For FG 52-1, the number of CSI-RS resources for L1-RSRP measurement within a slot is not counted in FG 2-24 (vivo)

MTK/Apple/QC: we need to count it in 2-24. Otherwise we need to introduce another UE capability.

Vivo: we have different understanding. 2-24 is already there for serving cell UE capability. But here we are discussing about capability for neighbor cell.

- Proposal 2: For FG 52-1 and FG 52-2, the number of CSI-RS resources is counted across all candidate cells. (vivo)

- Proposal 3: FG 52-3 is revised to Skip SSB based L1-RSRP measurement for neighboring cell CSI-RS-based L1-RSRP measurement. (vivo)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 52. NR_Mob_Ph4 | 52-1 | Number of CSI-RS resources for L1-RSRP measurement within a slot | - The max number of CSI-RS resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for L1-RSRP measurement | 63-1 | Yes | No | There is no limitation on the number of CSI-RS resources for L1 measurement within a slot. | Per BC | No | No | N/A | Candidate value: {1,2,3,4,5,6,7,8,16,32, 48,64} - Note: It is also counted in FG 2-24 | Optional with capability signaling |
|  | 52-2 | Number of total CSI-RS resources to be measured | - The max number of total CSI-RS resources of serving cells and candidate cells across all CCs for L1 measurement. Both LTM candidate cell(s) and serving cells for CSI-RS based L1-RSRP measurement are counted, regardless of whether CSI-RS based L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18]or not. | 63-1 | Yes | No | There is no limitation on the total number of CSI-RS resources of serving cells and neighbouring cells across all CCs for L1 measurement. | Per BC | No | No | N/A | Candidate values: [{2,4,8,12,16,32,64}] - Note: the value should be not smaller than UE capability of beamManagementSSB-CSI-RS (Component 2 of 2-24) | Optional with capability signaling |
|  | 52-3 | Skip SSB based L1-RSRP measurement for [neighboring or candidate] cell CSI-RS-based L1-RSRP measurement | - Indicates support for skipping SSB-based L1-RSRP during [neighboring or candidate] cell CSI-RS-based L1-RSRP measurement. | 63-1 | Yes | N/A | SSB based L1-RSRP cannot skipped while performing CSI-RS based L1-RSRP | Per UE | no | FR2-1 only | N/A |  | Optional with capability signalling |

# Topic #2: event triggered L1 measurement reporting

#### Issue 2-1: applicability of L1-RSRP reporting delay

Candidate options:

- Option 1: capture the following conditions in test case but not in core requirements. (CATT, Apple, [CMCC?], vivo, [HW?], [E///?])

  - No indicate TCI change during the evaluation period.

  - No configuration change w.r.t measurement RS during the evaluation period

  - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

Recommended WF:

- Agree on option 1.

Ad-hoc agreement:

- Capture the following conditions in test case but not in core requirements.

  - No indicate TCI change during the evaluation period.

  - No configuration change w.r.t measurement RS during the evaluation period

  - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

#### Issue 2-2: whether to introduce/update requirements for reporting criteria per measurement category

Candidate options:

- Option 1: define new requirements of Capabilities for Support of Event Triggering and Reporting Criteria UE capability for LTM (CATT, Nokia, CTC, OPPO, Xiaomi, MTK, CMCC, vivo, ZTE, E///)

  - Option 1a: Introduce reported UE capabilities for supported number of events triggered L1 measurement reporting. The candidate values are {4, 8, 12, 16}. (MTK)

  - Option 1b: vivo

    - RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM by considering intra-frequency measurement and inter-frequency measurement. For CSI-RS based L1 measurements for LTM, consider 1 layer of measurement for the reporting capability

    - Reporting criteria for LTM event triggered reporting is 4 per frequency layer.

    - The total number of reporting criteria for LTM event triggered reporting is 4*n, where n is the total number frequency layers for L1-RSRP measurement, including intra-frequency layer(s), inter-frequency layer(s) for SSB-based L1-RSRP measurement and 1 frequency layer for CSI-RS based L1-RSRP measurement if configured.

  - Option 1c: E///

    - RAN4 to agree 72 as the L1 measurement reporting criteria for a UE supporting event triggered measurement reporting

- Option 2: extend existing Ecat to support LTM intra- and inter-frequency L1 events for SSB and CSI-RS based reporting on top of the existing L3 events in section 9.1.4. (Nokia, OPPO)

Recommended WF:

- Agree on option 1 and further discuss option 1a, 1b and 1c.

Session Chair: encourage MTK, vivo and E/// to have offline discussion and come up with a unified proposal based on the direction of option 1.

Discussion:

Vivo: how about 40 in total?

MTK: need to discuss the definition of the reporting criteria. Whether it should be defined per cell, per carrier, or in total.

QC: important thing is how many event reporting can be configured.

Baseline for further discussion:

Reporting criteria for LTM event triggered reporting is [5/9] * 39-3-3 (Number of total cells UE can measure).

#### Issue 2-3: Impact on Cell switch and TCI activation requirements

Agreement in RAN4#114bis:

- For FR1:

  - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command,

    - Case 1: leaving condition of the event is configured and corresponding condition is not met,

      - the related cell switch delay and TCI state activation requirements are applicable

    - Case 2: leaving condition of the event is NOT configured, FFS

- For FR2: FFS

Candidate options:

- Option 1: In FR1, cell switch and TCI state requirements are applicable if the UE is configured with event-triggered L1 reporting and on leave reporting, and the UE has sent an event-triggered L1 report for the target cell and has not sent on leave report before reception of the cell switch command. (Nokia, Apple, Xiaomi, MTK, vivo, HW, QC)

  - Option 1a: Nokia

    - In FR1, cell switch and TCI state requirements are applicable if the UE is configured with event-triggered L1 reporting without on leave reporting, and the UE has sent an event-triggered L1 report for the target cell within [5 seconds] before reception of the cell switch command

    - Either limit the scenario where the UE is required to perform L1 measurements on the target cell according to measurement requirements, or limit proposals 1 and 2 to FR1 only.

  - Option 1b: add the following additional condition. (Apple)

    - Number of active TCI does not exceed that UE can support

- Option 2: extend to cover FR2 as well (vivo, QC)

- Option 3: RAN4 to revisit the definition of TCI state known condition to accommodate event triggered LTM reporting. Potential spec change is highlighted in the following: (E///)

  - “The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. Or UE has sent at least one L3-RSRP report with beam index corresponding to the candidate cell on which the L1 event is configured”.

Recommended WF:

- Agree on option 1 and further discussion option 2 and 3.

Online Agreement:

- In FR1, cell switch and TCI state requirements are applicable if the UE is configured with event-triggered L1 reporting and on leave reporting, and the UE has sent an event-triggered L1 report for the target cell and has not sent on leave report before reception of the cell switch command.

Adhoc agreement:

- Not to extend above agreements to FR2.

#### Issue 2-4: Impact on DL synchronization

Candidate options:

- Option 1: Add the following additional condition for Tfirst-RS and TRS-proc = 0: The target TCI state is on the active TCI state list, and the UE has sent an event-triggered L1 report associated to the target TCI state no longer than 160 ms before reception of the cell switch command for both FR1 and FR2. (Nokia)

Recommended WF:

- Discuss the option.

#### Issue 2-5: CR

- Please provide comments on CR below:

| TDoc | Title | Source | Comments |
| --- | --- | --- | --- |
| R4-2509161 | DraftCR for LTM L1 event-triggered reporting | Nokia |  |
| R4-2509699 | DraftCR on capabilities for Support of Event Triggering and Reporting Criteria | OPPO |  |
| R4-2510175 | DraftCR on event-triggered reporting for CSI-RS based L1 measurement | CMCC |  |
| R4-2510782 | Draft CR for event-triggered reporting on LTM | Ericsson |  |

# Topic #3: CSI-RS based L1 measurement

#### Issue 3-1: whether skipping step 2 for a serving cell when included in LTM candidate cell

- Proposals

  - Proposal 1: For CSI-RS based L1-RSRP measurement in FR2-1, step 2 can not be skipped when the candidate cell is a serving cell, e.g., PSCell or SCell. (MediaTek, CATT, China Telecom, Apple, Xiaomi, CMCC, vivo, Huawei, Ericsson)

    - Proposal 1a: Clarify in the measurement requirement applicability that the requirement is not applicable when a FR2 serving cell is included in the candidate cell for LTM. [Apple, OPPO]

  - Proposal 2: Prefer to keep step2 about SSB-based L1 measurements and reports on serving cell when it is in the candidate cell for LTM.   (ZTE)

  - Proposal 3: When CSI-RS resources are configured for an FR2 serving cell measurements, the applicability of requirements is determined as follows: (Qualcomm)

    - If the CSI-RS resources are configured in an LTM candidate configuration (LTM-Candidate), the same requirements as those defined for CSI-RS-based FR2 LTM candidate cell measurements apply.

    - Otherwise, the requirements defined for the FR2 serving cell apply.

  - Proposal 4: Nokia

    - If serving cell is also current LTM candidate cell, the UE shall perform CSI-RS beam management measurements according to the beam management configuration and requirements

    - At least step 2 can be skipped if the serving cell is current candidate cell.

    - Step 3 can be skipped if the candidate cell is current serving cell as UE can use or report CSI-RS beam management measurements.

- Recommended WF

  - Discuss if proposal 1 can be agreed.

1st round online outcome:

Option 1 direction: (Apple, MTK, HW, ZTE, vivo, CATT, E///)

For CSI-RS based L1-RSRP measurement in FR2-1, the new requirement skipping step 2 does not apply for serving cell.

Concern: QC

Option 2: (MTK, HW, Apple, QC)

remove the enhanced UE capability on step 2 skipping.

Concern: E///

#### Issue 3-2: Measurement restriction

- Proposals

  - Proposal 1: In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5 ms. (ZTE, Apple, Ericsson, Xiaomi, CATT)

  - Proposal 2: There are at most two separate windows within every P where P is min{CSI-RS periodicity of cells on one intra-frequency layer} when the periodicity of the configured CSI-RS resources is larger than 20 ms. (Huawei)

    - the starting point of the first window is [( the slot boundary of serving cell) mod P], where the corresponding slot contains the configured L1 CSI-RS resource of the serving cell.

    - The starting point of the second window if configured is determined by an offset of half of P in slots with regards to the starting point of the first window.

  - Proposal 3: When CSI-RS based L1 measurements cause scheduling restriction, all CSI-RS resources on one intra-frequency layer are configured within up to two separate windows where each window is up to 5 ms and the two windows, if configured, are separated by 20ms. (vivo, Apple)

  - Proposal 4: Nokia

    - CSI-RS measurement window is 5ms

    - Discuss further whether the number of windows for CSI-RS LTM candidate cell measurements can be increased

    - For UE CSI-RS measurement applicability: Periodicity of CSI-RS resource is between 10ms and 640ms

    - The starting point of the first window is the slot boundary of the serving cell, where the corresponding slot contains the configured CSI-RS resource.

    - Location of the next window is the slot boundary of the next LTM candidate cell CSI-RS in time domain (defined by CSI-RS configuration)

- Recommended WF

  - Discuss if proposal 1 can be agreed.

Online Agreement:

Further discuss and down-select one proposal from P1 and P2.

  - Proposal 1: In FR2, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5 ms. (ZTE, Apple, Ericsson, Xiaomi, CATT)

  - Proposal 2: There are at most two separate windows within every P where P is min{CSI-RS periodicity of cells on one intra-frequency layer} when the periodicity of the configured CSI-RS resources is larger than 20 ms. (HW, vivo)

    - the starting point of the first window is [( the slot boundary of serving cell) mod P], where the corresponding slot contains the configured L1 CSI-RS resource of the serving cell.

    - The starting point of the second window if configured is determined by an offset of half of P in slots with regards to the starting point of the first window.

#### Issue 3-3: how to handle the case when CSI-RS based L1 RSRP measurement on neighbour cell when colliding with serving cell CSI-RS for L1-RSRP measurement?

- Proposals

  - Proposal 1: If the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain, RAN4 to introduce measurement restriction as legacy. [CATT, OPPO, Xiaomi, Huawei]

  - Proposal 2: Either option 1 with scheduling restriction or option 2 with sharing factor of 2:1 between serving cell and neighbor cell could be considered. (Apple)

  - Proposal 3: For intra-f CSI-RS based L1-RSRP measurement on neighbour cell in FR2, introduce a sharing factor when it is overlapped or adjacent with CSI-RS configured for L1-RSRP measurement on other cells. (MediaTek, ZTE)

  - Proposal 4: For FR2 CSI-RS L1 measurement requirement, if the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain, introduce sharing factor similar to PL1_sharing. (Ericsson)

  - Proposal 5: If CSI-RS resources configured for L1 RSRP measurement for a serving cell and a neighboring cell with a different PCID than the serving cell collide in the time domain at the slot level, the scaling factor defined in Rel-18 for SSB-based FR2 LTM candidate cell applies. (Qualcomm)

  - Proposal 6: Nokia

    - If LTM candidate cell is a current serving cell, no collision handling needed.

    - Sharing factor can be introduced only for the scenario where LTM candidate cell is neighbouring cell. No sharing factor is defined for the scenario where serving cell is LTM candidate cell.

- Recommended WF

  - Discuss if proposal 1 can be agreed.

Online Agreement:

- If the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain, RAN4 to introduce measurement restriction as legacy.

#### Issue 3-4: how to handle the case when the CSI-RS based L1-RSRP measurement on neighbour cell in FR2-1 overlap with CSI-RS configured for RLM/BFD/CBD?

- Proposals

  - Proposal 1: For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, define measurement restriction when it is overlapped with CSI-RS configured for RLM/BFD/CBD. (MediaTek)

- Recommended WF

  - Discuss if proposal 1 can be agreed.

Online Agreement:

  - For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, define measurement restriction when it is overlapped with CSI-RS configured for RLM/BFD/CBD.

#### Issue 3-5: Early CSI acquisition

- Proposals

  - Proposal 1: Prefer not to define RAN4 requirements for early CSI acquisition. (CATT, Apple, China Telecom, Xiaomi, Huawei)

  - Proposal 2: (MediaTek)

    - For the optional feature (CSI acquisition before LTM CSC MAC-CE reception), not to define any RRM requirements in R19.

    - For the baseline (CSI acquisition after LTM CSC MAC-CE reception), additional CSI acquisition delay should not be added in cell switch delay requirements to avoid delaying the whole cell switch procedure. Not to define new cell switch delay requirements for the baseline.

  - Proposal 3: it is proposed to define timeline for early CSI acquisition during LTM cell switch. (ZTE)

  - Proposal 4: the timeline for early CSI acquisition during LTM cell switch is proposed as: (CMCC)

TCSI-acquisition = Tcmd + TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TCSI_reporting

TCSI_reporting is the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources

  - Proposal 5: RAN4 to define RRM requirements for target cell early CSI-acquisition. (vivo)

    - A component of TCSI_reporting is added to LTM cell switch delay if the target cell early CSI-acquisition is configured.

    - TCSI_reporting is defined as the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331.

  - Proposal 6: For the baseline CSI acquisition, RAN4 to define early CSI acquisition delay (TCSI_reporting_LTM) as the delay uncertainty (in ms) in acquiring the first available CSI reporting resources after end of max (TFirst_available_CSI + TCSI_processing, DLTM). (Ericsson)

  - Proposal 7: (Qualcomm)

    - For the baseline CSI acquisition (DL CSI acquisition after LTM CSC MAC-CE reception), RAN4 to not define requirements.

    - Proposal 4: For optional CSI acquisition (i.e., downlink CSI acquisition before LTM CSC MAC-CE reception), the same requirements and applicability rules as those for CSI-RS-based L1-RSRP measurements on candidate cells apply—meaning no separate requirements for early CSI acquisition.

  - Proposal 8: (Nokia):

    - For baseline CSI acquisition, confirm one of the following:

      - CSI-RS measurements for CSI acquisition have impact on cell switch delay

      - When configured, UE shall measure and process CSI-RS(s) for CSI acquisition without impact to cell switch delay when the network transmits CSI-RSs during DLTM (i.e., no impact to cell switch delay)

- Recommended WF

  - Discuss if proposal 1 can be agreed.

Online Agreement:

- No consensus on defining new RAN4 requirements for early CSI acquisition.

  - Clarification on the applicability of the existing requirement is not precluded if consensus on the exact clarification can be reached.

Adhoc agreement:

It is RAN4 common understanding that for baseline early CSI acquisition,

- If fine T/F tracking is included in cell switch delay, UE shall start CSI acquisition at least after UE finishes fine T/F tracking of target cell.

- Otherwise, UE shall start CSI acquisition at least after Tprocessing.

No need to capture above agreement in RAN4 spec.

#### Issue 3-6: how to capture requirement applicability v.s. UE capability?

- Proposals

  - Proposal 1: Clarify in the requiremetn applicability that RAN4 The requirements in the clause 9.14a are applicable to FR1 and FR2-1 for LTM when the configured CSI-RS based L1 RSRP measurement for LTM candidate cell doesn’t exceeding UE measurement capability. (Apple)

- Recommended WF

  - Discuss if proposal 1 can be agreed.

#### Issue 3-7: whether send LS to RAN1/2 on intra-frequency/inter-frequency definition for L1 measurement?

- Proposals

  - Proposal 1: It is proposed to send LS to RAN1/2 about RAN4 agreement on intra-frequency and inter-frequency definition for CSI-RS based L1 measurement. (Apple)

- Recommended WF

  - Agree on proposal 1.

Online Agreement:

Send LS to RAN1/2 about RAN4 agreement on intra-frequency and inter-frequency definition for CSI-RS based L1 measurement.

#### Issue 3-8: others

- Proposals

  - Proposal 1: Discuss whether the CSI-RS frequency location (w.r.t active / configured BWP) has impact on CSI-RS acquisition procedure before / after the cell switch command

  - Proposal 2: Discuss how many L1-RSRP CSI-RS measurements UE is capable of performing during each 5ms measurement window.

- Recommended WF

  - Discuss the proposals.

#### Issue 3-9: CR

- Please provide comments on CR below:

| T-doc number | Title | Company | Please provide Comments in the column below |
| --- | --- | --- | --- |
| R4-2509461 | R19 mobility CR for measurement restriction in serving cell | Apple |  |
| R4-2509467 | (NR_Mob_Ph4-Core) draft CR on CSI-RS based L1 measurement requirement | Apple |  |
| R4-2510101 | Draft CR on introduction of CSI-RS L1-RSRP measurement | MediaTek Inc. |  |
| R4-2510331 | Draft CR on measurement reporting requirements for CSI-RS based LTM | vivo |  |
| R4-2510454 | draftCR on measurement restriction for CSI-RS based L1-RSRP | ZTE Corporation, Sanechips |  |
| R4-2510574 | Draft CR on scheduling availability of UE during CSI-RS based L1-RSRP measurement | Huawei, HiSilicon |  |
| R4-2510784 | Draft CR for CSI-RS ased L1 measurements | Ericsson |  |
| R4-2511246 | DraftCR to 38.133 on LTM CSI-RS applicability | Nokia |  |

# Topic #4: conditional LTM

Issue 1-1-2: TCSI-RS_SFN_intra

- Candidate solutions:

- Proposal 1: TCSI-RS_SFN_intra refers to the corresponding requirements in R16 CSI-RS based L3 measurement. It is assumed that deriveSSB-IndexFromCell is always enabled for FR1 TDD and FR2. (Huawei, QC, OPPO, China Telecom)

      - OPPO: The associated SSB is needed to be configured.

- Proposal 2: If the CSI-RS periodicity configured for an FR1 LTM candidate cell exceeds the radio frame length, and deriveSSB-IndexFromCell is not enabled for the cell, TCSI-RS_SFN_intra should be included in the SSB-based L3 measurement requirement (a prerequisite for CSI-RS-based candidate cell L1-RSRP measurement). (QC)

- Proposal 3: Remove the TCSI-RS_SFN_intra (E///, Apple, MTK)

      - Note: reuse the applicaible condition in Rel-18 LTM

- Recommended WF

- Discuss is needed.

E///: Support P3 since the target cell is known

MTK: reuse the applicable condition in Rel-18 LTM, which is the target cell SFN is the same as the serving cell.

E///: reuse the same applicability rule.

QC: No ambiguity issue for Rel-18. Always PRACH in Rel-18. The assumption is different for Rel19 C-LTM. We can compromise to option 3 if we state in CR with some applicable conditions. We can copy the additional condition from RAN1 spec.

Agreement:

Remove the TCSI-RS_SFN_intra and reuse the applicable condition in Rel-18 LTM. In addition, further discuss whether additional condition is needed.

Discuss in CR directly.

Issue 1-2-3: CLTM requirements for candidate cells without active TCI states in FR2

- Candidate solutions:

- Proposal 1: RAN4 to revisit the agreement made in RAN4#114bis regarding CLTM requirements for candidate cells without active TCI states in FR2, following this update: (Nokia)

    - L1 measurement behavior, which was introduced in R18, is not changed. CLTM measurement time requirements (Tmeasure) do not apply for the candidate cells without active TCI states after TCI state activation for at least one candidate cell in FR2. Other CLTM requirements (cell switch interruption, etc.) apply.

Apple: Try to understand the intention.

Nokia: for the scenario of UE continue measurement, condition is met, and UE triggers switching, the existing interruption requirement applies.

QC: Understood. But we wonder whether we can verify this.

HW: It is up to UE implementation. Whether UE perform reporting depend on UE whether evaluate the reporting condition. What is the spec impact of this issue?

Nokia: Related to previous agreement. According to RAN2 spec, UE continues measurement. In RAN4, it is up to UE implementation whether to continue. We want to clarify that UE shall not trigger unknown interruption.

Apple: Technically agree with Nokia. But how to capture in RAN4 spec.

QC: Can be captured in the spec.

Agreement:

- Capture the following in the WF:

    - As starting point: L1 measurement behavior, which was introduced in R18, is not changed. CLTM measurement time requirements (Tmeasure) do not apply for the candidate cells without active TCI states after TCI state activation for at least one candidate cell in FR2. Other CLTM requirements (cell switch interruption, etc.) apply.

      - Further work on the wording offline

- FFS whether and how to capture in the CR.

Issue 1-2-2: The values and the conditions for Tfirst-RS = 0 and TRS-proc = 0

- Candidate solutions:

- Proposal 2: If UE has received a PDCCH order and consequently a TA MAC-CE, Tfirst-RS = 0 and TRS-proc = 0 based on successful preamble transmission and while the TA timer is running. (Nokia)

    - Concern: MTK

- Agreement:

  - Further check the status of P2 in the 2nd round.

Adhoc agreement:

This issue can be discussed during maintenance phase without impact on core part completion.

# Topic #4: performance

## Event-triggered L1 reporting

#### Issue 4-1-1: test case list for event-triggered L1 reporting

Candidate options:

- Option 1: Nokia

  - FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps)

  - FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps)

  - FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps)

  - At least 1 test case for event-triggered periodic reporting.

- Option 2: CATT

  - SSB based test cases:

    - TC for LTM Intra-frequency event triggered L1 measurement reporting tests without gap under non-DRX

    - TC for LTM Inter-frequency event triggered L1 measurement reporting tests with gaps under non-DRX

    - TC for LTM Intra-frequency event triggered L1 measurement reporting tests without gap under non-DRX with SSB index reading

  - CSI-RS based test case:

    - Only new TC for LTM Intra-frequency event triggered reporting tests without gap under non-DRX should be defined.

- Option 3: Apple, [vivo?]

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1

  - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR1

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2

  - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR2

- Option 4: CTC

  - LTM Intra-frequency SSB/CSI-RS based event triggered L1-RSRP measurement reporting tests without gap in FR1/FR2.

  - LTM Inter-frequency SSB based event triggered L1-RSRP measurement reporting tests with/without gap in FR1/FR2.

  - LTM Inter-frequency CSI-RS based event triggered L1-RSRP measurement reporting tests with gap in FR1/FR2.

- Option 5: OPPO

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR1

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR1

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR2

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR2

- Option 6: MTK

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2

- Option 7: CMCC

  - Both FR1 and FR2-1 are considered.

  - For SSB based event triggered L1 measurement reporting, intra-frequency measurement, inter-frequency measurement without gap, gap-based inter-frequency measurement need to be considered.

  - For CSI-RS based event triggered L1 measurement reporting, measurement without gap need to be considered.

- Option 8: ZTE

  - FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps)

  - FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps)

  - FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps)

- Option 9: HW

  - FR1 and FR2 LTM Intra-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement)

  - FR1 and FR2 LTM Intra-frequency event triggered reporting tests without gap (CSI-RS based L1-RSRP measurement)

  - FR1 and FR2 LTM Inter-frequency event triggered reporting tests with gap (SSB based L1-RSRP measurement)

  - FR1 and FR2 LTM Inter-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement)

- Option 10: E///

| Intra-f L1-RSRP measurement for LTM | - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |
| --- | --- |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated |
| Inter-f L1-RSRP measurement with MG | - Inter-f L1-RSRP measurement with MG in FR1 - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement with MG in FR2 - 2 neighbor cells |
| Inter-f L1-RSRP measurement without gap for LTM | - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated |

Recommended WF:

- Discuss the options.

For SSB based L1 RSRP measurement reporting:

- Approach 1: duplicate all R18 test cases and add event triggered reporting. UE which can pass R19 test cases can skip R18 test cases. (E///, Apple)

- Approach 2: keep R18 test cases, develop necessary new R19 test to verify reporting behavior. (MTK, HW)

For CSI-RS based L1 RSRP measurement reporting:

- Introduce test cases to verify 1) CSI-RS based L1-RSRP measurement and 2) CSI-RS based L1-RSRP measurement event triggered reporting.

- For UE supporting event triggered reporting, UE only needs to pass 2)

- For UE not supporting event triggered reporting, UE only needs to pass 1)

#### Issue 4-1-2: test configuration for event-triggered L1 reporting

Candidate options:

- Proposal 1: On high level, the test cases for event-triggered L1 reporting can have two cells (serving and candidate), and the signal level of one of these cells is to be changed during the test so that the reporting condition becomes met. UE shall send the report within the specified delay after the condition becomes met. (Nokia)

  - Use LTM3 as the reporting event in the test cases.

- Proposal 2: it is proposed to define test cases for LTM event 2/3/4/5. (CMCC)

- Proposal 3: In event triggered reporting tests for CSI-RS L1 RSRP, “3-step” measurement procedure shall be verified. (HW)

- Proposal 4: At least Event LTM3 and Event LTM4 be included in the RRM testcase. (E///)

Recommended WF:

- Agree on proposal 1. Discuss other proposals.

Adhoc agreement:

- On high level, the test cases for event-triggered L1 reporting can have two cells (serving and candidate), and the signal level of one of these cells is to be changed during the test so that the reporting condition becomes met. UE shall send the report within the specified delay after the condition becomes met.

## CSI-RS based L1 measurement

#### Issue 4-2-1: CSI-RS based L1 measurement accuracy

Candidate options:

- Option 1: CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell. (Nokia, Apple, OPPO, MTK, CMCC, vivo, HW, E///)

  - Option 1a: CATT

    - CSI-RS based L1-RSRP accuracy requirements defined in clauses 10.1.19.2 and 10.1.20.2 can be a baseline for CSI-RS based L1-RSRP requirements for neighbor cell.

    - RAN4 to define new clauses for CSI-RS based L1-RSRP requirements for neighbor cell in clauses 10.1.19D, 10.1.19E, 10.1.20A and 10.1.20B as R18 SSB based L1-RSRP requirements for neighbor cell.

  - Option 1b: Apple

| Accuracy requirements | Detail |
| --- | --- |
| Intra-frequency L1-RSRP accuracy requirements | 10.1.19D.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 10.1.19D.2.1 Absolute Accuracy 10.1.19D.2.2 Relative Accuracy |
|  | 10.1.20A.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR2 10.1.20A.2.1 Absolute Accuracy 10.1.20A.2.2 Absolute Accuracy |

  - Option 1c:

    - Reuse legacy accuracy requirements of CSI-RS based L1 measurements for serving cell under the following conditions:

    - The bandwidth of CSI-RS is 48 PRBs and the density is 3.

    - The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements.

    - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

    - Specify absolute and relative intra-frequency L1-RSRP accuracy requirements for FR1 and FR2

    - Reuse the legacy L1 report mapping specified in clause 10.1.6

Recommended WF:

- CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell when the following conditions are met:

  - Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

  - Conditions for L1-RSRP measurements are fulfilled according to annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

  - The bandwidth of CSI-RS is 48 PRBs and the density is 3.

  - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

- Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell.

- New requirements are defined in the following sections

| Accuracy requirements | Detail |
| --- | --- |
| Intra-frequency L1-RSRP accuracy requirements | 10.1.19D.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 10.1.19D.2.1 Absolute Accuracy 10.1.19D.2.2 Relative Accuracy |
|  | 10.1.20A.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR2 10.1.20A.2.1 Absolute Accuracy 10.1.20A.2.2 Absolute Accuracy |

Adhoc agreement:

- CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell when the following conditions are met:

  - Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

  - Conditions for L1-RSRP measurements are fulfilled according to annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

  - The bandwidth of CSI-RS is 48 PRBs and the density is 3.

  - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

- Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell.

- New requirements are defined in the following sections

| Accuracy requirements | Detail |
| --- | --- |
| Intra-frequency L1-RSRP accuracy requirements | 10.1.19D.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 10.1.19D.2.1 Absolute Accuracy 10.1.19D.2.2 Relative Accuracy |
|  | 10.1.20A.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR2 10.1.20A.2.1 Absolute Accuracy 10.1.20A.2.2 Relative Accuracy |

#### Issue 4-2-2: test case list for CSI-RS based L1 measurement

Candidate options

- Option 1: Nokia

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR1

  - Intra-frequency Semi-persistent CSI-RS L1-RSRP measurements with semi-persistent reporting on FR1

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with SSB-based beam sweeping

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with CSI-RS-based beam sweeping

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with CSI-RS based beam sweeping

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with SSB-based beam sweeping

- Option 2: CATT

  - RAN4 to define the test case for LTM CSI-RS based Intra-frequency L1-RSRP measurement in FR1 and FR2.

- Option 3: Apple, OPPO, [vivo?]

  - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR1

  - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR2

  - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1

  - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2

- Option 4: MTK

  - CSI-RS based L1 RSRP measurement for neighbour cell in FR1

  - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement

  - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement

  - CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1

  - CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2

- Option 5: CMCC

  - For CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for measurement delay. And both the case with step 2 and the case without step 2 are tested.

- Option 6: HW

  - As the CSI-RS based L1-RSRP measurement procedure has been verified by event triggered reporting test case for CSI-RS L1-RSRP, it is no need to specify specific test for CSI-RS based L1-RSRP measurement.

  - In event triggered reporting tests for CSI-RS L1 RSRP, “3-step” measurement procedure shall be verified.

  - FR1 and FR2 LTM Intra-frequency CSI-RS L1-RSRP accuracy test cases for neighbour cell are to be specified.

- Option 7: E///

| Intra-f L1-RSRP measurement for LTM | - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |
| --- | --- |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, |
| Inter-f L1-RSRP measurement without gap for LTM | - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated |

Recommended WF:

- Discuss the options.

#### Issue 4-2-3: test configuration for CSI-RS based L1 measurement - BW

Candidate options:

- Option 1: Test configuration is set to have 48 PRBs within the active BWP. (Nokia)

Recommended WF:

- Agree on option 1.

#### Issue 4-2-4: others

Candidate options:

- Proposal 1: Define test case(s) to verify early CSI acquisition if core requirements are specified. (vivo)

  - Proposal 1a: Define test case(s) to verify early CSI acquisition before the cell switch command, conditionally to core requirement definition. (Nokia)

  - Proposal 1b: Define test case(s) to verify early CSI acquisition after the cell switch command, conditionally to core requirement definition. (Nokia)

- Proposal 2: Discuss whether there is need to consider configuration where CSI-acquisition resource(s) are outside of the active BWP. (Nokia)

Recommended WF:

- Discuss the proposals.

## Conditional LTM

#### Issue 4-3-1: CLTM test scope – frequency range

Candidate options:

- Option 1: (Nokia, Apple, CTC, OPPO, MTK, vivo, ZTE, HW)

  - FR1 to FR1

  - FR2 to FR2

- Option 2: CMCC

  - PCell switch to a neighboring LTM candidate cell

    - FR1 cell to FR1 cell

    - FR1 cell to FR2 cell

    - FR2 cell to FR2 cell

    - FR2 cell to FR1 cell

  - PCell switch to an LTM candidate cell that is a serving SCell in MCG

    - FR1 cell to FR1 cell

    - FR2 cell to FR2 cell

- Option 3: E///

  - FR1 Pcell switch to a FR2 neighboring LTM candidate cell,

  - FR2 Pcell switch to a neighboring FR1 LTM candidate cell,

  - Pcell FR1 switch to an LTM candidate cell that is a FR1 serving SCell in MCG,

  - FR2 PCell switch to an LTM candidate cell that is a FR2 serving SCell in MCG.

Recommended WF:

- Agree on the following frequency range:

  - FR1 to FR1

  - FR2 to FR2

- Other options are not precluded.

Discussion:

CLTM test scope:

    - FR1 cell to FR2 cell

    - FR2 cell to FR1 cell

MTK: we don’t see the need for these two.

Adhoc agreement:

- CLTM test frequency range:

  - FR1 to FR1

  - FR2 to FR2

- Other options are not precluded.

#### Issue 4-3-2: CLTM test scope - intra/inter-frequency

Candidate options:

- Option 1: cover both intra and inter-frequency CLTM. (Nokia, Apple, CTC, vivo, ZTE, HW)

Recommended WF:

- Agree on option 1.

Discussion:

MTK: we only need to test intra-frequency. From cell swich procedure point of view, inter-frequency is similar with intra-frequency.

HW: need time to check.

#### Issue 4-3-3: CLTM test scope – L1/L3 based CLTM

Candidate options:

- Option 1: cover both L1 and L3 based CLTM. (Nokia, CATT, CTC, ZTE, E///)

  - Option 1a: RAN4 to discuss how to verify L1 and L3 measurement based candidate evaluation for CLTM such as whether to reuse partial LTM test procedure. (E///)

- Option 2: only test L1 based CLTM. (OPPO, MTK)

Recommended WF:

- Discuss the options

Nokia: we prefer to test L3 as well.

MTK: we support option 2. It would be same as CHO.

Nokia: we only need to test CLTM with early DL/UL sync. That part is different from CHO.

OPPO: L1 based CLTM can cover L3 CLTM from functionality point of view.

#### Issue 4-3-4: CLTM test scope – RACH-less and RACH-based CLTM

Candidate options:

- Option 1: cover both RACH-less and RACH based CLTM. (Nokia, CATT, CTC, OPPO, vivo, ZTE, HW)

- Option 2: RAN4 to discuss how to verify both RACH based and RACH-less procedure for CLTM such as re-use partial LTM test procedure. (E///)

Recommended WF:

- Discuss the options

E///: we can first agree on option 1, then further discuss test procedure.

MTK: we only need to test RACH based CLTM. RACH-less has been verified in R18 test.

Nokia: there is some difference. If the timer is still running when conditions are met, UE shall trigger RACH-less CLTM.

MTK: but triggering is the same.

#### Issue 4-3-5: CLTM test scope – early DL sync

Candidate options:

- Option 1: cover both CLTM with and without early DL sync. (Nokia)

- Option 2: only test CLTM with early DL sync. Do not test CLTM without early DL sync. (OPPO, MTK)

Recommended WF:

- Discuss the options

#### Issue 4-3-6: CLTM test scope – PCell/PSCell

Candidate options:

- Option 1: cover both PCell swith and PSCell switch. (ZTE)

Recommended WF:

- Discuss the options

#### Issue 4-3-7: subsequent CLTM test scope

Candidate options:

- Option 1: Apple

  - Intra-frequency subsequent CLTM from FR1 to FR1

  - Inter-frequency subsequent CLTM from FR2 to FR2

- Option 1a: MTK

  - subsequent CLTM from FR1 to FR1

  - subsequent CLTM from FR2 to FR2

- Option 2: OPPO

  - (SA) RACH based intra-frequency subsequent CLTM for PCell switch from FR1 to FR1

  - (SA) RACH less inter-frequency subsequent CLTM for PCell switch from FR2 to FR2

  - (NR-DC) RACH based intra-frequency subsequent CLTM for PSCell switch from FR1 to FR2

  - (NR-DC) RACH less inter-frequency subsequent CLTM for PSCell switch from FR2 to FR1

- Option 3: CMCC

  - PCell switch to a neighboring LTM candidate cell

    - FR1 cell to FR1 cell

    - FR1 cell to FR2 cell

    - FR2 cell to FR2 cell

    - FR2 cell to FR1 cell

  - PCell switch to an LTM candidate cell that is a serving SCell in MCG

    - FR1 cell to FR1 cell

    - FR2 cell to FR2 cell

- Option 4: HW

  - RACH based Intra-frequency subsequent conditional PCell switch

  - RACH based Inter-frequency subsequent conditional PCell switch

  - RACH less Intra-frequency subsequent conditional PCell switch

  - RACH less Inter-frequency subsequent conditional PCell switch

Recommended WF:

- Discuss the options

#### Issue 4-3-8: test configuration related proposals

Candidate options:

- Proposal 1: On high level, CLTM test cases to have serving cell and candidate cell, and the UE shall trigger cell switch based on signal level of one of these cells changing so that the cell switch condition becomes met. (Nokia)

- Proposal 2: Ensure in the test cases with early UL and DL synchronization that the UE has valid DL synchronization for the target TCI state and a valid TA at the time of condition becoming met. (Nokia)

- Proposal 3: The legacy test configurations and parameters used in R18 LTM test cases can be a baseline for conditional LTM test cases. (CATT)

Recommended WF:

- Discuss the options

#### Issue 4-3-9: test applicability

Candidate options:

- Proposal 1: for UE which can pass subsequent CLTM, test case for CLTM can be skipped, since subsequent CLTM naturally includes a complete CLTM procedure. (Apple, MTK)

Recommended WF:

- Discuss the options

#### Issue 4-3-10: others

Candidate options:

- Proposal 1: for inter-CU LTM and inter-CU subsequent LTM, it is proposed to discuss whether to define test cases. (CMCC)

- Proposal 2: test both PCell switch and PSCell swtich in (subsequent) CLTM.

- Proposal 3: For (subsequent) CLTM, there is no need to define test cases for PDCCH-order Random Access. (ZTE)

- Proposal 4: verify both SSB based L1 measurement , CSI-RS based L1 measurement triggered CLTM. (E///)

Recommended WF:

- Discuss the options