3GPP TSG-RAN WG4 Meeting #116	                                                	         																			R4-2512141

# Bengaluru, India, 25 – 29 August, 2025

Agenda item:			7.29.1

Source:	Apple

Title:	WF on RRM requirements for NR_Mob_Ph4_Part1

Document for:	approval

# Topic #1: General aspects and work plan

#### Issue 1-1: RAN4 UE feature list for event triggered L1 reporting

Agreement:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 52. NR_Mob_Ph4 | 52-1 | Number of CSI-RS resources for L1-RSRP measurement within a slot | - The max number of CSI-RS resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for L1-RSRP measurement | 63-1 | Yes | No | There is no limitation on the number of CSI-RS resources for L1 measurement within a slot. | Per BC | No | No | N/A | Candidate value: {1,2,3,4,5,6,7,8,16,32, 48,64} - Note: It is also counted in FG 2-24 | Optional with capability signaling |
|  | 52-2 | Number of total CSI-RS resources to be measured | - The max number of total CSI-RS resources of serving cells and candidate cells across all CCs for L1 measurement. Both LTM candidate cell(s) and serving cells for CSI-RS based L1-RSRP measurement are counted, regardless of whether CSI-RS based L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18 or not. | 63-1 | Yes | No | There is no limitation on the total number of CSI-RS resources of serving cells and neighbouring cells across all CCs for L1 measurement. | Per BC | No | No | N/A | Candidate values: {2,4,8,12,16,32,64} - Note: the value should be not smaller than UE capability of beamManagementSSB-CSI-RS (Component 2 of 2-24) | Optional with capability signaling |
|  | 52-3 | Skip SSB based L1-RSRP measurement for candidate cell CSI-RS-based L1-RSRP measurement | - Indicates support for skipping SSB-based L1-RSRP during neighboring cell CSI-RS-based L1-RSRP measurement. - Indicates support for skipping SSB-based L1-RSRP during both neighboring cell and serving cell CSI-RS-based L1-RSRP measurement. | 63-1 | Yes | N/A | For UE doesn’t support neither 1) nor 2): CSI-RS resources from neighbour cell shall be Type-D QCL’ed with the associated SSB for L1 measurement. CSI-RS resources configured for LTM L1-RSRP measurement from serving cell shall be Type-D QCL’ed with SSB for L1-RSRP measurement, or another CSI-RS in resource set configured with repetition ON. For UE support 1) but not 2): CSI-RS resources from neighbour cell do not need to be Type-D QCL’ed with the associated SSB for L1 measurement, but shall be Type-D QCL’ed with the associated SSB for L3 measurement. CSI-RS resources configured for LTM L1-RSRP measurement from serving cell shall be Type-D QCL’ed with SSB for L1-RSRP measurement, or another CSI-RS in resource set configured with repetition ON. For UE support 2) but not 1): CSI-RS resources from neighbour cell do not need to be Type-D QCL’ed with the associated SSB for L1 measurement, but shall be Type-D QCL’ed with the associated SSB for L3 measurement. CSI-RS resources configured for LTM L1-RSRP measurement from serving cell do not need to be Type-D QCL’ed with SSB for L1-RSRP measurement, or another CSI-RS in resource set configured with repetition ON. | Per UE | no | FR2-1 only | N/A | UE can only indicate support of 1) or 2) but not both. | Optional with capability signalling |

# Topic #2: event triggered L1 measurement reporting

#### Issue 2-1: applicability of L1-RSRP reporting delay

Agreement:

- Capture the following conditions in test case but not in core requirements.

  - No indicate TCI change during the evaluation period.

  - No configuration change w.r.t measurement RS during the evaluation period

  - After the condition that would trigger the event takes effect, the condition is assumed to be fixed during the whole L1 measurement period

#### Issue 2-2: whether to introduce/update requirements for reporting criteria per measurement category

Agreement:

- No consensus on the necessity to introduce/update requirements for reporting criteria per measurement category.

- The issue does not impact the WI core part completion, and interested companies can further discuss this issue in maintenance phase based on contribution driven.

#### Issue 2-3: Impact on Cell switch and TCI activation requirements

Agreement:

- In FR1, cell switch and TCI state requirements are applicable if the UE is configured with event-triggered L1 reporting and on leave reporting, and the UE has sent an event-triggered L1 report for the target cell and has not sent on leave report before reception of the cell switch command.

- Not to extend above agreements to FR2.

# Topic #3: CSI-RS based L1 measurement

#### Issue 3-1: whether skipping step 2 for a serving cell when included in LTM candidate cell

Agreement:

- Define UE capability of skipping SSB based L1-RSRP measurement for candidate cell CSI-RS-based L1-RSRP measurement, which include two Components:

1) Indicates support for skipping SSB-based L1-RSRP during neighboring cell CSI-RS-based L1-RSRP measurement.

2) Indicates support for skipping SSB-based L1-RSRP during both neighboring cell and serving cell CSI-RS-based L1-RSRP measurement.

- Not define additional/new RAN4 requirement for #2)

- Legacy requirement for serving cell operation such as RLM, BFD, CBD applies.

#### Issue 3-2: Measurement restriction

Agreement:

- In FR2-1, or in FR1 when CSI-RS based L1 measurement would cause scheduling restriction, all CSI-RS resources within a 40 ms window on one intra-frequency layer should be configured within up to two separate windows, each lasting up to 5 ms. The separation of the two windows is at least 4ms.

#### Issue 3-3: how to handle the case when CSI-RS based L1 RSRP measurement on neighbour cell when colliding with serving cell CSI-RS for L1-RSRP measurement?

Agreement:

- If the CSI-RS resources configured for L1 RSRP measurement in serving cell and neighbour cell are colliding in time domain, RAN4 to introduce measurement restriction as legacy.

#### Issue 3-4: how to handle the case when the CSI-RS based L1-RSRP measurement on neighbour cell in FR2-1 overlap with CSI-RS configured for RLM/BFD/CBD?

Agreement:

- For intra-f CSI-RS based L1-RSRP measurement on neighbor cell in FR2, define measurement restriction when it is overlapped with CSI-RS configured for RLM/BFD/CBD.

#### Issue 3-5: Early CSI acquisition

Agreement:

- No consensus on defining new RAN4 requirements for early CSI acquisition.

  - Clarification on the applicability of the existing requirement is not precluded if consensus on the exact clarification can be reached.

- It is RAN4 common understanding that for baseline early CSI acquisition,

  - If fine T/F tracking is included in cell switch delay, UE shall start CSI acquisition at least after UE finishes fine T/F tracking of target cell.

  - Otherwise, UE shall start CSI acquisition at least after Tprocessing.

- No need to capture above agreement in RAN4 spec.

#### Issue 3-7: whether send LS to RAN1/2 on intra-frequency/inter-frequency definition for L1 measurement?

Agreement:

- Send LS to RAN1/2 about RAN4 agreement on intra-frequency and inter-frequency definition for CSI-RS based L1 measurement.

# Topic #4: performance

## Event-triggered L1 reporting

#### Issue 4-1-1: test case list for event-triggered L1 reporting

Candidate options:

- Option 1: Nokia

  - FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps)

  - FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps)

  - FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps)

  - At least 1 test case for event-triggered periodic reporting.

- Option 2: CATT

  - SSB based test cases:

    - TC for LTM Intra-frequency event triggered L1 measurement reporting tests without gap under non-DRX

    - TC for LTM Inter-frequency event triggered L1 measurement reporting tests with gaps under non-DRX

    - TC for LTM Intra-frequency event triggered L1 measurement reporting tests without gap under non-DRX with SSB index reading

  - CSI-RS based test case:

    - Only new TC for LTM Intra-frequency event triggered reporting tests without gap under non-DRX should be defined.

- Option 3: Apple, [vivo?]

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1

  - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR1

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2

  - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR2

- Option 4: CTC

  - LTM Intra-frequency SSB/CSI-RS based event triggered L1-RSRP measurement reporting tests without gap in FR1/FR2.

  - LTM Inter-frequency SSB based event triggered L1-RSRP measurement reporting tests with/without gap in FR1/FR2.

  - LTM Inter-frequency CSI-RS based event triggered L1-RSRP measurement reporting tests with gap in FR1/FR2.

- Option 5: OPPO

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR1

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR1

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR2

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR2

- Option 6: MTK

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1

  - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2

- Option 7: CMCC

  - Both FR1 and FR2-1 are considered.

  - For SSB based event triggered L1 measurement reporting, intra-frequency measurement, inter-frequency measurement without gap, gap-based inter-frequency measurement need to be considered.

  - For CSI-RS based event triggered L1 measurement reporting, measurement without gap need to be considered.

- Option 8: ZTE

  - FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps)

  - FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps)

  - FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps)

- Option 9: HW

  - FR1 and FR2 LTM Intra-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement)

  - FR1 and FR2 LTM Intra-frequency event triggered reporting tests without gap (CSI-RS based L1-RSRP measurement)

  - FR1 and FR2 LTM Inter-frequency event triggered reporting tests with gap (SSB based L1-RSRP measurement)

  - FR1 and FR2 LTM Inter-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement)

- Option 10: E///

| Intra-f L1-RSRP measurement for LTM | - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |
| --- | --- |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated |
| Inter-f L1-RSRP measurement with MG | - Inter-f L1-RSRP measurement with MG in FR1 - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement with MG in FR2 - 2 neighbor cells |
| Inter-f L1-RSRP measurement without gap for LTM | - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated |

Recommended WF:

- Further discuss based on the following directions

  - For SSB based L1 RSRP measurement reporting:

    - Approach 1: duplicate all R18 test cases and add event triggered reporting. UE which can pass R19 test cases can skip R18 test cases. (E///, Apple)

    - Approach 2: keep R18 test cases, develop necessary new R19 test to verify reporting behavior. (MTK, HW)

  - For CSI-RS based L1 RSRP measurement reporting:

    - Introduce test cases to verify 1) CSI-RS based L1-RSRP measurement and 2) CSI-RS based L1-RSRP measurement event triggered reporting.

    - For UE supporting event triggered reporting, UE only needs to pass 2)

    - For UE not supporting event triggered reporting, UE only needs to pass 1)

#### Issue 4-1-2: test configuration for event-triggered L1 reporting

Candidate options:

- Proposal 1: On high level, the test cases for event-triggered L1 reporting can have two cells (serving and candidate), and the signal level of one of these cells is to be changed during the test so that the reporting condition becomes met. UE shall send the report within the specified delay after the condition becomes met. (Nokia)

  - Use LTM3 as the reporting event in the test cases.

- Proposal 2: it is proposed to define test cases for LTM event 2/3/4/5. (CMCC)

- Proposal 3: In event triggered reporting tests for CSI-RS L1 RSRP, “3-step” measurement procedure shall be verified. (HW)

- Proposal 4: At least Event LTM3 and Event LTM4 be included in the RRM testcase. (E///)

Agreement:

- On high level, the test cases for event-triggered L1 reporting can have two cells (serving and candidate), and the signal level of one of these cells is to be changed during the test so that the reporting condition becomes met. UE shall send the report within the specified delay after the condition becomes met.

## CSI-RS based L1 measurement

#### Issue 4-2-1: CSI-RS based L1 measurement accuracy

Agreement:

- CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell when the following conditions are met:

  - Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

  - Conditions for L1-RSRP measurements are fulfilled according to annex B.2.4.2 for a corresponding Band for each relevant CSI-RS.

  - The bandwidth of CSI-RS is 48 PRBs and the density is 3.

  - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

- Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell.

- New requirements are defined in the following sections

| Accuracy requirements | Detail |
| --- | --- |
| Intra-frequency L1-RSRP accuracy requirements | 10.1.19D.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 10.1.19D.2.1 Absolute Accuracy 10.1.19D.2.2 Relative Accuracy |
|  | 10.1.20A.2 CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR2 10.1.20A.2.1 Absolute Accuracy 10.1.20A.2.2 Relative Accuracy |

#### Issue 4-2-2: test case list for CSI-RS based L1 measurement

Candidate options

- Option 1: Nokia

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR1

  - Intra-frequency Semi-persistent CSI-RS L1-RSRP measurements with semi-persistent reporting on FR1

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with SSB-based beam sweeping

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with periodic reporting on FR2 with CSI-RS-based beam sweeping

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with CSI-RS based beam sweeping

  - Intra-frequency Periodic CSI-RS L1-RSRP measurements with event triggered reporting for FR2 with SSB-based beam sweeping

- Option 2: CATT

  - RAN4 to define the test case for LTM CSI-RS based Intra-frequency L1-RSRP measurement in FR1 and FR2.

- Option 3: Apple, OPPO, [vivo?]

  - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR1

  - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR2

  - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1

  - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2

- Option 4: MTK

  - CSI-RS based L1 RSRP measurement for neighbour cell in FR1

  - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 with SSB based L1-RSRP measurement

  - CSI-RS based L1 RSRP measurement for neighbour cell in FR2 without SSB based L1-RSRP measurement

  - CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1

  - CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2

- Option 5: CMCC

  - For CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for measurement delay. And both the case with step 2 and the case without step 2 are tested.

- Option 6: HW

  - As the CSI-RS based L1-RSRP measurement procedure has been verified by event triggered reporting test case for CSI-RS L1-RSRP, it is no need to specify specific test for CSI-RS based L1-RSRP measurement.

  - In event triggered reporting tests for CSI-RS L1 RSRP, “3-step” measurement procedure shall be verified.

  - FR1 and FR2 LTM Intra-frequency CSI-RS L1-RSRP accuracy test cases for neighbour cell are to be specified.

- Option 7: E///

| Intra-f L1-RSRP measurement for LTM | - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells |
| --- | --- |
|  | - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, |
| Inter-f L1-RSRP measurement without gap for LTM | - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells |
|  | - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated |

Recommended WF:

- Discuss the options.

#### Issue 4-2-3: test configuration for CSI-RS based L1 measurement - BW

Candidate options:

- Option 1: Test configuration is set to have 48 PRBs within the active BWP. (Nokia)

Recommended WF:

- Agree on option 1.

#### Issue 4-2-4: others

Candidate options:

- Proposal 1: Define test case(s) to verify early CSI acquisition if core requirements are specified. (vivo)

  - Proposal 1a: Define test case(s) to verify early CSI acquisition before the cell switch command, conditionally to core requirement definition. (Nokia)

  - Proposal 1b: Define test case(s) to verify early CSI acquisition after the cell switch command, conditionally to core requirement definition. (Nokia)

- Proposal 2: Discuss whether there is need to consider configuration where CSI-acquisition resource(s) are outside of the active BWP. (Nokia)

Recommended WF:

- Discuss the proposals.

## Conditional LTM

#### Issue 4-3-1: CLTM test scope – frequency range

Candidate options:

- Option 1: (Nokia, Apple, CTC, OPPO, MTK, vivo, ZTE, HW)

  - FR1 to FR1

  - FR2 to FR2

- Option 2: CMCC

  - PCell switch to a neighboring LTM candidate cell

    - FR1 cell to FR1 cell

    - FR1 cell to FR2 cell

    - FR2 cell to FR2 cell

    - FR2 cell to FR1 cell

  - PCell switch to an LTM candidate cell that is a serving SCell in MCG

    - FR1 cell to FR1 cell

    - FR2 cell to FR2 cell

- Option 3: E///

  - FR1 Pcell switch to a FR2 neighboring LTM candidate cell,

  - FR2 Pcell switch to a neighboring FR1 LTM candidate cell,

  - Pcell FR1 switch to an LTM candidate cell that is a FR1 serving SCell in MCG,

  - FR2 PCell switch to an LTM candidate cell that is a FR2 serving SCell in MCG.

Agreement:

- CLTM test frequency range:

  - FR1 to FR1

  - FR2 to FR2

- Other options are not precluded.

#### Issue 4-3-2: CLTM test scope - intra/inter-frequency

Candidate options:

- Option 1: cover both intra and inter-frequency CLTM. (Nokia, Apple, CTC, vivo, ZTE, HW)

Recommended WF:

- Continue discussion.

#### Issue 4-3-3: CLTM test scope – L1/L3 based CLTM

Candidate options:

- Option 1: cover both L1 and L3 based CLTM. (Nokia, CATT, CTC, ZTE, E///)

  - Option 1a: RAN4 to discuss how to verify L1 and L3 measurement based candidate evaluation for CLTM such as whether to reuse partial LTM test procedure. (E///)

- Option 2: only test L1 based CLTM. (OPPO, MTK)

Recommended WF:

- Discuss the options

#### Issue 4-3-4: CLTM test scope – RACH-less and RACH-based CLTM

Candidate options:

- Option 1: cover both RACH-less and RACH based CLTM. (Nokia, CATT, CTC, OPPO, vivo, ZTE, HW)

- Option 2: RAN4 to discuss how to verify both RACH based and RACH-less procedure for CLTM such as re-use partial LTM test procedure. (E///)

Recommended WF:

- Discuss the options

#### Issue 4-3-5: CLTM test scope – early DL sync

Candidate options:

- Option 1: cover both CLTM with and without early DL sync. (Nokia)

- Option 2: only test CLTM with early DL sync. Do not test CLTM without early DL sync. (OPPO, MTK)

Recommended WF:

- Discuss the options

#### Issue 4-3-6: CLTM test scope – PCell/PSCell

Candidate options:

- Option 1: cover both PCell swith and PSCell switch. (ZTE)

Recommended WF:

- Discuss the options

#### Issue 4-3-7: subsequent CLTM test scope

Candidate options:

- Option 1: Apple

  - Intra-frequency subsequent CLTM from FR1 to FR1

  - Inter-frequency subsequent CLTM from FR2 to FR2

- Option 1a: MTK

  - subsequent CLTM from FR1 to FR1

  - subsequent CLTM from FR2 to FR2

- Option 2: OPPO

  - (SA) RACH based intra-frequency subsequent CLTM for PCell switch from FR1 to FR1

  - (SA) RACH less inter-frequency subsequent CLTM for PCell switch from FR2 to FR2

  - (NR-DC) RACH based intra-frequency subsequent CLTM for PSCell switch from FR1 to FR2

  - (NR-DC) RACH less inter-frequency subsequent CLTM for PSCell switch from FR2 to FR1

- Option 3: CMCC

  - PCell switch to a neighboring LTM candidate cell

    - FR1 cell to FR1 cell

    - FR1 cell to FR2 cell

    - FR2 cell to FR2 cell

    - FR2 cell to FR1 cell

  - PCell switch to an LTM candidate cell that is a serving SCell in MCG

    - FR1 cell to FR1 cell

    - FR2 cell to FR2 cell

- Option 4: HW

  - RACH based Intra-frequency subsequent conditional PCell switch

  - RACH based Inter-frequency subsequent conditional PCell switch

  - RACH less Intra-frequency subsequent conditional PCell switch

  - RACH less Inter-frequency subsequent conditional PCell switch

Recommended WF:

- Discuss the options

#### Issue 4-3-8: test configuration related proposals

Candidate options:

- Proposal 1: On high level, CLTM test cases to have serving cell and candidate cell, and the UE shall trigger cell switch based on signal level of one of these cells changing so that the cell switch condition becomes met. (Nokia)

- Proposal 2: Ensure in the test cases with early UL and DL synchronization that the UE has valid DL synchronization for the target TCI state and a valid TA at the time of condition becoming met. (Nokia)

- Proposal 3: The legacy test configurations and parameters used in R18 LTM test cases can be a baseline for conditional LTM test cases. (CATT)

Recommended WF:

- Discuss the options

#### Issue 4-3-9: test applicability

Candidate options:

- Proposal 1: for UE which can pass subsequent CLTM, test case for CLTM can be skipped, since subsequent CLTM naturally includes a complete CLTM procedure. (Apple, MTK)

Recommended WF:

- Discuss the options

#### Issue 4-3-10: others

Candidate options:

- Proposal 1: for inter-CU LTM and inter-CU subsequent LTM, it is proposed to discuss whether to define test cases. (CMCC)

- Proposal 2: test both PCell switch and PSCell swtich in (subsequent) CLTM.

- Proposal 3: For (subsequent) CLTM, there is no need to define test cases for PDCCH-order Random Access. (ZTE)

- Proposal 4: verify both SSB based L1 measurement , CSI-RS based L1 measurement triggered CLTM. (E///)

Recommended WF:

- Discuss the options