3GPP TSG-RAN WG4 Meeting #116                                                                R4-2512142

Bengaluru, IN, 25th - 29th Aug 2025

Agenda item:			7.25.1

Source:	China Telecom

Title:	WF on RRM requirements for NR_Mob_Ph4_Part2

Document for:	Approval

Notes

In this document,

- < Agreement > represents the decisions made in this meeting

- < Way forward > represents the next step in later meetings

  - “FFS” does not mean RAN4 will make a down-selection for the item. More other proposals can be proposed.

Topic #1: RRM discussion

Tmeasure

Way forward in RAN4#115 For CSI-RS based L1-RSRP measurement: - For known cell, Tmeasure shall include the time period used to acquire the SFN information (TCSI-RS_SFN_intra) and CSI-RS based L1-RSRP measurement period. - Further discuss the conditions for TCSI-RS_SFN_intra = 0. - Further discuss whether to include unknown cell case. If included, add one more component of TPSS/SSS.

Issue 1-1-1: Tmeasure for CSI-RS based L1-RSRP measurement

< Agreement >

- Discuss in CR.

Issue 1-1-2: TCSI-RS_SFN_intra

< Agreement >

- Remove the TCSI-RS_SFN_intra and reuse the applicable condition in Rel-18 LTM. Discuss whether additional condition is needed in CR directly

Issue 1-1-3: The conditions for TCSI-RS_SFN_intra = 0

- Issue is closed based on the agreement of issue 1-1-2.

Issue 1-1-4: Whether to include unknown cell case

< Agreement >

- Only cover the known cell case.

Issue 1-1-5: Side condition for known cell

< Agreement >

- Follow the related agreements on CSI-RS based L1 measurement in part1.

Tfirst-RS and TRS-proc

Issue 1-2-1: Whether Tfirst-RS and TRS-proc are included in Tinterrupt

< Agreement >

- Tfirst-RS and TRS-proc are included in Tinterrupt.

Issue 1-2-2: The values and the conditions for Tfirst-RS = 0 and TRS-proc = 0

- Candidate solutions:

- Proposal 1:

1> if the target TCI state is in the serving cell active TCI state list

2> Tfirst-RS = 0 and TRS-proc= 0

1> else if the UE is configured with CLTM L1 intra- and/or inter-frequency measurements for the target cell, and

2> if the target TCI state is in the CLTM candidate cell active TCI state list, and

3> if the time between receiving the CLTM candidate cell TCI state activation MAC-CE and starting to execute cell switch towards the target cell is at least TCI state activation delay stated in section 8.25.3, and

3> if the time between receiving the CLTM candidate cell TCI state activation MAC-CE and starting to execute cell switch towards the target cell is not more than TCI state activation delay stated in section 8.25.3 + 160 ms, or

3> if the measurement period of the SSB associated to target TCI state is not larger than 160 ms after the CLTM candidate cell TCI state activation MAC-CE is received

4> Tfirst-RS = 0 and TRS-proc= 0

1> else if the target cell is an FR1 cell, and the UE is not configured with CLTM L1 intra- and/or inter-frequency measurements for the target cell, and

2> if the target TCI state is in the CLTM candidate cell active TCI state list, and

3> if the time between receiving the CLTM candidate cell TCI state activation MAC-CE and starting to execute cell switch towards the target cell is at least TCI state activation delay stated in section 8.25.3, and

3> if not more than TCI state activation delay stated in section 8. 25.3 + 480 ms for an intra-frequency target cell, or

NOTE: longer L3 measurement delay may be expected for 480 ms after TCI state activation delay stated in section 8.25.3.

3> if not more than TCI state activation delay stated in section 8. 25.3 + 160 ms for an inter-frequency target cell

4> Tfirst-RS = 0 and TRS-proc= 0

2> else if the time between the latest PDCCH ordered RACH preamble transmission on the target cell and starting to execute cell switch towards the target cell is not more than 160 ms.

3> Tfirst-RS = 0 and TRS-proc= 0

1> else:

2> Tfirst-RS is the time to the first SSB transmission on the target cell after TLTM-processing.

- Proposal 2: If UE has received a PDCCH order and consequently a TA MAC-CE, Tfirst-RS = 0 and TRS-proc = 0 based on successful preamble transmission and while the TA timer is running.

< Agreement >

- Proposal 1 is agreed.

- Proposal 2 can be discussed during maintenance phase without impact on core part completion.

Issue 1-2-3: CLTM requirements for candidate cells without active TCI states in FR2

< Agreement >

- Capture the following in the WF.

    - As starting point: L1 measurement behavior, which was introduced in R18, is not changed. CLTM measurement time requirements (Tmeasure) do not apply for the candidate cells without active TCI states after TCI state activation for at least one candidate cell in FR2. Other CLTM requirements (cell switch interruption, etc.) apply.

- FFS whether and how to capture in the CR