3GPP TSG-RAN WG4 Meeting # 116		 R4-2512143

Bengaluru, India, August 25th - 29th, 2025

Agenda item:			7.26.1

Source:	Moderator (Nokia)

Title:	Ad-hoc minutes for NR_XR_Ph3

Document for:	Approval

# Introduction

The issues in this document are a selection of the issues contained in the document

- R4-2509068, Topic summary for [116][225] NR_XR_Ph3

The RAN4 #116 meeting is the last meeting planned for RRM core requirements, therefore, the recommended priority for discussion is as follows:

Issues with online session agreement for information

- Issue 1-3-2: Formulas for measurement delay extension without DRX

Issues with tentative agreement before ad-hoc discussion

- Issue 1-3-3: Formulas for measurement delay extension with DRX

- Issue 1-3-4: Formulas for measurement delay extension for inter-RAT

- Issue 1-3-5: Formulas for measurement delay extension of concurrent gaps

- Issue 1-3-6: Formulas for measurement delay extension of L1/L2 triggered mobility

- Issue 1-5-1: UE capabilities for XR – tables

Issues for further discussion during ad hoc

- Issue 1-6-1: Specification impact

- Issue 1-1-1: UE power class

- Issue 1-3-7: Conditions for measurement skipping

- Issue 1-2-1: Type 2 gaps

- Issue 1-3-8: Measurement gap configurations relevant to skipping

- Issue 1-4-1: Skipping indication on frequency domain

- Issue 2-1-1: Workplan

- Issue 2-2-1: Scenarios for the definition of the test cases

- Issue 2-2-2: Accuracy requirements

- Issue 2-2-3: Per UE and per FR gaps

- Issue 2-2-4: Type 1 and Type 2 gaps

- Issue 2-2-6: Test target

# Ad-hoc minutes

#### Issue 1-3-2: Formulas for measurement delay extension without DRX

- Agreement from online session for information

  - Max( 200 ms, ( 8 + Lcancel) ´ Max(MGRP , SMTC period) ) ´ CSSFinter

  - where Lcancel is the number of cancelled MG occasions in the measurement period

#### Issue 1-3-3: Formulas for measurement delay extension with DRX

- Proposals

  - Option 1 (Nokia, ZTE, NTT DOCOMO): RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as

    - DRX cycle ≤ 320 ms

      - Max(200 ms ´ CSSFinter, Ceil(8 ´1.5) ´ Max(MGRP, SMTC period, DRX cycle) ´ CSSFinter + Lcancel × Max(MGRP, SMTC period, DRX cycle) )

    - DRX cycle > 320 ms

      - 8 ´ DRX cycle ´ CSSFinter + Lcancel ´ DRX cycle

  - Option 2 (China Telecom, Interdigital, Apple, Ericsson, MediaTek): Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

    - DRX cycle ≤ 320 ms

      - Max(200 ms, Ceil((8+ Lcancel) ´1.5) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter

    - DRX cycle > 320 ms

      - Ceil(8+ Lcancel) ´ DRX cycle ´ CSSFinter

  - Option 3 (Nokia): RAN4 to define measurement delay extension due to measurement gap occasion cancelation for inter-frequency measurements with gap as

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + Tcancel) ms

    - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter + Tcancel) ms

    - Where

      - DRX cycle ≤ 320 ms, Tcancel = Lcancel ´ Max(MGRP , SMTC period, DRX cycle)

      - DRX cycle > 320 ms, Tcancel = Lcancel ´ DRX cycle

  - Option 4 (vivo): The number of skipped gap occasions is one per each DRX cycle in the delay requirements.

    - .

- Recommended WF

  - Outcome of Issue 1-3-1 can be reused, but further discussion on Option 4 is needed.

- Tentative agreement

  - Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

    - DRX cycle ≤ 320 ms

      - Max(200 ms, Ceil((8+ Lcancel) ´1.5) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter

    - DRX cycle > 320 ms

      - Ceil(8+ Lcancel) ´ DRX cycle ´ CSSFinter

- Discussion:

  - Moderator: discuss option 4

    - Lcancel is the number of DRX cycles where at least one gap is cancelled

- Agreement

  - Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively:

    - DRX cycle ≤ 320 ms

      - Max(200 ms, Ceil((8+ Lcancel) ´1.5) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter

    - DRX cycle > 320 ms

      - Ceil(8+ Lcancel) ´ DRX cycle ´ CSSFinter

    - Lcancel is the number of DRX cycles where at least one gap is cancelled

#### Issue 1-3-4: Formulas for measurement delay extension for inter-RAT

- Proposals

  - Option 1 (Nokia): Extend the measurement delay for Inter-RAT E-UTRAN as

    - TIdentify,E-UTRAN FDD,cancel = TIdentify,E-UTRAN FDD +ceil( Lcancel×Kgap_EUTRA)×MGRP

    - TMeasure,E-UTRAN FDD,cancel = TMeasure,E-UTRAN FDD +ceil( Lcancel×Kgap_EUTRA)×MGRP

    - TIdentify,E-UTRAN TDD,cancel = TIdentify,E-UTRAN TDD +ceil( Lcancel×Kgap_EUTRA)×MGRP

    - TMeasure,E-UTRAN TDD,cancel = TMeasure,E-UTRAN TDD +ceil( Lcancel×Kgap_EUTRA)×MGRP

  - Option 1a (moderator): Extend the measurement delay for Inter-RAT E-UTRAN as

    - TIdentify,E-UTRAN FDD,cancel = TIdentify,E-UTRAN FDD +ceil( Lcancel×Kgap_EUTRA)×MGRP×CSSF

    - TMeasure,E-UTRAN FDD,cancel = TMeasure,E-UTRAN FDD +ceil( Lcancel×Kgap_EUTRA)×MGRP×CSSF

    - TIdentify,E-UTRAN TDD,cancel = TIdentify,E-UTRAN TDD +ceil( Lcancel×Kgap_EUTRA)×MGRP×CSSF

    - TMeasure,E-UTRAN TDD,cancel = TMeasure,E-UTRAN TDD +ceil( Lcancel×Kgap_EUTRA)×MGRP×CSSF

  - Option 2 (vivo): Define new Tinter1 for gap skipping in inter-RAT measurement requirements.

- Recommended WF

  - Discussion needed.

- Options

  - Option 1

    - Extend measurement delay by ceil( Lcancel×Kgap_EUTRA)×MGRP×CSSF

    - Details to be captured on the CR.

  - Option 2

    - Tinter1_cancel = Tinter1 – (MGL – RF retuning time) * Lcancel

  - Option 3

    - T1 = ceil( Lcancel×Kgap_EUTRA)×MGRP

    - Extend by 480 * ceil( T1 / 480 ) ×CSSF

- Comments

  - QC: Option 1, don’t work because delay is multiples of 480 ms. It should be multiples of 480 ms. For option 2, need to clarify what Lcancel is. Is it the total number of gaps on the meas period?

    - Lcancel definition needs to be fixed

  - Vivo: Lgaps can be the number of measurement gaps in a cycle of 480 ms

  - QC: We need to determine how many gaps are canceled in every cycle of 480 ms.

  - Apple: if we still need to follow the legacy framework it would be very complicated. If we have only 1 LTE and 1 gap, can we extend by only 1 MGRP?

  - QC: than you need to extend by 480 ms.

    - The assumption is ceiling multiples of 480 ms

  - MTK: Tinter1 is typical. We need new equation.

  - Moderator: further work offline

- Recommended WF

  - Discuss if Option 3 can be agreed

  - Option 3

    - T1 = ceil( Lcancel×Kgap_EUTRA)×MGRP

    - Extend by 480 * ceil( T1 / 480 ) ×CSSF

#### Issue 1-3-5: Formulas for measurement delay extension of concurrent gaps

- Proposals

  - Option 1 (Nokia): RAN4 to define measurement delay extension due to occasion cancelation for inter-frequency measurements with gap as

    - Max(200 ms × CSSFinter, ceil( 8 * Kgap)   × Max(MGRP , SMTC period) × CSSFinter + ceil( Lcancel ×Kgap ) × Max(MGRP , SMTC period)),

      - where Lcancel is the number of non-dropped cancelled MG occasions in the measurement period.

    - Use this formula as example and follow similar approach for the other cases.

    - Cancelation of measurement gap occasions related to a certain gap pattern does not affect the measurement delay for another configured gap pattern.

- Recommended WF

  - Agreement based on the outcome of Issue 1-3-1

    - Max( 200 ms, ceil( (8 + Lcancel) * Kgap) ´ Max(MGRP , SMTC period) ) ´ CSSFinter

    - where Lcancel is the number of cancelled MG occasions in the measurement period

#### Issue 1-3-6: Formulas for measurement delay extension of L1/L2 triggered mobility

- Proposals

  - Option 1 (Nokia): The measurement delay for L1/L2 triggered mobility is determined as follows: Max(Treport, Ceil(M * Kgap) × Max(MGRP, SSB period)) × CSSFinter + Ceil(Lcancel × Kgap) × Max(MGRP, SSB period))

- Recommended WF

  - Agreement based on outcome of 1-3-2 and CR R4-2510325

| Condition | TL1-RSRP_Measurement_Period_SSB_Inter |
| --- | --- |
| No DRX | Max(Treport, Ceil((M+Lcancel) * Kgap) Max(MGRP, SSB period) CSSFinter) |
| DRX cycle ≤ 320 ms | Max(Treport, Ceil((M+Lcancel) 1.5 * Kgap) Max(MGRP, SSB period, DRX cycle) CSSFinter) |
| DRX cycle > 320 ms | Ceil((M+Lcancel) * Kgap) DRX cycle CSSFinter |

#### Issue 1-5-1: UE capabilities for XR - tables

- Proposals

  - Option 1 (vivo):

| 57.NR_XR_Ph3 | 53-1 | Recommended ratio of gap occasions for cancelation | Indicates the UE is capable of reporting recommended ratio of measurement gap occasions for cancelation | 64-1 | Yes | N/A | The UE does not support reporting recommended ratio for gap cancelation | Per UE | no | no |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Recommended WF

  - Tentative agreement on Option 1

- Agreement

| 57.NR_XR_Ph3 | 53-1 | Recommended ratio of gap occasions for cancelation | Indicates the UE is capable of reporting recommended ratio of measurement gap occasions for cancelation | 64-1 | Yes | N/A | The UE does not support reporting recommended ratio for gap cancelation | Per UE | no | no |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

#### Issue 1-1-1: UE power class

- Proposals

  - Option 1 (Ericsson): The requirements for measurement gap cancellation apply for UE power class 3.

  - Option 2 (Nokia, Ericsson, vivo, CMCC, QC): Measurement gap cancelation applies for all power classes, except for PC6 (HST).

  - Option 3 (vivo, CMCC): No restriction on UE power classes whether the requirements for gap cancellation are applicable or not.

  - Option 4 (MediaTek): Further information and discussion on the proposal of limiting the scope to power class 3.

- Recommended WF

  - Please discuss whether there should be restriction to power class for XR MG cancellation

- Comments from companies

  - Ericsson: We can go option 2

  - Vivo: option 2 is ok

  - CMCC: 2 and 3 fine

  - MTK: Issue is FR1 and FR2? It should be only FR2

  - Vivo: we don’t see need to exclude

  - ZTE: if option 2 we need to know why exclude PC6?

  - QC: It impacts specification work. Separate requirements for PC6. Because of spec impact. We also have separate tables for FR1 HST UEs.

  - MTK: We can preclude FR1 and FR2 HST

- Agreement:

  - Measurement gap cancelation applies for all power classes, except for FR2 HST (PC6).

#### Issue 1-6-1: Specification impact

- Background information:

  - Agreement from RAN4 #114 R4-2502700

Issue 1-3: L3 Measurements without gaps with scheduling restrictions Issue 1-4: L1 measurements with scheduling restrictions and procedures Issue 1-5: L1/L2 triggered mobility measurements and procedures <Agreement> - During the skipped measurement gap occasion, the UE is not required to perform measurements (i.e. L3 measurements and L1 measurements), and the UE shall not cause scheduling restrictions. - Outside a gap occasion, skipping of L3 Measurements without gaps with scheduling restrictions is not supported. - Outside a gap occasion, skipping of L1 Measurements with scheduling restrictions is not supported.

- Proposals

  - Option 1 (ZTE): The impact of gap skipping could be captured in L3 measurement with gap, L3 measurement without gap, L1 measurement with gap, L1 measurement without gap chapters, including the impact on the scheduling restriction.

- Recommended WF

  - Please consider previous agreements and discuss whether RAN4 needs to capture impact on L3 and L1 without gaps due to XR MG cancellation.

- Comments from companies

  - Moderator: Fully overlapped case doesn’t have CRs at the moment.

  - ZTE: Anoter simple case is the partial overlap, the work is easier, maybe some clarification in the specification. For the fully overlapped case, the extension factor would be needed on the delays for gapless measuements.

  - MTK: If we consider this case the spec will be really complicated. Fully overlapped is edge case. We don’t have to specify all optimization cases.

  - Vivo: we already have agreement that if the gap is measured the UE is not causing restrictions. We might need to clarify UE behaviour in those cases.

  - MTK: We still have the requirement, the UE measure within the gap, but we don’t define new UE behaviour because of this case

  - Apple: the UE still need to meet the legacy requirements in this case?

  - ZTE: need to capture partially overlapped scheduling restrictions clarification, and tha the UE is not expected to measure gapless measurement on the canceled gap

  - MTK: need to change definition of Kp

- Agreement

  - For partially overlapped case

    - Capture clarification on the scheduling restriction clauses for L3 measuemrent without gap that no scheduling restriction apply on the canceled gaps

    - For L1 and L3 without gaps, clarification on the Kp that the calculation doesn’t include cancelled gaps on the Navailable

#### Issue 1-3-7: Conditions for measurement skipping

- Proposals from companies

  - Proposals for no conditions

    - Proposal 1 (China Telecom): No conditions are to be defined for measurement skipping.

    - Proposal 2 (Nokia): No conditions are needed for measurement skipping regarding maximum number of skipped occasions per measurement period or related to MG configuration

    - Proposal 3 (Qualcomm): Given that a UE can send preferred percentage of MG cancellations within a time period as a UAI, no need to define additional conditions on maximum number of MGs skipped during the measurement period and maximum acceptable time separation between two non-cancelled MGs etc.

  - Proposals on max number of occasions

    - Proposal 4 (MediaTek): RAN4 shall define the max number of occasions cancelled during a measurement period to be 50%.

    - Proposal 5 (vivo): For measurement gap skipping, the max number of gaps skipped during the measurement period should be limited/specified which should not be larger than UE capability reported via UAI.

    - Proposal 6 (Apple): max number of occasions cancelled during a measurement period shall be defined. Values from NR-U can be reused.

    - Proposal 7 (Ericsson, Interdigital): The maximum number of extensions are

|  | PSSS/SSS detection | SSB index detection | Measurement period |
| --- | --- | --- | --- |
| No DRX or max(DRX cycle, SMTC period, MGRP)≤40 ms | FR1: 12 FR2: 1.5×Mpss/sss_sync | FR1: 5 FR2: 1.5×MSSB_index | FR1: 12 FR2: 1.5×Mmeas_period |
| DRX cycle ≤ 320 ms | FR1: 8 FR2: Mpss/sss_sync | FR1: 3 FR2: MSSB_index | FR1: 8 FR2: Mmeas_period |
| DRX cycle > 320 ms | FR1: 5 FR2: 0.5×Mpss/sss_sync | FR1: 2 FR2: 0.5× MSSB_index | FR1: 5 FR2: 0.5× Mmeas_period |

    - Upon exceeding XPSS/SSS,gaps,max the UE is not required to meet the corresponding PSS/SSS detection requirement.

    - Upon exceeding Xind,gaps,max the UE shall restart the time index detection.

    - Upon exceeding Xmeas,max the UE shall restart the measurement.

  - Proposals on maximum separation between two occasions shall be

    - Proposal 8 (Ericsson): Maximum acceptable time separation between two measurement occasions available for RRM measurements may need to be defined, e.g.:

      - Any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known.

    - Proposal 9 (OPPO): RAN4 to define maximum time separation between two non-skipped gaps for measurement.

    - Proposal 10 (ZTE): The maximum separation could be defined as: the maximum separation guarantees the two non-successive non-skipped gaps are covered by a single measurement period.

- Proposals merged by the Moderator

  - Option 1 (Nokia, ZTE, QCMTK): No conditions are defined

    - Concerns: Ericsson, vivo, Apple

  - Option 2 (Ericsson, vivo, Apple, QC (compromise )), MTK: Define the maximum number of gaps skipped during the measurement period.

    - Concerns: Nokia, ZTE, QC

      - QC: Concerns on restarting the measurements?

      - QC, UE may restart

  - Option 3 (Ericsson, ZTE, Apple (open) Nokia (compromise)): Define a maximum separation between two non-skipped gaps for measurement

    - Concerns: QC, Nokia

      - QC: Concerns on restarting the measurements?

  - Option 4 (Apple (open)): Define the max number of occasions cancelled during a measurement period as a percentage of 50%.

    - Concerns: Ericsson, Nokia, ZTE, QC

- Recommended WF

  - Discussion on the conditions is needed. If the group agrees that conditions are necessary, we can further discuss based on one of the detailed proposals.

  - How Is CSSF is impacting option 2?

- Comments from companies

  - Option 1:  No conditions are defined

    - Supporting companies: Nokia, ZTE, QC, MTK

    - Companies with concerns: Ericsson, vivo, Apple

  - Option 2 (Ericsson, vivo, Apple, QC (compromise )), MTK: Define the maximum number of gaps skipped during the measurement period.

    - Supporting companies: Ericsson, vivo, Apple

    - Companies with concerns: Nokia, ZTE, Qualcomm

    - Compromise by Qualcomm

      - Acceptable, if UE may restart measurements if Lmax is met

  - Option 3: Define a maximum separation between two non-skipped gaps for measurement

    - Supporting companies: Ericsson, ZTE, Apple

    - Companies with concerns: Qualcomm, Nokia

      - Qualcomm: Concerns on restarting the measurements

  - Option 4 (Apple (open)): Define the max number of occasions cancelled during a measurement period as a percentage of 50%.

    - Supporting companies: Apple

    - Companies with concerns: Ericsson, Nokia, ZTE, QC

- Recommended WF based on discussion

  - Discuss based on the options

    - Option 3a: Define a maximum separation between two non-skipped gaps for measurement of 5 consecutive gaps?

      - After this limit the UE is allowed to restart the measurement.

    - Option 5:

      - The requirements apply provided that any two closest SMTC occasions available at the UE for the measurement shall be separated by no more than the maximum time requirement for the cell to remain known.

#### Issue 1-2-1: Type 2 gaps

- Proposals

  - Proposal 1 (Huawei): Based on RAN1 agreements, it is clear which MG is to be cancelled for non-colliding concurrent gaps.

  - Proposal 2 (Nokia): Follow RAN1 specification, i.e.

    - the DCI command with gap cancelation applies for the first gap occasion happening at least X ms after the DCI command,

    - if 2 non-colliding concurrent gaps are separated by less than X ms, the second gap can be cancelled if a DCI is received during the time window of X ms before the first gap occasion.

  - Proposal 3 (Qualcomm): MG cancelling DCI also applies to non-colliding concurrent MGs separated by <X ms and no spec change is needed in RAN4.

  - Proposal 4 (Interdigital): If two non-colliding concurrent gaps are separated by less than “X ms”, a gap cancelation would apply for the earliest measurement gap occasion that respects the “X ms” distance from the DCI reception, but not to the second gap that is less than “X ms” from the first one.

  - Proposal 5 (CMCC): for the applicability of the gap cancellation if two non colliding concurrent gaps are separated by less than X ms:

    - If concurrent gaps are separated by less than X ms and X is 3ms, it is the colliding case, and MG cancelling DCI applies to the MG surviving the collision handling rules.

    - If concurrent gaps are separated by less than X ms and X is 5ms, MG cancelling DCI cancels the MG occasion which is the first occasion that satisfy the X ms from the cancelling indication reception.

  - Proposal 6 (ZTE): For the gap canceling in the case of two non colliding concurrent gaps are separated by just larger than 4ms, NW could cancel any gap via different DCI transmission instance, the FFS part could be addressed.

  - Proposal 7 (vivo): Cancellation of concurrent gaps applies after gap collision handling and regardless of gap distance.

- Recommended WF

  - Companies need to discuss expected behaviour on that situation (Picture from ZTE paper):

  - Please have in mind existing RAN1 agreements/specification.

  - Please check if CR R4-2509959 already captures the proposals

- Comments from companies

  - Company A:

#### Issue 1-3-8: Measurement gap configurations relevant to skipping

- Options

  - Option 1 (Ericsson): RAN4 to discuss MG configurations relevant for MG cancellation, e.g.,

    - minimum MGL, e.g., MGL≥3 ms,

    - maximum MGRP, e.g., MGRP≤320 ms to avoid too long delays.

- Recommended WF

  - Please discuss if there is need for limiting which MG configurations can be skipped.

- Comments from companies

  - Company A:

#### Issue 1-4-1: Skipping indication on frequency domain

- Background information

  - Agreement from R4-2508285

## Issue 5-2: Skipping indication on frequency domain <Agreement>: - It is RAN4 common understanding that: If UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FRs. - Further discuss whether it has already been specified in RAN1 CR, and whether/how to capture in RAN4 spec.

- Proposals

  - Proposal 1 (Nokia): RAN4 to agree the applicability of the MG cancellation indication to the frequency range for per-UE / per-FR gaps as provided in the Draft CR38.133, clause 9.1.y.3 in R4-2509959.

  - Proposal 2 (vivo): Common understanding on per-FR/per-UE gap application of gap cancellation is also captured in RAN4 specification from RRM requirements perspective.

  - Proposal 3 (Huawei): Clarify in RAN4 spec about the applicable frequency range as follows:

    - The applicable range of measurement gap cancelling includes the scheduled Cell(s) and Cell(s) in the same frequency range when UE is configured with per-FR gap; otherwise, applicable range of measurement gap cancelling applies to both frequency ranges.

  - Proposal 4 (ZTE): When UE receives gap skipping indication in DCI,

    - For per-UE gap, the gap skipping is valid for the measurements on any frequency layer during the canceled gap occasion;

    - For per-FR gap, the gap skipping is valid for the measurements on any frequency layer of the same FR during the canceled gap occasion.

- Recommended WF

  - Discussion needed.

- Comments from companies

  - Company A

#### Issue 2-1-1: Workplan

- Background information

|  |  |  | RAN | R4RD | RAN | R4RD | R4RD | RAN | R4RD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| WI code | WI or SI | Target | 108 Jun.25 | 116 | 109 Sep.25 REL-19 freeze | 116bis | 117 | 110 Dec.25 | 118 |
| NR_XR_Ph3-Core | WI | Sep.25 |  | 0,25 |  |  |  |  |  |
| NR_XR_Ph3-Perf | WI | Mar.26 |  | 0,25 |  | 0,5 | 0,5 |  | 0,5 |

- Proposals

  - Option 1 (Nokia, Qualcomm): Approve workplan update on R4-2509960

| RAN4 meeting | Tasks |
| --- | --- |
| RAN4 #116 - August 2025 | - Approval of workplan - Initial discussion on the list of test cases |
| RAN4 #116-bis - October 2025 | - Conclusions on the list of test cases - Discussion on test parameters - Agreement of CR work split |
| RAN4 #117 - November 2025 | - Discussion on test parameters - First Draft CRs for RRM core requirements. - Draft CRs to 38.133 expected - First Big Draft CR endorsed |
| RAN4 #118 - February 2026 | - Discussion on remaining open issues for RRM perf. - Revision of CRs for RRM perf. requirements. - Approval of Big CR |

- Recommended WF

  - TBA

- Comments from companies

  - Company A:

#### Issue 2-2-1: Scenarios for the definition of the test cases

- Proposals

  - Proposal 1 (CMCC): it is proposed to define test cases for NR-SA.

  - Proposal 2 (CMCC): it is proposed to define test cases for FR1 and FR2-1.

- Recommended WF:

  - Discuss if Proposals 1 and 2 can be agreed.

- Comments from companies

  - Company A:

#### Issue 2-2-2: Accuracy requirements

- Background information

  - Agreement from RAN4 #113 R4-2420103

## Issue 3-1: How to capture impact of measurement skipping <Agreement>: - Reuse existing measurement accuracy requirements. - RAN4 shall keep the number of samples for a frequency layer for L3 measurements unimpacted due to gap skipping - For the delay requirements, FFS whether and how to extend the delay

- Proposals

  - Proposal 1 (CMCC): for measurement accuracy requirements, it is proposed to confirm previous agreements to reuse existing measurement accuracy requirements for gap cancellation.

  - Proposal 2 (Ericsson): RAN4 confirms the earlier agreement:

    - Reuse existing measurement accuracy requirements for UE under XR operation.

  - Proposal 3 (Ericsson): Define at least one accuracy test case for XR in FR1 and at least one accuracy test for XR in FR2-1.

- Recommended WF:

  - Discuss whether confirmation of previous agreement is needed, and whether accuracy test case should be defined.

- Comments from companies

  - Company A:

#### Issue 2-2-3: Per UE and per FR gaps

- Company Proposals

  - Proposal 1 (OPPO): Consider both per-FR gap and per-UE gap in the test case.

  - Proposal 2 (Nokia): RAN4 to focus on test cases for per-UE MGs.

  - Proposal 3 (Ericsson): Test cases are specified only for per-UE MGs.

  - Proposal 4 (Qualcomm): RAN4 to adopt per-UE measurement gaps as the baseline configuration, with a single MG cancellation.

- Proposals merged by the moderator

  - Option 1: Test cases are specified only for per-UE MGs

  - Option 2: Test cases are specified for both per-UE and per-FR MGs

- Recommended WF

  - Discussion needed

- Comments from companies

  - Company A:

#### Issue 2-2-4: Type 1 and Type 2 gaps

- Company Proposals

  - Proposal 1 (OPPO): Consider Rel-17 concurrent gaps in the test case.

  - Proposal 2 (CMCC): it is proposed to define test cases for Type 1 measurement gap and Type 2 measurement gap ( e.g. the colliding case).

  - Proposal 3 (ZTE): The skipping in both Type-1 MG and Type-2 MG should be verified in different tests.

  - Proposal 4 (Ericsson): Test cases for inter-frequency measurements with gaps are specified for Type 1 MG but not Type 2 MG.

  - Proposal 5 (Ericsson): RAN4 to discuss whether test cases with Type 2 MG can be deprioritized in Rel-19.

  - Proposal 6 (Qualcomm): One additional test case for concurrent MG configurations may also be introduced.

- Merged proposals by the moderator

  - Option 1: Test cases are defined for both Type 1 and Type 2 gaps

  - Option 2: Test cases are defined only for Type 1 gaps

- Recommended WF:

  - Discussion needed.

- Comments from companies

  - Company A:

#### Issue 2-2-5: DRX proposals

- Company proposals:

  - Proposal 1 (OPPO): Define the test cases for the cases without DRX and with DRX.

  - Proposal 2 (Nokia): Test cases without DRX should have higher priority than with DRX.

    - Test cases with DRX should cover at least inter-frequency measurements with Type 1 MG.

    - Inter-RAT with DRX and with Type 1 MG can be developed with lower (second) priority.

    - Focus on non-DRX test cases for Type 2 MGs.

  - Proposal 3 (Ericsson): Each type of test case is specified for either DRX or non-DRX but not both.

- Recommended WF:

  - Discuss the proposals having in mind the test coverage and workload.

- Comments from companies

  - Company A:

#### Issue 2-2-6: Test target

- Company proposals

  - Proposal 1 (OPPO): UE Tx/Rx behavior in the skipping MG occasions and the extended delay requirements should be verified in the same test case.

  - Proposal 2 (Nokia): The following aspects should be evaluated in XR test cases:

    - Measurement delay extension due to skipping

    - Capability of canceling measurement gap within Toffset of 5ms or 3ms depending on UE capability

    - Reception of PDCCH/PDSCH during canceled measurement gap occasions

    - Transmission of PUCCH/PUSCH during canceled measurement gap occasions

  - Proposal 3 (Huawei): RAN4 to define test cases for MG skipping to verify following behaviour and requirements:

    - The MG is skipped accordingly based on gNB DCI, and UE is scheduled during MG occasions and transmit ACK accordingly.

    - UE shall meet the extended measurement delay requirements.

  - Proposal 4 (Ericsson): Define at least one accuracy test case for XR in FR1 and at least one accuracy test for XR in FR2-1.

  - Proposal 5 (ZTE): The UE processing timeline of receiving the DCI based MG skipping indication should be verified independently or together with other test motivation.

- Proposals merged by the moderator:

  - RRM test cases should verify

    - Proposal 1: accuracy requirements

    - Proposal 2: delay extension with gap cancellation

    - Proposal 3: Reception/transmission without scheduling restrictions during cancelled gap occasions

    - Proposal 4: The UE processing timeline of receiving the DCI-based MG skipping indication

- Recommended WF:

  - Discuss which proposals are agreeable.

- Comments from companies

  - Company A:

#### Issue 2-2-7: List of test cases

- Company proposals

  - Proposal 1 (OPPO): Define test cases for:

    - Intra-frequency measurements with gaps

    - Inter-frequency measurements with gaps

    - Inter-RAT E-UTRAN FDD measurements

    - Inter-RAT E-UTRAN TDD measurements

    - Inter-frequency L1-RSRP with MG

  - Proposal 4 (CMCC): it is proposed to define test cases for intra-frequency measurement, inter-frequency measurement, and LTE inter-RAT measurement.

  - Proposal 5 (vivo): Define test cases to very core requirements for intra-frequency measurement with gaps, inter-frequency measurement with gaps, inter-frequency L1-RSRP measurement with gaps and inter-RAT E-UTRAN measurements when gap cancellation is enabled.

  - Proposal 6 (ZTE): The MG skipping has impact on L3 & L1 measurements with gap. The impact by measurement skipping on the following types of L3&L1 measurement with gap should be verified:

    - Intra-frequency measurement with gap

    - Inter-frequency measurement with gap

    - EUTRAN measurement with gap

    - L1 inter-frequency measurement with gap

  - Proposal 7 (Ericsson): A subset of relevant test cases is specified by RAN4 for UE measurements under XR operation.

  - Proposal 8 (Qualcomm): RAN4 to use an incremental test-case design approach to specify new test cases, where only additional parameters and configurations from the baseline test case are specified along with the new test requirements.

  - Proposal 9 (ZTE): The MG skipping also impacts the L3 & L1 measurements without gap when these measurement occasions partially or fully overlapping with MG occasions. Such impacts should be verified as well.

- Proposals merged by the moderator

  - Define test cases for

    - Proposal 1: Intra-frequency measurements with gaps

    - Proposal 2: Inter-frequency measurements with gaps

    - Proposal 3: Inter-RAT E-UTRAN FDD measurements

    - Proposal 4: Inter-RAT E-UTRAN TDD measurements

    - Proposal 5: Inter-frequency L1-RSRP with MG

    - Proposal 6: L3 and L1 measurements without gaps

- Recommended WF

  - Discuss the merged proposals 1 to 5, and whether reduced number of tests are defined for each of the proposals.

- Comments from companies

  - Company A:

#### Issue 2-2-8: Complete list of test cases for discussion

- Proposals

  - Proposal 1: Evaluate which test cases from the list below to be developed

| Gap type | Intra/inter/inter-RAT | Frequency range | DRX | Included/not included |
| --- | --- | --- | --- | --- |
| Type 1 gap per FR | Intra-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT E-UTRAN FDD | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT E-UTRAN TDD | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
| Type 1 gap per UE | Intra-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT E-UTRAN FDD | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT E-UTRAN TDD | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
| Type 2 gaps per FR | Intra-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT E-UTRAN FDD | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT E-UTRAN TDD | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
| Type 2 gaps per UE | Intra-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT E-UTRAN FDD | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
|  | Inter-RAT E-UTRAN TDD | FR1 (serving) | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 (serving) | ON |  |
|  |  |  | OFF |  |
| LTM measurements with gaps | Inter-frequency | FR1 | ON |  |
|  |  |  | OFF |  |
|  |  | FR2 | ON |  |
|  |  |  | OFF |  |

- Recommended WF:

  - Discuss the proposals. Use the outcome of the previous issues to delete or add items to the table.

#### Issue 2-3-1: Type 2 test configurations

- Proposals

  - Proposal 1 (Nokia): For Type 2 MGs test cases, include 2 gaps with partial overlap.

  - Proposal 2 (Nokia): For Type 2 MGs test cases, reuse measurement gap configuration of A.6.6.18.2.1, i.e.

    - Low priority MG pattern #1 with MGRP=40ms, offset=39ms

    - High priority MG pattern #2 with MGRP=80ms, offset=4ms

- Recommended WF:

  - Discuss the proposals

- Comments from companies

  - Company A:

#### Issue 2-4-1: Cancelation model for XR test cases

- Proposals

  - Proposal 1 (Nokia): Measurement gap cancelation is determined randomly by the test equipment.

  - Proposal 2 (Nokia): For Type 2 gaps, the measurement gap cancelation ratio is configured independently for each configured gap pattern.

- Recommended WF

  - Discuss the proposals

- Comments from companies

  - Company A:

#### Issue 2-4-2: Cancelation ratio

- Proposals

  - Proposal 1 (Nokia):

    - Measurement gap cancelation is determined randomly by the test equipment with a ratio of 50% for UE supporting 5ms Toffset

    - Measurement gap cancellation is determined randomly by the test equipment with a ratio of 40% for UE supporting 3ms Toffset

  - Proposal 2 (Nokia): For Type 2 MGs test cases, configure cancelation ratio as:

    - For MG pattern #1 with MGRP=40ms, offset=39ms, cancelation ratio of 0%

    - For MG pattern #2 with MGRP=80ms, offset=4 ms, cancelation ratio of 50% for UE supporting 5ms Toffset and 40% for UE supporting 3ms Toffset

  - Proposal 3 (vivo): Ratio of gap occasions to be cancelled during the measurement period is 20% or 40% in the tests.

- Recommended WF:

  - Discuss the proposals

- Comments from companies

  - Company A:

#### Issue 2-5-1: Toffset between DCI and start of MG

- Proposals

  - Proposal 1 (CMCC): it is proposed to define test cases for both 3ms offset and 5ms offset. And applibility rules can be considered, e.g. the UE supporing 3ms offset only need to pass the test with 3ms timeline, the UE not supporting 3ms offset need to pass the test with 5ms timeline.

  - Proposal 2 (vivo): Time offset for gap cancellation indication is 5ms in the tests.

  - Proposal 3 (Huawei): The processing time 3ms/5ms shall be verified in the test cases. If UE supporting additional 3ms processing capability, 3ms shall be verified.

- Recommended WF

  - Discuss the proposals

- Comments from companies

  - Company A:

#### Issue 2-5-2: Types of DCI with cancellation indication

- Proposals

  - Proposal 2 (vivo): In the tests, at least DCI 0-1 and DCI 1-1 are used for indication of gap cancellation.

- Recommended WF

  - Discuss the proposal

- Comments from companies

  - Company A:

#### Issue 2-5-3: Behaviour of Option 1 and Option 2 for indication of value ‘0’

- Proposals

  - Proposal 1 (vivo): RAN4 to discuss whether UE behavior of option 1 and option 2 for indication value ‘0’ should be tested.

- Recommended WF

  - Discuss the proposal

- Comments from companies

  - Company A: