3GPP TSG-WG4 Meeting # 116	R4-2512239

# Bangalore, India. August 25th – August 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.191 | CR | - | rev | - | Current version: | 0.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on A-IoT random access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Ambient_IoT_Solutions |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | RAN2 defines random access procedure for A-IoT devices. The corresponding RAN4 requirements will be missing in the RAN4 specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding description of random access requirements for A-IoT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification is incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | xxxx |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | TBD |  |  |  | TBD |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2511263 |  |  |  |  |  |  |  |  |

<Start of change 1>

### 9.1.1 Introduction of random access

This clause contains requirements for A-IoT device(s) random access procedure. The random access procedure is specified in clause 5.3 of TS 38.391, and the D2R physical layer transmission is specified in clause 6.1 of TS 38.291. The requirements for the CBRA type procedure are described in clause 9.1.2, whereas the requirements for the CFA type procedure are described in clause 9.1.3 of this specification.

<End of change1>