# 3GPP TSG-RAN WG4 Meeting #116	R4-2512293

# Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draftCR | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draft CR for NR measurements for positioning for RedCap UEs with NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-07-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The requirements of NR measurements for positioning for RedCap UEs with NTN is missing. The agreement in RAN4#114 meeting (R4-2502604) was: #### Issue 2-3-2: Network verified UE location for 1Rx/2Rx (e)RedCap UEs with FR1-NTN Agreement: - For 2Rx (e)RedCap UEs with FR1-NTN bands, the measurement period and measurement accuracy requirements for Network verified UE location defined in NTN can be reused. - For 1Rx (e)RedCap UEs with FR1-NTN bands, the measurement accuracy requirements defined for 1Rx RedCap positioning can be referred, and the same relaxation (as the measurement accuracy requirements defined for 1Rx TN-RedCap ) for accuracy should be reused. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the requirements of NR measurements for positioning for RedCap UEs with NTN is missing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements of NR measurements for positioning for RedCap UEs with NTN is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.9D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of Change 1>

## 9.9D	NR measurements for positioning for RedCap in Satellite Access

### 9.9D.1	Introduction

This clause contains requirements for RedCap UE capable of performing NR positioning measurements defined in TS 38.215 [4], including UE Rx-Tx time difference, when the PCell is served by a Satellite Access Node (SAN).

The measurement reporting delay can be longer for the measurement reporting requirements in this clause when IDC autonomous denial is configured.

#### 9.9D.1.1	General Aspects of Gap-based Measurement

The requirements in clause 9.9C.1.1shall apply.

#### 9.9D.1.2	General Aspects of Gapless Measurement

The requirements in clause 9.9C.1.2 shall apply.

#### 9.9D.1.3	Scheduling Availability of UE during PRS Measurement without Measurement Gaps

The requirements in clause 9.9C.1.3 shall apply.

### 9.9D.2	Void

### 9.9D.3	Void

### 9.9D.4	UE Rx-Tx time difference measurements

#### 9.9D.4.1 Introduction

The requirements in this clause shall apply for RedCap UE, when the UE has received nr-Multi-RTT-RequestLocationInformation message from LMF via LPP [34] requesting the UE to measure and report one or more UE Rx-Tx time difference measurements defined in TS 38.215 [4].

#### 9.9D.4.2 Requirements Applicability

For RedCap UE not in HD-FDD mode, the requirements in clause 9.9D.4 apply for periodic and triggered RedCap UE Rx-Tx time difference measurements, provided:

-	RedCap UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25D are met for a corresponding band.

-	SRS is configured on the PCell.

-	The RedCap UE transmits SRS within [-160, 160] msec of the DL PRS resource of the TRP corresponding to the PCell in the assistance data.

For RedCap UE in HD-FDD mode, the requirements in clause 9.9D.4 apply for periodic and triggered RedCap UE Rx-Tx time difference measurements, provided:

-	RedCap UE Rx-Tx time difference measurement related side conditions given in clause 10.1.25D are met for a corresponding band.

-	SRS is configured on the PCell.

-	The RedCap UE transmits SRS within [-160, 160] msec of the DL PRS resource of the TRP corresponding to the PCell in the assistance data.

-	For UE supporting Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE, when ntn-PrioritizeUl-semiStatichalfDuplexRedCapcollision is enabled, no collision occurs between PRS and UL transmission.

#### 9.9D.4.3	Measurement Capability

RedCap UE Rx-Tx time difference measurement capability is as indicated by the UE in NR-Multi-RTT-ProvideCapabilities, according to TS 37.355 [34].

#### 9.9D.4.4	Measurement Reporting Requirements

The requirements in clause 9.9C.4.4 shall apply except that:

-  the RedCap UE Rx-Tx time difference measurement accuracy for all measured DL PRS resources shall be fulfilled according to the accuracy requirements specified in clause 10.1.25D.2, and

-  the RedCap UE Rx-Tx time difference measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clause 10.1.25D.3.

#### 9.9D.4.5	Measurement Period Requirements

The requirements in clause 9.9C.4.5 shall apply except that:

-	clause identifier 9.1.5.2 is replaced with 9.1A.5.2

-	clause identifier 9.9C.4 is replaced with 9.9D.4

#### 9.9D.4.6	Measurement Period Requirements without Measurement Gaps

The requirements in clause 9.9C.4.6 shall apply.

- clause identifier 9.9C.4.6 is replaced with 9.9D.4.6.

# <End of Change 1>