3GPP TSG-RAN4 Meeting #116	R4-2512302

# Bangalore, India, 25th - 29th, August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | draft | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_Ph3-Core) Cell Re-selection for RedCap UEs with NTN in IDLE state |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The core requirements for RedCap UEs in NTN scenario in RRC IDLE state are quite stable, the new feature shall be captured in TS 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The core requirements for RedCap UEs in NTN scenario in RRC IDLE state are captured in TS38.133. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The core requirements for RedCap UEs in NTN scenario in RRC IDLE state will be absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2E, 4.2E.1, 4.2E.2.1, 4.2E.2.1.1, 4.2E.1.1.2, 4.2E.2.2, 4.2E.2.3, 4.2E.2.4, 4.2E.2.5, 4.2E.2.6, 4.2E.2.7, 4.2E.2.7.1, 4.2E.2.7.2, 4.2E.2.7.3, 4.2E.2.7.4, 4.2E.2.8, 4.2E.2.8.1, 4.2E.2.8.2, 4.2E.2.8.3, 4.2E.2.8.4, 4.2E.9, 4.2E.10, 4.2E.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

4.2E	Cell Re-selection for NR RedCap UE with Satellite Access

4.2E.1	Introduction

The cell reselection procedure allows the UE to select a more suitable cell and camp on it.

When the RedCap UE is in either Camped Normally state or Camped on Any Cell state on a cell, the RedCap UE shall attempt to detect, synchronize, and monitor intra-frequency , inter-frequency and inter-RAT cells indicated by the serving cell. For intra-frequency ,inter-frequency and inter-RAT cells, the serving cell may provide explicit neighbour list, or only carrier frequency information and bandwidth information. RedCap UE measurement activity is also controlled by measurement rules defined in TS 38.304 [1], allowing the RedCap UE to limit its measurement activity.

For intra-frequency cell re-selection, requirements defined in clause 4.2E.2.3 apply.

For inter-frequency cell re-selection, when TN carrier is not configured, requirements specified in clause 4.2E.2.4 apply and when NTN carrier is configured, requirements specified in clause 4.2E.2.10 apply.

For inter-RAT cell re-selection, requirements defined in clause 4.2E.2.11 apply.

The requirements specified in clause 4.2E shall apply for the quasi-earth fixed cell and the earth moving cell.

The requirements specified in clause 4.2E apply to FR1-NTN as defined in TS 38.101-5 [43].

The terms SSB and SMTC in this clause apply to CD-SSB only if not specified otherwise.

The 1 Rx RedCap UE for performing the cell reselection procedure [1] applies:

-	Qrxlevmin as the signaled value of Qrxlevmin [2] -1 dB.

-	Qqualmin as the signaled value of Qqualmin [2] -1 dB.

4.2E.2	Requirements for RedCap UE with Satellite Access

4.2E.2.1	UE measurement capability for RedCap with Satellite Access

4.2E.2.1.1	UE measurement capability for 1Rx RedCap UEs

For idle mode cell re-selection purposes, the UE shall be capable of monitoring at least:

-	Intra-frequency carrier, and

-	Depending on UE capability, 6 NR inter-frequency carriers.

4.2E.2.1.2	UE measurement capability for 2Rx RedCap UEs

The capability defined in section 4.2C.2.1 shall apply.

4.2E.2.2	Measurement and evaluation of serving cell for RedCap UEs

The RedCap UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell at least once every M1*N1 DRX cycle; where:

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and NSMTC =1, upon one SMTC configured at the UE,

-	M1=2.5 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 1<NSMTC  4,

-	otherwise M1=1.

Where, NSMTC is the number of SMTCs configured by SAN.

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least eDRX_IDLE cycle/2, when the RedCap UE is configured with eDRX cycle ≤10.24s; otherwise, DRX cycle/2.

If the UE is not configured with eDRX_IDLE cycle and the UE has evaluated according to table 4.2C.2.2-1 (only FR1) for both 1Rx RedCap and 2Rx RedCap in Nserv_RedCap consecutive DRX cycles that the serving cell does not fulfil the cell selection criterion S, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities. In this case the UE shall not relax measurements on any of the neighbour cells even if UE has fulfilled the criteria for meeting the relaxed measurement requirements defined clauses 4.2E.2.7 and 4.2E.2.8.

If the UE is configured with eDRX_IDLE cycle and has evaluated according to table 4.2X.2.2-1, in Nserv_RedCap consecutive eDRX cycles, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell, regardless of the measurement rules currently limiting UE measurement activities. In this case the UE shall not relax measurements on any of the neighbour cells even if UE has fulfilled the criteria for meeting the relaxed measurement requirements defined clauses 4.2E.2.7 and 4.2E.2.8.

For the UE configured with eDRX_IDLE cycle, Nserv_RedCap is specified in table 4.2X.2.2-1 for 1 Rx RedCap and 2 Rx RedCap in FR1.Additionally, if the UE is configured with ‘t-service’ [2], the UE shall start measurements of the neighbour cells indicated by the serving cell before ‘t-service’ is reached according to the requirements provided in clauses 4.2E.2.3 and 4.2E.2.4.

Also,

-	if distanceThresh and referenceLocation are configured by the network [2] and the UE supports location-based measurement initiation and has obtained its location information, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell if the distance between UE and the serving cell reference location – referenceLocation – is larger than distanceThresh. The requirements apply provided that the distance exceeds the distanceThresh by a margin of 50 m.

-	if distanceThresh and movingReferenceLocation are configured by the network [2] and the UE supports location-based measurement initiation and has obtained its location information, the UE shall initiate the measurements of all neighbour cells indicated by the serving cell if the distance between UE and the serving cell moving reference location – movingReferenceLocation – is larger than distanceThresh. The requirements apply provided that the distance exceeds the distanceThresh by a margin of 80 m.

If the UE is not configured with‘t-Service’ [2] in the serving cell and if the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information during the time T, the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1], where

-	T= 10 s if the UE is not configured with eDRX_IDLE cycle, or

-	T= MAX (10 s, one eDRX_IDLE cycle) if the UE is configured with eDRX_IDLE cycle in FR1

If the UE is configured with ‘t-Service’ in the serving cell then the UE shall initiate cell selection procedures for the selected PLMN as defined in TS 38.304 [1] when any of the following conditions is fulfilled:

-	If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information within T since time instance T1 provided that ‘t-Service’ > T1 or

-	If the UE in RRC_IDLE has not found any new suitable cell based on searches and measurements using the intra-frequency, inter-frequency and inter-RAT information indicated in the system information within T since the time instance ‘t-Service’.

-	Where, T1 is the time instance in seconds when the UE has determined that the serving cell does not fulfil the cell selection criterion S.

Table 4.2E.2.2-1: Nserv_RedCap for UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28 s periods) | Scaling Factor (N1) | Nserv_RedCap [number of DRX or eDRX cycles Note 3] |
| --- | --- | --- | --- | --- |
| 2.56 | N/A | N/A | 1 | N1*2 |
| 5.12 | N/A | N/A |  | N1*2 |
| 10.24 | N/A | N/A |  | N1*2 |
| NOTE 1: The number of DRX cycles in this table corresponds to the DRX cycles within PTWs, when PTW is configured. NOTE 2: The eDRX_IDLE cycle lengths are as specified in section 10.5.5.32 of TS 24.008 [42]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56 s, 5.12 s and 10.24 s. NOTE 4: The lower bound of PTW length is derived based on Nserv_RedCap *DRX_cycle1.28*1.28. |  |  |  |  |

4.2E.2.3	Measurements of intra-frequency NR cells for RedCap UE

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

If Srxlev > SIntraSearchP and Squal > SIntraSearchQ, and the distance between UE and serving cell reference location or serving cell moving reference location is smaller than distanceThresh if the distanceThresh is configured (see TS 38.304[1]) and UE has location information, then the UE is not required to perform measurement of intra-frequency.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS 38.304 [1] within Kmulti_SMTC * Tdetect,NR_Intra when that Treselection= 0 if the UE does not support Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14] or if the enhancedMeasurementNGSO-r17 is not enabled, or within Kmulti_SMTC *  Tdetect,NR_Intra_enh if the UE supports Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14] and the enhancedMeasurementNGSO-r17 is enabled. An intra frequency cell is considered to be detectable according to the conditions defined in annex B.1.6 for a corresponding Band.

The UE shall measure SS-RSRP and SS-RSRQ at least every Kmulti_SMTC * Tmeasure,NR_Intra if the UE does not support Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14] or if the enhancedMeasurementNGSO-r17 is not enabled, or every Kmulti_SMTC * Tmeasure,NR_Intra_enh if the UE supports Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14] and the enhancedMeasurementNGSO-r17 is enabled, for intra-frequency cells that are identified and measured according to the measurement rules.The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2. The UE shall not consider an NR neighbour cell for cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For UE in FR1-NTN:

If SMTCs do not overlap with each other,

-	, if GEO satellites are measured on the carrier;

-	, if LEO satellites are measured on the carrier;

-	If SMTCs partially overlap with each other,

-	, if only GEO satellites are measured on the carrier;

-	, if only LEO satellites are measured on the carrier;

Where

-	 Is the number of LEO satellites to be measured within i-th SMTC,

-	 Is the number of LEO satellites that UE can measure in parallel within an SMTC,

-	 Is the number of SMTCs that partially overlap with each other.

Note: for deriving Kmulti_SMTC for Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tdetect,NR_Intra, Tmeasure,NR_Intra or Tevaluate,NR_Intra.

The parameter Kmulti_SMTC is the scaling factor for measurements of multiple SMTCs which correspond to different satellites.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS 38.304 [1] within Kmulti_SMTC * Tevaluate,NR_Intra if the UE does not support Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14] or if the enhancedMeasurementNGSO-r17 is not enabled, or within Kmulti_SMTC * Tevaluate,NR_Intra_enh if the UE supports Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14] and the enhancedMeasurementNGSO-r17 is enabled, when Treselection = 0 as specified in table 4.2C.2.3-1(with FR1), table 4.2E.2.3-1 or table 4.2E.2.3-2 provided that:

-	when rangeToBestCell is not configured:

-	the cell is at least 3 dB better ranked in FR1 for 2Rx RedCap, or

-  the cell is at least 4 dB better ranked in FR1 for 1Rx RedCap.

-	when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value in TS 38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them.

-	the cell is at least 3 dB better ranked in FR1 if the current serving cell is among them for 2 Rx RedCap.

-	the cell is at least 4 dB better ranked in FR1 if the current serving cell is among them for 1 Rx RedCap.

The 1 Rx RedCap UE applies absThreshSS-BlocksConsolidation as the signalled value of absThreshSS-BlocksConsolidation [2] + 1 dB.

When evaluating cells for reselection, the SSB side conditions apply to both serving and non-serving intra-frequency cells.

If Treselection timer has a nonzero value and the intra-frequency cell is satisfied with the reselection criteria which are defined in TS 38.304 [1], the UE shall evaluate this intra-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

For both 1Rx RedCap and 2Rx RedCap not configured with eDRX_IDLE cycle, Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap are specified in table 4.2C.2.3-1.

For 1 Rx RedCap and 2 Rx RedCap configured with eDRX_IDLE cycle, Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap are specified in table 4.2X.2.3-1 for FR1, where the requirements apply provided that the serving cell is configured with eDRX_IDLE.

For both 1Rx RedCap and 2Rx RedCap, Tdetect,NR_Intra_enh_RedCap ,Tmeasure,NR_Intra_enh_RedCap and Tevaluate,NR_Intra_enh_RedCap are defined in table 4.2E.2.3-2.

The parameter Kmulti_SMTC,i is the scaling factor for measurement of multiple SMTCs or multiple satellites, which refers to clause 4.2C.2.3.

Table 4.2E.2.3-1: Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap for UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28 s periods) | Tdetect,NR_Intra_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Intra_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Intra_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| 5.12 | - | - | 117.76 (23) | 5.12 (1) | 10.24 (2) |
| 10.24 | - | - | 235.52 (23) | 10.24 (1) | 20.48 (2) |
| NOTE 1: The number of DRX cycles in this table corresponds to the DRX cycles within PTWs, when PTW is configured. NOTE 2: The eDRX_IDLE cycle lengths are as specified in section 10.5.5.32 of TS 24.008 [42]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56 s, 5.12 s and 10.24 s. NOTE 4: The lower bound of PTW length is derived based on . NOTE 5: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |  |

Table 4.2E.2.3-2: Tdetect,NR_Intra_enh_RedCap, Tmeasure,NR_Intra_enh_RedCap and Tevaluate,NR_Intra_enh_RedCap

| DRX cycle length [s] | Tdetect,NR_Intra_enh_RedCap [s] (number of DRX cycles) | Tmeasure,NR_Intra_enh_RedCap [s] (number of DRX cycles) | Tevaluate,NR_Intra_enh_RedCap [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | [3.2] x M2 ([10] x M2)Note 1 | 0.32 x M3 (1 x M3) Note 1 | 0.96 x M4 (3 x M4) Note 1 |
| 0.64 | [6.4] ([10]) | 0.64 (1) | 1.92 (3) |
| 1.28 | [11.52] ([9]) | 1.28 (1) | 3.84 (3) |
| 2.56 | [64] ([25]) | 2.56 (1) | 7.68 (3) |
| NOTE 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 2, M3 = M4 = 2.5 |  |  |  |

For any requirement in this section, when the UE transitions between any two states when being configured with eDRX_IDLE, changing eDRX_IDLE cycle length, the UE shall meet the transition requirement, which is the less stringent requirement of the two requirements corresponding to the first state and the second state, during the transition time interval which is the time corresponding to the transition requirement. After the transition time interval, the UE shall meet the requirement corresponding to the second state.

If ‘t-Service’ is broadcasted and applicable, UE shall be able to detect, measure, and evaluate neighbour cells before the serving cell stops serving the area regardless of whether the distance condition based on serving cell reference location or serving cell moving reference location is met or the legacy Srxlev/Squal condition are met, and when to start the detection, measurement and evaluation on neighbour cells is up to UE implementation. This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of the last updated value for t-Service is acquired by the UE for the first time to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Ttrigger.

Ttrigger = max(Kmulti_SMTC*Tdetect,NR_Intra, Kmulti_SMTC*Kcarrier* Tdetect,NR_Inter),

where

-	Kcarrier is the number of NR inter-frequency carriers indicated by the serving cell,

-	Tdetect,NR_Intra_RedCap refers to intra-frequency cell detection delay in IDLE/INACTIVE mode defined in tables 4.2C.2.3-1(with FR1) and 4.2E.2.3-1,

-	Tdetect,NR_Inter_RedCap refers to inter-frequency cell detection delay in IDLE/INACTIVE mode defined in table 4.2C.2.4-1(with FR1) and 4.2E.2.4-1.

The requirements in this clause apply provided that the number of SMTCs for intra-frequency carrier does not exceed the parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially for performing the measurements until all of the SMTCs can be measured. The selection of SMTCs to be used is up to UE implementation, and in this case, measurement period longer than the corresponding measurement period specified in tables 4.2C.2.3-1 (with FR1), 4.2E.2.3-1 and table 4.2E.2.3-2 is expected.

4.2E.2.4	Measurements of inter-frequency NR cells for RedCap UE

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the distance between UE and serving cell reference location or serving cell moving reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2E.2.9.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, or the distance between UE and serving cell reference location or serving cell moving reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. The requirements apply provided that the distance exceeds the distanceThresh by a margin of 50 m when referenceLocation is configured by the network or 80 m when movingReferenceLocation is configured by the network. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this clause.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS 38.304 [1] within  Kcarrier*Kmulti_SMTC*Tdetect,NRInter_RedCap if the UE does not support t Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14] or if the enhancedMeasurementLEO-r17 is not enabled, or within Kcarrier*Kmulti_SMTC*Tdetect,NR_Inter_RedCap_enh if the UE supports Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14] and the enhancedMeasurementLEO-r17 is enabled, if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met:

For 2 Rx RedCap by a margin of at least

5 dB in FR1 for reselections based on ranking or

6 dB in FR1 for SS-RSRP reselections based on absolute priorities or

4 dB in FR1 for SS-RSRQ reselections based on absolute priorities

For 1 Rx RedCap by a margin of at least

6 dB in FR1 for reselections based on ranking or

7 dB in FR1 for SS-RSRP reselections based on absolute priorities or

5 dB in FR1 for SS-RSRQ reselections based on absolute priorities.

The parameter Kcarrier is the number of NR inter-frequency carriers indicated by the serving cell.

The parameter Kmulti_SMTC is the scaling factor for measurement of multiple SMTCs or multiple satellites, which refers to clause 4.2C.2.4.

For UE in FR1-NTN:

-	If SMTCs do not overlap with each other,

-	, if GEO satellites are measured on the carrier;

-	, if LEO satellites are measured on the carrier;

-	If SMTCs partially overlap with each other,

-	, if only GEO satellites are measured on the carrier;

-	, if only LEO satellites are measured on the carrier;

where

is the number of LEO satellites to be measured within i-th SMTC,

is the number of LEO satellites that UE can measure in parallel within an SMTC, is the number of SMTCs that partially overlap with each other.

NOTE:	For deriving Kmulti_SMTC for Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter of frequency layer j, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tdetect,NR_Inter, Tmeasure,NR_Inter or Tevaluate,NR_Inter.

An inter-frequency cell is considered to be detectable according to the conditions defined in Annex B.1.7 for a corresponding Band.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure SS-RSRP or SS-RSRQ at least every Kcarrier*Kmulti_SMTC*Tmeasure,NR_Inter_RedCap if the UE does not support Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14] or if the enhancedMeasurementLEO-r17 is not enabled, or every Kcarrier*Kmulti_SMTC*Tmeasure,NR_Inter_RedCap_enh if the UE supports Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14] and the enhancedMeasurementLEO-r17is enabled, for identified lower or equal priority inter-frequency cells. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall filter SS-RSRP or SS-RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter_RedCap/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 38.304 [1] within Kcarrier*Kmulti_SMTC*Tevaluate,NR_Inter_RedCap if the UE does not support Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14] or if the enhancedMeasurementLEO-r17 is not enabled, or within Kcarrier*Kmulti_SMTC*Tevaluate,NR_Inter_RedCap_enh  if the UE supports Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14] and the enhancedMeasurementLEO-r17 is enabled, when Treselection = 0 as specified in tables 4.2C.2.4-1(with FR1) and 4.2E.2.4-1 provided that the reselection criteria is met by

-	the condition when performing equal priority reselection and

-	when rangeToBestCell is not configured:

-	the cell is at least 5 dB better ranked in FR1 for 2 Rx RedCap.

-	the cell is at least 6 dB better ranked in FR1 for 1 Rx RedCap.

-	when rangeToBestCell is configured:

-	the cell has the highest number of beams above the threshold absThreshSS-BlocksConsolidation among all detected cells whose cell-ranking criterion R value defined in TS 38.304 [1] is within rangeToBestCell of the cell-ranking criterion R value of the highest ranked cell.

-	if there are multiple such cells, the cell has the highest rank among them

-	the cell is at least 5 dB better ranked in FR1 if the current serving cell is among them, or 6 dB in FR1 for SS-RSRP reselections based on absolute priorities for 2 Rx RedCap or 4 dB in FR1 for SS-RSRQ reselections based on absolute priorities for 2 Rx RedCap.

-	the cell is at least 6 dB better ranked in FR1 if the current serving cell is among them, or 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities for 1 Rx RedCap.

The 1 Rx RedCap UE applies absThreshSS-BlocksConsolidation as the signalled value of absThreshSS-BlocksConsolidation [2] + 1 dB.

When evaluating cells for reselection, the SSB side conditions apply to both serving and inter-frequency cells.

If Treselection timer has a non-zero value and the inter-frequency cell is satisfied with the reselection criteria, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in table 4.2C.2.4-1(with FR1) under the following conditions:

-	TSMTC_intra = TSMTC_inter = 160 ms; where

-	TSMTC_intra is the periodicity of the SMTC configured for the intra-frequency carrier if no identified intra-frequency cell is in the PCI list of smtc2-LP on this intra-frequency carrier; TSMTC_intra is the periodicity of the smtc2-LP configured for the intra-frequency carrier if at least one identified intra-frequency cell is in the PCI list of smtc2-LP on this intra-frequency carrier. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed for TSMTC_intra. If the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected.

-	TSMTC_inter is the actual SMTC periodicity used by the inter-frequency cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the inter-frequency carrier is assumed for TSMTC_inter. If the actual SSB transmission periodicity is greater than the SMTC configured for the inter-frequency carrier, longer Tdetect, NR_inter is expected.

-	SMTC occasions configured for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the SMTC occasions configured for the intra-frequency carrier, and

-	SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the paging occasion in TS 38.304 [1].

For 1 Rx RedCap and 2 Rx RedCap not configured with eDRX_IDLE cycle, Tdetect,NR_Inter_RedCap, Tmeasure,NR_ Inter _RedCap and Tevaluate,NR_ Inter _RedCap are specified in table 4.2C.2.4-1 (with FR1).

For 1 Rx RedCap and 2 Rx RedCap configured with eDRX_IDLE cycle, Tdetect,NR_ Inter _RedCap, Tmeasure,NR_ Inter _RedCap and Tevaluate,NR_ Inter _RedCap are specified in table 4.2E.2.4-1 for FR1.

For both 1Rx RedCap and 2Rx RedCap, Tdetect,NR_Inter_RedCap_enh ,Tmeasure,NR_Intra_RedCap_enh and Tevaluate,NR_Intra_RedCap_enh refer to table 4.2E.2.4-2.

Table 4.2E.2.4-1: Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap for UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28 s periods) | Tdetect,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| 5.12 | - | - | 117.76 (23) | 5.12 (1) | 10.24 (2) |
| 10.24 | - | - | 235.52 (23) | 10.24 (1) | 20.48 (2) |
| NOTE 1: The number of DRX cycles in this table corresponds to the DRX cycles within PTWs, when PTW is configured. NOTE 2: The eDRX_IDLE cycle lengths are as specified in section 10.5.5.32 of TS 24.008 [42]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56 s, 5.12 s and 10.24 s. NOTE 4: The lower bound of PTW length is derived based on . |  |  |  |  |  |

Table 4.2E.2.4-2: Tdetect,NR_Inter_RedCap_enh, Tmeasure,NR_Inter_RedCap_enh and Tevaluate,NR_Inter_RedCap_enh

| DRX cycle length [s] | Tdetect,NR_Inter_RedCap_enh [s] (number of DRX cycles) | Tmeasure,NR_Inter_RedCap_enh [s] (number of DRX cycles) | Tevaluate,NR_Inter_RedCap_enh [s] (number of DRX cycles) |
| --- | --- | --- | --- |
| 0.32 | [3.84 x M2 ([12] x M2)] Note 1 | [0.32 x M3 ([1] x M3)] Note 1 | 0.96 x M4 (3 x M4) Note 1 |
| 0.64 | [7.68 ([12])] | [0.64 (1)] | 1.92 (3) |
| 1.28 | [12.8 ([10])] | 1.28 (1) | 3.84 (3) |
| 2.56 | [64] ([25]) | 2.56 (1) | 7.68 (3) |
| NOTE 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2 |  |  |  |

If t-Service is broadcasted and applicable, UE shall be able to detect, measure, and evaluate neighbour cells before the serving cell stops serving the area regardless of whether the distance condition based on serving cell reference location or serving cell moving reference location or the legacy Srxlev/Squal condition are met, and when to start detection, measurement, and evaluation is up to UE implementation. This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of the last updated value for t-Service is acquired by the UE for the first time to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Ttrigger, and Ttrigger = max(Kmulti_SMTC*Tdetect,NR_Intra, Kmulti_SMTC*Kcarrier* Tdetect,NR_Inter) when serving cell is below the search threshold, and Ttrigger = max(Kmulti_SMTC*Tdetect,NR_Intra, Nlayer* [60 s]) when serving cell is above the search threshold, where

-	Kcarrier is the number of NR inter-frequency carriers indicated by the serving cell,

-	Nlayer is the total number of higher priority NR carrier frequencies broadcasted in system information,

-	Tdetect,NR_Intra refers to HST intra-frequency cell detection delay in IDLE/INACTIVE mode defined table 4.2.2.3-2,

-	Tdetect,NR_Inter refers to HST inter-frequency cell detection delay in IDLE/INACTIVE mode defined table 4.2.2.4-2.

The requirements in this clause apply provided that the number of SMTCs for any inter-frequency carrier does not exceed the values indicated by parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially until all of the SMTCs can be measured, the selection of SMTCs to be used is up to UE implementation, and longer measurement delay than the corresponding measurement period specified in tables 4.2C.2.4-1 (with FR1), table 4.2E.2.4-1 and table 4.2C.2.4-2 is expected.

The requirements in this clause apply provided that the valid information for the satellite serving the target cell has been provided by the serving cell.

The requirements in this clause apply provided that SSB of neighbour cells are within the time shifted SMTC.

4.2E.2.5	Maximum interruption in paging reception

UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception.

The FDD and HD-FDD RedCap UE shall meet all applicable requirements specified in clause 4.2C.5, except that

-  ‘Tdetect,NR_Intra’ is replaced with ‘Tdetect,NR_Intra_RedCap’ , and

-  ‘Tdetect,NR_Inter’ is replaced with Tdetect,NR_Inter_RedCap , and

-	clause 4.2C.2.3 is replaced with 4.2E.2.3, and

-	clause 4.2C.2.4 is replaced with 4.2E.2.4.

The 1 Rx RedCap in HD-FDD shall meet all applicable requirements specified in clause 4.2C.2.5 under the following conditions

-	at least 1 SSB is available at the UE in the serving cell during the last 160 ms duration.

4.2E.2.6	Minimum requirement at transitions for RedCap UE

The requirements in clause 4.2.2.8 apply provided that target cell’s satellite is GEO.

4.2E.2.7	Measurements of intra-frequency NR cells for RedCap UE configured with relaxed measurement criterion

4.2E.2.7.1	Introduction

This clause contains the requirements for measurements on intra-frequency NR cells when Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ and when the UE is configured any of the following relaxed measurement criteria:

-	Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1],

-	Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4.9.2 in [1],

-	Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4.9.1 and 5.2.4.9.2 in [1] respectively.

4.2E.2.7.2	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

UE is configured with lowMobilityEvaluation [2] criterion and UE is not configured with cellEdgeEvaluation [2] criterion and UE has fulfilled the lowMobilityEvaluation [2] criterion, or

-	UE is configured with both lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] is not configured, and UE has fulfilled only the lowMobilityEvaluation [2] criterion.

The requirements defined in clause 4.2E.2.3 apply for this clause except that:

-	For a UE not configured with eDRX_IDLE, Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax refer to table 4.2B.2.9.9-1 and table 4.2B.2.9.9-2 with FR1 only for 1 Rx RedCap and 2 Rx RedCap respectively.

-	For a UE configured with eDRX_IDLE up-to 10.24 s, Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax refer to table 4.2B.2.9.9-3 and table 4.2B.2.9.9-4 for 1 Rx RedCap and 2 Rx RedCap respectively.

4.2E.2.7.3	Measurements for UE fulfilling not-at-cell edge criterion

This clause contains requirements for measurements on intra-frequency NR cells provided that:

UE is configured with cellEdgeEvaluation [2] criterion and UE is not configured with lowMobilityEvaluation [2] criterion and UE has fulfilled the cellEdgeEvaluation [2] criterion, or

-	UE is configured with both lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criteria and combineRelaxedMeasCondition [2] is not configured, and UE has fulfilled only the cellEdgeEvaluation [2] criterion.

The requirements defined in clause 4.2E.2.3 apply for this clause except that:

-	For a UE not configured with eDRX_IDLE, Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax refer to table 4.2B.2.9.10-1 and table 4.2B.2.9.10-2 with FR1 only for 1 Rx RedCap  and 2 Rx RedCap respectively.

-	For a UE configured with eDRX_IDLE up to 10.24s, Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax refer to table 4.2B.2.9.10-3 and table 4.2B.2.9.10-4 for 1 Rx RedCap and 2 Rx RedCap respectively.

4.2E.2.7.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria

This clause contains requirements for measurements on intra-frequency NR cells provided that:

-	UE is configured with both lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion, and

-	both criteria are fulfilled, and

-	less than 1 hour have passed since measurements for cell reselection were performed

In this case the UE is not required to meet Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap as defined in table 4.2E.2.3.

4.2E.2.8	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion

4.2E.2.8.1	Introduction

This clause contains the requirements for measurements on inter-frequency NR cells when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ and when the UE is configured any of the following relaxed measurement criteria:

-	Relaxed measurement criterion for UE with low mobility defined in clause 5.2.4.9.1 in [1],

-	Relaxed measurement criterion for UE not-at-cell edge defined in clause 5.2.4. 9.2 in [1],

-	Both low mobility criterion and not-at-cell edge criterion as defined in clauses 5.2.4. 9.1 and 5.2.4.9.2 in [1] respectively.

4.2E.2.8.2	Measurements for UE fulfilling low mobility criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	UE is configured with lowMobilityEvaluation [2] criterion and UE is not configured with cellEdgeEvaluation [2] criterion and UE has fulfilled the lowMobilityEvaluation [2] criterion, or

-	UE is configured with both lowMobilityEvaluation [2] and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] is not configured, and UE has fulfilled only the lowMobilityEvaluation [2] criterion.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements are defined as follows:

-	For a UE not configured with eDRX_IDLE, Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax refer to table 4.2B.2.10.9-1 and table 4.2B.2.10.9-2 for 1 Rx RedCap and 2 Rx RedCap respectively

-	For a UE configured with eDRX_IDLE up to 10.24s, Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax refer to table 4.2B.2.10.9-3 and table 4.2B.2.10.9-4 for 1 Rx RedCap and 2 Rx RedCap respectively.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and the UE is configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2B.2.7 and, K2 = 60. Otherwise, if the UE is not configured with highPriorityMeasRelax [2] then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2E.2.9.

4.2E.2.8.3	Measurements for UE fulfilling not-at-cell edge criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	UE is configured with cellEdgeEvaluation [2] criterion, and UE is not configured with lowMobilityEvaluation [2] criterion and UE has fulfilled the cellEdgeEvaluation [2] criterion or

-	UE is configured with both lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion and combineRelaxedMeasCondition [2] is not configured, and UE has fulfilled only the cellEdgeEvaluation [2] criterion.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the requirements defined in clause 4.2B.2.4 apply for this clause except that:

-	For a UE not configured with eDRX_IDLE, Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax are as specified in table 4.2B.2.10.10-1 and table 4.2B.2.10.10-2 with FR1 only for 1 Rx RedCap and 2 Rx RedCap respectively.

-	For a UE configured with eDRX_IDLE up to 10.24s, Tdetect,NR_Inter_RedCap_Relax, Tmeasure,NR_Inter_RedCap_Relax and Tevaluate,NR_Inter_RedCap_Relax are as specified in table 4.2B.2.10.10-3 and table 4.2B.2.10.10-4 for 1 Rx RedCap and 2 Rx RedCap respectively.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ and regardless of whether the UE is configured with highPriorityMeasRelax [2] or not, the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2E.2.9.

4.2E.2.8.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion

This clause contains requirements for measurements on inter-frequency NR cells provided that:

-	UE is configured with both lowMobilityEvaluation [2] criterion and cellEdgeEvaluation [2] criterion, and has fulfilled both criteria, and

-	less than 1 hour have passed since measurements for cell reselection were last performed

In this case the UE is not required to meet Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap as defined in clause 4.2E.2.4.

When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, the UE shall search for, measure and evaluate inter-frequency layers of higher, equal or lower priority at least every 1 hour.

When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, the UE shall search for inter-frequency layers of higher priority at least every K2*Thigher_priority_search where Thigher_priority_search is described in clause 4.2E.2.9 and K2=60.

The requirements defined in this clause apply regardless of eDRX_IDLE configurations.

4.2E.2.9	General requirements

When configured with eDRX_IDLE, the UE shall search every layer of higher priority at least every Thigher_priority_search, where Thigher_priority_search is defined as follows:

-	Thigher_priority_search = max(60, 15*eDRX_IDLE cycle length) * Nlayers seconds if eDRX_IDLE cycle ≤ 10.24 s,

Otherwise, if the UE is not configured with eDRX_IDLE cycle, the UE shall search every layer of higher priority at least every Thigher_priority_search = (60 * Nlayers) seconds.

Nlayers is the total number of higher priority NR and E-UTRA carrier frequencies broadcasted in system information.

For a UE configured with early measurement reporting, while T331 is running, Nlayers is the combined total number of higher priority NR and E-UTRA carrier frequencies broadcasted in system information.

4.2E.2.10	Measurements of inter-frequency NR cells with TN carrier

This clause applies for the inter-frequency cell re-selection for TN carriers, and NTN carriers if configured. The requirements in clause 4.2E.2.10 apply provided that network provides SIB19 and UE is configured with and one or more TN carriers.

UE is allowed to skip TN neighbour cells measurement in an area where there is no coverage of the frequency based on the provided TN cell coverage information and UE GNSS position information. Otherwise, UE shall perform TN measurement if its estimated distance to tn-ReferenceLocation is smaller than tn-DistanceRadius. The requirements apply provided that the actual distance between UE to tn-ReferenceLocation is smaller than tn-DistanceRadius – 50m.This clause considers the inter-frequency cell reselection from NTN to TN in FR1.

The UE shall be able to identify new inter-frequency cells and perform SS-RSRP or SS-RSRQ measurements of identified inter-frequency cells if carrier frequency information is provided by the serving cell, even if no explicit neighbour list with physical layer cell identities is provided.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the distance between UE and serving cell reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2E.2.9.

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, or the distance between UE and serving cell reference location is larger than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for and measure inter-frequency layers of higher, equal or lower priority in preparation for possible reselection. The requirements apply provided that the distance exceeds the distanceThresh by a margin of 50 m. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority layers shall be the same as that defined below in this clause.

The UE shall be able to evaluate whether a newly detectable inter-frequency cell meets the reselection criteria defined in TS 38.304 [1] within Kcarrier_TN * Tdetect,NR_Inter_TN + Kcarrier*Kmulti_SMTC*Tdetect,NR_Inter_NTN_RedCap if the UE does not support Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14]  or if the enhancedMeasurementLEO-r17 is not enabled, or within Kcarrier_TN * Tdetect,NR_Inter_TN + Kcarrier*Kmulti_SMTC*Tdetect,NR_Inter_NTN_RedCap_enh if the UE supports Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14]  and the enhancedMeasurementLEO-r17 is enabled, if at least carrier frequency information is provided for inter-frequency neighbour cells by the serving cells when Treselection = 0 provided that the reselection criteria is met:

For 2 Rx RedCap by a margin of at least

5 dB in FR1 for reselections based on ranking or

6 dB in FR1 for SS-RSRP reselections based on absolute priorities or

4 dB in FR1 for SS-RSRQ reselections based on absolute priorities

For 1 Rx RedCap by a margin of at least

6 dB in FR1 for reselections based on ranking or

7 dB in FR1 for SS-RSRP reselections based on absolute priorities or

5 dB in FR1 for SS-RSRQ reselections based on absolute priorities.

The parameter Kmulti_SMTC is the scaling factor for measurement of multiple SMTCs or multiple satellites

-	If SMTCs do not overlap with each other,

-	, if GEO satellites are measured on the carrier;

-	, if LEO satellites are measured on the carrier;

-	If SMTCs partially overlap with each other,

-	, if only GEO satellites are measured on the carrier;

-	, if only LEO satellites are measured on the carrier;

where

is the number of LEO satellites to be measured within i-th SMTC,

is the number of LEO satellites that UE can measure in parallel within an SMTC, is the number of SMTCs that partially overlap with each other.

NOTE:	For deriving Kmulti_SMTC for Tdetect,NR_Inter, Tmeasure,NR_Inter and Tevaluate,NR_Inter of frequency layer j, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tdetect,NR_Inter, Tmeasure,NR_Inter or Tevaluate,NR_Inter.

The parameter Kcarrier_TN is the number of NR TN inter-frequency carriers indicated by the serving cell, except for the frequency carrier where there is no coverage of that frequency based on the provide TN cell coverage information and UE GNSS position information.

Tdetect,NR_Inter_TN ,Tmeasure,NR_Inter_TN and Tevaluate,NR_Inter_TN are the NR TN inter-frequency cell re-selection requirement defined in table 4.2.2.4-1 in TS 38.133

Tdetect,NR_Inter_NTN_RedCap ,Tmeasure,NR_Inter_NTN_RedCap and Tevaluate,NR_Inter_NTN-RedCap are the NR NTN inter-frequency cell re-selection requirement defined in tables 4.2C.2.4-1 (with FR1) and 4.2C.2.4-1 in TS 38.133.

Tdetect,NR_Inter_NTN_RedCap_enh, Tmeasure,NR_Inter_NTN_RedCap_enh and Tevaluate,NR_Inter_NTN_RedCap_enh are the NR NTN inter-frequency cell re-selection requirement defined in table 4.2X.2.4-2 in TS 38.133.

An inter-frequency cell is considered to be detectable according to the conditions defined in Annex B.1.7 for a corresponding band.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter. If, after detecting a cell in a higher priority search, it is determined that re-selection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of re-selection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall measure SS-RSRP or SS-RSRQ at least every Kcarrier_TN * Tmeasure,NR_Inter_TN + Kcarrier*Kmulti_SMTC*Tmeasure,NR_Inter_NTN_RedCap if the UE does not support Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14]  or if the enhancedMeasurementLEO-r17 is not enabled, or every  Kcarrier_TN * Tmeasure,NR_Inter_TN + Kcarrier*Kmulti_SMTC*Tmeasure,NR_Inter_NTN_RedCap_enh if the UE supports Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14]  and the enhancedMeasurementLEO-r17is enabled, for identified lower or equal priority inter-frequency cells. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall filter SS-RSRP or SS-RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter_NTN_RedCap_enh/2.

The UE shall not consider a NR neighbour cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For an inter-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the inter-frequency cell has met reselection criterion defined TS 38.304 [1] within  Kcarrier_TN * Tevaluate,NR_Inter_TN + Kcarrier*Kmulti_SMTC*Tevaluate,NR_Inter_NTN_RedCap if the UE does not support Enhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14] or if the enhancedMeasurementLEO-r17 is not enabled, or within  Kcarrier_TN * Tevaluate,NR_Inter_TN + Kcarrier*Kmulti_SMTC*Tevaluate,NR_Inter_NTN_RedCap_enh  if the UE supportsEnhanced RRM requirements for measurements in IDLE and INACTIVE modes defined in TS 38.306 [14]  and the enhancedMeasurementLEO-r17 is enabled, when Treselection = 0 as specified in tables 4.2C.2.4-1 (with FR1) and table 4.2E.2.3-1 provided that the reselection criteria is met by

-	For 2 Rx RedCap by a margin of at least

5 dB in FR1 for reselections based on ranking or

6 dB in FR1 for SS-RSRP reselections based on absolute priorities or

4 dB in FR1 for SS-RSRQ reselections based on absolute priorities

For 1 Rx RedCap by a margin of at least

6 dB in FR1 for reselections based on ranking or

7 dB in FR1 for SS-RSRP reselections based on absolute priorities or

5 dB in FR1 for SS-RSRQ reselections based on absolute priorities.

When evaluating cells for reselection, the SSB side conditions apply to both serving and inter-frequency cells.

If Treselection timer has a non-zero value and the inter-frequency cell is satisfied with the reselection criteria, the UE shall evaluate this inter-frequency cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect that cell.

The UE is not expected to meet the measurement requirements for an inter-frequency carrier under DRX cycle=320 ms defined in table 4.2C.2.4-1 under the following conditions:

TSMTC_intra = TSMTC_inter = 160 ms; where

-	TSMTC_intra is the periodicity of the SMTC configured for the intra-frequency carrier if no identified intra-frequency cell is in the PCI list of smtc2-LP on this intra-frequency carrier; TSMTC_intra is the periodicity of the smtc2-LP configured for the intra-frequency carrier if at least one identified intra-frequency cell is in the PCI list of smtc2-LP on this intra-frequency carrier. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed for TSMTC_intra. If the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected.

-	TSMTC_inter is the actual SMTC periodicity used by the inter-frequency cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the inter-frequency carrier is assumed for TSMTC_inter. If the actual SSB transmission periodicity is greater than the SMTC configured for the inter-frequency carrier, longer Tdetect, NR_inter is expected.

-	SMTC occasions configured for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the SMTC occasions configured for the intra-frequency carrier, and

-	SMTC occasions configured for the intra-frequency carrier and for the inter-frequency carrier occur up to 1 ms before the start or up to 1 ms after the end of the paging occasion in TS 38.304 [1].

If t-Service is broadcasted and applicable, UE shall be able to detect, measure, and evaluate neighbour cells before the serving cell stops serving the area regardless of whether the distance condition based on serving cell reference location or the legacy Srxlev/Squal condition are met, and when to start detection, measurement, and evaluation is up to UE implementation. This requirement does not apply when the time span from the last slot of SI transmission within SI modification period where the broadcasting of the last updated value for t-Service is acquired by the UE for the first time to the first slot when the cell is scheduled to stop serving the area according to the broadcasted information is less than Ttrigger, and Ttrigger = max(Tdetect,NR_Intra, Kcarrier* Tdetect,NR_Inter) when serving cell is below the search threshold, and Ttrigger = max(Tdetect,NR_Intra, Nlayer* [60s]) when serving cell is above the search threshold, where

-	Kcarrier is the number of NR inter-frequency carriers indicated by the serving cell,

-	Nlayer is the total number of higher priority NR carrier frequencies broadcasted in system information,

-	Tdetect,NR_Intra refers to HST intra-frequency cell detection delay in IDLE/INACTIVE mode defined table 4.2.2.3-2,

-	Tdetect,NR_Inter refers to HST inter-frequency cell detection delay in IDLE/INACTIVE mode defined table 4.2.2.4-2.

The requirements in this clause apply provided that the number of SMTCs for any inter-frequency carrier does not exceed the values indicated by parallelSMTC-r17, otherwise UE may select one or subset of all the configured SMTCs sequentially until all of the SMTCs can be measured, the selection of SMTCs to be used is up to UE implementation, and longer measurement delay than the corresponding measurement period specified in table 4.2C.2.4-1 and table 4.2E.2.4-2 is expected.

For 1 Rx RedCap and 2 Rx RedCap not configured with eDRX_IDLE cycle, Tdetect,NR_Inter_RedCap, Tmeasure,NR_ Inter _RedCap and Tevaluate,NR_ Inter _RedCap are specified in table 4.2C.2.4-1 with FR1.

For 1 Rx RedCap and 2 Rx RedCap configured with eDRX_IDLE cycle, Tdetect,NR_ Inter _RedCap, Tmeasure,NR_ Inter _RedCap and Tevaluate,NR_ Inter _RedCap are specified in table 4.2E.2.4-1 for FR1.

For both 1Rx RedCap and 2Rx RedCap, Tdetect,NR_Inter_RedCap_enh ,Tmeasure,NR_Intra_RedCap_enh ,Tevaluate,NR_Intra_RedCap_enh are defined in table 4.2E.2.4-2.

4.2E.2.11	Measurements of inter-RAT E-UTRAN cells with TN carrier

This clause applies for the inter-RAT cell re-selection for TN carriers. The requirements in clause 4.2C.2.11 apply provided that network provides SIB19 and UE is configured with and one or more TN carriers.

UE is allowed to skip TN neighbour cells measurement in an area where there is no coverage of the frequency based on the provided TN cell coverage information and UE GNSS position information.  Otherwise, UE shall perform TN measurement if its estimated distance to tn-ReferenceLocation is smaller than tn-DistanceRadius. The requirements apply provided that the actual distance between UE to tn-ReferenceLocation is smaller than tn-DistanceRadius – 50m.This clause considers the inter-RAT cell reselection from NTN to E-UTRAN TN in FR1.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2E.2.9

If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ then the UE shall search for and measure inter-RAT E-UTRAN layers of higher, lower priority in preparation for possible reselection. In this scenario, the minimum rate at which the UE is required to search for and measure higher priority inter-RAT E-UTRAN layers shall be the same as that defined below for lower priority RATs.

The requirements in this clause apply to inter-RAT E-UTRAN FDD measurements and E-UTRA TDD measurements. When the measurement rules indicate that inter-RAT E-UTRAN cells are to be measured, the UE shall measure RSRP and RSRQ of detected E-UTRA cells in the neighbour frequency list at the minimum measurement rate specified in this clause.

The parameter NEUTRA_carrier is the number of EUTRA TN carriers indicated by the serving cell, except for the frequency carrier where there is no coverage of that frequency based on the provide TN cell coverage information and UE GNSS position information.

The UE shall filter RSRP and RSRQ measurements of each measured E-UTRA cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least half the minimum specified measurement period.

An inter-RAT E-UTRA cell is considered to be detectable provided the following conditions are fulfilled:

-	the same conditions as for inter-frequency RSRP measurements specified in TS 36.133 [15, Annex B.1.2] are fulfilled for a corresponding Band, and

-	the same conditions as for inter-frequency RSRQ measurements specified in TS 36.133 [15, Annex B.1.2] are fulfilled for a corresponding Band.

-	SCH conditions specified in TS 36.133 [15, Annex B.1.2] are fulfilled for a corresponding Band

The UE shall be able to evaluate whether a newly detectable inter-RAT E-UTRAN cell meets the reselection criteria defined in TS 38.304 [1] within NEUTRA_carrier * Tdetect,EUTRAN when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ when Treselection = 0 provided that the reselection criteria is met by a margin of at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities.

Cells which have been detected shall be measured at least every NEUTRA_carrier * Tmeasure,EUTRAN when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ.

When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,EUTRAN. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell.

If the UE detects, on an inter-RAT E-UTRAN carrier, a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

The UE shall not consider an inter-RAT E-UTRA cell in cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

For a cell that has been already detected, but has not been reselected to, the filtering shall be such that a UE not configured with eDRX_IDLE cycle shall be capable of evaluating that an already identified inter-RAT E-UTRA cell has met reselection criterion defined in TS 38.304 [1] within NEUTRA_carrier_RedCap * Tevaluate,EUTRAN_RedCap when Treselection = 0 as specified in table 4.2.2.5-1provided that the reselection criteria is met by a margin of at least 6 dB for RSRP reselections based on absolute priorities or 4 dB for RSRQ reselections based on absolute priorities for 2Rx RedCap and at least 6dB for RSRP reselections based on absolute priorities or 4dB for RSRQ reselections based on absolute priorities for 1 Rx RedCap.

If the Treselection timer has a non-zero value and an inter-RAT E-UTRA cell satisfies the reselection criteria defined in TS 38.304 [1], the UE shall evaluate this E-UTRA cell for the Treselection time. If this cell remains satisfied with the reselection criteria within this duration, then the UE shall reselect to this cell.

When the distance between the UE and tn-ReferenceLocation is larger than tn-DistanceRadius +50m, the UE is allowed to not perform measurements on the TN frequency in the corresponding area.

For 1 Rx RedCap and 2 Rx RedCap configured with eDRX_IDLE cycle, Tdetect,EUTRAN_RedCap, Tmeasure,EUTRAN_RedCap and Tevaluate, E-UTRAN_RedCap are specified in table 4.2B.2.5-2.

For UE not configured with eDRX_IDLE cycle, Tdetect,EUTRAN_RedCap, Tmeasure,EUTRAN_RedCap and Tevaluate, E-UTRAN_RedCap are specified in table 4.2B.2.5-1.