3GPP TSG-RAN4 Meeting #116          		R4-2512353

Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.191 | CR |  | rev |  | Current version: | 0.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big TP on RRM requirements for Ambient IoT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-09-02 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This big TP includes following endorsed Tdocs at RAN4#116 meeting. Tdoc; Title; Source company. R4-2512235; TP for TS 38.191 to introduce CBRA correct behaviour when transmitting A-IoT MSG1; CATT. R4-2512236; draftCR on Rel19 AIoT (9.1.2.3 CBRA: Correct behaviour when not receiving A-IoT MSG2); Xiaomi. R4-2512237; DraftCR on D2R Timing requirements; CMCC. R4-2510606; Draft CR on CFA RRM requirements for A-IoT; Huawei, HiSilicon. R4-2512238; CBRA: Correct behaviour when receiving A-IoT MSG2; ZTECorporation,Sanechips. R4-2512239; Draft CR on A-IoT random access; Ericsson. |
| Summary of change: |  | Introduce RRM core requirements for Ambient IoT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM core requirements for Ambient IoT are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1, 9.2, 9.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of 1st change >>

9.1	Random access

9.1.1	Introduction of random access

This clause contains requirements for A-IoT device(s) random access procedure. The random access procedure is specified in clause 5.3 of TS 38.391, and the D2R physical layer transmission is specified in clause 6.1 of TS 38.291. The requirements for the CBRA type procedure are described in clause 9.1.2, whereas the requirements for the CFA type procedure are described in clause 9.1.3 of this specification.

## << End of 1st change >>

<< Start of 2nd change >>

9.1.2 	Requierments for contention based random access

9.1.2.1	Correct behaviour when transmitting A-IoT Msg1

When an A-IoT device is indicated to perform CBRA by the A-IoT Paging message according to the procedure described in clause 5.2 in TS 38.391, the A-IoT device shall have the capability to randomly select an access occasion among access occasions configured in the A-IoT paging message according to the procedure described in clause 5.3.1.1 and shall have the capability to transmit the A-IoT Msg1 (i.e., random ID message) on this selected access occasion according to the procedure described in clause 5.3.1.2 in TS 38.391.

## << End of 2nd change >>

<< Start of 3rd change >>

9.1.2.2 	CBRA: Correct behaviour when receiving A-IoT MSG2

The UE shall again perform the Random Access Resource selection procedure defined clause 5.3.1 in TS 38.391.

## << End of 3rd change >>

<< Start of 4th change >>

9.1.2.3	CBRA: Correct behaviour when not receiving A-IoT MSG2

The UE shall again perform the random access selection procedure defined in clause 5.3.1 in TS 38.391.

## << End of 4th change >>

<< Start of 5th change >>

9.1.3	Requirements for contention free access

9.1.3.1	CFA: Correct behaviour when transmitting First D2R message

Upon reception of A-IoT Paging message, if the RA type is indicated as CRA in paging message, UE shall initiate contention-free access, and initiate D2R message transmission, as defined in TS 38.391 clause 5.2 and 5.3.2.

## << End of 5th change >>

<< Start of 6th change >>

9.2	D2R timing

9.2.1 	D2R transmit timing

For D2R transmission, the reference point for the device transmission timing is TR→D after the end of the corresponding R2D transmission for the device as define in TS 38.291. The device transmission timing error shall be less than or equal to Te.  The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last transition edge of PRDCH intended for the device in the corresponding R2D transmission to the starting time of the D2R transmission.

9.3	TD2R_min

A device is not required to monitor a corresponding R2D transmission earlier than TD2R_min after the end of a D2R transmission from the device. The value of TD2R_min is max {3* TchipD2R, 10 us}, where TchipD2R is the duration of a D2R chip as specified in TS38.291.

## << End of 6th change >>