3GPP TSG-RAN WG4 Meeting #116	R4-2512522

# Bangaluru, India, August 25th – 29th 2025

Agenda item:	7.12.2

Source:	Ericsson

Title:	TP for TR38.753: Content of chapter 7

Document for:	Endorsement

# Introduction

In RAN4#115 meeting, alignment simulation parameters were discussed, and preliminary results were delivered based on candidate SCM models.  In this contribution, further changes are provided to remove extra SCM model and add potential metrics for 8 layers.

# References

[1] 	R4-2508776, Way Forward for [115][321] FS_NR_demod_SCM, Nokia

[2]	R4-2508621, Summary of alignment simulation for FS_NR_demod_SCM, Apple, Ericsson

[3]	R4-2508622, Simulation assumptions for SCM, Huawei, MTK

[4]	R4-2510711, Discussion on NR SCM general aspects, Ericsson

Text Proposal

# 7	Alignment of Spatial Channel Models

Simulation results and analysis of different test cases with different channel models are captured in chapter 6 for model comparison. Chapter 7 for alignment is to capture companies’ simulation results on agreed performance metric per channel model and also the span and average value of companies results.

The following alignment test cases are included in this chapter:

FR1 SU-MIMO PDSCH 4Tx4Rx with 4 layers. The SNR of 30% and 70% maximum throughput are captured. The detailed parameter assumptions can be found in Table 6.2-1 and 6.2-2.

FR1 SU-MIMO PDSCH 8Tx8Rx with 8 layers. The SNR of 30% and 70% maximum throughput per codeword are captured. The detailed parameter assumptions can be found in Table 6.2-1 and 6.2-2.

FR1 SU-MIMO PMI 8Tx4Rx with 4 layers and Type-I codebook. The SNR of 70% and 90% maximum throughput are captured. The detailed parameter assumptions can be found in Table 6.1-1 and 6.1-2.

FR1 SU-MIMO PMI 8Tx4Rx with 4 layers and eType-II codebook. The SNR of 70% and 90% maximum throughput are captured. The detailed parameter assumptions can be found in Table 6.1-1 and 6.1-2.

There are two Spatial Channel Models, one CDL based model, known as rCDL-C1 (reduced CDL-C) and one TDL based model known as xTDL-C1 (extended TDL-C) which are captured for alignment simulation. Details of these models are in chapters 5.1 and 5.2 respectively.

The BS antenna configuration for CDL alignment is one antenna element per subarray.

4Tx case: (M, N, P, Ms, Ns) = (1, 2, 2, 1, 1).

8Tx case: (M, N, P, Ms, Ns) = (1, 4, 2, 1, 1).

The Doppler shift configurations are 3km/h for CDL based model and 10Hz for TDL based model.

## 7.1		rCDL-C1 model results alignment

Table 7.1-1 Simulation result summary for FR1 SU-MIMO PDSCH 4Tx4Rx with 4 layers

| SNR at Norm. Throughput [dB] | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Source #6 | Source #7 | Source #8 | Average | Span |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30% | 8.5 |  | 6 |  | 6.4 |  | 6.6 |  | 6.4 |  | 7 |  | 5.9 |  | 6.7 |  | 6.7 |  | 2.6 |  |
| 70% | 15.5 |  | 14.3 |  | 16.5 |  | 15.6 |  | 15.9 |  | 16.6 |  | 14.7 |  | 15.6 |  | 15.6 |  | 2.3 |  |

Observations for rCDL-C1

- 7 out of 8 sources could achieve SNR span < 2.5dB at both 30% and 70% normalized throughput.

Table 7.1-2 Simulation result summary for FR1 SU-MIMO PDSCH 8Tx8Rx with 8 layers

| SNR at Norm. Throughput [dB] |  |  | Source #1 |  | Source #2 |  | Source #3 |  | Source #4 |  | Source #5 |  | Source #6 |  | Source #7 |  | Average |  |  | Span |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CW1 | 30% | 6.4 |  | 4.9 |  | 4.8 |  | 6.1 |  | 6.8 |  | 6.9 |  | 6.3 |  | 6.0 |  | 2.1 |
|  | 70% | 16.9 |  | 14.2 |  | 16.5 |  | 16.7 |  | 17.5 |  | 17.2 |  | 18.5 |  | 16.8 |  | 4.3 |
| CW2 | 30% | 10.1 |  | 15.7 |  | 10 |  | 9.9 |  | 11.6 |  | 12.3 |  | 12.4 |  | 11.7 |  | 5.8 |
|  | 70% | 23.7 |  | 26 |  | 24.4 |  | 24 |  | 24.7 |  | 23.4 |  | 24.4 |  | 24.4 |  | 2.6 |

Observation for rCDL-C1:

- 6 out of 7 sources could achieve SNR span <2.5dB for both CW1 and CW2 at both 30% and 70% normalized throughput.

Table 7.1-3 Simulation result summary for FR1 SU-MIMO PMI 8Tx4Rx with 4 layers, follow PMI

| SNR at Norm. Throughput [dB] |  |  | Source #1 |  |  | Source #2 |  |  | Source #3 |  |  | Source #4 |  |  | Source #5 |  |  | Source #6 |  |  | Source #7 |  |  | Source #8 |  |  | Source #9 |  |  | Average |  |  | Span |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Type-I | 70% |  | 12.4 |  |  | 11.0 |  |  | 9.7 |  |  | 10.3 |  |  | 10.2 |  |  | 9.7 |  |  | 9.5 |  |  | 10.2 |  |  | 9.8 |  |  | 10.4 |  |  | 2.9 |  |  |
|  | 90% |  | 14.4 |  |  | 13.1 |  |  | 12.5 |  |  | 12.8 |  |  | 12.6 |  |  | 12.6 |  |  | 12.0 |  |  | 12.5 |  |  | 12.3 |  |  | 12.7 |  |  | 2.4 |  |  |
| eType-II |  | 70% |  | 14.0 |  |  | 14.3 |  |  | 8.4 |  |  | 11.6 |  |  | 10.1 |  |  | 10.8 |  |  | 8.2 |  |  | 13.8 |  |  | 10.8 |  |  | 11.5 |  |  | 6.1 |  |
|  |  | 90% |  | 15.9 |  |  | 17.1 |  |  | 12.3 |  |  | 14.0 |  |  | 12.6 |  |  | 13.9 |  |  | 10.6 |  |  | 16.8 |  |  | 12.9 |  |  | 14.0 |  |  | 6.5 |  |

Observation for rCDL-C1:

- In type-I codebook case, 8 out of 9 sources could achieve SNR span < 2.5dB at 70% and 90% normalized throughput.

Observation for rCDL-C1:

- In eType-II codebook case, three clusters of results can be observed:

- Cluster 1: include source #3, #7 are in 70% normalized throughput and source #7 in 90% normalized throughput.

- Cluster 2: include source #4, #5, #6, #9 in 70% normalized throughput and source #3, #4, #5, #6, #9 in 90% normalized throughput. The span of this cluster < 2.5dB for both 70% and 90% normalized throughput.

- Cluster 3: include source #1, #2, #8. The span of this cluster < 2.5dB for both 70% and 90% normalized throughput.

## 7.2		xTDL-C1 model results alignment

Table 7.3-1 Simulation result summary for FR1 SU-MIMO PDSCH 4Tx4Rx with 4 layers

| SNR at Norm. Throughput [dB] | Source #1 | Source #2 | Source #3 | Source #4 | Source #5 | Average | Span |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 30% | 5.3 | 5.8 | 6 | 6 | 5.7 | 5.8 | 0.7 |
| 70% | 13.5 | 15.9 | 15.4 | 15.8 | 14 | 14.9 | 2.4 |

Observation for xTDL-C1:

- 5 out of 5 sources could achieve SNR span < 2.5dB for both 30% and 70% normalized throughput.

Table 7.3-2 Simulation result summary for FR1 SU-MIMO PDSCH 8Tx8Rx with 8 layers

| SNR at Norm. Throughput [dB] |  | Source #1 | Source #2 | Source #3 | Source #4 | Average | Span |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CW1 | 30% | 4 | 2.3 | 5.7 | 7.1 | 4.8 | 4.8 |
|  | 70% | 14.5 | 11.1 | 14.8 | 15.4 | 14 | 4.3 |
| CW2 | 30% | 10.8 | 11 | 11 | 8.4 | 10.3 | 2.6 |
|  | 70% | 21.2 | 19.7 | 20.4 | 16.6 | 19.5 | 4.6 |

Observation for xTDL-C1:

- For CW1 at 30% normalized throughput, 4 sources results are almost evenly distributed with ~1.5dB offset.

- 3 out of 4 sources could achieve SNR span < 2.5dB for CW1 at 70% normalized throughput.

- 3 out of 4 sources could achieve SNR span < 2.5dB for CW2 at both 30% and 70% normalized throughput.

Table 7.3-3 Simulation result summary for FR1 SU-MIMO PMI 8Tx4Rx with 4 layers, follow PMI

| SNR at Norm. Throughput [dB] |  | Source #1 | Source #2 | Source #3 | Source #4 | Average | Span |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Type-I | 70% | 12.3 | 13.0 | 10.8 | 11.3 | 11.9 | 2.2 |
|  | 90% | 14.0 | 15.0 | 13.6 | 12.8 | 13.9 | 2.2 |
| eType-II | 70% | 13.5 | 14.7 | 11.6 | 10.9 | 12.7 | 3.8 |
|  | 90% | 17.0 | 16.8 | 17.3 | 12.4 | 15.9 | 4.9 |

Observation for xTDL-C1:

- In type-I codebook case, 4 out of 4 sources could achieve SNR span< 2.5dB at both 70% and 90% normalized throughput.

Observation for xTDL-C1:

- In eType-II codebook case, there are two clusters of result that can be observed at 70% normalized throughput. Sources #1 and #2 are in one cluster and sources #3 and #4 are in another cluster. The span of each cluster could achieve SNR span< 2.5dB. 3 out of 4 sources could achieve SNR span< 2.5dB at 90% normalized throughput.