# 3GPP TSG-RAN WG4 Meeting #116	R4-2512543

# Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | DraftCR | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on restriction of simultaneous operation of FR1-NTN and FR2-NTN numerology for Ku band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-8-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In RAN#107 meeting, there was an updated on R19 WI NR_NTN_Ku_bands (RP-250799). The restriction was added by the note that simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE. We think this restriction should be reflected in the specification. In the last meeting, we discussed how to capture this note but there was no agreement (see WF R4-2508640). In this draft CR, we provide a wording to address this issue. Tx-Rx frequency seperation agreed in R4-2508640 is not included in any draftCR yet |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add restriction for simultaneous operation of FR1-NTN and FR2-NTN band is not supported in the UE. - Add Tx-Rx frequency seperation as agreed in R4-2508640. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Simultaneous operation of FR1-NTN and FR2-NTN numerology is allowed in the UE for Ku band. Tx-Rx frequency seperation not defined |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.5, 5.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.3.5	UE channel bandwidth per operating band

The requirements in this specification apply to the combination of channel bandwidths, SCS and operating bands shown in Table 5.3.5-1 for FR1-NTN and table 5.3.5-2 for FR2-NTN. The transmission bandwidth configuration in Table 5.3.2-1 and Table 5.3.2-2 shall be supported for each of the specified channel bandwidths. The channel bandwidths are specified for both the Tx and Rx path. For UE that implement both FR1-NTN and FR2-NTN bands for the same RF frequencies, simultaneous operation is not supported.

< Unchanged sections omitted >

### 5.4.4	TX–RX frequency separation

The default TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation for operating bands is specified in Table 5.4.4-1 for FR1-NTN.

Table 5.4.4-1: UE TX-RX frequency separation (FR1-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n256 | 190 MHz1 165 to 215 MHz2 |
| n255 | -101.5 MHz1 -72.5 to -130.5 MHz2 |
| n254 | 862 – 885 MHz |
| n252 | 180 MHz1 165 to 195 MHz2 |
| n248 | -3 795 to -1 255 MHz |
| n247 | -3 295 to -1 005 MHz |
| NOTE 1: Default Tx-Rx separation NOTE 2: The verification of flexible Tx-Rx frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3.2 |  |

Table 5.4.4-2: UE TX-RX frequency separation (FR2-NTN)

| NTN Satellite Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| n509 | -3 750 to -1 300 MHz |
| n508 | -3 250 to -1 050 MHz |