3GPP TSG-RAN WG4 Meeting #116                                                        R4-2512574

# Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.941 | CR |  | rev | 1 | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TR 37.941: Impact of BS-to-BS isolation analysis on the legacy co-location requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BS_RF_req_evo-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To capture the impacts of BS-to-BS isolation analysis on the legacy co-location requirements |  |  |  |  |  |  |  |  |
| Summary of change: |  | To capture the impacts of BS-to-BS isolation analysis on the legacy co-location requirements: - Transmitter OFF power; 2) Co-location spurious emission requirement/Protection of BS receiver of own or different BS 3)Transmitter intermodulation requirement 4)Co-located OOBB requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some background information of co-location requirement is missing in this TR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the change>

6.4.4.3	The impact of BS-to-BS isolation analysis on the co-location requirements

6.4.4.3.1	General

The co-location requirements are requirements which are based on a given BS-to-BS co-location scenario, for BS type 1-C and 1-H it is assumed that this scenario has an MCL of 30dB. The intention with these requirements is to ensure that BSs serving different bands can be co-sited at the same site and/or tower.

Table 6.4.4.3.1-1: Overview of NR co-location requirements

| Requirement | Reference to clauses in NR BS specification TS 38.104 | Description |
| --- | --- | --- |
| Transmitter spurious co-location emission | BS type 1-C and BS type 1-H in Clause 6.6.5.2.4. BS type 1-O in Clause 9.7.5.2.5. | This requirement is defined to protect BS operating in other 3GPP band(s) co-located at the same site. |
| Receiver out-of-band blocking | BS type 1-C and BS type 1-H in Clause 7.5.3. BS type 1-O in Clause 10.6.2.2. | This requirement is defined to guarantee UL operation when a BS is co-located to another BS operating at another 3GPP band(s). |
| Transmitter OFF power | BS type 1-C in Clause 6.4.1.2. BS type 1-H in Clause 6.4.1.3. BS type 1-O is Clause 9.5.2.2. | This requirement is defined to protect other BSs in the same network in neighbouring cells. |
| Transmitter intermodulation | BS type 1-C in Clause 6.7.2. BS type 1-H in Clause 6.7.3. BS type 1-O in Clause 9.8.2. | This requirement guarantees transmitter robustness with respect to an external interferer. |

Originally all requirements were derived for non-AAS BS (BS type 1-C). In Rel-13, hybrid AAS BS was introduced (In NR specifications referred to as BS type 1-H) and in Rel-15 all OTA AAS BS was introduced (In NR specifications referred to as BS type 1-O). For BS type 1-C and BS type 1-H, all requirements are defined at the transceiver ports (also known as the Transceiver Array Boundary (TAB) connector(s)). For BS type 1-O all co-location requirements are defined based on a concept of a co-location reference antenna. In conformance testing a practical implementation of this antenna is defined and is called  as the  Co-Location Test Antenna (CLTA).

Looking at the details related to the technical background for each individual requirement it can be noticed that the BS-to-BS coupling loss assumed is of different type depending on requirement. In Table 6.4.4.3.1-2, some further details for each requirement are presented.

Table 6.4.4.3.1-2: Requirement derivation assumptions

| Requirement | Frequency aspect | Description |
| --- | --- | --- |
| Transmitter spurious emission | Inter band | The requirement is defined to protect another band, hence coupling between bands are relevant. The approach used to derive the requirement level is defined to consider A-to-A coupling. |
| Receiver out-of-band blocking | Inter band | The requirement is defined to protect another band, hence coupling between bands are relevant. Since the receiver blocking related to induvial receiver branches, A-to-SA is considered when the BS type 1-C and 1-H requirement level is derived. The BS type 1-O requirement is based on receiver sensitivity so whilst the means of interference to each receiver branch is from A-SA the requirement is based on the A-A system performance |
| Transmitter OFF power | Intra band | The requirement is defined for TDD operation to protect the neighbouring BS in the same network. The approach used to derive the requirement level is defined to consider A-to-A coupling within the same band. The A-to-A coupling for this requirement should be based on to neighbouring BS and not side-by-side. |
| Transmitter intermodulation | Intra band | This requirement is defined to guarantee than unwanted emission requirements is met in the case of an interferer signal. The interferer signal is injected within the band. Since the requirement is related to induvial transmitter branches, A-to-SA is considered when the BS type 1-C and 1-Hrequirement level is derived. The BS type 1-O requirement is based on TRP emissions so whilst the means of interference to each Tx branch is from A-SA coupling the requirement is based on the spurious output from the entire array. |

It is worth to notice that the analysising BS-to-BS coupling for all requirements listed in Table 6.4.4.3-2, may require a numer of different scenarios A complete technical background for all requirements would require a coupling evaluation including the following cases:

- Intra band A-to-A for relevant band combinations

- Inter band A-to-SA for relevant band combinations

- Intra band A-to-A for relevant bands considering coupling to neighbouring BS

- Intra band A-to-SA for relevant bands

The relevance of each of the assumed MCL values to each of the BS types and how they are used in deriving the core and conformance requirements also needs to be clearly stated. Analysing the results presented in Table 6.4.4.2-1 and Table 6.4.4.2-2 would indicate that assuming BS-to-BS port coupling loss equal to a value larger than 30 dB would be reasonable for bands in the upper region of FR1.

In the following clauses, impact on individual requirements is further elaborated.

6.4.4.3.2	Transmitter OFF power requirement

FR1 NR BS transmitter OFF power is defined in terms of PSD in 1MHz and derived according to the following approach:

Transmit OFF power < kT0  + NF – 6dB + MCL + 10log(1MHz)

< -174 + 5 – 6 + MCL+60

< -115dBm/MHz + MCL

Considering the worst case is that BSs are assumed to be co-located, and for BS type 1-O the requirement is specified at the output of the CLRA the minimum coupling loss between co-located BSs would have no impact on the definition of core requirement. For the existing BS type 1-C the transmitter OFF power -85dBm/MHz, is specified at the antenna connectors and it is based on the assumption of MCL between two co-located BS as 30 dB so the core requirement would be affected by a change in the minimum coupling loss assumption.

6.4.4.3.3	Co-location spurious emission requirement/Protection of BS receiver of own or different BS

The protection of own or different BS is based on desensitizing a victim receiver by 0.8dB with the assumption the victim receiver has the same performance as the system under test.

For BS type 1-O the interference power level is specified at the co-located antenna output and is given by:

Spurious Emission LevelWA_1-O ＜ kT0 + NF  + 10*log10(100kHz) －7dB

= -174dBm/Hz + 5dB + 50dB－7dB = -126dBm

Note the figure in the requirement is scaled based on 8 TRX so is actually -126 +9 = -115dBm

The Spurious Emission Level of  BS type 1-C si specified at the antenna port and is adjusted by the MCL:

Spurious Emission LevelWA_1-C ＜ kT0 + NF  + 10*log10(100kHz) －7dB + MCL

= -174dBm/Hz + 5dB + 50dB－7dB + MCL

Where :

- MCL refer to the minimum coupling loss between co-located BSs, therefore the BS-to-BS isolation analysis would have the impact on the definition of the BS type 1-C co-location spurious emission requirement/Protection of BS receiver of own or different BS. MCL is assumed as 30dB in the existing specification.

- The BS of different classes will have different Noise figures. Generally, in most 3GPP spec, it is assumed that the    WA NF = 5dB, MR NF = 10dB, and LA NF = 13dB.

6.4.4.3.4	Transmitter intermodulation requirement

For FR1 transmitter intermodulation requirement, interfering signal level is equal to Rated total output power (Prated,t,AC) in the operating band. For BS type 1-O it is specified at the input to the co-located antenna, for BS type 1-C it is specified at the antenna conector and as such is affected by – MCL, therefore the BS-to-BS isolation analysis would have the impact on the definition of BS type 1-C transmitter intermodulation requirement. MCL refers to the minimum coupling loss which is assumed to be 30dB in the existing specification.

6.4.4.3.5	Co-located OOB Blocking requirement

For FR1 co-located OOB Blocking requirement, it s assmed that the co-located interfering system is of the same class as the DUT, interfering signal level at the interfereing co-located system for WA, MR and LA is calculated based on the following assumption:

- 46dBm

- 38dBm

- 24dBm

For BS type 1-O the interferer level is specified at the co-located antenna input.

For BS type 1-C the interferer level is specified at the antenna connector and as such is affected by the MCL:

- 46dBm-MCL

- 38dBm-MCL

- 24dBm-MCL

Where MCL refer to the minimum coupling loss which is 30dB in the past for 3GPP, therefore the BS-to-BS isolation analysis would have the impact on the definition of BS type 1-C receiver’s co-located OOB Blocking requirement.

<End of CR>