3GPP TSG-RAN WG4 Meeting #116                                                  	R4-2512649

# Bengaluru, India, 25th – 29th August, 2025

# Agenda item:			7.7.1

# Source:					Eutelsat Group, SES, Thales

# Title:						Way forward for Ku band ACLR and ACS specification band

# Document for:		Approval

# Introduction

The TN-NTN co-existence study for Ku band concluded in the WF [1] This WF proposes ACLR and ACS values for UE and SAN based on TN-NTN coexistence in Table X. This Tdoc takes those figures as input into the final figures to be specified for the UE and SAN RF which will consider further factors such as inter-SAN co-channel interference, practicality, cost etc.

# Discussion

The conclusion of the NTN/TN coexistence study in [1] was as follows.

Table 4: Candidate ACLR and ACS of NR-NTN Ku band

|  | SAN |  | VSAT |  |  |
| --- | --- | --- | --- | --- | --- |
|  | GEO | LEO 600 | Parabolic | Phased array 45x45 | Phased array 78x78 |
| ACLR (dBc) | Note 1 | Note 1 | GEO: 20.52 LEO 17.49 | GEO: 21.79 LEO: 12.52 | GEO: 21.67 |
| ACS (dBc) | 17.22 | 22.54 | GEO: 45.21 LEO: Note 2 | Note 2 | GEO: 35.4 |
| Note 1: NTN/TN coexistence simulations indicate that there is no interference condition. Note 2: The current simulation assumptions do not allow the required ACIR obtained from simulation. |  |  |  |  |  |

The LEO inter-SAN analysis in [2] proposed 19 dB for SAN ACLR and 18 dB for UE ACLR using the simulation assumptions values for UE ACS of 25 dB and SAN ACS of 27.7 dB.

The 19 dB in [2] for SAN LEO is for a particular deployment with higher spectral efficiency for a particular satellite operation, and it was agreed that a lower figure of 16 dB would be acceptable to enable alternative deployments.

[2] used the wrong figure for LEO ACS of 27.7, when it is 24 so figures need to be revised. Further simulation has resulted in the 19 dB UE ACLR changing to 18 dB.

The following table shows several sources of values that could be used to make a final decision. Hese are either the co-ex result, the Ka spec or from [2].

Table 4: Candidate UE/SAN ACLR/ACS values

|  | SAN |  | VSAT |  |  |
| --- | --- | --- | --- | --- | --- |
|  | GEO | LEO 600 | Parabolic | Phased array 45x45 | Phased array 78x78 |
| ACLR (dBc) | [16-18] | [16-18] | GEO 20.52(co-ex) LEO 17.49(co-ex) | LEO 14 (Ka) LEO 12.52 (co-ex) LEO 18 [2] GEO 14 (Ka) GEO 21.79 (co-ex) GEO 18 same as LEO | GEO 14 (Ka) GEO 18 (same as 45x45) |
| ACS (dBc) | 20 (Note 1) | 26 (Note 1) | GEO 45.21 (co-ex) | GEO 25 (Ka) LEO 25 (Ka) | GEO 35.4 (co-ex) GEO 25 (Same as GEO 45x45) GEO 25 (Ka) |
| Note 1: The SAN ACS was tightened 2 dB from Ka to shift the burden on ACIR from the VSAT. |  |  |  |  |  |

The VSAT ACS is derived from co-ex results and taking several factors into considerations including the likelihood of encountering similar geometrical conditions (i.e., a very weak NTN link and a very strong adjacent TN link) in realistic VSAT operations. The final VSAT ACS was tightened 1 dB compared to Ka-band as part of the discussions on ACLR to get close to the co-ex results.

Agreement:

|  | SAN |  | VSAT |  |
| --- | --- | --- | --- | --- |
|  | GEO | LEO 600 | Type 4,5 | Type 6 |
| ACLR (dBc) | 16 | 16 | 19 | 18 |
| ACS (dBc) | 20 | 26 | 26 | 26 |

# References

[1] R4-2512648, “Way forward for [116][313] NR_NTN_Ku_Band_Coexistence”, Ericsson

[2] R4-2509245, “Intra-SAN (own-system interference) ACLR analysis for Ku band” Eutelsat Group