3GPP TSG-RAN WG4 Meeting #116	R4-2512654

Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | - | rev | - | Current version: | 19.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TS38.101-5 Add 9.5.3 requirement for NTN Ku bands |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ku_bands-Core |  |  |  |  |  | Date: |  |  | 2025-07-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of NTN Ku bands. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1.Add EN 302 977 as reference. 2.Add spurious emissions requirement for NTN Ku bands. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spurious emissions requirement for NTN Ku bands is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 9.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of change >>

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.521-5: "NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements".

[3]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[4]		3GPP TS 38.108: "NR; Satellite Node radio transmission and reception"

[5]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[6]	3GPP TS 38.101-4: "NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements".

[7]	3GPP TS 38.213: "NR; Physical layer procedures for control"

[8]	3GPP TS 38.331: " Radio Resource Control (RRC) protocol specification".

[9]	3GPP TS 38.300: "NR; NR and NG-RAN Overall description; Stage-2".

[10]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[11]	3GPP TS 38.306: "User Equipment (UE) radio access capabilities".

[12]	3GPP TR 38.811: "Study on New Radio (NR) to support non-terrestrial networks".

[13]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[14]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[15]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[16]	Recommendation ITU-R SM.329, "Unwanted emissions in the spurious domain".

[17]	EN 303 978, Satellite Earth Stations and Systems (SES); Harmonised Standard for Earth Stations on Mobile Platforms (ESOMP) transmitting towards satellites in geostationary orbit, operating in the 27,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU, v2.1.2, 2016-10.

[18]	EN 301 459, Satellite Earth Stations and Systems (SES); Harmonised Standard for Satellite Interactive Terminals (SIT) and Satellite User Terminals (SUT) transmitting towards satellites in geostationary orbit, operating in the 29,5 GHz to 30,0 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU, v2.1.1, 2016-05.

[19]	IEEE Std 149: "IEEE Standard Test Procedures for Antennas", IEEE.

[20]	EN 302 977, Satellite Earth Stations and Systems (SES); Harmonised Standard for Vehicle-Mounted Earth Stations (VMES) operating in the 14/12 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU, v2.1.1, 2016-06.

<< Next change >>

9.5.3	Spurious Emissions

9.5.3.1	General

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements in line with SM.329 [16] and NR operating band requirement to address UE co-existence. Spurious emissions are measured as TRP.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 9.5.3.1-1 starting from the edge of the assigned NR channel bandwidth. The spurious emission limits in Table 9.5.3.1-2 apply for all transmitter band configurations (NRB) and channel bandwidths. The requirement is verified in beam locked mode with the test metric of TRP (Link=TX beam peak direction, Meas=TRP grid).

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 9.5.3.1-1: Boundary between NR out of band and spurious emission domain

| Channel bandwidth | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| OOB boundary FOOB (MHz) | 100 | 200 | 400 | 800 |

Table 9.5.3.1-2: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth |
| --- | --- | --- |
| 30 MHz ≤ f ≤ 2nd harmonic of the upper frequency edge of the UL operating band in GHz | -13 dBm | 4 kHz |

9.5.3.2	On-axis spurious requirement

9.5.3.2.1	Applicability

The regional On-axis spurious requirement is applicable to NTN VSAT operating in band n512 and bands with network signalling values “NS_205N”and “NS_206N”. The On-axis spurious emissions are measured as EIRP.

These limits are applicable to the complete NTN VSAT equipment, including cabling between the units.

9.5.3.2.2	“Emissions disabled” and “Carrier-off” states

9.5.3.2.2.1	Requirement for band n512

The requirements specified in table 9.5.3.2.2-1 apply to NTN VSAT in “Emissions disabled” and “Carrier-off” states. They apply outside the transmission bandwidth.

Table 9.5.3.2.2.1-1: On-axis spurious limits in “Emissions disabled” and “Carrier-off” states

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 27.5 – 30.0 | 19 | 1 |

9.5.3.2.2.2	Requirement for network signalling values “NS_205N” and “NS_206N”

When “NS_205N” or “NS_206N” is indicated in the cell, the requirements specified in table 9.5.3.2.2-1 apply to NTN VSAT in “Emissions disabled” and “Carrier-off” states. They apply outside the transmission bandwidth.

Table 9.5.3.2.2.2-1: On-axis spurious limits in “Emissions disabled” and “Carrier-off” states

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 14.0 – 14.5 | 9 | 100 |

9.5.3.2.3	“Carrier-on” state

9.5.3.2.3.1	Minimum requirements for band n512

The requirements specified in Tables 9.5.3.2.3.1-1 and 9.5.3.2.3.1-2 apply to NTN VSAT in “Carrier-on”.

The requirement specified in Table 9.5.3.2.3.1-1 apply outside a bandwidth of 5 times the occupied bandwidth centred on the carrier centre frequency.

The requirement specified in Table 9.5.3.2.3.1-2 apply inside a bandwidth of 5 times the occupied bandwidth centred on the carrier centre frequency, and outside the transmission bandwidth.

NOTE:	The on-axis spurious radiations, outside the frequency range 27.5 GHz to 30.0 GHz, are indirectly limited by sub-clause 9.5.3.3.

Table 9.5.5.2.2.3.1-1: On-axis spurious limits in “Carrier-on” state - outside

| Frequency range (GHz) | NTN VSAT type | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| 27.5 – 30.0 | 4, 5 | 44 - K (NOTE) | 1 |
|  | 1, 2, 3 | 4 - K (NOTE) | 1 |
| NOTE: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one NTN VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [17] for Mobile VSAT or sub-clause 4.2.4.2 in [18] for Fixed VSAT. The manufacturer shall declare the value of N. |  |  |  |

Table 9.5.5.2.2.3.1-2: On-axis spurious limits in “Carrier-on” state - inside

| Frequency range (GHz) | NTN VSAT type | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- | --- |
| 27.5 – 30.0 | 4, 5 | 58 - K (NOTE) | 1 |
|  | 1, 2, 3 | 48 - K (NOTE) | 1 |
| NOTE: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one NTN VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [17] for Mobile VSAT or sub-clause 4.2.4.2 in [18] for Fixed VSAT. The manufacturer shall declare the value of N. |  |  |  |

9.5.3.2.3.2	Requirements for network signalling values “NS_205N” and “NS_206N”

When “NS_205N” or “NS_206N” is indicated in the cell, the requirements specified in Tables 9.5.3.2.3.2-1 and 9.5.3.2.3.2-2 apply to NTN VSAT in “Carrier-on”.

The requirement specified in Table 9.5.3.2.3.2-1 apply outside a bandwidth of 5 times the occupied bandwidth centred on the carrier centre frequency.

The requirement specified in Table 9.5.3.2.3.2-2 apply inside a bandwidth of 5 times the occupied bandwidth centred on the carrier centre frequency, and outside the transmission bandwidth.

NOTE:	The on-axis spurious radiations, outside the frequency range 14.0 GHz to 14.5 GHz, are indirectly limited by sub-clause 9.5.3.3.

Table 9.5.5.3.2.3.2-1: On-axis spurious limits in “Carrier-on” state - outside

| Frequency range (GHz) | NTN VSAT type | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- | --- |
| 14.0 – 14.5 | 4, 5, 6 | 34 - K (NOTE) | 100 |
| NOTE: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one NTN VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [20]. The manufacturer shall declare the value of N. |  |  |  |

Table 9.5.5.3.2.3.2-2: On-axis spurious limits in “Carrier-on” state - inside

| Frequency range (GHz) | NTN VSAT type | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- | --- |
| 14.0 – 14.5 | 4, 5, 6 | 48 - K (NOTE) | 100 |
| NOTE: K=10log10(N) with N the number of terminals simultaneously transmitting at the same EIRP on a given carrier frequency in the same measurement bandwidth. K = 0 if only one NTN VSAT transmits at any one time on a given carrier frequency. See sub-clause 4.2.2.2.1 in [20]. The manufacturer shall declare the value of N. |  |  |  |

9.5.3.3	Off-axis spurious requirement

9.5.3.3.1	Applicability

The regional Off-axis spurious requirement is applicable to NTN VSAT operating in band n512 and bands with network signalling values “NS_205N”and “NS_206N”. The Off-axis spurious emissions are measured as EIRP.

These limits are applicable to the complete NTN VSAT equipment, including cabling between the units.

9.5.3.3.2	General

The requirements specified in table 9.5.3.3.2-1 apply to NTN VSAT at 10 meters distance from the NTN VSAT.

Table 9.5.3.3.2-1: Radiated field strength limits at 10m from the NTN VSAT

| Frequency range (MHz) | EIRP Limit (dBμV/m) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 30 – 230 | 30 | 120 |
| 230 – 1000 | 37 | 120 |

9.5.3.3.3	“Emissions disabled” state

The requirements specified in table 9.5.3.3.3-1 apply to NTN VSAT in “Emissions disabled” state for all off-axis angles greater than 7° or greater than the minimum elevation angle supported, whichever is lower.

Table 9.5.3.3.3-1: Off-axis spurious limits in “Emissions disabled” state

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (kHz) |
| --- | --- | --- |
| 1.0 – 2.0 | -48 | 100 |
| 2.0 – 10.7 | -42 | 100 |
| 10.7 – 21.2 | -36 | 100 |
| 21.2 – 60.0 | -30 | 100 |

9.5.3.3.4	“Carrier-on” and “Carrier-off” states

9.5.3.3.4.1	Minimum requirements for band n512

The requirements specified in table 9.5.3.3.4.1-1 apply to NTN VSAT in “Carrier-on” and “Carrier-off” states for all off-axis angles greater than 7° or greater than the minimum elevation angle supported, whichever is lower.

Table 9.5.3.3.4.1-1: Off-axis spurious limits in “Carrier-on” and “Carrier-off” states

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 1.00 – 3.40 | -41 | 0.1 |
| 3.40 – 10.70 | -35 | 0.1 |
| 10.70 – 21.20 | -29 | 0.1 |
| 21.20 – 27.35 | -23 | 0.1 |
| 27.35 – 27.50 | -5 (Note 1) | 1 |
|  | -23 (Note 2) | 0.1 |
| 27.50 – 29.35 | -5 (Note 1) | 1 |
|  | -23 (Note 2) | 0.1 |
| 29.35 – 29.50 | -5 | 1 |
| 30.00 – 30.15 | -5 | 1 |
| 30.15 – 60.00 | -23 | 0.1 |
| NOTE 1: For mobile VSAT transmitting in the frequency range 29.5 – 30.0 GHz NOTE 2: For mobile VSAT transmitting in the frequency range 27.5 – 29.5 GHz |  |  |

9.5.3.3.4.2	Requirements for network signalling values “NS_205N” and “NS_206N”

When “NS_205N” or “NS_206N” is indicated in the cell, the requirements specified in table 9.5.3.3.4.2-1 apply to NTN VSAT in “Carrier-on” and “Carrier-off” states for all off-axis angles greater than 7° or greater than the minimum elevation angle supported, whichever is lower.

Table 9.5.3.3.4.2-1: Off-axis spurious limits in “Carrier-on” and “Carrier-off” states

| Frequency range (GHz) | EIRP Limit (dBm) | Measurement bandwidth (MHz) |
| --- | --- | --- |
| 1.00 – 2.00 | -47 | 0.1 |
| 2.00 – 3.40 | -41 | 0.1 |
| 3.40 – 10.70 | -35 | 0.1 |
| 10.70 – 13.75 | -29 | 0.1 |
| 13.75 – 14.00 | 5 (see note) | 10 |
| 14.50 – 14.75 | 5 (see note) | 10 |
| 14.70 – 21.20 | -29 | 0.1 |
| 21.20 – 60.00 | -23 | 0.1 |
| NOTE: This limit may be exceeded in a frequency band which shall not exceed 50 MHz, centred on the carrier frequency, provided that the on-axis EIRP density at the considered frequency is 50 dB below the maximum on-axis EIRP density of the signal (within the nominated bandwidth) expressed in dBW/100 kHz. |  |  |

<< End change >>