3GPP TSG-RAN WG4 Meeting #116	R4-2512842

Bangalore, India, 25th - 29th Aug, 2025

Source:	KDDI, Qualcomm, Samsung

Title:	TP to TR 38.719-02-01 for adding CA_n18A-n77(2A) with UL n77(2A)

Agenda item:			6.3.3

Document for:	Approval

- Introduction

The revised basket WID was approved in RAN#103[1].

Fallbacks are already in spec [2].

- References

[1] RP-250214, Revised WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells, Ericsson, ZTE, Huawei, HiSilicon, Mar.,2025, RAN#107

[2] 3GPP TS 38101-1 V19.2.0(2025-06)

Text Proposal

<Start of Text Proposal>

5.26	CA_n18-n77

5.26.1	Common for 1 band UL and 2 bands UL CA

5.26.1.1	Operating bands for CA

Table 5.26.1.1-1:  CA band combination of band nX+nY

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n18 | 815 MHz – 830 MHz | 860 MHz – 875 MHz | FDD |
| n77 | 3300 MHz – 4200 MHz | 3300 MHz – 4200 MHz | TDD |

5.26.1.2	Channel bandwidths per operating band for CA

Table 5.26.1.2-1: Supported bandwidths per CA band combination of band nX+nY

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n18A-n77(2A) | CA_n77(2A) | n18 | 5, 10, 15 | 0 |
|  |  | n77 | CA_n77(2A)_BCS0 |  |

5.26.1.3	UE co-existence studies for 1 band UL

Table 5.26.1.3-1 summarizes frequency ranges where harmonics and/or harmonics mixing occur for CA_ nX-nY.

Table 5.26.1.3-1 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | n18 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 815 | 1630 | 2445 | 3260 | 4075 |  |
| n77 | fLow | fHigh | 830 | 1660 | 2490 | 3320 | 4150 |  |
| DL1 | 3300 | 4200 | N/A |  |  | D | D | UL harmonic |
| DL23 | 6600 | 8400 |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 | 9900 | 12600 |  |  | N/A |  | N/A |  |
| DL4 | 13200 | 16800 |  | N/A | N/A | N/A | N/A |  |
| DL54 | 16500 | 21000 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | The two direct hits from the band n18 to the band n77: the 4th and the 5th harmonic to the band n77, have been specified in Rel-17 and hence can be omitted. |  |  |  |  |  |
| UL/DL harmonics |  | n77 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow | 3300 | 6600 | 9900 | 13200 | 16500 |  |
| n18 | fLow | fHigh | 4200 | 8400 | 12600 | 16800 | 21000 |  |
| DL1 | 860 | 875 | N/A |  |  |  |  | UL harmonic |
| DL23 | 1720 | 1750 |  | N/A |  | N/A | N/A | Harmonic mixing |
| DL34 | 2580 | 2625 |  |  | N/A |  | N/A |  |
| DL4 | 3440 | 3500 | D | N/A | N/A | N/A | N/A |  |
| DL54 | 4300 | 4375 |  |  | N/A | N/A | N/A |  |
| Analysis |  |  | There is a harmonic mixing direct hit case, i.e., UL4/DL1. |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only apply for DL>3GHz NOTE4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Table 5.26.1.3-2 summarizes frequency ranges where IMD products caused by one UL band with 2CC intra-band UL CA may occur for CA_ n18-n77

Table 5.26.1.3-2: Intra-band ULCA IMD overlap with the other DL band analysis.

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n77 fUL | 3300 | 4200 | Order | flow | fhigh |
| n18 fDL | 860 | 875 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 (1-1) | Min2CCBW | Max2CCBW |
|  | 10 | 600 |  | 10 | 600 |
| Close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh |  | 20 | 1200 |
| IMD3 (2-1) | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 2700 | 4800 |  | 30 | 1800 |
| IMD5 (3-2) | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
|  | 2100 | 5400 | Order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 1500 | 6000 |  | 6000 | 9000 |
| IMD9 (5-4) | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 900 | 6600 |  | 5400 | 9600 |
| IMD11 (6-5) | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
|  | 300 | 7200 | Order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | -300 | 7800 |  | 9300 | 13200 |
| Analysis | IMD4 and IMD11 due to one UL band with 600MHz separation BW exist for this band configuration. |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

5.26.1.4	∆TIB,c and ∆RIB,c values

TIB,c and RIB,c values have already been defined so this section is omitted.

5.26.1.5	REFSENS requirements

Reference sensitivity exceptions due to the two direct harmonic hits from UL n18 to DL n77 have been specified in Rel-17 and hence are omitted.

Reference sensitivity exceptions due to harmonic mixing from UL n77 for DL CA_n18-n77 is shown in Table 5.26.1.5.

Table 5.26.1.5 (for Table 7.3A.4-4): Reference sensitivity exceptions and uplink/downlink configurations due to harmonic mixing from a PC3 aggressor NR UL band for DL NR CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n77 | n18 | 10 | 15 | 25 | 5 | 5.4 | NOTE 8 | UL1/DL4X |
| n77 | n18 | 10 | 15 | 25 | 15 | 2.5 | NOTE 8 | UL1/DL4X |
| Unaffected part (NOTE 8) is omitted. NOTE X: For a UE which supports this band combination only when the Band n77 frequency range restriction of 3400 – 4100 MHz applies, the MSD test point(s) cannot be verified for the band combination and the test point(s) can be skipped. |  |  |  |  |  |  |  |  |

Based on co-existence studies on Table 5.26.1.3-2, Band n18 is victim of IMD4 (2, -2), IMD6 (3, -3) and IMD11 (6, -5) due to n77(2A) one UL band with 2CC intra-band UL CA. Higher IMD, i.e., IMD6, IMD11 can be ignored. The 2UL case analysis and the MSD values are shown in Table 5.26.1.5-1.

Table 5.26.1.5-1 (for Table 7.3A.5-1): 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n18-n77 | n18 | N/A | N/A | N/A | N/A | N/A | FDD | IMD27 |
|  | n7712 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
|  | n18 | N/A | 5 | N/A | 870 | 8.6 | FDD | IMD4 |
|  | n7712 | 3480 | 10 | 1 (RBSTART=25) | 3480 | N/A | TDD | N/A |
|  |  | 3915 | 10 | 1 (RBSTART=25) | 3800 |  |  |  |
| NOTE 7: In current release the maximum separation bandwidth class is 600MHz, therefore, no IMD MSD requirement apply for this CA configuration when two uplink sub blocks are assigned within CA_77(2A). NOTE 12: This band supports intra-band non-contiguous uplink configuration. |  |  |  |  |  |  |  |  |

5.26.1.6	OOB blocking exception requirements

OOB exception for this CA combination has already been defined in Rel-17, so this section is omitted.

<End of Text Proposal>