3GPP TSG-RAN4 Meeting # 116	R4-2512856

Bangalore, IN. 25th – 29th, Aug. 2025

# Source:	ZTE Corporation, Sanechips

# Title:	TP for PRD 01 to IMD MSD test point with two UL band for two-band DL CA

# Agenda Item:	6.3.3

# Document for:	Approval

- Introduction

This TP for PRD 02[1] is to introduce IMD MSD test point with two UL band for two-band DL CA.

- Reference

[1] Permanent Reference Document (PRD);Rules, guidelines and ways of working for introduction of band combinations in NR and LTE, v0.3.0

- Text Proposal

----- Start of TP -----

### 5.5.2	Inter-band CA

#### <texts are omitted>

#### 5.5.2.1.2	Two UL Bands

#### 5.5.2.1.2.1	One UL CC in each band

Pre-Condition for dual UL MSD Analysis with 2UL CCs

For two-band inter-band combinations with 2UL CCs in two bands, i.e. 1UL CC in each band, the Pre-Condition for dual UL MSD Analysis with 2UL CCs are:

○  Only FDD-FDD and TDD-FDD combination need to be considered.

Guideline on co-existence studies for one UL CC in each band

Table 5.5.2.1.2.1-1 lists Band nX + Band nY 2 bands UL CA(2CC) 2nd, 3rd, 4th and 5th order IMD for the UE-to-UE coexistence analysis.

Table 5.5.2.1.2.1-1: Band nX and Band nY for 2CC UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | – |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | – |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | – |  | – |  |
| NOTE : For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Guidelines for dual UL IMD MSD order

Up to IMD5 order need to be considered.

○   If only one IMD order occurs per victim band, the MSD value shall be defined in the specifications.

○   If the DL band may be affected by a mix of even and odd order IMD products, then the MSD value of the lowest even and the lowest odd order IMD, shall be defined in the specifications.

•    When applicable, a footnote shall be attached to the DL band to indicate that MSD may occur for higher order IMD products, and these orders shall be specified in the footnote.

○    If the DL band may be affected only by multiple even order IMD products, or only by multiple odd order IMD products, then the MSD value of the lowest even order IMD or the MSD value of the lowest odd order IMD, shall be defined in the specifications.

•    The lowest order IMD is recommended as worst case to represent the whole spectrum of the inter-band CA combinations.

•    Optionally, a second MSD test point may be specified on a case-by-case basis to account for additional IMD orders. It is recommended this second MSD test point corresponds to the lowest even or the lowest odd order IMD.

•    Any additional IMD order that is not specified shall be indicated by a note in the table.

Guidelines for dual UL IMD MSD test points

Dual UL MSD test points are captured in Reference Sensitivity exceptions due to intermodulation interference in clause 7.3A.5 in TS38.101-1 with the following parameters:

○ FDD/TDD band Parameters

- DL Fc and UL Fc

- UL LCRB (i.e. full RB allocation)

- UL/DL BW (i.e. the supported smallest channel bandwidth for UL and DL each band)

- Duplex mode (i.e. FDD or TDD)

#### 5.5.2.1.2.2	One UL CC in one band and two UL CCs in the other band

Pre-Condition for triple beat MSD Analysis with 3UL CCs

For two-band inter-band combinations with 3UL CCs in two bands, i.e. 1UL CC in one band, and 2UL CCs in the other UL band, the Pre-Condition for triple beat MSD Analysis with 3UL CCs are:

○  Only FDD-FDD and TDD-FDD combination need to be considered with 3 uplink (UL) component carriers (CC) in two clusters within FR1.

○  The 2 UL bands are part of the same band group or belong to adjacent band groups as defined in table 2a.1

Guideline on co-existence studies for one UL CC in each band

Table 5.5.2.1.2.2-1 lists Band nX + Band nY 2 bands UL CA(3CC) triple beat IMD analysis for Band nX- Band nY

Table 5.5.2.1.2.2-1-1: Band nX and Band nY for 3CC UL IMD products

| Band / CA1 | nX |  | CA_nYB/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | – |  | – |  |
| fDL | – |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | – |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | – |  | – |  |
| Analysis | Editor’s Note: The analysis conclusion for the 3UL CC IMD shall be included here, even if no issues are identified. |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table x.x.x, the analysis can be ignored. NOTE 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

Guidelines for triple beat IMD MSD order

○   For the case when the victim band may be affected by a 1st order triple-beat product proponents  should systematically check if the downlink band may be affected by dual uplink IMD3 interference. If the test point is missing, a dual UL IMD3 MSD test point should be specified.

○  A 1st order triple beat product is a 3rd order non-linearity which may affect its own FDD DL band, and it should be captured as interference source "IMD3", see the calculation in table 5.5.2.1.2.2-1 above.

Guidelines for triple beat IMD MSD test points

Triple beat MSD test points are captured in Reference Sensitivity exceptions due to intermodulation interference due to 2UL CA (clause 7.3A.5 in TS38.101-1) with the following parameters:

○ UL CA band (TDD or FDD) Parameters

- Must have non-contiguous RB allocation in UL LCRB (i.e. 1RB + 1RB)

- Duplex mode (ie. FDD or TDD)

- Spacing between RB allocations in each CC must equal the FDD Victim band duplex offset

- UL CC1 BW and UL CC1 Fc

- UL CC2 BW and UL CC2 Fc

○ Non-UL CA FDD band Parameters

- DL Fc and UL Fc

- TX LCRB (i.e. full RB allocation)

- UL/DL BW (i.e. the supported smallest channel bandwidth for UL and DL each band)

- Duplex mode (i.e. FDD)

----- End of TP -----