3GPP TSG-RAN WG4 #116bis	R4-2513040

Prague, Czech Republic, October 13-17, 2025

Agenda item:	8.7		RRM

Source:	Qualcomm Incorporated

Title:	On radio resource management aspects of 6GR

Document for:	Discussion

# Introduction

The 6G study item was approved in RAN #108 [1]. It also includes a study on the 6GR core and performance requirements. Topic (5) d) covers radio resource management (RRM) aspects for 6GR and topic (5) f) handles aspects related to testability to be driven by RAN4.

- 6GR core and performance requirements

- General RF aspects [RAN4]

    - Intra-3GPP co-existence studies

    - Study RF related system parameters and requirements

- BS RF requirement aspects including band [RAN4]

    - BS RF requirement and testing framework aiming at improvements and/or simplification compared to 5G NR, including MSR and AAS operation

    - Study how to improve 6G BS core, conformance specifications, including structure and drafting principles

- UE RF requirement aspects including band and band combination [RAN4]

    - UE RF requirement framework aiming at improvements and/or simplification compared to 5G NR

    - Study how to improve 6G UE RF specification(s), including structure, drafting principles, and database for band combination

    - Study UE RF capabilities considering different device types and implementations

- RRM aspects for 6GR [RAN4, RAN1, RAN2]

    - RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR

    - Study how to improve 6G requirement specification, including structure and drafting principles

- Demodulation and performance aspects [RAN4]

    - Demodulation and performance requirement framework and key assumptions, aiming at improvements and/or simplification compared to 5G NR for UE and BS

    - Study how to improve 6G demodulation and performance specifications, including structure and drafting principles for UE and BS

- Aspects related to the testability [RAN4]

    - Testability methodology framework and key assumptions, aiming to ensure that requirements can be properly tested considering the applicability and feasibility of conductive and/or OTA testing with reasonable complexity

- Other aspects

    - Handling irregular channel bandwidths including the definition [RAN4, RAN1]

    - Definition of ‘frequency range(s)’[RAN4]

In addition, a study on the 6G scenarios and requirements is conducted in RAN plenary [2]. Its outcome is summarized in the technical report 38.914 [3].

In this contribution, we discuss our views of the goals of the RAN4-driven radio resource management aspects of the 6G study item including enhancements for the requirement framework.

# Views for the 6G Study Item for RRM

A new study item for 6G radio has been approved in [1]. The justification for the study emphasizes the practical user experience, systems extensibility and performance excellence of the new radio access technology. In particular, the new 6GR must ensure an acceptable performance-complexity trade-off with enhanced spectral efficiency and improved spectrum utilization with the goal to deliver superior performance with respect to 5G NR. Regarding the 6G core and performance requirements, the 6G study item states that the RRM requirements and procedure aspects should be reassessed aiming at improvements and/or simplifications compared to 5G NR.

In our view this means that RAN4 should focus on enhancing the procedural aspects and the requirement framework for radio resource management to ensure that RRM procedures are tested under conditions that reflect field-relevant scenarios as close as possible.

Observation 1: Existing RAN4 RRM tests do not always ensure testing under field relevant conditions.

Proposal 1: RAN4 should focus on enhancing the testing of RRM procedures to ensure that functionality and performance is tested under conditions that reflect field-relevant scenarios.

RAN4 should also ensure that artificial scenarios and test cases are avoided as much as possible and real UE implementations are tested. Existing RRM core and performance requirements should be re-evaluated whether they still reflect state-of-the art implementation or whether after around 10 years of 5G development and optimization, existing requirements allow further enhancement. This may for example allow for optimizing timelines, interruptions and, in general, several of the existing RRM procedures.

Proposal 2: RAN4 should study how to ensure that real UE implementations are tested as much as possible.

Proposal 3: RAN4 should re-evaluate existing RRM core and performance requirements whether they still reflect state-of-the art UE implementations.

RAN4 should also investigate in the 6G study item whether the set of RRM procedures can be reduced, and requirements can be defined only for key procedures. Corner cases or less common scenarios may not require defining core and performance requirements. That would also help to reduce the complexity and the readability of the RRM specification significantly. For example, for event-triggered reports requirements may only be defined if a clear network action follows within a well-defined interval. One of the goals of the 6G study item should be to define a slim RRM specification that focuses on the main procedures and does not define requirements for corner cases. Similarly, RAN4 should also aim to avoid defining too many different options for the same feature and focus on a very few core procedures. An example is SCell activation that got enhanced in 5G NR since Rel-15 but only the Rel-15 procedures are commercialized in our understanding.

Proposal 4: RAN4 should study in the 6G study item whether the amount of RRM procedures can be reduced. Requirements should only be defined for key RRM procedures.

Many of the existing RRM performance test cases are meanwhile very relaxed for good UE implementations. Still, it can be observed in many field deployments that RRM procedures of the network follow the minimal requirements of the RAN4 specification although corresponding implementations in the devices have become very mature meanwhile. By following minimal requirements network efficiency is degraded significantly. RAN4 should investigate how it becomes feasible for the network to follow real UE performance instead of minimal requirements.

Observation 2: It is observed in field deployments that RRM procedures of some networks follow the minimal requirements of the RAN4 specification.

Proposal 5: RAN4 should investigate how the network can be enabled to follow true UE performance in its RRM procedures instead of assuming that all UEs just support minimal requirements.

We think it is also important to address the interworking of RAN4 and RAN5 in the 6GR study item. We believe RAN4 does not need to define all details of the parameters and configuration of the RRM performance tests.  Typically, RRM features become relevant for the field several years after they have been specified by RAN4. Hence, RAN4 is not in a good position to decide on all parameters of the performance test cases. Instead, RAN5 is better suited to define the detailed parameter settings of the test cases to address the importance for field deployments. We think that RAN4 should define only the scope and the framework for the RRM performance tests, but details of the parameter configuration could be addressed by RAN5.

Observation 3: RRM features are usually deployed in the field several years later after they are specified by RAN4. Hence, RAN4 is not in a good position to decide on important field-relevant parameter settings of the RRM performance test cases.

Proposal 6: We propose to discuss between RAN4 and RAN5 whether in 6GR RAN4 could focus on the scope and framework for defining RRM performance tests and RAN5 could specify the detailed parameter configurations of the RRM performance tests.

## Mobility Aspects

Unified measurement and mobility

There have been multiple evolutions to RRC connected UE mobility in 5G as shown below:

- RRC based baseline handover in Rel-15

- Conditional handover in Rel-16

- Dual Active Protocol Stack (DAPS) in Rel-16

- Inter-Cell Beam Management (ICBM) in Rel-17

- Lower-layer Triggered Mobility (LTM) in Rel_18

- Enhanced LTM in Rel-19

To the best of our understanding at the time of writing, Conditional Handover and DAPS have not yet been widely deployed in commercial networks, each for different reasons. In contrast, we’ve observed significant traction from infrastructure vendors around LTM. Moreover, based on perspectives shared by companies in 3GPP and other workshops, it is evident that LTM is being positioned as the baseline framework for unified measurement and mobility in 6G.

That said, we are not yet fully convinced of the benefits of LTM, especially considering the expected complexity. This is particularly true considering extensive internal analyses on UE implementation challenges and evaluations of the anticipated performance gains.

Below are the identified UE implementation challenges and practical use case limitations:

- Excessive Candidate Cells for L1-RSRP Measurements

  - May compete with serving cell measurement and beam management opportunities, especially in FR2.

- Tight Timeline for Early PRACH Transmission

  - Minimum PRACH preparation time can be as short as 3.5 ms, which may be tighter than the timeline for PDCCH-order PRACH transmission to the serving cell.

- Inter-Frequency LTM Scenario

  - Due to the lack of MG patterns designed for L1 measurements, inter-frequency L1-RSRP measurements for LTM candidate cells are slower than intra-frequency ones.

- CSI-RS Based L1-RSRP Measurements for Asynchronous Cells

  - CSI-RS requires FFT processing, making L1-RSRP measurements impractical unless cells are tightly synchronized.

- Concurrency Issues

  - Mobility and MIMO features were developed independently, increasing the risk of UE overload from serving and neighbor cell measurements - including CSI measurement and TCI state activation. This may lead UEs to report LTM capability conservatively due to computational constraints.

- Balanced and Fair Metrics for Handover Decisions

  - UEs may only support LTM for intra-frequency FR1 cells, while others rely on traditional L3 measurements. This can bias handover decisions and introduce metric imbalance, especially when CSI-RS based LTM measurements are limited to specific neighbor cells.

- Sparse and On-Demand SSBs

  - Emerging demands for sparse or on-demand SSBs for network energy saving may complicate the LTM neighbor cell measurement framework beyond what was seen in 5G.

Observation 4: Although LTM is being positioned as the baseline framework for unified measurement and mobility in 6G, as reflected in 3GPP discussions and industry workshops, its benefits and practicality remain uncertain due to the following anticipated implementation complexity and constraints related to deployment and applicability.

- Excessive Candidate Cells for L1-RSRP Measurements

  - May compete with serving cell measurement and beam management opportunities, especially in FR2.

- Tight Timeline for Early PRACH Transmission

  - Minimum PRACH preparation time can be as short as 3.5 ms, which may be tighter than the timeline for PDCCH-order PRACH transmission to the serving cell.

- Inter-Frequency LTM Scenario

  - Due to the lack of MG patterns designed for L1 measurements, inter-frequency L1-RSRP measurements for LTM candidate cells are slower than intra-frequency ones.

- CSI-RS Based L1-RSRP Measurements for Asynchronous Cells

  - CSI-RS requires FFT processing, making L1-RSRP measurements impractical unless cells are tightly synchronized.

- Concurrency Issues

  - Mobility and MIMO features were developed independently, increasing the risk of UE overload from serving and neighbor cell measurements - including CSI measurement and TCI state activation. This may lead UEs to report LTM capability conservatively due to computational constraints.

- Balanced and Fair Metrics for Handover Decisions

  - UEs may only support LTM for intra-frequency FR1 cells, while others rely on traditional L3 measurements. This can bias handover decisions and introduce metric imbalance, especially when CSI-RS based LTM measurements are limited to specific neighbor cells.

- Sparse and On-Demand SSBs

  - Emerging demands for sparse or on-demand SSBs for network energy saving may complicate the LTM neighbor cell measurement framework beyond what was seen in 5G.

Proposal 7: If a unified measurement and mobility framework for 6G is defined based on the 5G LTM feature, RAN4 should re-evaluate the benefits of the feature, thoroughly analyze the implementation challenges arising from its complexity and associated RRM requirements, and investigate how to address the identified issues to the extent that the feature can be implemented with reasonable complexity.

Excessive latency in handover requirements with limited relevance to field deployments

There are several instances where existing mobility requirements lack demonstrable effectiveness and field relevance. One notable example involves RRM-spec-defined “unknown” cells, where the resulting mobility requirements often lead to excessively long interruption durations and delays. These requirements are not representative of realistic deployment scenarios and pose questions about the purpose of defining such relaxed requirements.

Observation 5: The 5G RRM specification contains several instances where the defined mobility requirements result in excessively long interruption durations and delays, for example, the “unknown” cell handover scenario. These requirements do not reflect realistic deployment conditions and raise questions about the rationale behind specifying such relaxed requirements.

Additionally, field observations indicate that “inter-frequency” handovers predominantly occur in specific configurations, most commonly between FDD and TDD bands. Despite this, current inter-frequency mobility requirements are defined in a scenario-agnostic manner, without anchoring to concrete use cases. This generalized approach complicates optimization efforts and may result in requirements that are either overly conservative or misaligned with practical needs.

Observation 6: Current inter-frequency mobility requirements are defined in a scenario-agnostic manner, despite field evidence showing that such handovers predominantly occur in specific configurations (e.g., FDD↔TDD). This generalized approach complicates optimization and may lead to overly conservative or misaligned requirements.

Defining mobility requirements that must apply uniformly across all possible scenarios can dilute their technical precision and operational applicability. Moreover, it may adversely affect the readability of the RAN4 specification by introducing excessive conditional logic, such as numerous ‘if’, ‘else if’, and ‘else’ statements. Unless RAN4 explicitly agrees to differentiate requirements based on scenario types (which should be avoided unless strongly justified), it is more practical and effective to focus on scenarios with clear field relevance and deployment evidence.

Observation 7: Defining mobility requirements uniformly across all scenarios can dilute technical precision and operational relevance. It also risks reducing the readability of the RAN4 specification due to excessive conditional logic. A more practical approach would be to focus on scenarios with clear field relevance and deployment evidence, unless strong justification exists for broader applicability.

Proposal 8: 6G mobility requirements should be constrained to scenarios where the resulting handover latencies or interruption durations are technically meaningful and operationally relevant. Requirements that lead to excessively large values should be avoided.

Target Handover Latency and Interruption Requirements

Field data analysis from 5G networks indicates that user data forwarding from the source cell to the target cell is not always as fast as the mobile UE’s handover latency. Without enhancements on the network side, further optimization of UE handover latency may have limited practical value, as the gating factor from the end-user perspective is the overall data path delay — not just the UE’s mobility performance.

In 6G, it is important to study and account for network-side data forwarding latency when defining achievable handover latency targets. This approach would allow RAN4 to avoid unnecessarily aggressive latency and interruption requirements, thereby reducing UE implementation burden and avoiding optimizations that may not yield meaningful field benefits.

Proposal 9: RAN4 to study the practically achievable end-to-end handover latency target, taking into account user-plane data forwarding latency, to better align handover requirements with practical effectiveness.

## Activation/Deactivation of Cells

Evolution of SCell/SCG activation

Several enhancements related to SCell and SCG activation have been introduced through RAN1, RAN2, and RAN4 efforts, as outlined below:

- Rel-15:

  - MAC-based single SCell activation

- Rel-16:

  - MAC-based multiple SCell activation

  - RRC based SCell activation (TCI state activation upon the SCell activation was introduced in Rel-17)

- Rel-17:

  - PUCCH SCell activation

  - Aperiodic temporary TRS based SCell activation latency enhancement (limited to known SCell)

  - SCG activation

  - Conditional PSCell addition

- Rel-18:

  - Event-like L3 report based unknown SCell activation latency enhancement

  - SSB-less inter-band NES SCell activation

- Rel-19:

  - Idle/Inactive mode L3 measurement based SCell activation latency enhancement

  - OD-SSB based NES SCell activation

  - LB-LB CA

Since Rel-15, each release has introduced multiple SCell/SCG-related enhancement features, as listed above. As a result, the current SCell activation requirements have become increasingly complex. However, almost all enhancement features — except for MAC-based single and multiple SCell activation — have not been adopted in commercial networks. In other words, only the features that UEs are mandated to support have seen successful field deployment. Despite this near-zero adoption rate, there were attempts to further optimize SCell activation latency for Rel-20. This disconnect between specification and field relevance should be seriously considered as a lesson learned when defining SCell-related features for 6G.

Observation 8: Despite multiple SCell/SCG enhancements introduced since Rel-15, only mandatory MAC-based activation features have seen commercial adoption. This highlights a gap between specification complexity and field relevance.

Proposal 10: For 6G carrier aggregation features, RAN4 should investigate which SCell activation features have been commercially deployed or are planned for deployment and prioritize those based on proven deployment evidence to avoid unnecessary complexity and ensure practical impact.

RRM requirements

RRM requirements associated with SCell activation are defined for the following aspects:

- Interruption: verified through missing HARQ feedback on active serving cells:

  - Upon deactivated SCell measurements

  - During SCell activation and deactivation

- Latency: verified through the first non-zero CQI report for the to-be-activated SCell

  - Latency varies significantly depending on multiple factors, including known vs. unknown SCell, band relationships between serving cells, measurement cycle of the SCell, etc.

In 5G, interruption requirements during deactivated SCell measurement and SCell activation are overly conservative. Given that UEs typically operate under high traffic demand when SCell activation is triggered, excessively long interruption durations across all serving cells may undermine the intended performance benefits.

Observation 9: 5G interruption requirements during SCell activation result in blanking scheduling opportunities across all serving cells for extended periods, which may not reflect practical traffic conditions.

Proposal 11: In 6G, interruption requirements during deactivated SCell measurement and SCell activation should be defined to align with state-of-the-art UE implementations. For example, the interruption duration for intra-band CA in 5G is excessively long and may not reflect practical deployment needs.

SCell activation latency requirements in 5G are overly complex and heavily dependent on the SCell’s state — particularly whether it is ‘known’ or ‘unknown’ — along with several other factors such as measurement and reporting periods, intra-/inter-band configurations, SSB-less and SMTC-less setups, and co-location conditions like RTD/RPD-based same-Tx-Beam.

However, based on real product analysis, these latency requirements do not consistently vary across all defined factors. In many cases, the total activation latency specified in RAN4 is significantly longer than what is observed in real-world implementations, offering limited practical effectiveness. As a result, SCell deactivation decisions may be overly conservative due to concerns about long reactivation delays, leading to increased UE power consumption.

One example is that the RAN4 requirement assumes the UE uses the SSB of the to-be-activated SCell, which aligns in the time domain with all other SSBs of serving cells in the same band — a condition not necessarily required in actual implementations. In other words, although AGC operation may cause interruptions to other serving cells, AGC does not necessarily need to occur only when the SSBs from all serving cells in the same band are time-aligned.

Another example is the assumption that the UE requires one SSB each for coarse AGC, fine AGC, coarse time synchronization, and fine time/frequency synchronization. Additionally, RAN4 assumes five SSB samples are needed for pathloss measurement during PUCCH-based SCell activation.

These examples illustrate how RAN4 requirements diverge from real-world implementations and contribute to unnecessarily large SCell activation latency on paper.

On the other hand, the RAN4 requirement has simplified SCell addition in a way that activation latency does not scale proportionally with the number of SCells added or activated via RRC. This can make UE implementation challenging, especially in scenarios where multiple SCells are added simultaneously. For instance, when several SCells are added and activated, physical layer activation may not begin until the UE brings up the baseband and RF chains for all cells, introducing delays.

Observation 10: RAN4’s SCell activation latency requirements in 5G are overly complex and misaligned with real-world implementations. Assumptions such as strict SSB alignment and multi-step synchronization inflate latency on paper, leading to conservative deactivation behavior and increased UE power consumption.

Proposal 12: For 6G carrier aggregation, RAN4 should re-evaluate the assumptions made in the 5G SCell activation requirement definition, based on state-of-the-art UE implementations, and investigate whether and to what extent SCell activation latency modeling can be simplified to better align requirements with practical UE behavior and implementation realities.

## Measurement and Gaps

Measurement gaps are critical for enabling inter-frequency and inter-RAT measurements. However, current NR implementations exhibit inefficiencies due to long, static gap configurations that are not always aligned with UE-specific needs. For example, measurement gap lengths (MGLs) are designed to accommodate various SSB durations, considering the number of SSBs in bursts and their SCS. Yet, measurement objects (MOs) may be dynamically updated (added/removed), and networks often lack the resources to reconfigure MGs for each UE in response to these changes. As a result, in many deployments, MG configurations remain static and do not reflect the actual measurement requirements of the UE, leading to wasted resources and reduced scheduling flexibility.

Observation 11: Existing gap configurations are often long and predefined, resulting in underutilization when measurements are not actively needed.

Observation 12: UEs currently have limited ability to influence gap configurations, resulting in suboptimal measurement scheduling and resource usage.

Additionally, NR has introduced a variety of gap types across releases—such as per-UE/per-FR gaps, pre-configured MGs, NCSG, autonomous gaps, and positioning gaps—many of which can operate concurrently. This proliferation increases complexity on both the UE and network sides, often resulting in low adoption of certain features (e.g., NCSG), thereby undermining the value of standardization efforts.

Observation 13: The introduction of multiple gap types and their concurrent operation increases system complexity, contributing to low adoption and underutilization of MG-related features.

Thus, we believe the upcoming 6GR should undertake a comprehensive study of the measurement gap (MG) framework to identify inefficiencies—both technical and operational—and explore solutions that enable gaps only when necessary. In particular, RAN4 should prioritize the investigation of mechanisms such as NCSG-like approaches that support interruption-free measurements. Early infrastructure support for such mechanisms is crucial to ensure timely deployment and avoid the risk of these features being sidelined.

Moreover, it is important that RAN4 avoids further fragmentation of the MG framework by introducing too many distinct gap types. Instead, a unified and flexible MG framework should be adopted to simplify implementation and improve feature adoption across vendors and deployments.

Proposal 13: RAN4 should study the overall measurement gap framework in 6GR and identify all inefficiencies—technical and operational—that hinder optimal gap usage.

Proposal 14: RAN4 should identify and evaluate mechanisms that enable interruption-free measurements, with a focus on deployability from the beginning of 6GR.

In addition, we propose that RAN4 studies mechanisms to support UE-initiated gap configurations. As previously noted, networks often do not dynamically update MG configurations in response to changes in measurement objects, leading to inefficient use of gaps. UEs, however, have the most accurate view of their mobility conditions and can optimize their measurement scheduling to minimize the number and duration of gaps required. Therefore, it is logical to allow UEs to indicate preferred MG configurations that balance mobility needs with throughput considerations.

Proposal 15: RAN4 should study how to enable UEs to indicate preferred MG configurations, allowing for more adaptive and efficient measurement scheduling.

## Enhancements of Interruptions

According to [6], 5G NR allows interruptions for EN-DC, SA, NE-DC and NR-DC for various scenarios like PSCell/SCell addition/release, SCell activation/deactivation, during measurements on SCC and many other cases. In many cases, interruptions on some or all active serving cells are allowed in case that certain actions happen on other serving cells. For example, in case that a deactivated SCell is activated, an interruption of the SMTC duration plus an additional slot in any active serving cell is allowed that is in the same FR1 band as the SCell to be activated.

It is obvious that this can lead to inefficiencies for the network if the network simply follows in its scheduling decisions the RAN4 interruption requirements or in case that the network would be able to honor the true interruption duration in the device, but the signaling is not supported. For several of the interruption scenarios, the true interruptions may be shorter than the existing requirements or in some cases, no interruptions may be needed at all. Also, over time the UE implementation may have improved, and interruption durations became shorter or even fully disappeared.

Observation 14: 5G NR allows for many scenarios interruptions on the active serving cells that impact the efficiency of the network significantly.

Observation 15: In specific scenarios, the true interruption of state-of-the-art UEs may be shorter than the minimal requirement in RAN4. In some scenarios, even no interruption may be needed.

Hence, it is important that RAN4 re-investigates all scenarios with interruptions whether the requirement can be relaxed or whether interruption is still needed.

Proposal 16: RAN4 should study all existing interruption scenarios in 5G NR whether they are still applicable to 6GR, whether the existing requirements should be refined and whether interruptions are needed in certain scenarios not yet covered in 5G NR.

The minimal requirement of the duration of the interruption depends on the scenario. Each of those scenarios should be analyzed separately in RAN4 [6]. For example, for intra-band carrier aggregation for SCell addition/release, SCell activation/deactivation and SCell measurements the interruption length depends on the duration TSMTC duration of the SMTC window. In all these different scenarios very different call flows apply in the UE. Hence, it is not obvious why the minimal requirement for these scenarios depends in all cases on the duration of the SMTC. In the case of measurements on deactivated SCell it is not obvious at all why interruptions are needed.

### RF Tune Time

In the context of interruption also the RF tune time for measurement gaps should be mentioned. In the previous section, the necessity of enhancing the gap framework was discussed. Figure 2-2 shows the general definition of a gap that includes the measurement duration and a switching time at the beginning and the end of a gap that is free of measurements. The allowed switching time in [6] is 0.5 ms for FR1 and 0.25 ms for FR2. Depending on the subcarrier spacing, this duration is half a slot for 15 kHz, one slot for 30 kHz and two slots for 120 kHz.

Figure 2-2: Gap Duration and Switching Time

Like interruptions, the switching time causes overhead and reduces network efficiency. For 30 kHz and 120 kHz, the existing minimal requirement can consume one or two full slots. In this case, overhead can only be reduced if the true switching time is lower than the minimal requirement. However, for 15 kHz, the minimal requirement for the switching time is just half a slot. In this case it may be possible to use the second half of the slot for other purposes, e.g., data transmission on the carrier the RF is tuned from. It should be studied whether and how this half-slot of 0.5 ms can be used efficiently.

Observation 16: For 30 kHz and 120 kHz, the minimal requirement for the RF tune time of a measurement gap consumes one or two full slots.

Observation 17: For 15 kHz, the minimal requirement for the RF tune time of a measurement gap consumes half a slot. The second half of the slot is neither used for RF switching nor used for data transmission and wasted.

Proposal 17: RAN4 should study for the case of 15 kHz subcarrier spacing whether the unused half slot (0.5 ms) in the duration of a measurement gap can be used efficiently, e.g., for data transmission.

# RRM-specific Categories

After multiple 5G releases, UE capability size has significantly increased, and RRM performance has improved well beyond the baseline. However, due to complex RRM requirement definitions and numerous optional UE capabilities, it is nearly impossible to intuitively assess UE RRM performance such as latency and interruption, based on supported features.

Observation 18: 5G has significantly expanded UE capability size and improved RRM performance beyond baseline levels. The complexity of RRM requirements and optional UE features makes intuitive evaluation of RRM performance (e.g., latency, interruption) almost impossible.

We believe it would be beneficial to provide the industry with a more intuitive framework to guide implementation and deployment decisions. The framework should clearly outline the UE’s RRM performance based on all capabilities supported by the UE and allow easy comparison against a baseline (e.g., the bare minimum latency for handover, SCell activation, etc.). For example, the framework can outline the following aspects:

- Measurement aspect: Latency and Accuracy

- Mobility aspect: Latency and Interruption

- Interruption aspect: Interruption ratio/length for intra- and inter-band

- Carrier Aggregation aspect: Latency for Single/Multi SCell activation

For each RRM aspect, a set of baseline RRM performance levels can be defined, with additional sets introduced as new features emerge. Furthermore, the framework can include a mapping that links each RRM aspect to the corresponding set of optional UE capabilities, enabling a more intuitive and comprehensive understanding of overall UE performance.

Proposal 18: For 6G, RAN4 should study how to provide a more intuitive framework to guide implementation and deployment decisions by labeling UE RRM performance, based on all capabilities supported by the UE, in a standardized format. The framework could include a mapping that links each RRM aspect to its corresponding set of optional UE capabilities, enabling a more intuitive and comprehensive understanding of overall UE performance.

# Additional Comments to Miscellaneous Requirements

RAN4 has assumed the RX beam sweep factor of eight in many FR2 measurement periods of Rel-15. RAN4 assumed that UE needs to sweep eight RX beams in FR2 to cover angle of arrivals in relevant azimuth and elevation domain and detect or measure an unknown reference signal. This scaling factor of 8 significantly increases many evaluation periods in FR2. This long evaluation period slows down the mobility procedures significantly.

Observation 19: RAN4 has assumed the RX beam sweep factor to be eight in many FR2 measurement periods of Rel-15. This scaling factor of eight significantly increases evaluation periods in FR2.

RAN4 has introduced new UE capabilities in Rel-18 and Rel-19 that allow UE to signal support of reduced beam sweeping factors for serving and neighbor cells. RAN4 should study if and how to reduce RX beam sweeping factor in FR2 evaluation period requirements of 6G.

Observation 20: RAN4 has introduced new UE capabilities in Rel-18 and Rel-19 that allow UE to signal support of reduced FR2 RX beam sweeping factors for serving and neighbor cells.

Proposal 19: RAN4 should study if and how to reduce RX beam sweeping factor in FR2 evaluation period related requirements in 6G.

Setting aside the FR2 RX beam sweeping factor, some other settings and assumptions of evaluation periods in 5G might be more relaxed than necessary. Results of RAN5 tests suggest that 5G UEs pass many FR1 and FR2 evaluation period related tests with a lot of margins. This stems from multiple reasons, e.g.: core requirements got defined for the worst-case scenario of TDL and AWGN channels, but performance tests were defined for AWGN in most cases.

RAN4 should re-evaluate different evaluation periods and the settings of corresponding performance tests.

Observation 21: Results of RAN5 tests suggest that 5G UEs pass many FR1 and FR2 evaluation period related tests with a lot of margins.

Observation 22: Core requirements of many 5G RAN4 RRM features were derived based on the worst-case performance among TDL-A/B/C and AWGN channels, but performance tests were defined for AWGN in most cases.

Proposal 20: RAN4 should evaluate measurement period related requirements of FR1 and FR2, and study if and how to reduce them in 6G.

Proposal 21: RAN4 should study whether to use TDL, instead of AWGN, in more performance tests of 6G RRM.

Figure 4-1: Timeline of known TCI state switch in 5G

5G RAN4 spec allows UE to receive one SSB to update its loop setting in both “known” and “unknown” TCI state switch procedure before UE is expected to communicate with the new TCI state. SSB in PCell has primarily been restricted to cell defined periodic SSB in 5G. Hence, RAN4 performance tests allow UE 20 ms time to receive one CD-SSB to update its loop setting during both “known” and “unknown” TCI state switch process. This increases the timeline of “known” and “unknown” TCI state switch. Figure 4-1 shows the timeline of known TCI state switch in 5G.

Besides, 5G RAN4 spec allows UE to receive eight periodic CSI-RS (SSB) signals to refine its RX beams during “unknown” TCI state switch procedure where the target TCI state is CSI-RS (SSB). This significantly increases the timeline of “unknown” TCI state switch procedure.

The extended timeline of unknown TCI state switch may become prominent in AI-ML based beam management, where the target TCI state, QCL-ed to UE’s predicted RS, will be classified as “unknown” if it is not QCLed to a known measured RS and if UE does not support the capability of predicting TX-RX beam pairs.

Hence, reducing the timelines of TCI state switch procedure would be very beneficial in 6G. RAN4 should study to reduce the TCI state switch timeline.

Observation 23: RAN4 performance tests allow UE 20ms time to receive one CD-SSB to update its loop setting during both “known” and “unknown” TCI state switch process.

Observation 24: 5G RAN4 spec allows UE to receive eight periodic CSI-RS (SSB) signals to refine its RX beams during “unknown” TCI state switch procedure where the target TCI state is QCled to CSI-RS (SSB).

Observation 25: The extended timeline of unknown TCI state switch is prominent in AI-ML based beam management, where the target TCI state, QCL-ed to UE’s predicted RS, will be classified as “unknown” if it is not QCLed to a known measured RS and if UE does not support the capability of predicting TX-RX beam pairs.

Proposal 22: RAN4 should study the mechanisms to reduce TCI state switch timeline in 6G.

# Conclusions

In this paper we provided our views on radio resource management in the 6GR study item. Our observations and proposals are summarized as follows:

Views for the 6G Study Item for RRM

Proposal 1: RAN4 should focus on enhancing the testing of RRM procedures to ensure that functionality and performance is tested under conditions that reflect field-relevant scenarios.

Proposal 2: RAN4 should study how to ensure that real UE implementations are tested as much as possible.

Proposal 3: RAN4 should re-evaluate existing RRM core and performance requirements whether they still reflect state-of-the art UE implementations.

Proposal 4: RAN4 should study in the 6G study item whether the amount of RRM procedures can be reduced. Requirements should only be defined for key RRM procedures.

Observation 2: It is observed in field deployments that RRM procedures of some networks follow the minimal requirements of the RAN4 specification.

Proposal 5: RAN4 should investigate how the network can be enabled to follow true UE performance in its RRM procedures instead of assuming that all UEs just support minimal requirements.

Observation 3: RRM features are usually deployed in the field several years later after they are specified by RAN4. Hence, RAN4 is not in a good position to decide on important field-relevant parameter settings of the RRM performance test cases.

Proposal 6: We propose to discuss between RAN4 and RAN5 whether in 6GR RAN4 could focus on the scope and framework for defining RRM performance tests and RAN5 could specify the detailed parameter configurations of the RRM performance tests.

Mobility Aspects

Observation 4: Although LTM is being positioned as the baseline framework for unified measurement and mobility in 6G, as reflected in 3GPP discussions and industry workshops, its benefits and practicality remain uncertain due to the following anticipated implementation complexity and constraints related to deployment and applicability.

- Excessive Candidate Cells for L1-RSRP Measurements

  - May compete with serving cell measurement and beam management opportunities, especially in FR2.

- Tight Timeline for Early PRACH Transmission

  - Minimum PRACH preparation time can be as short as 3.5 ms, which may be tighter than the timeline for PDCCH-order PRACH transmission to the serving cell.

- Inter-Frequency LTM Scenario

  - Due to the lack of MG patterns designed for L1 measurements, inter-frequency L1-RSRP measurements for LTM candidate cells are slower than intra-frequency ones.

- CSI-RS Based L1-RSRP Measurements for Asynchronous Cells

  - CSI-RS requires FFT processing, making L1-RSRP measurements impractical unless cells are tightly synchronized.

- Concurrency Issues

  - Mobility and MIMO features were developed independently, increasing the risk of UE overload from serving and neighbor cell measurements - including CSI measurement and TCI state activation. This may lead UEs to report LTM capability conservatively due to computational constraints.

- Balanced and Fair Metrics for Handover Decisions

  - UEs may only support LTM for intra-frequency FR1 cells, while others rely on traditional L3 measurements. This can bias handover decisions and introduce metric imbalance, especially when CSI-RS based LTM measurements are limited to specific neighbor cells.

- Sparse and On-Demand SSBs

  - Emerging demands for sparse or on-demand SSBs for network energy saving may complicate the LTM neighbor cell measurement framework beyond what was seen in 5G.

Proposal 7: If a unified measurement and mobility framework for 6G is defined based on the 5G LTM feature, RAN4 should re-evaluate the benefits of the feature, thoroughly analyze the implementation challenges arising from its complexity and associated RRM requirements, and investigate how to address the identified issues to the extent that the feature can be implemented with reasonable complexity.

Observation 5: The 5G RRM specification contains several instances where the defined mobility requirements result in excessively long interruption durations and delays, for example, the “unknown” cell handover scenario. These requirements do not reflect realistic deployment conditions and raise questions about the rationale behind specifying such relaxed requirements.

Observation 6: Current inter-frequency mobility requirements are defined in a scenario-agnostic manner, despite field evidence showing that such handovers predominantly occur in specific configurations (e.g., FDD↔TDD). This generalized approach complicates optimization and may lead to overly conservative or misaligned requirements.

Observation 7: Defining mobility requirements uniformly across all scenarios can dilute technical precision and operational relevance. It also risks reducing the readability of the RAN4 specification due to excessive conditional logic. A more practical approach would be to focus on scenarios with clear field relevance and deployment evidence, unless strong justification exists for broader applicability.

Observation 8: Despite multiple SCell/SCG enhancements introduced since Rel-15, only mandatory MAC-based activation features have seen commercial adoption. This highlights a gap between specification complexity and field relevance.

Proposal 9: RAN4 to study the practically achievable end-to-end handover latency target, taking into account user-plane data forwarding latency, to better align handover requirements with practical effectiveness.

Activation/Deactivation of Cells

Observation 8: Despite multiple SCell/SCG enhancements introduced since Rel-15, only mandatory MAC-based activation features have seen commercial adoption. This highlights a gap between specification complexity and field relevance.

Proposal 10: For 6G carrier aggregation features, RAN4 should investigate which SCell activation features have been commercially deployed or are planned for deployment and prioritize those based on proven deployment evidence to avoid unnecessary complexity and ensure practical impact.

Observation 9: 5G interruption requirements during SCell activation result in blanking scheduling opportunities across all serving cells for extended periods, which may not reflect practical traffic conditions.

Proposal 11: In 6G, interruption requirements during deactivated SCell measurement and SCell activation should be defined to align with state-of-the-art UE implementations. For example, the interruption duration for intra-band CA in 5G is excessively long and may not reflect practical deployment needs.

Observation 10: RAN4’s SCell activation latency requirements in 5G are overly complex and misaligned with real-world implementations. Assumptions such as strict SSB alignment and multi-step synchronization inflate latency on paper, leading to conservative deactivation behavior and increased UE power consumption.

Proposal 12: For 6G carrier aggregation, RAN4 should re-evaluate the assumptions made in the 5G SCell activation requirement definition, based on state-of-the-art UE implementations, and investigate whether and to what extent SCell activation latency modeling can be simplified to better align requirements with practical UE behavior and implementation realities.

Measurement and Gaps

Observation 11: Existing gap configurations are often long and predefined, resulting in underutilization when measurements are not actively needed.

Observation 12: UEs currently have limited ability to influence gap configurations, resulting in suboptimal measurement scheduling and resource usage.

Observation 13: The introduction of multiple gap types and their concurrent operation increases system complexity, contributing to low adoption and underutilization of MG-related features.

Proposal 13: RAN4 should study the overall measurement gap framework in 6GR and identify all inefficiencies—technical and operational—that hinder optimal gap usage.

Proposal 14: RAN4 should identify and evaluate mechanisms that enable interruption-free measurements, with a focus on deployability from the beginning of 6GR.

Proposal 15: RAN4 should study how to enable UEs to indicate preferred MG configurations, allowing for more adaptive and efficient measurement scheduling.

Enhancements of Interruptions

Observation 14: 5G NR allows for many scenarios interruptions on the active serving cells that impact the efficiency of the network significantly.

Observation 15: In specific scenarios, the true interruption of state-of-the-art UEs may be shorter than the minimal requirement in RAN4. In some scenarios, even no interruption may be needed.

Proposal 16: RAN4 should study all existing interruption scenarios in 5G NR whether they are still applicable to 6GR, whether the existing requirements should be refined and whether interruptions are needed in certain scenarios not yet covered in 5G NR.

Observation 16: For 30 kHz and 120 kHz, the minimal requirement for the RF tune time of a measurement gap consumes one or two full slots.

Observation 17: For 15 kHz, the minimal requirement for the RF tune time of a measurement gap consumes half a slot. The second half of the slot is neither used for RF switching nor used for data transmission and wasted.

Proposal 17: RAN4 should study for the case of 15 kHz subcarrier spacing whether the unused half slot (0.5 ms) in the duration of a measurement gap can be used efficiently, e.g., for data transmission.

RRM-specific Categories

Observation 18: 5G has significantly expanded UE capability size and improved RRM performance beyond baseline levels. The complexity of RRM requirements and optional UE features makes intuitive evaluation of RRM performance (e.g., latency, interruption) almost impossible.

Proposal 18: For 6G, RAN4 should study how to provide a more intuitive framework to guide implementation and deployment decisions by labeling UE RRM performance, based on all capabilities supported by the UE, in a standardized format. The framework could include a mapping that links each RRM aspect to its corresponding set of optional UE capabilities, enabling a more intuitive and comprehensive understanding of overall UE performance.

Additional Comments to Miscellaneous Requirements

Observation 19: RAN4 has assumed the RX beam sweep factor to be eight in many FR2 measurement periods of Rel-15. This scaling factor of eight significantly increases evaluation periods in FR2.

Observation 20: RAN4 has introduced new UE capabilities in Rel-18 and Rel-19 that allow UE to signal support of reduced FR2 RX beam sweeping factors for serving and neighbor cells.

Proposal 19: RAN4 should study if and how to reduce RX beam sweeping factor in FR2 evaluation period related requirements in 6G.

Observation 21: Results of RAN5 tests suggest that 5G UEs pass many FR1 and FR2 evaluation period related tests with a lot of margins.

Observation 22: Core requirements of many 5G RAN4 RRM features were derived based on the worst-case performance among TDL-A/B/C and AWGN channels, but performance tests were defined for AWGN in most cases.

Proposal 20: RAN4 should evaluate measurement period related requirements of FR1 and FR2, and study if and how to reduce them in 6G.

Proposal 21: RAN4 should study whether to use TDL, instead of AWGN, in more performance tests of 6G RRM.

Observation 23: RAN4 performance tests allow UE 20ms time to receive one CD-SSB to update its loop setting during both “known” and “unknown” TCI state switch process.

Observation 24: 5G RAN4 spec allows UE to receive eight periodic CSI-RS (SSB) signals to refine its RX beams during “unknown” TCI state switch procedure where the target TCI state is QCled to CSI-RS (SSB).

Observation 25: The extended timeline of unknown TCI state switch is prominent in AI-ML based beam management, where the target TCI state, QCL-ed to UE’s predicted RS, will be classified as “unknown” if it is not QCLed to a known measured RS and if UE does not support the capability of predicting TX-RX beam pairs.

Proposal 22: RAN4 should study the mechanisms to reduce TCI state switch timeline in 6G.

# References

- RP-251881, New SID: Study on 6G Radio, RAN #108, June 2025

- RP-251395, Revised SID: Study on 6G Scenarios and requirements, RAN #108, June 2025

- TR 38.914, Study on 6G Scenarios and Requirements, v0.1.0, June 2025