3GPP TSG-RAN WG4 Meeting #116bis			R4-2513051

Prague, Czech, Oct 13th – 17th, 2025

Agenda item:	8.11

Title:	Discussion on sensing for 6GR

Source:	Samsung

Document for:	Approval

# Introduction

In RAN#108, the Integrated Sensing and Communication (ISAC) on 6G Radio (6GR) has been approved as a new study item [2], where RAN4 working group is expected to study and discuss the physical functions and procedures, RF, coexistence, and testability for radio sensing.

- Sensing – Studies to be based on use cases and associated requirements, as defined in [TR38.914] - PHY functions and procedures for sensing technology (e.g., waveform. reference signals, measurement feedback, etc…) [RAN1, RAN4] - Evaluate sensing performance and if necessary, extend channel modelling, for the selected use cases [RAN1] - Aspects of integration with communication services [RAN1] - higher layer procedures and protocol aspects [RAN2] - RAN4 aspects of sensing including RF, coexistence, and testability in coordination with other WGs [RAN4] Note: RAN1 identify detailed requirements, if triggered by TSG RAN

In RAN#109, the key performance indicators and use cases of ISAC have been discussed, and the core technical performance requirements and basic use cases have correspondingly added to TR38.914. [3]

## 5.1 Key performance indicators - Sensing-related capabilities are measured in terms of the following technical performance requirements: - Detection Probability and False Alarm Probability: Detection probability is the probability of correctly detecting the presence of the sensing object, and false alarm probability is the associated probability that a sensing object is detected when no sensing object is actually present. - Horizontal/Vertical Localization Accuracy: It is defined as the difference between the estimated horizontal/vertical location and the actual horizontal/vertical location of the sensing object. The required value of localization accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the location estimation error. - Velocity Accuracy: Velocity accuracy is defined as the difference between the estimated velocity and the actual velocity of the sensing object. The required value of velocity accuracy shall be obtained assuming [90%/95%] confidence level, which is the [90th/95th] percentile point of the cumulative distribution function (CDF) of the velocity estimation error. - [Sensing Resolution: TBD] ## 5.4 Requirements of New and Existing Services - The 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects, at least including UAVs, human, vehicles and AGVs.

In this contribution, we discuss deployment scenario-related aspects for Rel-20 ISAC evaluation assumptions and performance evaluation.

# Discussion

Integrated Sensing and Communication have been envisioned as one of the most promising vertical service in 6GR, which is expected to enable the dedicated communication system with addition environmental perception capability, and thus provide better mobile user experience in the newly emerging scenarios, such as low-altitude economy, automotive, and digital twins, etc. To capture the cross-domain service capability, the framework of physical interfaces, signal processing procedures and device behaviors needs to be updated to match the performance requirements for both sensing service and communication service for 6GR.

## 2.1 High Level Viewpoint of 6GR ISAC Study

Different from the traditional communication feature in LTE and NR, the design of ISAC system is required to merge the sensing and data transmission functions into a shared hardware platform. Although the radar sensing system and the wireless communications systems are evolving towards similar frequency bands, the physical interfaces and the corresponding signal processing procedure remains distinct to a large extent. To fully excavate sensing performance, the signal processing framework as well as air interfaces of communication system need to be upgraded and optimized completely. However, such renewal of the communication system is time-consuming and onerous, and it could be unaffordable for 3GPP standardization discussions. Therefore, it is important to find a balance between the sensing performance and the system upgrade complexity. Based on this idea, it is recommended that 6GR ISAC standardization discussion avoid pursuing extreme sensing performance but instead focus on providing reasonable sensing performance for high-commercial-potential scenarios with limited system framework and physical interface modifications.

ISAC standardization discussion should avoid pursuing extreme sensing performance but instead focus on providing reasonable sensing performance for high-commercial-potential scenarios with limited system framework and physical interface modifications.

On the other hand, in an ISAC system, the sensing and communication services share the same radio-frequency, computation, and other hardware resources. The occupation of resources for the sensing service could affect the normal execution of communication tasks, and thus lead to a performance downgrade for the communication service. In fact, according to the investigations in academia [4], the optimal sensing performance and the optimal communication performance are the Pareto optimal, and the optima of sensing and communication system constitute a Pareto front, which indicates that the sensing performance and the communication performance satisfy tradeoff relationship. The tradeoff relationship is illustrated in Fig. 1, but the true shape of Pareto optimal boundary (front) is defined by the detailed sensing and communication integration method. It can be inferred that the favorable sensing performance can only be achieved by sacrificing the communication performance.

Fig 1. Demonstration of Pareto optimal relationship between sensing performance and communication performance.

The sensing and communication performance can only achieve optimality from the Pareto optimality perspective. In other words, the sensing performance and the communication performance satisfy tradeoff relationship.

Concerning that the sensing function is just one of the vertical services, which should not exert significant negative impact on the communication service, it is recommended that RAN4 should study ISAC/designate sensing performance with limited communication performance cost. To achieve this, it is also recommended that before the confirmation of detailed sensing performance, the tradeoff relationship of sensing and communication service should be investigated in advance.

The tradeoff relationship of sensing and communication service could be investigated.

According to Observation 2, it is proved that the sensing performance and communication performance satisfy tradeoff relationship. An upgrade of sensing performance may undermine the potential communication performance due to the shared hardware resources or interference from sensing signals. Henceforth, it is also important to monitor communication performance downgrade of an ISAC system compared to a dedicated communication system to keep the cost of communication service at the affordable level.

6GR ISAC study keeps focusing on the communication performance of an ISAC system to maintain sensing service cost of the ISAC system at the affordable level.

## 2.2 Use Case Discussion

In RAN#109, the key performance indicators (KPI) related to the sensing service have been discussed, and the several sensing performance metrics is approved in TR38.914. However, the sensing performance requirements largely depend on the specific sensing use cases. For example, the importance of detecting cargos for the AGVs in a factory may not be as equal important as that of detecting intrusion human/vehicles in a highway. On the other hand, different use cases could focus different aspects of sensing information acquisitions and possess different sensing performance requirements. For the indoor human activity sensing scenario, the it is important to identify what activity is the person doing, which is related to the detection probability/false alarm probability. While, for the UAV monitoring scenarios, the location and velocity of the UAV is also of equal importance as the detection. Therefore, it is necessary for RAN4 to identify the general use cases of ISAC first, then begins to investigate the ISAC system performance and develop specific performance requirements based on the selected use case.

RAN4 could identify the use cases of ISAC in advance, then investigate the ISAC system performance and develop specific performance requirements based on the selected use case.

### 2.2.1 6GR ISAC Use Case

In RAN#109, it is approved that the 6GR and 6G RAN architecture shall at least support use cases of detection and/or tracking of passive objects such as UAVs, human, vehicles and AGVs. As the specific ISAC application scenarios could be too numerous for RAN4 to discussion one by one, it is suggested that RAN4 focus on the use case with high commercial-application value. On the one hand, UAVs has shown it commercial application potential in outdoor activities like low-altitude economy, traffic surveillance these years. On the other hand, as the industry evolves towards intelligent and unmanned directions, AGVs become more and more important in indoor manufacturing factories. Thus, it is suggested that RAN4 could consider UAV sensing target in outdoor scenario and AGV sensing target in indoor scenario as representative use cases for evaluations and simulation.

RAN4 adopts UAV sensing target in outdoor scenario and AGV sensing target in indoor scenario as representative use cases for evaluations and simulation.

### 2.2.2 The Starting Point of RAN4 6GR ISAC Study

With the high-level viewpoint of 6GR ISAC studies, where to start the investigation of ISAC is of interest. In 5G-A, the study items “Study on channel modelling for integrated sensing and communications (ISAC) for NR” and “Study on Integrated Sensing and Communication (ISAC) for NR” have been approved in RAN#103 and RAN#108, respectively. Since then, the technical investigation and corresponding performance evaluation for ISAC system based on UAV use case have been conducted by RAN1 working group and valuable discussion results have been concluded. On the other hand, the new study item of ISAC in 6GR has also been approved by RAN#108, which aims at general implementation scenarios with more abundant use cases and sensing modes. Although the ISAC technology investigation in 3GPP standardization has been separated into two parts, i.e., 5G-A and 6GR, and studied independently, the fundamental issues that how to integrate sensing capability to the communication system using the existing communication hardware without damaging the data transmission capability for the two studies remain the same. The conclusions derived in the 5G-A ISAC study are still valuable in 6GR, and discussion results for UAV use case may be applicable to different use case in 6GR. Therefore, it is recommended that RAN4 starts ISAC discussion based on the fundamental conclusions derived by RAN1 5G-A ISAC study in the UAV use case, and further carries out detailed 6GR ISAC research and discussions.

The fundamental issues for 5G-A ISAC study and 6GR ISAC study remain the same, and the conclusions derived in 5G-A UAV use case may still applicable for the use cases in 6GR.

The starting point of RAN4 ISAC discussion builds upon the fundamental conclusions derived by RAN1 5G-A ISAC study in the UAV use case, and further carries out detailed 6GR ISAC research and discussions.

## 2.3 Sensing Mode Discussion

As is stated, multitudinous sensing scenarios places various performance requirements for sensing system. For instance, the UAV or vehicle detection and tracking scenarios demand ISAC system to possess large sensing coverage in order to detect the remote object in sky. The human gesture detection scenarios require ISAC system to have sufficient high sensing resolution in order to serve for indoor human activities identification task. A single ISAC system may not support such diverse performance requirements from different use cases. To cope with these requirements, ISAC systems can be classified to different types based on six sensing modes, which includes: 1) Transmit and Receive Point (TRP) monostatic sensing mode, 2) UE monostatic sensing mode, 3) TRP-to-TRP bistatic sensing mode, 4) UE-to-UE bistatic sensing mode, 5) TRP-to-UE bistatic sensing model, 6) UE-to-TRP bistatic sensing model. Each sensing mode has its own advantages and disadvantages. On the one hand, for the monostatic sensing modes, the sensing task is generally performed employing full-duplex method. In these cases, the node is fully in charge of sensing task and is with no necessity of sharing information with other nodes using extra reference signals, which is more implementation efficient. On the other hand, TRPs have more computation resources and less power supply limitation than UEs and thus the TRP related sensing system may have larger sensing coverage and better sensing capability. Considering the advantages of TRP related sensing, it is reasonable that RAN4 consider the TRP related sensing modes at the beginning of 6GR ISAC system study.

RAN4 considers TRP monostatic sensing mode, TRP-to-TRP bistatic sensing mode, TRP-to-UE sensing mode, and UE-to-TRP sensing mode in ISAC technology study at first.

## 2.4 Frequency Band Discussion

Although the RAN1 NR ISAC study prioritizes FR1 for ISAC evaluation, significant effort should also be dedicated to FR2 assessment, given its superior sensing performance enabled by large available bandwidth. Additionally, FR2 for sensing could also leverage FR2 deployment.

For an evaluation baseline, the calibration assumptions for Rel-19 ISAC channel model [5] can be considered. Specifically, it is proposed to consider 6 GHz carrier frequency with 30 kHz SCS for FR1, and 30 GHz carrier frequency with 120 kHz SCS for FR2-1. Note that larger SCS may enhance high-mobility performance but reduces maximum sensing range.

Both FR1 and FR2 should be considered for RAN4 6GR ISAC study.

## 2.5 Multiplexing Method for ISAC

For multiplexing sensing and communication, three options can be considered, i.e., time-division multiplexing (TDM), frequency-division multiplexing (FDM) and spatial-division multiplexing (SDM).

For TDM method, sensing can be allocated with separate time domain resources with entire bandwidth, which is beneficial for achieving good range estimation performance. In this case, switching time may need to be considered when transitioning between sensing and communication. Note that for the monotonic sensing, simultaneous DL and UL (full-duplex) is needed.

For TDM, dedicated time resources (with full exploitation of the entire allocated bandwidth) are beneficial for range estimation, and the switching time between communication and sensing needs to be considered.

Fig. 2 Example of multiplexing methods for ISAC system

For FDM method, the assumptions of resource allocation between sensing and communication need clarification. Two primary scenarios are typically considered: 1) where frequency resources for sensing are fully separated from those for communication, i.e., sensing and communication occupy mutually exclusive portions of frequency ranges in a band, and 2) where sensing RS mapping occupies a subset of subcarriers of a given frequency range and the remaining subcarriers are multiplexed for DL communication signals.

However, it is worth noting that for the FDM method, only a part of bandwidth is allocated to the sensing service. Thus, the sensing performance (e.g., sensing resolution) may be affected according to this frequency bandwidth configuration.

For FDM, the multiplex method can be classified to two scenarios: 1) multiplexing in a sub-band level, which divides the whole channel bandwidth into two mutually exclusive frequency ranges and 2) multiplexing in a subcarrier-level while occupying the same entire channel bandwidth. The clarification is able to be made based on RAN1’s further investigations.

Although technically feasible, SDM can be deprioritized at least for performance evaluation purposes due to (1) the increased receiver processing complexity and (2) precoding restrictions given the objective of separating the spatial streams.

For TRP monostatic sensing, RAN4 should prioritize TDM and FDM method, while deprioritizing SDM due to its processing complexity and restrictions on precoding selection.

## 2.4 Interference Discussion

Due to the added sensing service, the ISAC system may suffer from multiple kind of interference, including self-interference, neighbouring cell interference. The self-interference refers to the received undesired signal from the transmit antennas (working in either communication or sensing mode) to receive antennas (working in either communication or sensing mode) of the same sector in the same site, as is illustrated in Fig 3(a). The neighbouring cell interference can be classified to two types, co-site inter-sector co-channel interference and inter-site TRP-TRP co-channel interference, which are shown in Fig. 3(b), respectively. Generally, the existence of these interference signals would affect the sensing result and may undermine the performance of communication service. Thus, to figure out the influence of the interference, it is recommended RAN4 study the interference signal modelling and the corresponding impacts on the sensing and communication performance.

Fig. 3(a) Demonstration of self-interference			Fig. 3(b) Demonstration of neighbouring cell interference

The interference suffered by ISAC system can be classified into two types: 1) self-interference, which can be defined as the received undesired signal from the transmit antennas of the same sector in the same site 2) neighbouring cell interference, including 2.1) co-site inter-sector co-channel interference and 2.2) inter-site TRP-TRP co-channel interference, which can be defined as the received undesired signal from the transmit antennas of neighbouring sector in the same site and the received undesired signal from the transmit antennas of neighbouring sites, respectively.

### 4.4.1 Self Interference Modelling

The self-interference exists in multiple scenarios for ISAC systems. The following discussion will be expanded from the following two aspects: a) TDM case, b) FDM case.

4.4.1.a) Time-division multiplexing case

Based on the discussion in section 2.5, it can be derived that the sensing signals and the communication signals are completely isolated in the time division. Thus, there is no self-interference between the sensing service and communication service. However, for the TRP monostatic sensing mode, the sensing service requires full-duplex operation of TRPs. This full-duplex operation occurs when sensing signals are transmitted and received at the same time. In this scenario, the power of the transmitted signal can affect the receiver itself. However, it is worth noting that the self-interference may not exist if a signal reception switch is adopted at the TRP, which means that the signal reception switch is on only if the sensing signal transmission is finished.

When an ISAC system is working in TDM mode, there is no self-interference between sensing and communication service. However, the self-interference between the sensing transmitter and the sensing receiver may exist under some circumstances.

4.4.1.b) Frequency-division multiplexing case

For the FDM ISAC multiplexing case, due to the co-existence of sensing service and communication service at the frequency domain, the self-interference inevitably exists between the sensing signals and communication signals. In this case, the UL communication signal reception could be influenced by the transmitted sensing signal or reflected sensing signals. On the other hand, the sensing signal reception process could also be affected by either UL communication signals or the transmitted DL communication signals from BS. Further, the self-interference may also exist between the sensing transmitter and sensing receiver.

For the self-interference signal from the UL communication, the self-interference cancellation can be conducted by spatial isolation, frequency isolation, RF interference cancellation (IC), digital IC, etc. Companies can report the assumption of self-interference cancellation capability based on their own observations.

When an ISAC system is working in FDM mode, the self-interference exists between 1) the sensing receiver and communication transmitter, 2) the communication receiver and the sensing transmitter, as well as 3) the sensing transmitter and the sensing receiver.

### 4.4.2 Neighbouring Cell Interference Modeling

In real-world scenarios, sensing will be operated across multiple cells rather than a single cell. From a monostatic perspective, not only is self-interference encountered, but signals transmitted from other cells also acts as additional interference to the receiver, which is shown in Fig. 3(b). Therefore, in this case, it is necessary to model the neighbouring cell interference and figure out the corresponding impact.

(1) co-site inter-sector co-channel interference

Receivers of each TRP responsible for different sectors in the same site may be affected by interference from sensing transmission signals from TRPs in neighbouring sectors. However, the propagation in these near-field areas is currently not considered in the existing channel model. Notice that the previous TR [6] has modelled interference in the same way as self-interference, which is that the transmission power passes through multiple isolation mechanisms and acts as interference at the receiver. Therefore, in this study, we can refer to this and apply noise boosting accordingly. However, since interference isolation methods such as antenna isolation and digital cancellation are implementation-dependent, each company can select feasible values, conduct evaluation based on them, and report the used values as assumptions.

(2) inter-site TRP-TRP co-channel interference

Another aspect of interference comes from sensing transmission signals emitted by TRPs located in different sites. This interference could have a critical impact, especially when the TRPs are facing each other. The inter-site TRP-TRP co-channel interference can be modelled using the same approach as the modelling of the bistatic background channel.

### 4.4.3 Influence of interference signals on sensing

Unlike the interference in communication system, the interference in sensing signal reception is less damaging. For the communication signal reception, the extra interference signal would lead to mis-detection of communication symbols or deteriorate channel decoding process, and thus result in poor transmission throughput / high bit error rate. However, for the sensing service, the interference signals may lead to saturation of RF components making sensing system inoperable. Otherwise, the interference can be regarded as an extra point/block in the doppler-delay diagram (sensing result). If the sensing devices have prior knowledge of this interference, the interference can be neglected in further sensing result analysis.

However, how to identify the interference and the desired target need to be further studied, and RAN4 could investigate the interference modelling and decides parameters (e.g., power strength, doppler, delay) /criteria to distinguish the interference signals from the desired target signal based on the sensing result.

## 2.5 Performance Evaluation Discussion

In RAN#109, four sensing related KPIs have been approved, including detection probability & false alarm probability, horizontal/vertical localization accuracy, velocity accuracy, and sensing resolution. Therefore, RAN4 could adopt detection probability & false alarm probability, horizontal/vertical localization accuracy, velocity accuracy, and sensing resolution as performance metrics for sensing. However, since RAN1 has not started 6GR ISAC study yet, there are still many blanks in the standardization discussions. Other performance metrics might be needed for further ISAC performance evaluations. The other necessary performance metrics and the detailed definition of performance metrics could be updated according to the sufficient progress in RAN1.

According to the detection theory, the detection probability and the false alarm probability can be formulated as functions of signal-to-noise ratio (SNR) at sensing receiver [11]. Although, the horizontal/vertical localization accuracy, velocity accuracy can not be formulated to functions with respect to (w.r.t.) the receive SNR directly, the lower bound of the of localization accuracy, i.e., Cramér–Rao bound (CRB), can be proven as a function of receive SNR in many cases [12]. From the simplification perspective, it is reasonable for RAN4 adopts receive SNR of sensing signals as the intermediate performance indicator for sensing performance evaluation.

For the simplification perspective, RAN4 could adopt receive SNR of sensing signals as the intermediate performance indicator for sensing performance evaluation.

For the passive objection detection use case, the detection performance, i.e., detection probability and the false alarm probability, is closely related to the received sensing signal power which depends on the radar cross section (RCS) and the distance of TRP transmitter-detection object-TRP receiver. To evaluate the sensing performance, RAN4 should investigate the modelling of the common detection target RCS, and define typical RCS values for common detection targets. On the other hand, to identify the distance of TRP transmitter-detection object-TRP receiver, the spatial distribution for detection target also needs to be investigated.

RAN4 could investigate the modelling of the detection target RCS, and define typical RCS values for common detection targets.

RAN4 could investigate detection target distribution and discuss the feasible target distribution parameters.

# Conclusion

- ISAC standardization discussion should avoid pursuing extreme sensing performance but instead focus on providing reasonable sensing performance for high-commercial-potential scenarios with limited system framework and physical interface modifications.

The sensing and communication performance can only achieve optimality from the Pareto optimality perspective. In other words, the sensing performance and the communication performance satisfy tradeoff relationship.

The tradeoff relationship of sensing and communication service could be investigated.

6GR ISAC study keeps focusing on the communication performance of an ISAC system to maintain sensing service cost of the ISAC system at the affordable level.

RAN4 could identify the use cases of ISAC in advance, then investigate the ISAC system performance and develop specific performance requirements based on the selected use case.

- RAN4 adopts UAV sensing target in outdoor scenario and AGV sensing target in indoor scenario as representative use cases for evaluations and simulation.

The fundamental issues for 5G-A ISAC study and 6GR ISAC study remain the same, and the conclusions derived in 5G-A UAV use case may still applicable for the use cases in 6GR.

- The starting point of RAN4 ISAC discussion builds upon the fundamental conclusions derived by RAN1 5G-A ISAC study in the UAV use case, and further carries out detailed 6GR ISAC research and discussions.

RAN4 considers TRP monostatic sensing mode, TRP-to-TRP bistatic sensing mode, TRP-to-UE sensing mode, and UE-to-TRP sensing mode in ISAC technology study at first.

Both FR1 and FR2 should be considered for RAN4 6GR ISAC study.

For TDM, dedicated time resources (with full exploitation of the entire allocated bandwidth) are beneficial for range estimation, and the switching time between communication and sensing needs to be considered.

For FDM, the multiplex method can be classified to two scenarios: 1) multiplexing in a sub-band level, which divides the whole channel bandwidth into two mutually exclusive frequency ranges and 2) multiplexing in a subcarrier-level while occupying the same entire channel bandwidth. The clarification is able to be made based on RAN1’s further investigations.

For TRP monostatic sensing, RAN4 should prioritize TDM and FDM method, while deprioritizing SDM due to its processing complexity and restrictions on precoding selection.

The interference suffered by ISAC system can be classified into two types: 1) self-interference, which can be defined as the received undesired signal from the transmit antennas of the same sector in the same site 2) neighbouring cell interference, including 2.1) co-site inter-sector co-channel interference and 2.2) inter-site TRP-TRP co-channel interference, which can be defined as the received undesired signal from the transmit antennas of neighbouring sector in the same site and the received undesired signal from the transmit antennas of neighbouring sites, respectively.

When an ISAC system is working in TDM mode, there is no self-interference between sensing and communication service. However, the self-interference between the sensing transmitter and the sensing receiver may exist under some circumstances.

When an ISAC system is working in FDM mode, the self-interference exists between 1) the sensing receiver and communication transmitter, 2) the communication receiver and the sensing transmitter, as well as 3) the sensing transmitter and the sensing receiver.

For the simplification perspective, RAN4 could adopt receive SNR of sensing signals as the intermediate performance indicator for sensing performance evaluation.

RAN4 could investigate the modelling of the detection target RCS, and define typical RCS values for common detection targets.

RAN4 could investigate detection target distribution and discuss the feasible target distribution parameters.

# References

RP-251861 “New SID: Study on Integrated Sensing and Communication (ISAC) for NR,” RAN#108

RP-251881, “New SID: Study on 6G Radio,” NTT DOCOMO (Moderator), 3GPP TSG RAN Meeting #108, Prague, Czech Republic, June 9-13, 2025.

RP-252882 “TR38.914 ‘Study on 6G Scenarios and Requirements (Release 20)’”

Y. Xiong, F. Liu, Y. Cui, W. Yuan, T. X. Han, and G. Caire, “On the fundamental tradeoff of integrated sensing and communications under Gaussian channels,” IEEE Trans. Inf. Theory, vol. 69, no. 9, pp. 5723–5751, Sep. 2023.

TR38.901 “Study on channel model for frequencies from 0.5 to 100 GHz (Release 19)”

TR38.858 “Study on evolution of NR duplex operation (Release 18)”

R1-2506548 “On evaluation assumptions for 6GR air interface”, Moderator (Huawei), 3GPP TSG RAN1 Meeting #122

R1-2505580 “Discussion on ISAC Evaluation assumptions and performance evaluation”, Samsung, 3GPP TSG RAN1 Meeting #122

R1-2505582 “Design of 6GR air interface”, Samsung, 3GPP TSG RAN1 Meeting #122

R1-2505583 “Evaluation assumptions for 6GR”, Samsung, 3GPP TSG RAN1 Meeting #122

Steven M. Kay. Fundamentals of statistical signal processing: Estimation theory, Pearson, 1995.

Steven M. Kay. Fundamentals of statistical signal processing: Detection theory, Pearson, 1995.