3GPP TSG-RAN WG4 Meeting #116bis		   R4-2513113
Prague, Czech Republic, 13 – 17 Oct 2025

Agenda Item:	8.7

Source:	OPPO

Title:	Views on RAN4 driven non-AI RRM for 6G

Document for:	Discussion

Introduction

With spectrum becoming more fragmented and devices more heterogeneous, a unified yet flexible 6G technology framework will be essential, and corresponding RRM requirements should also be studied to ensure alignment with existing and new services, address real-world concerns about UE implementation complexity and two sides’ energy saving, and meet good performance of mobility, scheduling and control.

According to the approved 6G SID [1, RP-251881] and WF [2, R4-2511652] for 6G RAN4 planning, companies are encouraged to provide analysis and views on at least RAN4 driven non-AI RRM topics.

Potential 6G RRM topics

The RRM aspects for next generation access technologies should be carried out in at least, but not limited to, the following areas,

- RRM requirement and procedure aspects aiming at improvements and/or simplification compared to 5G NR

- Study how to improve 6G requirement specification, including structure and drafting principles

- Mobility for 6G (for all RRC states), including related RRM

In this contribution, we will provide our initial considerations on RRM aspects for 6G.

Overview of 6G RRM

What is for 6G RRM

The RRM requirements in 4G LTE and 5G NR, standardized by RAN4, serve as UE performance validation and the critical bridge between protocol design of different RAN WGs. RAN4 is responsible to define the minimum RRM core and performance requirements to ensure UEs meet consistency, reliability and interoperability across diverse scenarios (e.g., CA/DC, MIMO, multi-TRP, high-speed train). This involves:

- evaluating physical layer impact from RAN1, e.g., new reference signal, scheduling, beam management, etc.

- implementing protocol stack timelines from RAN2, e.g., RRC/MAC procedures and related UE behaviour.

- addressing RF hardware and software constraints, e.g., UE capability-specific rules, such as relaxing measurement accuracy for RedCap UEs (1Rx antenna) or Rx beam sweeping factor for FR2 UE.

- ensuring backward compatibility when RAN1/RAN2 introduces enhancements (e.g., multi-TRP in Rel-16), RAN4 updates RRM tests (e.g., TRP-specific BFD) while maintaining legacy compliance.

In short, RAN1 physical layer design, RAN2 protocol procedure design, and RAN4 RF assumption have impact on RAN4 RRM.

RAN1 physical layer design, RAN2 protocol procedure design, and RAN4 RF assumption have impact on RAN4 RRM.

Provided that new features and new use cases supported by 6G, what kind of RRM requirements UEs need to meet are also an interesting and challenging issue. For 6G UE operating in multi-carrier, multi-cell, multi-band or even multi-vendor deployments, the RRM requirements are expected to become more complicated. From RRM requirements’ perspective, more efficiency and less energy consumption, higher throughput and less interruption, seem to be basic KPIs for next generation technique innovation.

From RRM requirements’ perspective, more efficiency and less energy consumption, higher throughput and less interruption, should be considered as target for next generation technique innovation.

From R15 to R20, in total 6 releases for NR, RRM requirements are continuously iterated to support RAN1/2 led new features, e.g., NR-U operation, sidelink operation, CA/DC enhancement, L1/L2 triggered mobility (LTM), UE power saving, Network Energy Saving (NES), SBFD, XR, etc.

Two examples of the evolution on power saving and CA/DC enhancement in 5G are listed as below.

Evolution of UE and network energy saving

Evolution of CA/DC enhancement

The discussion of 6G physical layer signal/channel/procedure, coverage and energy saving schemes have been started in RAN1 from August meeting. Based on the functionality of physical layer and protocol stack timeline, RAN4 will discuss how UE performs transmission and reception including measurement, and investigate what are the minimum RRM requirements for UE.

For instance, once RAN1 figured out the reference signals for measurement and RAN2 designed RRC reconfiguration and procedures for handover, then RAN4 would define and test the delay and interruption time. In this case, RAN4 can convert RAN1/RAN2 designs into testable metrics, ensure UE performance and consistency across diverse scenarios, and adjust test boundaries for new features.

In our view, RAN4 can start the study of RRM impact by RAN1/2 new design for 6G as early as possible, especially physical layer signal/procedure, coverage and energy saving schemes. The outcome of RAN4 study can also help RAN1/2’s design to become more reasonable and feasible.

The RRM impact of RAN1/2 new design for 6G, such as physical layer signal/procedure, coverage and energy saving schemes, needs to be studied in RAN4 as early as possible.

Meanwhile, RAN4 also complete some RAN4 led work items, e.g., Measurement Gap enhancement (GAP), High speed train related enhancement (HST), multi-Rx reception (Multi-Rx). They are RAN4 driven topics, which have less impact to RAN1/2. In our view, equivalent functionality shall also be considered for the 6G.

The equivalent aspects of RAN4 led work items in 5G (such as GAP, HST, Multi Rx, etc.) can be reconsidered for 6G.

The table below summarizes the RRM requirements in TS 38.133 from R15 to R19. RAN4 can follow the RRM requirements framework of 5G for 6G study, and pick the useful features from both the first release (i.e., R15) and subsequent releases (e.g., R16 to R20) as the starting points of 6G RRM requirements.

Besides, all potential device types which are being discussed in RAN1/2 are to be included. Like normal UE, Redcap UE or IoT-like devices in 5G, UEs in 6G may also have different Rx antennas, channel bandwidth, duplex-mode, etc., and different RRM requirements and corresponding applicability need to be studied for them.

RAN4 to take Table 1 as starting point for 6G study of RRM, and both basic R15 features and some enhanced features in later releases of NR can be considered in 6G Day 1.

RRM requirements framework

| Feature | R15 (Baseline) | R16/R17/R18/R19/R20 (Enhancement) |
| --- | --- | --- |
| Idle-state Mobility | - DRX based cell (re)selection - Detection, evaluation, and measurement - Paging interruption | - Idle CA/DC measurement, e.g., EMR; - RRM measurement relaxation, e.g., low mobility and not-at-cell edge - Fast CA/DC setup, e.g., eEMR - OD-SIB1 impact - LP-WUS/WUR, e.g., measurement relaxation and offloading |
| Connected-State Mobility | Handover: - SSB based on L3 measurements. - Delay and interruption requirements CA operation: - Single SCell (de)activation - SCell addition/removal | Handover: - L1/L2 triggered mobility, e.g., SSB/CSI-RS based L1-RSRP measurement - CHO, CLTM SCell/PScell operation: - PUSCH SCell activation - Direct SCell activation - Multiple SCell activation - SCG activation - SSB-less SCell - On-demand SSB for SCell - MAC-CE based LTM and SCell activation |
| Timing | Timing & Synchronization - UE Transmission Timing: TA range and accuracy. - Measurement Timing: Alignment of RSRP measurement windows with SSB periods (i.e.,160ms). | - Multi-TAG - Reference Cell for SSB-less cell - One shot large UL timing adjustment |
| Scheduling | - BWP switching - TCI state switching - Interruption | - Carrier Switching - Antenna switching - UCBW change |
| Measurement procedure | L3 Measurements: - Measurement gap (MG) - delay and accuracy for intra-/inter-frequency, inter-RAT L1 Measurements: - RLM/BFD/CBD/L1-SINR | - Gap enhancement, e.g., needforgap, concurrent gap, pre-MG, NCSG, MUSIM, positioning gap - SCell BFD - Unified TCI - BWP without restriction |
| Note 1: All potential device types which are discussed in RAN1/2 could be included, e.g., normal UE, Redcap UE or IoT-like devices. |  |  |

Interests on RAN4 driven RRM topics

How to select other topics which are more RAN4 centric needs to be discussed in RAN4. In our view, the selected features can be used as baseline for 6G discussion in RRM area.

From the experience of 5G RRM work, measurement and gap design are clearly RAN4 driven RRM topics for 6G, e.g., Idle and connected mode UE measurement procedure, measurement gap enhancement. We can start the measurement and gap related discussion in RAN4, and do not need to wait for the progress or conclusion form other WGs.

Besides, RF and baseband assumption are also RAN4 specific, where RAN4 driven topics can also be started. The impact due to new spectrum allocation (e.g., FR3) can be studied parallelly in RAN4 RF and RRM. For example, antenna & RF architecture, frequency bands & bandwidth for 6G UE are going to be studied in RAN4 RF, which can also trigger the discussion of RRM impact, e.g., how UE Rx branches (e.g., 1Rx or 2Rx) affect measurement delay and accuracy, whether Rx beam sweeping is needed for FR3, which RF tuning time for FR1 or FR2 can be used for FR3. They may impact on the discussion of RRM measurement delay or measurement gap length.

RF and baseband assumption are RAN4 specific, which can be taken as RAN4 driven topics as well.

Study the RRM impact of new spectrum of 6G and possible new UE RF/baseband architecture(s).

For other features which are also going to be discussed in RAN1 or RAN2, we think RAN4 can also investigate whether some of them can be studied in RAN4 parallelly. These can involve:

- MIMO operation, multi-TRP

- mobility

- initial access

- spectrum aggregation

- RRM related coverage and energy efficiency

- other PHY signal/channel/procedure related RRM

Measurement and gap related topics can be firstly considered as RAN4 driven for 6G.

RAN4 to investigate which features can be studied in parallel with RAN1/2.

MIMO operation, multi-TRP

mobility

initial access

spectrum aggregation

RRM related coverage and energy efficiency

other PHY signal/channel/procedure related RRM

Different solutions of UE measurement from 5G are on the table now, like L1-RSRP based LTM or L3-RSRP based RRM measurement, AO-SSB or OD-SSB based measurement for SCell, 2 searchers for basic CSSF or 3 searchers CSSF enhancement for measurement outside gap. RAN4 needs to investigate a harmonized design for 6G RRM measurement, to enable better UE performance, and optimize the delay and gap/interruption requirements for 6G RRM.

Taking CSSF as an example, the current searcher assumption for CSSF for SA mode can be summarized as follows (just focus on SSB and without considering CSI-RS measurements), introducing the detailed usage of searchers:

- The 1st searcher is dedicated to PCC. The 2nd searcher is shared among the rest CCs.

- The 2nd searcher is dedicated to PSCC for 50% of the time, and for the other 50% of the searcher is shared with the rest of the SCCs.

- Among the SCCs, the SCC where neighbour cell measurement is also required occupies 50% of the time, which the rest 50% is equally shared with the rest of the SCCs (and further extended to inter-frequency and inter-RAT MOs without gaps in later releases).

The searcher assumption can be discussed at the start of 6G, and it is better to consider all possible assumptions of searchers, like the total number of searchers, the usages of different searchers, and applicable measurements types. Comprehensive searcher assumption will simplify RRM measurement requirements a lot.

Different solutions and requirements from 5G are on the table as reference for 6G, such as L1-RSRP based LTM or L3-RSRP based RRM measurement, AO-SSB or OD-SSB based measurement, basic CSSF or enhanced CSSF.

Regarding the observations, it is worthy to consider a harmonized design for 6G RRM. Both L1 and L3 measurement, always-on and on-demand signals, and other comprehensive assumptions for RRM measurement, need to be considered in 6G Day1.

RAN4 to investigate a harmonized design for 6G RRM, at least considering both L1 and L3 measurement, always-on and on-demand signals, and other comprehensive assumptions for RRM measurement.

From our side, the following RRM functions are suggested with higher priority to be studied in RAN4, at least including:

- unified measurement and report, e.g., in order to simplify UE capabilities and RRM requirements for supporting both L3 and L1/L2 triggered mobility

- unified GAP design, e.g., due to diverse gaps like pre-MG/NCSG/concurrent gap, and complicated gap skipping or collision rules.

- spectrum aggregation operation, e.g., impact on RRM measurement due to enhanced CA or SCMC (single cell multi-carriers) being discussed in other RAN WGs

- UE and network energy savings, e.g., support of RRM measurement relaxation, on-demand measurement, DRX, LP-WUR based measurement relaxation/offloading, low-capability device types (like Redcap UE, IoT-like device in 5G) and other solutions are not precluded depending on RAN1/2 conclusion

Illustration of measurement and report in SCMC operation

RAN4 to study the following RRM topics with high priority in 6G study phase:

Unified L1 and L3 measurement and report

Unified GAP design

RRM impact due to new spectrum aggregation operation, e.g., SCMC, enhanced CA

RRM impact due to UE and network energy savings, e.g., OD-SSB based measurement, low-power wake-up receiver, or low-capability device types.

RRM related UE capability

To be safe, UE measurement capabilities for 6G need to cover those for 5G, including the minimum number of measured frequency layers, the minimum number of cells, the assumption of searchers, the resources of measured RSs, measurement gaps, etc.

In addition, the assumption of searcher and measurement engines for measurement (including outside and within gap) can be reconsidered. For example, RAN4 can study whether 3 searchers introduced in R19 can be considered as baseline for 6G day 1, owing to support of more spectrum and measurement demands for 6G UE, e.g., FR1/2/3 and inter-RAT 2/3/4/5G measurements. And whether the searchers are separated or shared by L1 and L3 measurement can be also studied in RAN4 if L1 and L3 unified measurements were introduced.

UE measurement capabilities for 6G are assumed to be able to cover those for 5G, at least including frequency layers, cells, searchers, RSs, measurement gaps.

In 5G NR, a lot of UE capabilities related to RRM were introduced for UE, like fast/relaxed measurement, with or without scheduling restriction, interruption free or not, support of different baseband processing time. Also, as mentioned above, device type is being discussed in RAN1/2, and all possible UE categories like UE with different Rx, channel bandwidth, stationary or high-speed, or even IoT-like devices. These should be considered as well when RAN4 defines the minimum RRM requirements.

Therefore, RAN4 should study and agree on the principle to define the minimum requirements for UE, and avoid the diverse and complicated UE capabilities in the specification, which may introduce too many corner cases and duplicated test cases, making our spec quite hard to read. 6G RRM requirement should skip some corner cases and leave such issues up to UE implementation.

RAN4 to consider the minimum UE requirements and avoid too many UE capabilities and corner cases in 6G RRM discussion, leaving more flexibility for implementation.

Unified RRM Measurement

Measurement framework

For 5G NR RRM measurement, UE and network have already moved forward to meet better mobility performance, shorter delay with diverse configurations and UE capabilities. We think the basic structure of 5G RRM requirements would be not changed too much such as the components of cell reselection for Idle mode UE, and cell identification (i.e., cell detection, evaluation, and measurement) for connected mode UE can be kept unchanged.

Basic UE measurement procedure for 5G Idle and connected mode

The basic structure of measurement procedure for idle and connected mode can be reused.

For UE measurements in RRC connected mode, potential issues to be discussed for RRM can include: the definition of intra-frequency and inter-frequency, the applicability rules for gap-based or gap-less measurement, the sharing factor due to L1/L3 conflict, carrier specific scaling factor, known/unknown cell definition.

Potential issues to be discussed for RRM measurement requirements

We prefer to reuse NR basic assumption and procedure for 6G, e.g.,

- Idle/connected mode measurement procedure

- Intra and inter-frequency definition

- Scenarios of gap-based or gap-less

- Sharing factor, e.g., CSSF, P, Kp

- Known/unknown cell definition, e.g., detectable, measurable

Reuse NR basic assumption and procedure for 6G, e.g.,

Idle/connected mode measurement procedure

Intra and inter-frequency definition

Scenarios of gap-based or gap-less

Sharing factor, e.g., CSSF, P, Kp

Known/unknown cell definition

Unified L1 and L3 measurement

In 5G, both L1/L2 triggered mobility and L3 trigger handover are introduced for UE. Even they are both for mobility management, a lot of different UE capabilities and RRM measurement requirements are required. For example, RRM requirements of LTM apply with some restrictions, e.g., LTM candidate cells belong to L3 MOs, cell detection by L3 could be the pre-condition for LTM. Candidate cells can include serving cell.

Comparison of L3 mobility and LTM cell switch

To provide flexibility of mobility management, RAN4 can further study a unified solution to support both L1 and L3 measurement. RAN4 can start from cross-layer harmonization for same function firstly, e.g., L1-RSRP based neighbor cell measurement in LTM and L3-RSRP for mobility.

- Identify the need of unified configuration for same function, e.g., RS, MO, MTC, Report

- Evaluate RRM impact of unified cell switch/handover procedure, e.g., triggers/conditions/reports

- Assess the difference of measurement requirements and measurement restriction

RAN4 to study a unified solution to support both L1 and L3 measurement, at least including:

Identify the need of unified configuration for same function, e.g., RS, MO, MTC, GAP

Evaluate RRM impact of unified cell switch/handover, e.g., triggers/conditions/reports

Assess the difference of measurement requirements and measurement restriction

Besides, RAN4 can also investigate and evaluate cross-function harmonization for same layer, e.g., L1-RSRP in MIMO and L1-RSRP in LTM. It may more rely on RAN1/RAN2 design, e.g., procedure, configuration, event, etc. We can study the feasibility and how to apply or extend ICBM and inter-cell multi-TRP to L1 mobility (LTM), which may also rely on RAN1 and RAN2’s design on MIMO operation and Mobility in 6G SI.

From our side, we slightly prefer to consider the cross-layer harmonization for RRM measurement with higher priority, i.e., L1 and L3 based neighbor cell measurement, and further consider the cross-function harmonization for the same layer, i.e., L1-RSRP in MIMO and LTM. For example, RAN4 can focus on cross-layer L1 and L3 unified measurements in 2025 Q4, and further evaluate the RRM impact of cross-function L1 measurement like MIMO and LTM from 2026 Q1 based on initial assumption or conclusion from other WGs.

Unified L1 and/or L3 measurement

RAN4 to focus on cross-layer L1 and L3 unified measurements firstly in 2025 Q4. Further evaluate RRM impact of cross-function L1 measurement like MIMO and LTM based on initial assumption or conclusion from other WGs.

Measurement GAP

GAP framework

Due to UE’s hardware restrictions (e.g., RF tuning, transceiver or other Rx processing units’ adjustment), UE may not be able to maintain connection with the serving cell when performing measurements, especially when the target measurement resources is not contained within the active BWP. In this case, gap (with explicit location and length) is a good choice to enable measurement and meanwhile avoid unknown data loss. Clearly, the gap plays an important role in measurement and will be used in 6G. This section will discuss the gap related issues.

Structure of intra-frequency SSB measurement

Firstly, we would like to give a brief description about the relationship between gap and a target measurement frequency, i.e., whether or when a gap is needed for a certain frequency. NR goes through a tough discussion over several releases and reaches a complete but complicated structure. As shown in figure 9 above, the measurement on an intra-frequency SSB could be classified as

- measurement with gap

- measurement with NCSG

- measurement without gap and interruption is unclear

- measurement without gap and with interruption

- measurement without gap and without interruption

depending on the different conditions and UE capability reports.

NR Rel-15 defines RRM requirements for intra-frequency without gap and with gap. And whether gap is needed for intra-frequency SSB is dependent on one of the following conditions:

- the SSB is completely contained in the active BWP of the UE

- the active downlink BWP is initial BWP.

Further in R16, needForGap = {‘gap’, ’no-gap’} report is introduced and UE could indicate whether the target intra-frequency could be measured without gap. To enable NCSG in R17, needForGapNCSG = {‘gap’, ’ncsg’, ’nogap-noncsg’} report is further introduced. It should be noticed that the ‘gap’ case is repeatedly considered in R16 and R17 reports. Besides, it is unclear whether interruption is required for R16 ‘no-gap’ case. Therefore, R18 further divides them into two sub-cases, i.e., ‘no-gap-no-interruption’ and ‘no-gap-with-interruption’, and the latter one is quite similar as NCSG in our understanding. The similar situation is also observed for inter-frequency SSB measurement. Such method of classifying MO is too complicated and redundant.

The requirements for measurement with gap, with NCSG, without gap, without gap without interruption, without gap with interruption, are too complicated due to complex classification of MO.

Therefore, RAN4 in 6G should consider a unified solution for measurement configuration and simplify the corresponding RRM requirements, including the MO classifying method and UE capability report.

Strive for a simple and clear classification of MO (e.g., with gap, with NCSG, without gap, without gap without interruption, without gap with interruption) in the first release of 6G.

Consider a simple and unified UE capability report to cover the existing NR reports, including needForGap, needForGapNCSG, needForInterruption.

GAP pattern

As per gap patterns, 4 MG patterns with MGL= {3ms, 6ms} and MGRP= {40ms, 80ms} are defined in LTE. NR introduced up to 26 MG patterns and 24 NCSG patterns, including all the LTE gaps for inter-RAT LTE measurements, and 20ms/160ms periodicity considering the flexible SSB/CSI-RS design. Besides, FR2 specific MGL (e.g., 5.5ms) are introduced due to the short RF switching time. The gap patterns in 6G should match with target measurement objectives. As mentioned in [1], 5G NR and 6G interworking and coexistence should be supported, which means the gap pattern in 6G should cover at least 6G measurement and 5G NR measurement.

On the other hand, it is unrealistic to define many gap patterns perfectly matching with for every SSB/CSI-RS configuration. Too many gap patterns will also lead to complicated gap applicability rules as defined in clause 9.1.2 of TS 38.133. In our view, 6G should avoid this situation and strive to reach a small set of gap patterns.

A small set of gap patterns can avoid complicated UE capabilities and gap applicability rules.

The gap patterns in 6G should match with target measurement purposes, including both 5G NR measurement and 6G measurement.

NR measurement gap patterns

| MG patterns |  | MGRP | 20ms | 40ms | 80ms | 160ms |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 | MGL | 3ms | #10 | #2 | #3 | #11 |
|  |  | 4ms | #6 | #7 | #8 | #9 |
|  |  | 6ms | #4 | #0 | #1 | #5 |
| FR2 | MGL | 1.5ms | #20 | #21 | #22 | #23 |
|  |  | 3.5ms | #16 | #17 | #18 | #19 |
|  |  | 5.5ms | #12 | #13 | #14 | #15 |

One approach to reduce the number of gap patterns is to focus on the most common and typical use cases. For 5G NR measurement, NR gap patterns should be supported, but not all of them. In our view, the mandatory NR gap patterns are generally aligned with typical measurement objective configuration in practice and should be prioritized. The remaining non-mandatory gap patterns could be deprioritized or even excluded. In Rel-15, MG pattern ID 0 and 1 are mandatorily supported without UE capability report, and MG pattern ID 13, 14, 17, 18 and 19 are additionally supported for FR2 with UE capability report. In Rel-16, MG pattern ID 2, 3 and 11 are mandatory for NR only measurement in NR SA and NR-DC scenarios. If only NR mandatory gap patterns highlighted in Table 2 are considered, the number of gap patterns could be reduced to 10.

Besides, some of gap patterns are quite similar and could be merged. For example, MG pattern ID 0 with MGL=6ms and MG pattern ID 13 with MGL=5.5ms, the 0.5ms MGL difference comes from RF switch time for FR1 and FR2. And this is more obvious for NCSG, for example NCSG ID 0 and NCSG ID 13 are the same from the perspective of ML and VIRP. In 6G, the RF and processing hardware may not be improved too much compared with 5G. Chances are that the RF switch time in NR (0.5ms for FR1 and 0.25ms for FR2) will be reused. Besides, 6G aims to support new FR3 and a more complicated RF structure may be considered for UE. We don’t see much possibility and necessity to reduce the switch time. Single RF switch time will not only reduce gap patterns, but also simplify UE implementation. In our view, 0.5ms for NR FR1 can be considered as baseline.

Select a subset of NR gap patterns for inter-RAT NR measurement in 6G

Option 1: prioritize NR mandatory gap patterns

Option 2: study FR-agnostic gap patterns assuming 0.5ms RF switch time for all FRs

For 6G measurement, the above NR gap patterns can be applicable as well or some new 6G-specific gap patterns may be required. The details are dependent on the reference signal or SMTC design in RAN1 and should be postponed after more conclusions are reached. For example, to achieve energy efficiency and power saving gain, one approach is to extend the periodicity of SSB/SMTC in 6G, e.g., to 320ms or even longer. Then, 6G specific gap patterns aligning with the new SSB/SMTC design should be introduced.

Study 6G-specific gap patterns, at least considering new design of reference signals or MTC.

UE capability for support of NR gap pattern

supportedGapPattern Indicates measurement gap pattern(s) optionally supported by the UE for NR SA, for NR-DC, for NE-DC and for independent measurement gap configuration on FR2 in (NG)EN-DC. The leading / leftmost bit (bit 0) corresponds to the gap pattern 2, the next bit corresponds to the gap pattern 3, as specified in TS 38.133 [5] and so on. The UE shall set the bits corresponding to the measurement gap pattern 13, 14, 17, 18 and 19 to 1 if the UE is an NR standalone capable UE that supports a band in FR2 or if the UE is an (NG)EN-DC capable UE that supports independentGapConfig and supports a band in FR2.

supportedGapPattern-NRonly-r16 Indicates measurement gap pattern(s) optionally supported by the UE for NR SA and NR-DC when the frequencies to be measured within this measurement gap are all NR frequencies. The leading / leftmost bit (bit 0) corresponds to the gap pattern 2, the next bit corresponds to the gap pattern 3 and so on. The UE shall set the bits corresponding to the measurement gap pattern 2, 3 and 11 to 1.

supportedGapPattern-NRonly-NEDC-r16 Indicates whether the UE supports gap patterns 2, 3 and 11 in NE-DC when the frequencies to be measured within this measurement gap are all NR frequencies.

As shown above, many UE capabilities on the support of gap patterns are considered in NR. In R15, UE could report the support of MG pattern ID 2~23 via supportedGapPattern signalling. Further in R16, two more UE capabilities on the supported gap patterns for NR only measurement were introduced. RAN4 also discussed the mismatch reports on supported gap patterns and reached a conclusion in [3]. Such the multiple UE capabilities are complicated, and 6G should simplify it.

Simplify UE capabilities on the support of gap patterns in 6G.

Another aspect is about gap type. Firstly, we think per-UE gap should be considered as baseline for 6G, especially for low-capability UE with single searcher or FFT processing unit. NR discussed other options such as per frequency layer, per SMTC group or per FR, and ended up with per-FR gap as the trade-off between flexibility and complexity. Independent searchers are assumed for per-FR gap and the measurement in one FR will not cause interruption to the other FR. We are open to discuss per-FR or even per-CC, per-CC group gap. But it relies on the UE structure and FR design in 6G, which are not clear so far. It is better to wait for more progress.

Consider per-UE gap as baseline, and open to discuss per-FR, per-CC (group) gap.

Unified GAP

NR R15 and R16 focus on the fundamental design of measurement gaps. NR Rel-17 MGE WID introduces the following three enhanced features:

- Pre-MG: support UE autonomous or NW-controlled MG (de)activation in case of active BWP switching, MO addition/removal, and SCell activation/deactivation and so on. In this way, the frequent RRC signalling to reconfigure MG can be avoided and the related latency can be reduced.

  - With these benefits, pre-MG is suggested to be supported in 6G.

- Concurrent MG: support multiple concurrent gaps in parallel, where each of them is associated with different MOs. Compared with single per-UE gap or single per-FR gap for each FR, con-MG is beneficial for the case when the target MOs are mis-aligned in the time domain. For example, if MG is required for MO1 (160ms periodicity and 0ms offset) and MO2 (160ms periodicity and 40ms offset), then NW have to configure a MG with 40ms periodicity as shown in Figure 10(a) without con-MG, and half of MG occasions are redundant. For UE supporting con-MG, NW can configure two MG with 160ms periodicity as shown in Figure 10(b).

  - Concurrent MG is suggested to be considered, if similar configuration of MOs will occur in 6G.

Examples of MG and MO configuration

- NCSG: achieve a good trade-off between measurement/mobility and data throughput by reducing the interruption due to measurements via vacant RF chains. Compared with normal MG, only two small interruptions (1ms or 0.75ms) are allowed in VIL and data transmission could be maintained during ML.

  - With above benefits, NCSG is suggested to be considered in 6G.

From RRM perspective, R17 only defines requirements for each individual feature and further the joint operations for case 1 con-MG with Pre-MG and case 2 con-MG with NCSG are considered in Rel-18. However, the RRC signalling in RAN2 already considered the joint operation from R17 and these is a mis-alignment between RAN2 configuration and RAN4 RRM requirements.

As below is a brief summary for measurement gap enhancement in RAN4 from R17 to R19. Devise types of gaps and different solutions for gap activation/gap collision/gap skipping are discussed and specified in different work items in RAN4, which make current RRM requirements related to measurement gap in TS38.133 quite obscure and difficult to understand.

The features of RAN4 measurement gap enhancement

As 6G is expected to be more efficient and flexible than 5G NR, a unified GAP design covering all above enhanced features should be studied, including new joint combinations.

To enable more efficient and flexible measurement, unified GAP needs to be studied to cover the functions of pre-configured gap, parallel configured gap, and small gaps.

However, there also existing some problems of the above gaps.

- Example#1: NCSG is required for deactivated SCell measurement. When the SCell is activated, this SCell may be measured without GAP, depending on whether the SSB is contained in the active BWP. In this case, the fast switch between NCSG and SMTC (e.g., implemented by Pre-NCSG activation), or transition between NCSG and MG is preferred. In NR, RAN4 discussed this scenario in case of con-MG with NCSG, and reached a conclusion that the deactivated SCell is measured with NCSG by default, which is a compromised solution.

- Example#2: when MO1 with 1ms SMTC duration and MO2 with 5ms SMTC duration are expected to be measured in the same MG, network can only configure a MG with 6ms MGL to cover both MOs in 5G NR. For the MG occasion measuring MO1, 4ms duration are wasted. To improve efficiency, the MGL adapted with the MO can be considered.

- Example#3: The Pre-MG (de)activation mechanism is highly dependent on the BWP. For NW-controlled mechanism, the (de)activation bit is configured for each BWP and UE is required to check this information for every active DL BWP. For UE autonomous mechanism, Pre-MG should be activated if UE cannot perform the measurement without gap, regardless whether some or all target SSBs are contained in the active DL BWPs. The current Pre-MG may not be flexible enough for 6G. Especially, Pre-MG status indication is carried in each BWP configuration and the (de)activation is quite complicated. We think per-UE signalling based scheme may be more reliable and easier. In addition, dynamic signalling like MAC CE or DCI can be considered, which is already supported in NR, e.g., pre-configured MG activated by MAC CE in R18 Positioning WI.

- Example#4: R19 XR agreed on DCI based gap cancelling solution. But RAN4 debated for a long time on the cons and pros of semi static and dynamic GAP skipping. RAN4 should be involved earlier in 6G and study the scheme of GAP skipping or cancelling, and evaluate the feasibility and RRM impact of different skipping rules.

Unified Gap

In summary, RAN4 needs to come up with a solution to enable efficient adaptation among all supported GAPs for UE in 6G. In our view, the following aspects can be considered in RAN4:

- Unified GAP configuration, e.g., enabling Pre-MG, con-MG, NCSG

- Simplified GAP (de)activation, e.g., per UE/FR/carrier level (de)activation

- Efficient GAP change or gap adaptation

- Semi static and dynamic GAP skipping or cancelling

- Unified solution for gap collision

Study the following aspects for unified GAP design:

Unified GAP configuration, e.g., enabling Pre-MG, con-MG, NCSG

Simplified GAP (de)activation, e.g., per UE/FR/carrier level (de)activation

Efficient GAP change or gap adaptation

Semi static and dynamic GAP skipping or cancelling

Unified solution for GAP collision

GAP sharing

When network signals “01”, “10” or “11” with RRC parameter MeasGapSharingScheme [2][16]and the value of X is defined as in table 9.1.2.1-1, and - Kintra = 1 / X * 100, - Kinter = 1 / (100 – X) * 100, When network signals “00” indicating equal splitting gap sharing, X is not applied. The RRC parameter MeasGapSharingScheme shall be applied to the calculation of carrier specific scaling factor as specified in clause 9.1.5.2.1. - Table 9.1.2.1-1: Value of parameter X for EN-DC measurement gap sharing measGapSharingScheme; Value of X (%). ‘00’; Equal splitting. ‘01’; 25. ‘10’; 50. ‘11’; 75. NOTE: It is left to UE implementation to determine which measurement gap sharing scheme in the table to be applied, when MeasGapSharingScheme is absent and there is no stored value in the field..

When multiple MOs are expected to be measured in a gap, gap sharing is considered. As shown above, an RRC parameter MeasGapSharingScheme is used to configure the gap sharing ratio between intra-frequency and inter-frequency carriers, where inter-frequency carriers include inter-frequency and inter-RAT in NR R15. The reason is that intra-frequency mobility is the most typical scenario and should be separately considered. In the later releases, more types of measurements like PRS and L1-RSRP measurements are further supported to be measured within gap.

However, the gap sharing scheme is not updated accordingly and we only consider these new measurement type as inter-frequency carriers. In 6G, more measurement types should be considered for gap sharing scheme. One example is shown as Table 4, where the sharing ratios for intra-frequency, inter-frequency, inter-RAT NR and L1 measurements could be explicitly configured. Moreover, the last column is left as others for the potential new measurement types in the later releases, like PRS measurement or sensing, and some code-points could be reserved for them.

An example of gap sharing scheme

| Gap sharing | Intra-frequency | Inter-frequency | Inter-RAT NR | L1 measurement | Others (e.g., PRS, sensing) |
| --- | --- | --- | --- | --- | --- |
| ‘000’ | Equal splitting |  |  |  |  |
| ‘001’ | 25 | 25 | 25 | 25 | 0 |
| ‘010’ | 50 | 20 | 20 | 10 | 0 |
| ‘011’ | 75 | 10 | 10 | 5 | 0 |
| ‘100’ ~ ‘111’ | Reserved |  |  |  |  |

For GAP sharing scheme, consider more measurement types.

Conclusion

We provide the following observations and proposals.

What is 6G RRM

- RAN1 physical layer design, RAN2 protocol procedure design, and RAN4 RF assumption have impact on RAN4 RRM.

- From RRM requirements’ perspective, more efficiency and less energy consumption, higher throughput and less interruption, should be considered as target for next generation technique innovation.

The RRM impact of RAN1/2 new design for 6G, such as physical layer signal/procedure, coverage and energy saving schemes, needs to be studied in RAN4 as early as possible.

The equivalent aspects of RAN4 led work items in 5G (such as GAP, HST, Multi Rx, etc.) can be reconsidered for 6G.

RAN4 to take Table 1 as starting point for 6G study of RRM, and both basic R15 features and some enhanced features in later releases of NR can be considered in 6G Day 1.

- RRM requirements framework

| Feature | R15 (Baseline) | R16/R17/R18/R19/R20 (Enhancement) |
| --- | --- | --- |
| Idle-state Mobility | - DRX based cell (re)selection - Detection, evaluation, and measurement - Paging interruption | - Idle CA/DC measurement, e.g., EMR; - RRM measurement relaxation, e.g., low mobility and not-at-cell edge - Fast CA/DC setup, e.g., eEMR - OD-SIB1 impact - LP-WUS/WUR, e.g., measurement relaxation and offloading |
| Connected-State Mobility | Handover: - SSB based on L3 measurements. - Delay and interruption requirements CA operation: - Single SCell (de)activation - SCell addition/removal | Handover: - L1/L2 triggered mobility, e.g., SSB/CSI-RS based L1-RSRP measurement - CHO, CLTM SCell/PScell operation: - PUSCH SCell activation - Direct SCell activation - Multiple SCell activation - SCG activation - SSB-less SCell - On-demand SSB for SCell - MAC-CE based LTM and SCell activation |
| Timing | Timing & Synchronization - UE Transmission Timing: TA range and accuracy. - Measurement Timing: Alignment of RSRP measurement windows with SSB periods (i.e.,160ms). | - Multi-TAG - Reference Cell for SSB-less cell - One shot large UL timing adjustment |
| Scheduling | - BWP switching - TCI state switching - Interruption | - Carrier Switching - Antenna switching - UCBW change |
| Measurement procedure | L3 Measurements: - Measurement gap (MG) - delay and accuracy for intra-/inter-frequency, inter-RAT L1 Measurements: - RLM/BFD/CBD/L1-SINR | - Gap enhancement, e.g., needforgap, concurrent gap, pre-MG, NCSG, MUSIM, positioning gap - SCell BFD - Unified TCI - BWP without restriction |
| Note 1: All potential device types which are discussed in RAN1/2 could be included, e.g., normal UE, Redcap UE or IoT-like devices. |  |  |

Interests on RAN4 driven RRM topics

RF and baseband assumption are RAN4 specific, which can be taken as RAN4 driven topics as well.

Study the RRM impact of new spectrum of 6G and possible new UE RF/baseband architecture(s).

Measurement and gap related topics can be firstly considered as RAN4 driven for 6G.

RAN4 to investigate which features can be studied in parallel with RAN1/2.

MIMO operation, multi-TRP

mobility

initial access

spectrum aggregation

RRM related coverage and energy efficiency

other PHY signal/channel/procedure related RRM

Different solutions and requirements from 5G are on the table as reference for 6G, such as L1-RSRP based LTM or L3-RSRP based RRM measurement, AO-SSB or OD-SSB based measurement, basic CSSF or enhanced CSSF.

RAN4 to investigate a harmonized design for 6G RRM, at least considering both L1 and L3 measurement, always-on and on-demand signals, and other comprehensive assumptions for RRM measurement.

RAN4 to study the following RRM topics with high priority in 6G study phase:

Unified L1 and L3 measurement and report

Unified GAP design

RRM impact due to new spectrum aggregation operation, e.g., SCMC, enhanced CA

RRM impact due to UE and network energy savings, e.g., OD-SSB based measurement, low-power wake-up receiver, or low-capability device types.

RRM related UE capability

UE measurement capabilities for 6G are assumed to be able to cover those for 5G, at least including frequency layers, cells, searchers, RSs, measurement gaps.

RAN4 to consider the minimum UE requirements and avoid too many UE capabilities and corner cases in 6G RRM discussion, leaving more flexibility for implementation.

Unified RRM Measurement

The basic structure of measurement procedure for idle and connected mode can be reused.

Reuse NR basic assumption and procedure for 6G, e.g.,

Idle/connected mode measurement procedure

Intra and inter-frequency definition

Scenarios of gap-based or gap-less

Sharing factor, e.g., CSSF, P, Kp

Known/unknown cell definition

RAN4 to study a unified solution to support both L1 and L3 measurement, at least including:

Identify the need of unified configuration for same function, e.g., RS, MO, MTC, GAP

Evaluate RRM impact of unified cell switch/handover, e.g., triggers/conditions/reports

Assess the difference of measurement requirements and measurement restriction

RAN4 to focus on cross-layer L1 and L3 unified measurements firstly in 2025 Q4. Further evaluate RRM impact of cross-function L1 measurement like MIMO and LTM based on initial assumption or conclusion from other WGs.

GAP framework

The requirements for measurement with gap, with NCSG, without gap, without gap without interruption, without gap with interruption, are too complicated due to complex classification of MO.

Strive for a simple and clear classification of MO (e.g., with gap, with NCSG, without gap, without gap without interruption, without gap with interruption) in the first release of 6G.

Consider a simple and unified UE capability report to cover the existing NR reports, including needForGap, needForGapNCSG, needForInterruption.

GAP pattern

A small set of gap patterns can avoid complicated UE capabilities and gap applicability rules.

The gap patterns in 6G should match with target measurement purposes, including both 5G NR measurement and 6G measurement.

Select a subset of NR gap patterns for inter-RAT NR measurement in 6G

Option 1: prioritize NR mandatory gap patterns

Option 2: study FR-agnostic gap patterns assuming 0.5ms RF switch time for all FRs

Study 6G-specific gap patterns, at least considering new design of reference signals or MTC.

Simplify UE capabilities on the support of gap patterns in 6G.

Consider per-UE gap as baseline, and open to discuss per-FR, per-CC (group) gap.

Unified GAP

To enable more efficient and flexible measurement, unified GAP needs to be studied to cover the functions of pre-configured gap, parallel configured gap, and small gaps.

Study the following aspects for unified GAP design:

Unified GAP configuration, e.g., enabling Pre-MG, con-MG, NCSG

Simplified GAP (de)activation, e.g., per UE/FR/carrier level (de)activation

Efficient GAP change or gap adaptation

Semi static and dynamic GAP skipping or cancelling

Unified solution for GAP collision

GAP sharing

For GAP sharing scheme, consider more measurement types.

References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO, RANP #108

- R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 Chair, RAN4 #116

- R4-2508379, LS on mandatory gaps capability, Ericsson, RAN4 #115

- TS 38.133-i90