3GPP TSG-RAN WG4 Meeting #116bis									R4-2513134

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	8.6

Source:	CMCC

Title:	Discussion on 6G spectrum

Document for:	Discussion

Introduction

In RAN#108, the technology study item for 6G radio (6GR) has been approved [1]. The anticipated services of 6GR range from immersive communication and massive communication to integrated sensing and communication. Furthermore, 6G services can be provided by both terrestrial network (TN) and non-terrestrial network (NTN).

According to chairman’s suggestion, this scope includes band/band combination definition and simplification, the definition of frequency ranges, 6G spectrum related other aspects, including the regulatory status survey..

- In this contribution, we show out view on 6G spectrum.

Discussion

  - The definition of frequency ranges

The RAN led study on 6G Scenarios and requirements has been approved at RAN#108 [1]. Among its objectives, it includes

- Develop 3GPP requirements for 6G Radio for these practical deployment scenarios to ensure substantial gains in all relevant bands: overall performance, user experience, TCO reduction including at least:

  - …

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band

  - …

- Based on the discussion in RAN1 and RAN plenary, we have following agreements

Regarding the RAN1 agreement below, the following is agreed: Keep using “around 7GHz” instead of using detailed frequency range definition in RAN1’s and RAN4’s study. - Still keep “FFS: for around 15GHz” without replacing it with sub-24.25GHz.

- It’s RAN4’s scope to discuss the frequency range for FR1 and RAN4 can further discuss based on previous RAN1 discussion options as below:

- How to categorize the frequency range? Should the spectrum resource from 7.125GHz to 8.4GHz be categorized into the lower group or the higher group?

  - Option 1: extend FR1 to 8.4 GHz and define a separate mid-high band (8.4–24.25 GHz) with its own single numerology.

  - Option 2: extend FR1 to 8.4 GHz and merge 8.4–24.25 GHz into FR2, creating a unified FR2 family.

  - Option 3: define separate FR for 7.125 – 24.25 GHz.

- Proposal 1: it’s RAN4’s scope to discuss the frequency range for FR1 based on the options proposed in RAN1 as above.

Following list the progress of around 7GHz in ITU.

6 425 – 7 125 MHz range has been identified as IMT spectrum regional at WRC-23. Despite its previous strong opposition, India has shifted its position and now offers support IMT identification for 6425-7125MHz. Besides, following list the candidate IMT spectrum under WRC-27 agenda item 1.7

- 4400-4800MHz: the major proponents is Russia. US tends to abandon 7125-8400MHz and shift its position to support these frequency range

- 7125-8400MHz: this frequency range was opposed by many countries and regional groups, including CEPT, due to strategic usages. These strategic usages are, and will remain, critical in all of this frequency band. Besides, this frequency range is intensively used by a large number of government satellite systems for down-links (i.e. 7 250-7 750 MHz) and up-links (i.e. 7 900-8400 MHz). The upper part of the band (i.e. 8 025-8 400 MHz) is also used for the downlink of Earth observation data by a large majority of Earth Observation satellites. On the other side, Japan and Korea tends to support such frequency range.

- 14.8-15.35GHz: now no countries/groups strongly support such bands

Observation 1: above list the regulatory status survey of 6425-7125MHz, 4400-4800MHz, 7125-8400MHz and 14.8-15.35GHz.

- During the RAN plenary, some satellite operators suggest to postpone any 6G study and normative work with with regard to IMT identification, on the frequency range [7250 - 7750 MHz] and [7900 - 8400 MHz], at least until WRC27 final decision. The 6G study phase will end in June of 2027, the detailed band definition of 7125-8400MHz can wait for conclusion of WRC-27. now at study phase, RAN can just use around 7GHz as 6G new spectrum since at least 6425-7125MHz has been identified as IMT spectrum.

- Proposal 2: it’s work item scope of detailed band definition among 7125-8400MHz based on the conclusion of WRC-27. at study phase, RAN4 study system parameter/RF requirements using around 7GHz as 6G new spectrum.

Band/band combination definition and simplification

- During 5G stage, RAN 4 specified 70+ single bands and 10000+ band combinations numbers for CA, EN-DC, NE-DC, NR-DC, SUL which introduce huge workload for RAN4 requirement definition and RAN5 testing. As a results, increasing the RF front end complexity for UE when it has to support as much band combination as possible.

- Following options are proposed to simplify RAN4 and RAN5 workload of band combinations:

- Option 1: use band group concept. For the bands belong to the same band group, switching instead CA is suggested. For the bands cross band group, the CA is suggested. This is something like what is done for the LBCA WID which use switching instead of CA for less than 1GHz spectrum. And RAN4 can further study the applicable frequency range, e.g. further check whether can extend the upper frequency to 2GHz. The main benefit is to reduce RF front end complexity and reduce insertion loss caused by duplexer to enlarge UL/DL coverage.

- Option 2:reuse current 5G RF requirements for 6G for the legacy band/band combinations and further study the method to reduce conformance testing workload.

- Option 3: define new bands for the lower frequency range.

- RAN4 can further study the band/band combination definitions and simplification based on above options.

- Proposal 3: RAN4 can further study the options to reduce band combination workload and reduce RF front end complexity based on following options:

- Option 1: use band group concept. For the bands belong to the same band group, switching instead CA is suggested. For the bands cross band group, the CA is suggested.

  - RAN4 can further study the applicable frequency range, e.g. further check whether can extend the upper frequency to 2GHz.

- Option 2:reuse current 5G RF requirements for 6G for the legacy band/band combinations and further study the method to reduce conformance testing workload.

- Option 3: define new bands for the lower frequency range.

- Option 4: decouple DL and DL and support feasible combinations

Conclusion

This contribution makes the following observations and proposals.

- How to categorize the frequency range? Should the spectrum resource from 7.125GHz to 8.4GHz be categorized into the lower group or the higher group?

  - Option 1: extend FR1 to 8.4 GHz and define a separate mid-high band (8.4–24.25 GHz) with its own single numerology.

  - Option 2: extend FR1 to 8.4 GHz and merge 8.4–24.25 GHz into FR2, creating a unified FR2 family.

  - Option 3: define separate FR for 7.125 – 24.25 GHz.

- Proposal 1: it’s RAN4’s scope to discuss the frequency range for FR1 based on the options proposed in RAN1 as above.

- 6 425 – 7 125 MHz range has been identified as IMT spectrum regional at WRC-23. Despite its previous strong opposition, India has shifted its position and now offers support IMT identification for 6425-7125MHz. Besides, following list the candidate IMT spectrum under WRC-27 agenda item 1.7

- 4400-4800MHz: the major proponents is Russia. US tends to abandon 7125-8400MHz and shift its position to support these frequency range

- 7125-8400MHz: this frequency range was opposed by many countries and regional groups, including CEPT, due to strategic usages. These strategic usages are, and will remain, critical in all of this frequency band. Besides, this frequency range is intensively used by a large number of government satellite systems for down-links (i.e. 7 250-7 750 MHz) and up-links (i.e. 7 900-8400 MHz). The upper part of the band (i.e. 8 025-8 400 MHz) is also used for the downlink of Earth observation data by a large majority of Earth Observation satellites. On the other side, Japan and Korea tends to support such frequency range.

- 14.8-15.35GHz: now no countries/groups strongly support such bands

Observation 1: above list the regulatory status survey of 6425-7125MHz, 4400-4800MHz, 7125-8400MHz and 14.8-15.35GHz.

- Proposal 2: it’s work item’ scope for detailed band definition among 7125-8400MHz based on the conclusion of WRC-27. At study phase, RAN4 study system parameter/RF requirements using around 7GHz as 6G new spectrum.

- Proposal 3: RAN4 can further study the options to reduce band combination workload and reduce RF front end complexity:

- Option 1: use band group concept. For the bands belong to the same band group, switching instead CA is suggested. For the bands cross band group, the CA is suggested.

  - RAN4 can further study the applicable frequency range, e.g. further check whether can extend the upper frequency to 2GHz.

- Option 2:reuse current 5G RF requirements for 6G for the legacy band/band combinations and further study the method to reduce conformance testing workload.

- Option 3: define new bands for the lower frequency range.

- Option 4: decouple DL and DL and support feasible combinations

References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO