3GPP TSG-RAN WG4 Meeting#116bis		R4-2513147

Prague, Czech, Oct 13th -17th, 2025

Agenda item:	8.3

Source: 	Spreadtrum, UNISOC

Title: 	Views on 6G system parameters

Document for:	Discussion

Introduction

In RAN4#116 meeting, a Plan [1] about RAN4 6G guidance for information was provided by RAN4 chair. In this contribution, we want to provide some initial views on 6G system parameters.

6G topics; Scope; Scope in Q4’2025 and Q1’2026. System parameters; waveform, modulation, CBW, FFT, numerology, #Rx, #Tx, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types; waveform, modulation, CBW, FFT, numerology, #Rx, #Tx, synchronization signal and raster, spectrum utilization, irregular channel bandwidth, device types. General RF and UE RF; non-spectrum UE RF, MIMO operation, duplexing, spectrum aggregation, initial access, multi-TRP, UE RF related coverage and energy efficiency and other joint UE-BS RF issues; non-spectrum and non-AI UE RF, spectrum aggregation，UE RF related coverage and energy efficiency. BS RF; BS RF, coexistence study, BS RF energy efficiency, duplexing, MSR; non-spectrum and non-AI BS RF, MSR, coexistence study. Spectrum; band/band combination definition and simplification, defintion of frequency ranges, spectrum related regulatory survey; band/band combination definition and simplification, defintion of frequency ranges, spectrum related regulatory survey. RRM; RAN4 driven non-AI RRM topics, MIMO operation, mobility, initial access, spectrum aggregation, multi-TRP, RRM related coverage and energy efficiency, other PHY signal/channel/procedure related RRM; RAN4 driven non-AI RRM topics,. Demod; RAN4 driven non-AI demod topics, MIMO operation, channel coding, multi TRP, demod related coverage and energy efficiency, other PHY signal/ channel/procedure related demod; RAN4 driven non-AI demod topics. AI; RAN4 driven use cases, RAN4 impacts of other WG driven use cases, RAN for AI; RAN4 driven use cases. 5G and 6G spectrum sharing; multi-RAT spectrum sharing for migration, mobility between 5G NR and 6G; Topics, if identified, which are not RAN1/2 dependent. Sensing; PHY functions and procedure, RF, coexistence and testability; Topics, if identified, which are not RAN1/2 dependent. OTA and testability; Testability methodology framework and key assumptions, including both AI and non-AI features; Testability methodology framework and key assumptions, including both AI and non-AI features. RAN4 operation efficiency; CR, spec improvement and modernization in general, new tools for band/ band combinations, other aspects; CR, spec improvement and modernization in general, new tools for band/ band combinations, other aspects.

Discussion

Waveform

Orthogonal Frequency Division Multiplexing (OFDM)-based waveforms, i.e., multiple carrier waveform cyclic prefix OFDM (CP-OFDM) and single carrier waveform discrete Fourier transform spreading OFDM (DFT-s-OFDM), were used for the LTE and NR systems. As thoroughly investigated in LTE and NR, the primary advantages of OFDM-based waveforms are their high spectral efficiency with multicarrier characteristics, strong anti-multipath interference and inter-symbol interference ability and MIMO compatibility. This efficiency can be further augmented through higher-order modulation schemes.

According to the RAN1’s 122 meeting agreements, CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink and CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink. DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink.

Agreement CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. Agreement CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink Note: proponents to identify at least the target use cases, signals/channels to use the waveform, and how the proposal is intended (if applicable) to support multiplexing with CP-OFDM, including MRSS, and how multi-user multiplexing is supported, etc.

Even though power limitations are less critical in the downlink compared to the uplink. However, for massive IoT usage scenarios, MCL is potential higher than eMBB usage scenarios. In our understanding, it is valuable to introduce DFT-s-OFDM offers coverage enhancement and power consumption for massive IoT usage scenarios for downlink. It also provides lower PAPR compared with CP-OFDM as well as improved robustness against phase noise and frequency offsets. In the meantime, there is no need to realize multilayer for massive IoT usage scenarios, one layer maybe enough for massive IoT usage scenarios. From RAN4 UE perspective, for this waveform, we can study the RFSFENS and demodulation performance and other requirements.

Proposal 1: DFT-s-OFDM waveform can be supported for 6GR for downlink (e.g., massive IoT usage scenarios). From RAN4 UE perspective, for this waveform, we can study the RFSFENS, demodulation performance and other requirements.

Modulation

According to the agreements in RAN1#122 meeting, 5G NR uniform modulation are supported as basis for study for data channel.

Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

From LTE to NR, the demand for higher data rates has led to a continuous increase in QAM modulation orders, until NR reached 1024QAM for downlink and 256QAM for uplink. RF impairments are one of the main concerns for going into even higher order modulation. The primary contributors to RF impairments are IQ mismatch, RF nonlinearity, and phase noise.

According to RAN1’s agreements, other modulation schemes will be further discussed. From RAN4 perspective, for 1024 QAM UL, SNR needs to be higher than 30dBm, EVM requirement needs to be less than 3.16%. Based on WIFI spec requirements, when UE supports 1024QAM UL, SNR should be higher than 35dB. General speaking, in real scenarios, SNR may need to be achieved 35dB due to interference and multipath fading etc., Then EVM requirements needs to be less than 1.78%. It is a big challenge for UE to achieve 1024 QAM with high cost and power consumption. The coverage range reduces a lot for high order modulation. Only when UE is very close to gNB could sustain the 1024QAM for UL. For DL, NR has supported 1024QAM. Based on the TPR in 6G SID, user experience data rate is more important than peak data rate. In our understanding, the highest modulation order can be 256 QAM in 6GR UL in Day1 and 1024 QAM in 6GR DL in day1.

Proposal 2: For 6GR Day1, the highest modulation order can be 256QAM for UL, the highest modulation order can be 1024QAM for DL.

Channel bandwidth

In LTE and 5G, different channel bandwidth for different band/spectrum. For 6GR, FR3 is introduced. In NR, the maximum channel bandwidth is 100M as table 1 in TS 38.101-1.For 6GR, we can extend 200MHz as the maximum channel bandwidth for U6G and FR3 frequency to increase throughput and improve user experience.

Table 1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

For sub-1GHz to sub-6GHz per band, we can reuse the definition of NR. Whether sub-6GHz can be extended to 200MHz can be further studied.

Proposal 3: For 6GR Day1, the channel bandwidth as proposed that:

- 200MHz as the maximum channel bandwidth for FR3 and U6G;

- For sub-1GHz to sub-6GHz, we can reuse the definition of NR. Whether sub-6GHz can be extended to 200MHz can be further studied.

FFT and numerology

As is known to us, bandwidth is basically equal to the number of sub carriers multiplied by the value of SCS. The number of sub-carriers is up to the N of FFT. SCS is up to numerology. Therefore, CBW, FFT and SCS are interrelated and influenced. Based on the analysis in the previous chapter about CBW, FFT with 8K for 200MHz can be considered. For SCS, in order to simplify the design of 6G system, one single numerology and SCS per band/frequency range is necessary. In the meantime, regarding the co-existence with NR, we propose that for FDD, SCS can be 15 kHz and the maximum FFT can be 4096. For TDD, SCS can be 30 kHz and the maximum FFT can be 8192. For FR2-1, we can use 120 kHz SCS and the maximum FFT can be 4096.

Proposal 4: For FDD, SCS can be 15 kHz and the maximum FFT can be 4096; for TDD, SCS can be 30 kHz and the maximum FFT can be 8192. For FR2-1, we can use 120 kHz SCS and the maximum FFT can be 4096.

Tx/Rx MIMO

Tx/Rx MIMO improves the capacity, coverage and reliability of communication systems exponentially without increasing spectrum bandwidth and transmission power. It is one of the critical technologies of 5G NR. Similarly, it is still one of the critical technologies of 6GR. Regarding the cost and limitation of the form factor, different numbers of Tx/Rx MIMO for different device types. 2Tx/4Rx is defined for handheld UE in 5G. Like other types, 1Tx/2Rx for XR and (e)Redcap UE. 1Tx/1Rx for IoT devices in 5G. In our understanding, we can reuse the NR for Tx/Rx MIMO as the baseline for 6GR.

Proposal 5: Reuse the NR for Tx/Rx MIMO as the baseline for 6GR.

- 2T4R can be handheld UE. 4Tx/8Rx can be FWA UE.

- 1Tx/2Rx can be for XR and (e)Redcap UE.

- 1Tx/1Rx can be for IoT devices.

Synchronization signal and raster

Sync. Raster

In order to save power from the network, many companies think that SSB can be extended (e.g., 80ms/160ms). Along with the expansion of SSB period mentioned, if the sync raster is unchanged, then the cell search time of UE will be longer. User experience in 6GR cannot be worse than 5G, thus, a more sparse sync raster compared to NR for 6GR is necessary. In addition, a more sparse sync raster will save energy consumption and decrease complexity in the cell search of UE. Therefore, 6GR should consider a more sparse Sync raster design, especially for low-capability device types.

However, the principle in NR for sync raster is to guarantee at least one SSB in one CBW. The formula is that sync raster ≤ min CBW- SSB CBW+ channel raster. In 5G, we have defined so many sync raster due to the principle. Nevertheless, according to the survey of the network deployments, operators only use a part of these sync raster in practically. From this point of view, whether the principle in NR for sync raster to be kept in 6GR needs to be discussed. If we should follow the principle, increasing min CBW or decreasing SSB CBW (depending on RAN1’s design) can make the sync raster be sparser. If we didn’t follow the principle, maybe we could define fixed and limited sync raster to achieve sparse sync raster.

Proposal 6: More sparse sync raster design can be considered in 6GR. Whether the principle in NR for sync raster should be followed in 6GR needs to be discussed.

Spectrum utilization

From spectrum perspective, FR3 (7.125GHz to 24GHz) is identified as 6G new band. In addition, existing frequency bands including at least the full range of FR1 (up to 7.125GHz) and FR2-1 (24.25 GHz – 52.6GHz) should also be supported. Multiple diverse spectrum resources are shown in Figure 1. In order to maximize the utilization of the 6G new bands and existing frequency bands, an efficient and simple spectrum utilization and operation is critical for 6GR day 1.

Figure1: candidate spectrum resources for 6GR [2]

According to the RAN#109 meeting, there are two proposals agreed, these two proposals are shown as follow.

Proposal 1: 6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR. Proposal 2: 6GR aims to support flexible utilization of spectrum resources for DL and UL over different carriers/bands.

For proposal1, how to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR. It is observed that there are so many fragmented spectrum which have smaller bandwidth especially in low FR1 bands, to meet the high data rate requirement of 6G, multiple non-continuous carriers with small bandwidth can be aggregated to achieve one wider bandwidth. If these multiple carriers are all managed as independent cells, the spectrum utilization efficiency will be low due to multiple cell management overhead and common signalling overhead. Thus, aggregating multiple fragmented carriers as a single cell can be studied to save common channels/signals overhead, simplify cell management and achieve fast carrier switching/activation. At least multiple intra-band carriers can be considered using this MCSC (Multi-Carrier Single Cell) mechanism. From RAN4 perspective, UE architecture, interference and other requirements need to be considered. On top of MCSC, NR CA mechanism should be maximally reused to increase user throughput.

Proposal 8: For supporting improved spectrum utilization and operations over one or more carriers/bands, these mechanisms can be considered.

- CA mechanism.

- MCSC (Multi-Carrier Single Cell) mechanism.

Proposal 9: For MCSC, from RAN4 perspective, UE architecture, interference and other requirements need to be considered.

For proposal 2, how to support flexible utilization of spectrum resources for DL and UL over different carriers/bands. Based on some company’s information, it means decoupling DL and UL resources. For example, FDD band for UL to achieve better coverage, TDD band for DL to achieve high throughput. However, if we use 5G existing mechanism, such as SUL, SUL needs to be band combination with other NR bands which cannot be existing independently, it is not flexible enough and high spectrum efficiency; if we use new mechanism, whether we need to redefine the definition of bands, whether the existing FDD modes include duplexer can be used, whether we need to redesign filter for these decouple mechanism. In addition, some DL and UL spectrum resources will interfere with each other, which means that the uplink and downlink spectrum cannot be paired freely and arbitrarily.

Proposal 10: To support flexible utilization of spectrum resources for DL and UL over different carriers/bands, these aspects need to consider.

- Whether to redefine the definition of bands

- Whether the UE architecture is changed(e.g., filter)

- Interference between DL and UL spectrum resources

Irregular channel bandwidth

In NR, the channel bandwidths were limited to increments of 5 MHz, 3MHz was introduced in later releases. And the study item of irregular channel bandwidth was agreed to study 6/7/11/12/13 MHz in Rel-18. Rel-19 and Rel-20 agreed to specify 7MHz and 6MHz requirements. These irregular channel bandwidths come from operators’ requests to fully utilize spectrum resources. Due to the scarcity of spectrum resources, which causes the spectrum fragmentation of many operators’ spectrum. Operator’s spectrum will not be the same as the CBW defined in RAN4. However, it needs so much time if we specify every irregular channel bandwidth like NR. Therefore, we need to study a proper solution to address the operators’ request and don’t take so much time to specify for these irregular channel bandwidth and avoid the UE test burden in 6GR.

Proposal 11: A solution to address the irregular channel bandwidth with high efficiency (e.g., flexible BW) in 6G can be studied.

Device types

Design Principe

Firstly, with the new devices with urgent market demands (e.g., Redcap and 2RX XR) were introduced in 5G NR later release, which causes network deployment to be more complex. 5G NR cannot provide a good eco-environment for those UE types introduced in later releases.

Secondly, flexible UE capability report mechanism was adopted in 5G NR to support various features/use cases. However, statistically speaking, just only 70% of features are commercialized. Which means various features are over designed.

Lastly, there are various device types applied in 5GNR, which may cause market fragmentation, some device types haven’t formed a large-scale market.

Based on the above analysis, we propose the following design principles.

    - A limited set of device types to avoid market fragmentation

    - Device type should be categorized based on main communication usage scenarios (e.g eMBB, Massive IoT) together with some device hardware limitations (e.g. form factor)

    - Scalable and forward-compatible design

      - In order to achieve scalable and forward-compatible design, we have defined three kinds of capabilities.

      - A common basic mandatory function set is defined for all device types;

      - On top of that, a different additional mandatory capability set is defined for each device type;

      - Further, each device type may additionally indicate optional capability/feature sets for different new usage scenarios or new services.

Proposal 12: we proposed the design principle for device types as follows.

    - A limited set of device types to avoid market fragmentation in 6G day1

    - Device type should be categorized based on main communication usage scenarios (e.g eMBB, Massive IoT) together with some device hardware limitations (e.g. form factor)

    - Scalable and forward-compatible design

      - -  A common basic mandatory function set is defined for all device types;

      - -  on top of it, a different additional mandatory capability set is defined for each device type;

      - - Further, each device type may additionally indicate optional capability/feature sets for different new usage scenarios or new services.

6G device types

Based on the above design principles, device types are categorized based on two main usage scenarios, including eMBB and massive IoT, in addition, different form factors with different communication capabilities. Therefore, we propose to define three kinds of device types, including type A/B/C. Type A and type B belong to the eMBB usage scenario, and type C belongs to the massive IoT usage scenario. In order to achieve scalable and forward-compatible design to avoid market fragmentation, we have defined three kinds of capability/feature. A common basic mandatory function set is defined for all device types, such as waveform, frame structure, channel coding, initial access procedure, DL and UL control channel design, scheduling /HARQ operation and other capability/features. On top of that, a different additional mandatory capability set is defined for each device type, further, regarding new usage scenario or new services that will be introduced in the future, and optional additional capability sets are defined for each device type.

Based on the design principle and above analysis, we can take table 2 to define device types as a starting point.

Table 2: 6GR UE/Device type example

| 6G communication usage scenarios | Example device type | Scalable and forward-compatible design to avoid market fragmentation |  |  |  |  | Example devices/use cases |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A common basic mandatory function set for all device types | A different additional mandatory capability set for each device type | Optional capability/feature sets for different new usage scenarios or new services. |  |  |  |
| eMBB | TypeA | - Waveform - Frame structure - Channel coding - Initial access design - DL and UL control channel design - Scheduling/HARQ operation - BS/UE energy saving features etc. | - 200M - 2T/4R - Modulation: 256 QAM DL/UL - Power class: 3 | Optional set A1 - #MIMO layers - Processing time - Spectrum aggregation capabilities Power clas | Set A2 … 4T/8R | … | Smart phone, CPE/FW, VUE, VSAT, Robots… |
|  | Type B |  | - 100M/200M - 1T2R - Modulation: 256 QAM DL/UL - Power class: 3 | Optional set B1 - #MIMO layers - Processing time - Spectrum aggregation capabilities - Power class Access control, etc. | Set B2 … | … | 2RX XR, High-end watch |
| Massive IoT | Type C |  | - 20M - 1T1R - Modulation:64 QAM DL/UL Power class: 3 or 5 | Optional set C1 - Processing time - Power class - Access control Scaling factor, etc. | Set C2 … | … | Wearables, Industrial sensors, Low-end watch, Video surveillance … |

Proposal 13: The Table 2 could be considered as the starting point to achieve scalable and forward-compatible design.

Conclusions

In this paper, we provide our initial views on 6G system parameters, we have made the following proposals.

Proposal 1: DFT-s-OFDM waveform can be supported for 6GR for downlink (e.g., massive IoT usage scenarios). From RAN4 UE perspective, for this waveform, we can study the RFSFENS, demodulation performance and other requirements.

Proposal 2: For 6GR Day1, the highest modulation order can be 256QAM for UL, the highest modulation order can be 1024QAM for DL.

Proposal 3: For 6GR Day1, the channel bandwidth as proposed that:

- 200MHz as the maximum channel bandwidth for FR3 and U6G;

- For sub-1GHz to sub-6GHz, we can reuse the definition of NR. Whether sub-6GHz can be extended to 200MHz can be further studied.

Proposal 4: For FDD, SCS can be 15 kHz and the maximum FFT can be 4096; for TDD, SCS can be 30 kHz and the maximum FFT can be 8192. For FR2-1, we can use 120 kHz SCS and the maximum FFT can be 4096.

Proposal 5: Reuse the NR for Tx/Rx MIMO as the baseline for 6GR.

- 2T4R can be handheld UE. 4Tx/8Rx can be FWA UE.

- 1Tx/2Rx can be for XR and (e)Redcap UE.

- 1Tx/1Rx can be for IoT devices.

Proposal 6: More sparse sync raster design can be considered in 6GR. Whether the principle in NR for sync raster should be followed in 6GR needs to be discussed.

Proposal 7: For the re-farming bands, 6GR channel raster can be 10 kHz when these bands in NR can support 10 kHz. For other bands, channel raster can be SCS in 6GR.

Proposal 8: For supporting improved spectrum utilization and operations over one or more carriers/bands, these mechanisms can be considered.

- CA mechanism.

- MCSC (Multi-Carrier Single Cell) mechanism.

Proposal 9: For MCSC, from RAN4 perspective, UE architecture, interference and other requirements need to be considered.

Proposal 10: To support flexible utilization of spectrum resources for DL and UL over different carriers/bands, these aspects need to consider.

- Whether to redefine the definition of bands

- Whether the UE architecture is changed(e.g., filter)

- Interference between DL and UL spectrum resources

Proposal 11: A solution to address the irregular channel bandwidth with high efficiency (e.g., flexible BW) in 6G can be studied.

Proposal 12: we proposed the design principle for device types as follows.

    - A limited set of device types to avoid market fragmentation in 6G day1

    - Device type should be categorized based on main communication usage scenarios (e.g eMBB, Massive IoT) together with some device hardware limitations (e.g. form factor)

    - Scalable and forward-compatible design

      - -  A common basic mandatory function set is defined for all device types;

      - -  on top of it, a different additional mandatory capability set is defined for each device type;

      - - Further, each device type may additionally indicate optional capability/feature sets for different new usage scenarios or new services.

Proposal 13: The Table 2 could be considered as the starting point to achieve scalable and forward-compatible design.

Table 2: 6GR UE/Device type example

| 6G communication usage scenarios | Example device type | Scalable and forward-compatible design to avoid market fragmentation |  |  |  |  | Example devices/use cases |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A common basic mandatory function set for all device types | A different additional mandatory capability set for each device type | Optional capability/feature sets for different new usage scenarios or new services. |  |  |  |
| eMBB | TypeA | - Waveform - Frame structure - Channel coding - Initial access design - DL and UL control channel design - Scheduling/HARQ operation - BS/UE energy saving features etc. | - 200M - 2T/4R - Modulation: 256 QAM DL/UL - Power class: 3 | Optional set A1 - #MIMO layers - Processing time - Spectrum aggregation capabilities Power clas | Set A2 … 4T/8R | … | Smart phone, CPE/FW, VUE, VSAT, Robots… |
|  | Type B |  | - 100M/200M - 1T2R - Modulation: 256 QAM DL/UL - Power class: 3 | Optional set B1 - #MIMO layers - Processing time - Spectrum aggregation capabilities - Power class Access control, etc. | Set B2 … | … | 2RX XR, High-end watch |
| Massive IoT | Type C |  | - 20M - 1T1R - Modulation:64 QAM DL/UL Power class: 3 or 5 | Optional set C1 - Processing time - Power class - Access control Scaling factor, etc. | Set C2 … | … | Wearables, Industrial sensors, Low-end watch, Video surveillance … |

- Reference

- R4-2511652, RAN4 6G Plan and Meeting Arrangement, RAN4 chair. TSG-RAN4#116 meeting

- 6GWS-250071, MediaTek Views 6G Day1 Radio, MediaTek