3GPP TSG RAN WG4 Meeting #116bis		R4-2513173

Prague, Czech, October 13th – 17th , 2025

Title: 	Discussion on R20 HPUE coexistence study

Agenda Item:	7.1.3

Source: 	CATT

Document for:	Approval

1 Introduction

In the RAN#109 meeting, a new WID [1] for UE RF enhancement phase 5 was approved as one of R20 RAN4 5G-A package. The objectives are listed below.

High power UE (HPUE) for NR single carrier operation - Specify the UE RF requirements for single carrier TDD to support 4Tx for CPE/FWA UE with 4x26dBm PA configurations - Co-existence study for ACLR, specify ACLR requirements if needed - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 4x4 UL MIMO and TxD - Specify the band specific requirements for exemplary bands - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - PC1.5 with 2Tx for FDD bands for handheld and FWA UE - Specify the UE RF requirements including maximum output power (MOP), MPR requirements, etc. - Supporting 2x2 UL MIMO and TxD - Baseline PA configuration: 2 x 26 dBm - Specify the band specific requirements for exemplary bands - Example bands: n1, n25 - PC1.5 1Tx for TDD bands for FWA UE - Study the possibility of applying PC1.5 with 2Tx MPR/A-MPR requirements to PC1.5 with 1Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 1 x 29 dBm - Example bands: n41, n77/n78, n79 - This work is to start in 2026 Q4 - 2Tx for TDD bands for FWA UE - Study the possibility of applying PC1 with 4Tx A-MPR/MPR requirements to PC1 with 2Tx, if feasible, specify the corresponding UE RF requirements, including maximum output power (MOP), MPR/A-MPR requirements, etc. - Baseline PA configuration: 2 x 29 dBm - Example bands: n41, n77/n78, n79, n104 - Maximum output power is up to 32 dBm - RAN4 to determine whether to reuse PC1 or introduce new power class - This work is to start in 2026 Q4

For 32dBm MOP with 4x26dBm PA configurations, Co-existence study for ACLR is needed. In this paper, we’d like to provide our initial views on this topic.

2 Discussion

Based on the WID, the exemplary TDD bands n41, n78/n77, n79 and n104 are considered. Generally, RAN4 only consider Sub3GHz as the carrier frequency for coexistence study. However, the frequency range for these exemplary TDD bands are from 2.6GHz ~ 7.125GHz. Referring to the TR 38.922 [2], the AAS BS parameters for 1710~4990MHz are summarized below.

4.4.1.2 Antenna parameters The BS antenna parameters are listed in Table 4.4.1.2-1. Table 4.4.1.2-1: IMT parameters relevant for 1710 to 4990 MHz Parameter; Macro Rural; Macro suburban; Macro urban; Micro urban. Am; 30 dB; 30 dB; 30 dB; 30 dB. SLAv; 30 dB; 30 dB; 30 dB; 30 dB. 3dB; 90 deg.; 90 deg.; 90 deg.; 90 deg.. 3dB; 65 deg.; 65 deg.; 65 deg.; 65 deg.. GE,max; 6.4 dBi; 6.4 dBi; 6.4 dBi; 6.4 dBi. Msub; 3; 3; 3; N/A. dv,sub; 0.7m; 0.7m; 0.7m; N/A. subtilt; 3 deg.; 3 deg.; 3 deg.; N/A. M; 4; 4; 4; 8. N; 8; 8; 8; 8. dh; 0.5m; 0.5m; 0.5m; 0.5m. dv; 2.1m; 2.1m; 2.1m; 0.7m. etilt; 90≤θescan≤100 deg.; 90≤θescan≤100 deg.; 90≤θescan≤100 deg.; 90≤θescan≤120 deg.. escan; -60≤φescan≤60 deg.; -60≤φescan≤60 deg.; -60≤φescan≤60 deg.; -60≤φescan≤60 deg.. ; 1; 1; 1; 1. Ptx; 46 dBm; 46 dBm; 46 dBm; 37 dBm. mech; 3 deg.; 6 deg.; 6 deg.; N/A.

Referring to the TR 38.922 [2], the AAS BS parameters for 7125 to 8400 MHz are summarized below.

5.4.1.2 Antenna parameters The BS antenna parameters are listed in Table 5.4.1.2-1. Table 5.4.1.2-1: IMT parameters relevant for 7125 to 8400 MHz Parameter; Macro suburban; Macro urban; Micro urban. Am; 30 dB; 30 dB; 30 dB. SLAv; 30 dB; 30 dB; 30 dB. 3dB; 90 deg.; 90 deg.; 90 deg.. 3dB; 65 deg.; 65 deg.; 65 deg.. GE,max; 6.4 dBi; 6.4 dBi; 6.4 dBi. Msub; 3; 3; N/A. dv,sub; 0.7m; 0.7m; N/A. subtilt; 3 deg.; 3 deg.; N/A. M; 8; 8; 8. N; 16; 16; 8. dh; 0.5m; 0.5m; 0.5m. dv; 2.1m; 2.1m; 0.7m. etilt; 90≤θescan≤100 deg.; 90≤θescan≤100 deg.; 90≤θescan≤120 deg.. escan; -60≤φescan≤60 deg.; -60≤φescan≤60 deg.; -60≤φescan≤60 deg.. ; 1; 1; 1. Ptx; 46 dBm; 46 dBm; 37 dBm. mech; 6 deg.; 6 deg.; N/A.

Referring to the TR 38.921 [3], the non-Sub-Array AAS BS parameters for 6425 to 7125 MHz are summarized below.

Table 8.1.2-1: BS array antenna parameters Parameter; Macro Sub-urban; Macro Urban; Micro Urban; Small cell indoor. Am (dB); 30; 30; 30; 30. SLAv (dB); 30; 30; 30; 30. 3dB (deg.); 90; 90; 90; 90. 3dB (deg.); 65; 90; 90; 90. GE,max (dBi); 6.4; 5.5; 5.5; 5.5. LE (dB); 2.0; 2.0; 2.0; 2.0. (M, N); (16, 8); (16, 8); (8,8); (4, 4). Number of supported polarizations, P; 2; 2; 2; 2. dh (m); 0.5; 0.5; 0.5; 0.5. dv (m); 0.7; 0.5; 0.5; 0.5. Horizontal coverage range (deg.); +/- 60; +/- 60; +/- 60; N/A. Vertical coverage range (deg.); 90 to 100; 90 to 120; 90 to 120; N/A. Conducted power (before ohmic loss) per antenna element, Ptx (dBm); 22 (Note 4); 22 (Note 4); 16 (Note 5); 9 (Note 6). Mechanical downtilt (deg.); 6; 10; N/A; N/A (Note 7). Note 1: MxN means there are M vertical and N horizontal elements Note 2: LE is included in GE,max Note 3: The vertical coverage range includes the mechanical downtilt. Note 4: The conducted power per element assumes 16x8x2 elements (i.e. power per H/V polarized element). Note 5: The conducted power per element assumes 8x8x2 elements (i.e. power per H/V polarized element). Note 6: The conducted power per element assumes 4x4x2 elements (i.e. power per H/V polarized element). Note 7: Boresight direction is perpendicular to the ceiling..

Proposal 1: For band n104 coexistence study, RAN4 need to discuss which AAS BS parameters should be used, non-Sub-Array AAS BS parameters for 6425 to 7125 MHz in TR 38.921 or Sub-Array AAS BS parameters for 7125 to 8400 MHz in TR 38.922.

From frequency perspective, the following relationship can be found. From BS/UE technical implementation perspective, it’s possible to share one antenna between band n41 and n79, or between band n77/n78 and n104.

| Basic Frequency | n41 (2496~2690MHz) | n77/n78 (3300~4200MHz) |
| --- | --- | --- |
| 2nd harmonic Frequency | 4492MHz ~ 5380MHz | 6600~8400MHz |
| Higher Frequency bands | n79 (4400~5000MHz) | n104 (6425~7125MHz) |

The following scenarios are proposed to be considered.

| No. | Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 2.6GHz, 4.9GHz, 7 GHz | Urban macro |  |
| 2 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 2.6GHz, 4.9GHz, 7 GHz | Indoor hotspot | Down-prioritized |
| 3 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 2.6GHz, 4.9GHz, 7 GHz | Dense urban | Down-prioritized |
| 4 | eMBB | NR, 100MHz | NR, 100MHz | UL to DL | 2.6GHz, 4.9GHz, 7 GHz | Urban macro | If SBFD is considered |
| 5 | eMBB | NR, 100MHz | NR, 100MHz | UL to DL | 2.6GHz, 4.9GHz, 7 GHz | Indoor hotspot | If SBFD is considered |

Proposal 2: RAN4 can consider the following scenarios for coexistence study of UL output power 32dBm.

| No. | Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 2.6GHz, 4.9GHz, 7 GHz | Urban macro |  |
| 2 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 2.6GHz, 4.9GHz, 7 GHz | Indoor hotspot | Down-prioritized |
| 3 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 2.6GHz, 4.9GHz, 7 GHz | Dense urban | Down-prioritized |
| 4 | eMBB | NR, 100MHz | NR, 100MHz | UL to DL | 2.6GHz, 4.9GHz, 7 GHz | Urban macro | If SBFD is considered |
| 5 | eMBB | NR, 100MHz | NR, 100MHz | UL to DL | 2.6GHz, 4.9GHz, 7 GHz | Indoor hotspot | If SBFD is considered |

In the TR 38.921, the 0.45km ISD was assumed for Macro Urban.

| Parameters | Values | Remark |
| --- | --- | --- |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |  |
| Inter-site distance | 0.45 km (urban) (Note 1) 0.9 km (suburban) | Based on cell range: 0.3 km (urban) 0.6 km (suburban) |

Proposal 3: RAN4 can consider to reuse the 0.45km ISD assumption for Macro Urban scenario.

3 Summary

Proposal 1: For band n104 coexistence study, RAN4 need to discuss which AAS BS parameters should be used, non-Sub-Array AAS BS parameters for 6425 to 7125 MHz in TR 38.921 or Sub-Array AAS BS parameters for 7125 to 8400 MHz in TR 38.922.

Proposal 2: RAN4 can consider the following scenarios for coexistence study of UL output power 32dBm.

| No. | Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 2.6GHz, 4.9GHz, 7 GHz | Urban macro |  |
| 2 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 2.6GHz, 4.9GHz, 7 GHz | Indoor hotspot | Down-prioritized |
| 3 | eMBB | NR, 100MHz | NR, 100MHz | UL to UL | 2.6GHz, 4.9GHz, 7 GHz | Dense urban | Down-prioritized |
| 4 | eMBB | NR, 100MHz | NR, 100MHz | UL to DL | 2.6GHz, 4.9GHz, 7 GHz | Urban macro | If SBFD is considered |
| 5 | eMBB | NR, 100MHz | NR, 100MHz | UL to DL | 2.6GHz, 4.9GHz, 7 GHz | Indoor hotspot | If SBFD is considered |

Proposal 3: RAN4 can consider to reuse the 0.45km ISD assumption for Macro Urban scenario.

Reference

[1] RP-252942, “New WID: UE RF enhancements for NR FR1, Phase 5”, Moderator (Vivo)

[2] TR 38.922, “Study on International Mobile Telecommunications (IMT) parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz”