3GPP TSG-RAN WG4 Meeting #116bis                                                                        R4-2513184

Prague, CZ, 13th –17th Oct, 2025

Source: 	CATT

Title: 	Considerations on UE RF requirements for UAV

Agenda item:	7.7.4

Document for:	Approval

- Introduction

In RAN #109 meeting, a new WI [1] for UAV was set up and RAN4 work should be started in this meeting. In this contribution, we focus on the objectives in SID, share some views and proposals on UE RF requirements which will be impacted.

- Discussion

The objectives of the WID are shown as below, some UE RF requirements related items are highlighted.

The core part of WI: - Scenario: - BSs for UAV service are deployed on the ground by adjusting the tilt angle upwards. The coverage height is up to 600~1000m. The ISD for BSs for UAV service is about 2~4km. - A direct radio link between BS on the ground and UAV UE. - Specify RF requirements for UAV UE to support higher UL output power. - Example bands: - TDD with same TDD pattern, including n41, n77, n78, n79, - FDD: n3 - Consider UAV UE with omni-direction antenna or with antenna array - Consider UAV UE with up to 33dBm max total conductive output power - Analyse co-existence for UAV and considering the details relevant to UAV including antenna array model, and specify requirements if needed. - Specify UE RF requirements, e.g., maximum output power, configured transmission power, SEM. - Specify RRM requirements, if needed Note 1: No RAN1 impact are expected

General

The main application scenarios of UAV include low altitude logistics, travel, tourism, control and supervision, medical rescue, etc. As ATG UE, the newly introduced UAV UE in Rel-20 also can support antenna array. Following the same way, all RF requirements need to be defined on transceiver array boundary (TAB) connectors in addition to omni-directional antenna connectors. When we discuss the requirements, we should consider both architecture especially when there are different requirements.

Proposal 1: RF requirements need to be defined on both omni-directional antenna connectors and transceiver array boundary (TAB) connectors.

Maximum output power

For ATG UE, no power class was introduced, UE can report its rated maximum output power at maximum modulation order and full PRB configurations with granularity as 1dB from 23~40dBm. For UAV, as the application scenario is more similar as NR with 2~4km ISD, we think define power class as NR UE is appropriate. New power class should be defined as 33dBm for the example bands in WID.

MPR & A-MPR

MPR is PC-specific so simulation should be done for new PC or new PA model. A-MPR is determined by regional requirements which need to be investigated further.

Configured transmitted power

There should no big issue to change configured transmitted power, once new MPR and A-MPR are introduced, configured transmitted power can be improved accordingly.

Proposal 2: Introduce a new power class for UAV UE for 33dBm max total conductive output power for the example bands. MPR and configured transmitted power will be improved according to new power class. Regional requirements for example bands need to be investigated further for additional emission and A-MPR requirements.

Minimum output power & Maximum input level

Minimum output power & Maximum input power consider the UAV BS and UAV UE are nearest from each other or with the minimum coupling loss between them, these two requirements can based on link level calculation to obtain.

Proposal 3: Minimum output power and Maximum input level should base on link calculation.

ACLR & ACS

ACLR/ACS requirements depend on the coexistence study or analysis. The adjacent channel simulation includes two cases:

- NR and UAV in adjacent channel from same or different operator;

- UAV to UAV in adjacent channel from another operator.

In-band blocking

In-band blocking level should come from Monte-Carlo simulation by interference statistics. The simulation can reuse the coexistence simulation with interference level statistics, the interference comes from below two cases:

- NR BS in adjacent channel from same or different operator;

- UAV BS in adjacent channel from another operator.

Proposal 4: ACLR/ACS requirements and In-band blocking level should base on coexistence study or analysis.

- Conclusion

This contribution provides our views on UE RF requirements which will be impacted. The following proposals are concluded as follows:

Proposal 1: RF requirements need to be defined on both omni-directional antenna connectors and transceiver array boundary (TAB) connectors.

Proposal 2: Introduce a new power class for UAV UE for 33dBm max total conductive output power for the example bands. MPR and configured transmitted power will be improved according to new power class. Regional requirements for example bands need to be investigated further for additional emission and A-MPR requirements.

Proposal 3: Minimum output power and Maximum input level should base on link calculation.

Proposal 4: ACLR/ACS requirements and In-band blocking level should base on coexistence study or analysis.

Proposal 5: Except above proposals, other UE RF requirements are expected reuse NR UE requirements or previous UAV UE requirements except the clarification of test antenna connectors as proposal 1.

- References

- RP-252954,  New WID on enhancement of NR RF and RRM requirements for uncrewed aerial vehicle (UAV)