3GPP TSG RAN WG4 Meeting #116-bis		R4-2513209

Prague, Czech, Oct 13th – 17th, 2025

Title: 	Discussion on R20 OTA test methodology

Agenda Item:	7.3.3

Source: 	CATT

Document for:	Discussion

1 Introduction

RAN#109 agreed the new work item for OTA with following core part scope in [1]:

- Define enhanced test methodologies to cover NTN Ka and Ku bands - Study how to extend the applicability of RAN4 defined OTA test methods to support Ku and Ka testing - FR2 OTA test methods (DFF, IFF, NFTF) extension is the first priority - FR1 OTA test method extension is not precluded, if also applicable - Study whether and how to support circular polarization measurements - Linear polarization is the baseline - Specify the required quiet zone to cover VSAT device type based on industry needs - 60cm QZ as a starting point - Specify preliminary MU assessment for test methodologies

In this contribution, we share our views about the test band priority and FR2 OTA test method for NTN VSAT device.

2 Discussion

2.1	NTN VSAT test band priority

As per the approved ts 38.101-5 big CR [2], The frequency range of Ku and Ka band in [2] are listed as below:

Table 5.2.3-1: Satellite operating bands in FR2-NTN Satellite operating band; Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high; Duplex mode. n5121; 27500 MHz - 30000 MHz; 17300 MHz - 20200 MHz; FDD. n5112; 28350 MHz - 30000 MHz; 17300 MHz - 20200 MHz; FDD. n5103; 27500 MHz - 28350 MHz; 17300 MHz - 20200 MHz; FDD. n509; 14000 MHz - 14500 MHz; 10700 MHz – 127504,5 MHz; FDD. n508; 13750 MHz - 14000 MHz; 10700 MHz – 127504,5 MHz; FDD. NOTE 1: This band is applicable in the countries subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries subject to or referring to FCC 47 CFR part 25. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202). NOTE 4: For Region 2, the downlink is limited to 10700 – 12700 MHz. NOTE 5: For the US, the downlink is limited to 10700 – 12200 MHz for Mobile VSAT..

It can be observed that the Ka-bands (n512, n511, n510) are allocated within the FR2 range (24.25 GHz to 52.6 GHz), whereas Ku-bands (n509, n508) are in the gap between FR1 and FR2 range. Compared to Ku-bands, Ka-bands are more compatible to leverage the existing FR2 OTA test methods with some modifications for VSAT device.

Proposal 1：	Study the OTA test methods on Ka-band as a starting point.

2.2	Extension of legacy FR2 OTA test method

In this WI, the extension of legacy FR2 OTA test methods (DFF, IFF, NFTF) will be studied for VSAT device. The description for DFF method in [4] is extracted as below:

The minimum distance between the antenna array integrated anywhere within the DUT and the measurement antenna needs to meet the far-field distance, RFF = 2D2/ Figure 5.2.1.2-2: Illustration of DFF System for range length definition The setup in Figure 5.2.1.2-3 is used to derive the minimum range length for NR FR2 DFF systems where the sphere enclosing the DUT matches the QZ and the DUT antenna with radiating aperture diameter D located in the corner of the DUT. With this setup, the minimum range length, RDFF, can be determined as RDFF = RQZ – D/2 + RFF = RQZ – D/2 + 2D2/ Figure 5.2.1.2-3: Illustration of DFF System for minimum range length definition

As per 6.1.1 in [3], the antenna configuration for VSAT is 60 cm equivalent aperture diameter. Assuming RQZ = RDUT = 60cm and 15GHz for Ku band and 30GHz for Ka band, in table 1 we can roughly estimate the required minimum range length for DFF measurement for VSAT device：

Table 1: Minimum Range Length of DFF System for D = 60cm

| f [GHz] QZ [cm] | 15 | 30 |
| --- | --- | --- |
| 60 | 36.3m | 72.3m |

Obviously, the range length of DFF method is quite large. That will result in significant path loss and high construction costs. In that case, the DFF method is not quite suitable for VSAT testing.

Observation 1：DFF test method is note suitable for VSAT device due to significant path loss and high construction costs

The IFF method (CATR system) measurement setup demonstrated in [4] is extracted as below:

Figure 5.2.3.1-1: IFF method 1 (CATR) measurement setup of UE RF characteristic

Compared to DFF method, CATR does not require a measurement distance of  to achieve a plane wave. Shorter measurement distance brings less path loss. That makes IFF method much more appropriate for VSAT device OTA measurement. It’s reasonable to treat the study of IFF method as first priority for VSAT OTA measurement.

Observation 2：IFF test method is appropriate for VSAT device and should be taken as first priority

Besides DFF and IFF, NFTF method is described in [4] as well. This method reconstructs the far-field 3D radiation pattern by collecting both amplitude and phase information over a spherical surface in the near-field. The measurement setup demonstrated in [4] is extracted as below:

Figure 5.2.4.1-1: Typical NFTF measurement setup of EIRP/TRP measurements

NFTF method could provide significant measurement time reduction via parallelized sensing and less path loss. But considering NFTF method are not suitable for EIS testing, maybe it could be considered as second priority for VSAT device OTA measurements.

Observation 3：NFTF test method can be considered as second priority for VSAT device

Proposal 2：	Consider IFF via CATR test method as first priority and NFTF test method as second priority for VSAT device OTA testing.

2.3	Test metric for NTN VSAT

The performance requirements and corresponding test metric for NTN VSAT are defined in [5], and can be summarized as below table:

|  | Characteristics | Test metric |
| --- | --- | --- |
| Tx | maximum output power: EIRP, TRP | EIRP(Link=Tx beam peak direction, Meas=Link angle), TRP |
|  | Transmit OFF power | TRP (Link=TX beam peak direction, Meas=TRP grid) |
|  | Transmit ON/OFF time mask | EIRP(Link=TX beam peak direction, Meas=Link angle) |
|  | Frequency Error |  |
|  | Error vector magnitude |  |
|  | Occupied bandwidth |  |
|  | Spectrum emission mask | TRP (Link=TX beam peak direction, Meas=TRP grid) |
|  | Adjacent channel leakage ratio |  |
|  | Spurious Emissions |  |
|  | On-axis spurious requirement | EIRP(Link=TX beam peak direction, Meas=Link angle) |
|  | Off-axis spurious requirement |  |
|  | “Carrier-on” and “Carrier-off” states |  |
| Rx | OTA reference sensitivity level | EIS (Link=RX beam peak direction, Meas=Link Angle) |
|  | Maximum input level |  |
|  | Adjacent channel selectivity |  |
|  | Blocking characteristics |  |

And in [5], Example measurement grid for EIRP and EIS are illustrated like below:

Figure 1 Example measurement grid for EIRP or EIS

But the example measurement grids for TRP are not defined in [5], we can assume the classical TRP measurement grid in [4] will be used. As specified in G.1.2 in [4], two different measurement grid types are considered: 1) constant step size grid with  =  =15°; 2) constant density grid. Both types have 266 measurement points over the whole sphere.

But for NTN VSAT device, only the upper hemisphere or even less zone matters. Different from NTN handheld UE, the radiated performance requirements for VSAT is already defined. There’s no time and need to consider partial radiated performance like PRP or PRS for VSAT. To save testing time, enhancement of classical TRP measurement could be considered. One possible approach is to consider applying different sampling step in both  and for upper hemisphere and lower hemisphere. For example, perform the TRP measurement with constant step size grid  =  =15° in the upper hemisphere and  =  =30°in the lower hemisphere.

Proposal 3:	Improve classical TRP measurement method to reduce test time. Coarse measurement grid for the lower hemisphere could be considered.

3 Conclusion

In this paper, we submit following observations and proposals for NTN VSAT OTA testing:

Proposal 1：	Study the OTA test methods on Ka-band as a starting point.

Observation 1：DFF test method is note suitable for VSAT device due to significant path loss and high construction costs

Observation 2：IFF test method is appropriate for VSAT device and should be taken as first priority

Observation 3：NFTF test method can be considered as second priority for VSAT device

Proposal 2：	Consider IFF via CATR test method as first priority and NFTF test method as second priority for VSAT device OTA testing.

Proposal 3:	Improve classical TRP measurement method to reduce test time. Coarse measurement grid for the lower hemisphere could be considered.

Reference

[1]	RP-252903 WID: Enhancement of UE OTA test method and requirements for NR

[2]	R4-2510831 Big CR for 38.101-5 for the addition of Ku bands, Eutelsat Group

[3]	TR 38.821 Solutions for NR to support non-terrestrial networks (NTN)

[4]	TR 38.810 Study on test methods

[5]	TS 38.101-5 User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements (Release 19)