3GPP RAN WG4 Meeting #116bis	R4-2513225

Prague, Czech, October 13th – October 17th, 2025

Title: 	Discussion on the RRM core requirements of AIML air ph2

Source: 	CATT

Agenda item:	7.9.4

Document for:	Discussion

Introduction

In RAN#109 meeting, RAN approved the new WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2 [1] with the following objectives for RAN4:

… Inter-vendor training collaboration for two-sided AI/ML models - Fully defined/specified reference model (“Direction C”) with RAN1 scalability study outcome taken into account [RAN4/RAN1] – check-point in RAN#110 upon RAN4 feedback - Specification of standardized encoder model structure plus parameter exchange (“Direction A, sub-option 3a-1” without target CSI sharing) leveraging defined/reference model of “Direction C” and taking RAN1 scalability study outcome into account [RAN4/RAN1/RAN2/RAN3] – check-point in RAN#110 upon SA WG feedback - Specification of standardized dataset format/content plus dataset exchange (“Direction A, sub-option 4-1”) [RAN1/RAN2/RAN3/RAN4] – check-point in RAN#110 upon SA WG feedback Data collection [RAN2]: - Further study and specify if deemed necessary, standards-based UE data collection at least for UE-side model training in close collaboration with SA WGs and taking into account the agreed principles during the Rel-19 study – check-point in RAN#110 based on SA WG feedback and SA2 study on data collection Interoperability and RRM requirement [RAN4]: - To define the requirement for the encoder, specify the standardized test decoder to guarantee interoperability - The fully specified test decoder, data set, and corresponding reference encoder in RAN4 to be used for inter-vendor collaboration - Necessary RRM requirement for specified LCM procedures. - NOTE: offline training is assumed for the purpose of this project. Strive to leverage on RAN1’s study on scalable model structure in FS_NR_AIML_Air.

In this contribution, we will discuss the potential impacts of AI/ML based CSI compression on RRM core requirements and present our views.

Discussion

Regarding the potential impacts on RRM requirements, the discussions and agreements in Rel-19 AIML air ph1 can be referred to. For example, RAN4 agreed to define AI/ML functionality activation delay requirement for BM and the agreement is as follows [2]:

RAN4#116: Issue 2-4: Activation delay Agreement: - UE has to be ready to start measurements for inference after sending RRC reconfiguration complete - Inference delay (might include also model loading depending on UE implementation) will be included in the inference reporting delay - This applies to all reporting schemes

If RAN2 agree to reuse the signalling framework for applicability report in Rel-19 AI/ML based BM, then this agreement can also apply for AI/ML based CSI compression.

In addition, RAN4 may need to define LCM requirements for performance monitoring for this use case, but this needs more RAN1/2 design. Therefore, RAN4 can discuss how to define requirements for performance monitoring in a later phase.

Another procedure for which RAN4 need to define LCM requirements is fallback scheme, i.e., when AI/ML model performance is worse than a certain level and UE is indicated by network to deactivate AI/ML models and fallback to non-AI methods, UE needs to be ready to report the CSI in a with the legacy method in specified time duration.

Proposal 1: For AI/ML functionality activation delay, the following agreement achieved in R19 AI/ML BM applies if RAN2 agree to reuse the signalling framework for applicability report:

-	UE has to be ready to start measurements for inference after sending RRC reconfiguration complete.

Proposal 2: RAN4 to discuss how to define LCM requirements for performance monitoring and fallback procedures after RAN1/2 have more concrete progress.

Conclusions

This contribution discussed the potential impacts of AI/ML based CSI compression on RRM core requirements and some observations and proposals are provided:

Proposal 1: For AI/ML functionality activation delay, the following agreement achieved in R19 AI/ML BM applies if RAN2 agree to reuse the signalling framework for applicability report:

-	UE has to be ready to start measurements for inference after sending RRC reconfiguration complete.

Proposal 2: RAN4 to discuss how to define LCM requirements for performance monitoring and fallback procedures after RAN1/2 have more concrete progress.

References

[1] RP-252445, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2, Qualcomm, RAN#109.

[2] R4-2511890, WF on AI/ML for air interface for [116][131], Qualcomm Incorporated, RAN4#116.