3GPP TSG RAN WG4 Meeting #116bis		R4-2513236

Prague, Czech Republic, Oct 13th – 17th , 2025

Title: 	Discussion on system parameters and UE RF requirements for 6MHz channel bandwidth

Agenda Item:	7.1.5.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN#109, a work item for further UE RF enhancement was approved, and one of the objectives is to add 6MHz channel bandwidth support [1]:

6MHz channel bandwidth - Specify the generic core requirements to support 6 MHz NR FR 1 channel bandwidth [RAN 4] - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements (OTA and conducted) for UL and DL - Specify the band specific requirements to introduce 6 MHz for NR band n5, n8, n12, n85, n29 [RAN 4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed - Additional Maximum Power Reduction (A-MPR) - Once RAN 4 has introduced the new channel bandwidth, RAN 2 will introduce the necessary signalling for this channel bandwidth [RAN 2] - Note This should be similar to the signalling for 7 MHz channel bandwidth - Once RAN 4 has introduced the new channel bandwidth, RAN 3 will introduce the necessary signalling for this channel bandwidth [RAN 3] - Note This should be similar to the signalling for 7 MHz channel bandwidth NOTE: Once this WI is completed, the 6 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI. The support of 6MHz bandwidth is release independent. General for all the areas - Specify release independence requirements for the core part in TS 38.307, if needed and feasible.

In this contribution, we discuss the spectrum utilization for 6MHz channel bandwidth and propose  PRBs as the maximum transmission bandwidth configuration accordingly. Moreover, we also discuss the impacted UE RF requirements such as REFSENS, and BCS4/5 etc.

# 2 Discussion

## 2.1 Spectrum utilization

Currently 3GPP has specified these regular channel bandwidths (integer times of 5MHz) in addition to an exceptional 3MHz introduced in Rel-18 mainly for some special scenarios. And the corresponding spectrum utilization is specified as the following table [2] [3]:

And the corresponding spectrum utilization in percentage of channel bandwidth could be found as the following table:

As other channel bandwidths introduced later than Rel-15, e.g., 35MHz and 45MHz, and 6MHz in Rel-19, a simple method for deriving spectrum utilization of a new channel bandwidth is so-called linear fitting. Obviously, there could be two different fittings:

- Linear fitting is done for spectrum utilization in terms of number of PRBs

- Linear fitting is done for spectrum utilization in terms of percentages

Since 6MHz channel bandwidth only includes SCS 15kHz, the fitting parameters could be seen in the following Table – 2.1-1:

Table – 2.1-1 Linear fitting parameters

| Parameters | PRB number | Percentage |
| --- | --- | --- |
| Slope | 5.4347 | 1.45E-03 |
| Interception | -2.2592 | 0.912 |

Accordingly, the spectrum utilization for 6MHz could be derived as 30.349 and 31.863 respectively, which means the number of PRBs for 6MHz channel bandwidth is either 30 or 31 based on different methods, corresponding to a minimum guard band 292.5kHz and 202.5kHz respectively. Where the minimum guard band size could be calculated in the same way as other regular channel bandwidth, i.e. GBChannel = (BWChannel x 1000 (kHz) - NRB x SCS x 12) / 2 - SCS/2.

On one hand, comparing the guard band sizes shows that for a 5 MHz channel the guard band is 242.5 kHz, and for a 6 MHz channel it is 342.5 kHz, whereas for a 6 MHz channel bandwidth with 202.5 kHz guard the increase is not proportional to the bandwidth expansion. On the other hand, allocating a larger number of PRBs in a narrower channel bandwidth has practical benefits. Therefore, selecting 31 PRBs for 6 MHz is considered a better choice.

The Table 2.1-2 below shows the result as 202.5kHz for SCS 15kHz.

Table – 2.1-2 Minimum guard band for 6MHz channel bandwidth (kHz)

| SCS (kHz) | 6 |
| --- | --- |
|  | MHz |
| 15 | 202.5 |
| 30 | N/A |
| 60 | N/A |

Proposal 1: RAN4 to specify the maximum transmission bandwidth configuration of 6MHz channel bandwidth as 31 PRBs for SCS 15kHz.

Proposal 2: RAN4 to specify the minimum guard band of 6MHz channel bandwidth as 202.5kHz for SCS 15kHz.

## 2.2 UE RF requirements

For an FDD band, whether REFSENS can be scaled with transmission bandwidth configuration of different channel bandwidth depends on the carrier’s duplex gap and duplex distance. However, these have been reflected in the current REFSENS requirements for band n5, n8, n12, n29 and n85, hence it is reasonable to derive REFSENS for the new channel bandwidth.

To derive a reasonable REFSENS value for 6MHz channel bandwidth with SCS 15kHz, the first task is to decide the corresponding spectrum utilization. And as we propose 31 PRBs for 6MHz channel bandwidth with SCS 15kHz above, the following derivation is done according to this assumption.

Secondly, the scaling factor by referring to the spectrum utilization of 5MHz channel bandwidth can be illustrated as the following Table – 2.2-1.

Table – 2.2-1 Scaling factor with respect to the spectrum utilization @ 5MHz CBW

| SCS (kHz) | 3 | 5 | 10 | 15 | 20 | 25 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
|  | Scaling Factor (dB) | Scaling Factor (dB) | Scaling Factor (dB) | Scaling Factor (dB) | Scaling Factor (dB) | Scaling Factor (dB) | Scaling Factor (dB) |
| 15 | -2.2 | 0.0 | 3.2 | 5.0 | 6.3 | 7.3 | 8.1 |
| 30 | N/A | 0.0 | 3.4 | 5.4 | 6.7 | 7.7 | 8.5 |
| 60 | N/A | N/A | 0.0 | 2.1 | 3.4 | 4.5 | 5.4 |

Then for band n5, the current REFSENS is specified as Table – 2.2-2.

Table – 2.2-2: REFSENS of band n5

| Operating Band | SCS kHz | 3 | 5 | 10 | 15 | 20 | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz |
|  |  | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) |
| n5 | 15 |  | -98 | -94.8 | -93 | -86.8 | -84.8 |
|  | 30 |  |  | -95.1 | -93.1 | -88.6 | -84.9 |

It could be observed that up to 15MHz channel bandwidth, REFSENS is aligned with the corresponding scaling factor for both bands. Hence, the scaling factor could be applied to REFSENS for 6MHz, which is 10*log10(31/25) = 0.9 dB, therefore, REFSENS for 6MHz CBW with SCS 15kHz would be -98 + 0.9 = -97.1 dBm for band n5, and , i.e., the following Table – 2.2-3:

Table – 2.2-3 REFSENS for band n5 with 6MHz CBW

| Operating Band | SCS kHz | 6 |
| --- | --- | --- |
|  |  | MHz |
|  |  | (dBm) |
| n5 | 15 | -97.1 |
|  | 30 | N/A |

Observation 1: For band n5, a scaling factor of 0.9dB can be applied to REFSENS of 6MHz on top of that of 5MHz CBW.

Proposal 3: RAN4 to specify REFSENS as -97.1 dBm for 6MHz CBW with SCS 15kHz for band n5.

Similarly, the current REFSENS for band n8, n12 and n85 are specified as the following Table – 2.2-4/5/6 respectively:

Table – 2.2-4 REFSENS for band n8

| Operating Band | SCS kHz | 3 | 5 | 10 | 15 | 20 | 25 | 30 MHz (dBm) | 35 MHz (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MHz | MHz | MHz | MHz |  |  |
|  |  | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) |  |  |
| n8 | 15 |  | -97 | -93.8 | -91.4 | -85.8 | -83.6 | -81.3 | -78.4 |
|  | 30 |  |  | -94.1 | -91.7 | -87.2 | -84.7 | -81.4 | -78.5 |

Table – 2.2-5: REFSENS of band n12

| Operating Band | SCS kHz | 3 | 5 | 10 | 15 |
| --- | --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MHz | MHz |
|  |  | (dBm) | (dBm) | (dBm) | (dBm) |
| n12 | 15 |  | -97 | -93.8 | -84 |
|  | 30 |  |  | -94.1 | -84.1 |

Table – 2.2-6: REFSENS of band n85

| Operating Band | SCS kHz | 3 | 5 | 10 | 15 |
| --- | --- | --- | --- | --- | --- |
|  |  | MHz | MHz | MHz | MHz |
|  |  | (dBm) | (dBm) | (dBm) | (dBm) |
| n85 | 15 | -99.2 | -97 | -93.8 | -84 |
|  | 30 |  |  | -94.1 | -84.1 |

It could be observed that up to 10MHz channel bandwidth, REFSENS is aligned with the corresponding scaling factor for all these 3 bands. Hence, the scaling factor of 0.9dB could be applied to REFSENS for 6MHz, therefore, REFSENS for 6MHz CBW with SCS 15kHz would be -97 + 0.9 = -96.1 dBm, i.e., the following Table – 2.2-7:

Table – 2.2-7 REFSENS for band n8/n12/n85 with 6MHz CBW

| Operating Band | SCS kHz | 6 |
| --- | --- | --- |
|  |  | MHz |
|  |  | (dBm) |
| n8 | 15 | -96.1 |
|  | 30 | N/A |
| n12 | 15 | -96.1 |
|  | 30 | N/A |
| n85 | 15 | -96.1 |
|  | 30 | N/A |

Proposal 4: RAN4 to specify REFSENS as -96.1 dBm for 6MHz CBW with SCS 15kHz for band n8, n12 and n85.

# 3 Conclusion

In this contribution, we have the following observation and proposals for system parameters and UE RF requirements for 6MHz channel bandwidth:

Observation 1: For band n5, a scaling factor of 0.9dB can be applied to REFSENS of 6MHz on top of that of 5MHz CBW.

Proposal 1: RAN4 to specify the maximum transmission bandwidth configuration of 6MHz channel bandwidth as 31 PRBs for SCS 15kHz.

Proposal 2: RAN4 to specify the minimum guard band of 6MHz channel bandwidth as 202.5kHz for SCS 15kHz.

Proposal 3: RAN4 to specify REFSENS as -97.1 dBm for 6MHz CBW with SCS 15kHz for band n5.

Proposal 4: RAN4 to specify REFSENS as -96.1 dBm for 6MHz CBW with SCS 15kHz for band n8, n12 and n85.

# Reference

[1] RP-252952, “New WID: UE RF enhancements for NR FR1, Phase 5”, Moderator(vivo)

[2] TS 38.101-1 v19.2.0

[3] TS 38.104 v19.1.0