3GPP TSG RAN WG4 Meeting #116bis		R4-2513243

Prague, Czech Republic, Oct 13th – 17th , 2025

Title: 	Discussion on BS conformance testing for LP-WUS/WUR

Agenda Item:	6.16.2

Source: 	CATT

Document for:	Discussion

# 1 Introduction

With the 100% completion of core part of the work item LP-WUS/WUR [1], the focus of this WID becomes performance part, though 25% completion for this part is declared, and CRs to TS 38.141-1/38.141-2 are also approved [2][3].

In this contribution, we present our views on aspects related to BS conformance testing.

# 2 Discussion

According to approved CR to TS 38.104 on BS RF core requirements for LP-WUS, the following clauses are updated accordingly [4]:

| Requirements | Conducted | Radiated |
| --- | --- | --- |
| Modulation Quality | 6.5.2.1 | 9.6.2.1 |
| Occupied bandwidth | 6.6.2.2 | 9.7.2.2 |
| ACLR | 6.6.3.1 | 9.7.3.1 |
| OBUE | 6.6.4.1 | 9.7.4.1 |
| Tx spurious emission | 6.6.5.1 | 9.7.5.1 |
| Tx IMD | 6.7.2.1 | 9.8.1 |

In the CRs to TS 38.141-1 and TS 38.141-2, only ACLR, OBUE, Tx spurious emissions and Tx intermodulation requirements were updated, while the modulation quality and occupied bandwidth requirements remained unchanged, aside from manufacturer’s declarations [2][3].

It is understandable that Section 6.5.2.1 is untouched since no modulation quality requirement is specified, however, for occupied bandwidth, a test similar to NB-IoT in-band operation should be considered, which requires a new test mode definition dedicated for LP-WUS (could be labelled as NR-LP-TM). The following illustrates how NB-IoT test is required in TS 38.141-1:

For a BS declared to be capable of NB-IoT operation in NR in-band (D.41), test shall be performed using N-TM according to clause 4.9.2.2.9.

Proposal 1: RAN4 to introduce a new test mode (NR-LP-TM) for LP-WUS.

The new test mode NR-LP-TM should be applicable for testing occupied bandwidth, ACLR, OBUE, Tx spurious emission and Tx IMD for LP-WUS.

Proposal 2: NR-LP-TM should be applicable for testing occupied bandwidth, ACLR, OBUE, Tx spurious emission and Tx IMD for BS supporting LP-WUS operation.

# 3 Conclusion

In this contribution, we have the following proposals for BS conformance testing for LP-WUS:

Proposal 1: RAN4 to introduce a new test mode (NR-LP-TM) for LP-WUS.

Proposal 2: NR-LP-TM should be applicable for testing occupied bandwidth, ACLR, OBUE, Tx spurious emission and Tx IMD for BS supporting LP-WUS operation.

# Reference

[1] RP-252854, “Status report for WI: Low-power wake-up signal and receiver for NR (LP-WUS/WUR); rapporteurs: vivo, NTT DOCOMO”, vivo, NTT DOCOMO

[2] R4-2512596, “BigCR to TS 38.141-1 on support for LP-WUS”, CATT, ZTE

[3] R4-2512597, “BigCR to TS 38.141-2 on support for LP-WUS”, CATT, ZTE

[4] R4-2512699, “BigCR to TS 38.104 on LPWUS”, Huawei, HiSilicon, vivo, Nokia, ZTE, CATT, Qualcomm, Ericsson, CMCC