3GPP TSG RAN WG4 Meeting #116bis	R4-2513269

Prague, CZ, 13th – 17th, Oct. 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Views on 6G system parameter

# Agenda item:	8.3

# Document for:	Approval

# 1 Introduction

The new SID[1] was approved in RAN #108 meeting, and the work for RAN4 will be started in RAN4 #116bis meeting.

In this contribution, we give some initial views on the 6GR system parameters.

# 2	Discussion

## 2.1	Waveform

The agreements about waveform in last RAN1#122 meeting are captured as below:

Agreement (first agreement for 6G!!) CP-OFDM and DFT-s-OFDM waveforms as defined in 5G NR are supported as the basis for 6GR for uplink - Enhancements/modifications on CP-OFDM/DFT-s-OFDM will be studied as potential additions - Other OFDM based waveforms are not precluded. Agreement CP-OFDM waveform as defined in 5G NR is supported as the basis for 6GR for downlink - Enhancements/modifications on CP-OFDM will be studied as potential additions - DFT-s-OFDM or any other OFDM-based waveform will be studied as a potential additional waveform for downlink

It can be seen that the basis waveform for 6G are agreed in RAN1 that the same with 5G, where CP-OFDM and DFT-s-OFDM waveforms in UL and CP-OFDM waveform in DL.

Meanwhile, other OFDM based waveforms are not precluded which will also be discussed in RAN1 #122 bis in Oct. Meeting.

Observation 1. For 6GR, CP-OFDM and DFT-s-OFDM waveforms are the basis for UL and DFT-s-OFDM waveform is the basis for DL, Other OFDM based waveforms are not precluded and under discussing in RAN1.

The main purposes for the new waveform are to improve spectrum efficiency, power efficiency and coverage. For the coverage improvement, design waveform with lower Peak-to-Average Power Ratio (PAPR) is desirable.The improved PAPR can alleviate the impacts of PA non-linearity, which can achieve better in-band and out-of-band emission requirements, enabling higher transmission power with smaller power back off. It can non only achieve higher spectrum utilization, but also improve power efficiency. For 6G design, this design approach should be considered for both DL and UL.

- For UL, DFT-s-OFDM is used in uplink primarily due to its low PAPR characteristics. However, only single-layer DFT-s-OFDM is currently supported in 5G NR, which limits the achievable throughput and spatial diversity. For 6G design, continuous enhancement on DFT-s-OFDM to boost performance can be considered.

- In addition to that, new scenarios such as ISAC and some other high-mobility scenarios like UAV should be taken in account. ISAC has been recognized as a key feature for future 6G systems. As a basic research direction, waveform design for sensing should be considered. However, as the core idea of ISAC, i.e., to integrate communication and sensing functionalities within a single wireless system, a unified waveform design with flexible resource sharing/allocation is expected to achieve both high-performance communication and accurate sensing. For example, the following aspects should be considered:

- Compatibility with communication systems

- Coverage requirements for both sensing and communication functions

- Sensing accuracy and detection probability

Observation 2. For 6GR, waveform design aims to improve spectrum efficiency, power efficiency and coverage. It should consider both DL and UL, and support multiple scenarios and needs such as ISAC as well.

From RAN4 perspective, usually both in-band emission (i.e. EVM) and out-of-band emission(i.e. ACLR/SEM/SE) requirements are used for the metric for waveform. For 6GR, waveform design should strive to achieve higher spectrum utilization and higher transmission power to improve the coverage and power efficiency.

One common method for the waveform design is to use filtering as spectral confinement technique. Then EVM requirement may be increased due to amplify imbalance caused by uneven ripple and phase distortion caused by group delay in the filter pass-band. Especially for high order modulation, the RBs at the edges of the constellation are closer to the adjacent RBs and are thus more susceptible to the influence of noise and interference. As a result, the EVM in the edge RBs may differ from that of the symbols in the center of the constellation diagram, thus there may exist have high risk to meet the EVM in edge RBs.

For out-of-band emission (i.e. ACLR/SEM/SE) requirements, the waveform design should guarantee the emission decline rapidly in the guard band. However, higher spectrum utilization means lower guard band, which make it is challenge to meet the out-of-band emission requirements. Moreover, higher transmission power is another challenge factor since it may cause more non-linear PA produce falls into out of band. In the reality implementation,  implementation margins are usually allowed in the requirements.

Except the in-band emission and out-of-band emission shall be met for waveform design, some other aspects are also need to be taken in account, such as complexity and ect. For the complexity, it is desirable that the waveform can be implemented in practical with acceptable cost such as the filter order is not too large.

- In addition to that, when we see the 5G NR spectrum utilization, we found the spectrum utilization for 5MHz/10MHz channel bandwidths with 30kHz are less than 90% while all other channel bandwidths has more than 90% spectrum utilization. Thus improving the spectrum utilization for 5MHz/10MHz channel bandwidths with 30kHz may also be considered.

Takes into the above consideration, we think the metric for the 6GR waveform design should at least include the following aspect:

- EVM

- SEM/ACLR/Spurious emission

- Complexity

Except some agreed basic waveforms in last RAN1 agreements, RAN1 will also discuss some waveform design in parallel in Oct. Meeting. Considering it is the first meeting to discuss 6G waveform issue in RAN4, we think it would better to postpone the waveform discussion in Oct. Meeting.

Proposal 1: From RAN4 perspective, 6GR waveform design should consider the following metrics:

- EVM

- SEM/ACLR/Spurious emission

- Complexity

Proposal 2: Postpone the waveform discussion in RAN4 to 2026 Q1 when initial RAN1 progress is made.

## 2.2	Channel bandwidth, FFT size and numerology

In 5G NR, maximum 100MHz channel bandwidth with 30/60kHz SCS and maximum 50MHz channel bandwidth with 15kHz SCS are supported for 5G NR FR1, and maximum 400MHz channel bandwidth with 120kHz SCS is supported for 5G NR FR2-1. All are based on the maximum 4096 FFT size.

For 6GR SCS, the agreements in last RAN1#122 meeting are:

Agreement - 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH. - For sub 6GHz - The following subcarrier spacing is at least supported - 15kHz SCS for FDD, 30kHz SCS for TDD - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz - FFS: 7.5kHz SCS for sub1GHz (FDD) - Whether to discuss the FFS will be subject to RANP decision. - For around 7GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz - FFS: For around 15GHz - The following subcarrier spacing options can be studied - 30kHz, 60kHz, 120kHz - Whether to discuss it will be subject to RANP decision - For between 24.25GHz - 52.6GHz - Subcarrier spacing 120kHz is supported - FFS whether to allow using additional subcarrier spacing for SSB - FFS subcarrier spacing for PRACH and up to initial access discussion.

In general, we think the 5G numerology design principle should be used as baseline, i.e., SCS based on 2n *15 kHz, and for 6GR, multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) shall be avoided.

Observation 3. 60kHz SCS is precluded for between 24.25GHz - 52.6GHz, and 15kHz SCS is precluded for around 7GHz.

For maximum channel bandwidth, the 4096 FFT size was adopted for 5G NR although 8192 FFT size was also discussed, it was abandoned at the end due to the potential implementation complexity. Consequently, maximum channel bandwidths of 100MHz and 400MHz are supported for 5G NR FR1 and FR2-1, respectively. For NR FR1, maximum channel bandwidths of 50MHz and 100MHz are supported for 15kHz and 30/60kHz SCS respectively, and maximum channel bandwidth of 400MHz is optionally supported for NR FR2-1.

For 6GR, at least 8192 FFT size is achievable in order to achieve better performance than 5G NR, also the relative channel bandwidth ratio is about 4%~6% for one PA design. For sub-6GHz, we think 100MHz maximum channel bandwidth would be applicable for 15kHz SCS and 200MHz channel bandwidth would be applicable for 30kHz SCS. Similarly, for FR2-1, 800MHz maximum channel bandwidth would be applicable.

For the supported SCS for FR1 FDD bands and TDD bands in for sub-6GHz, it was also discussed in the previous RAN1/RAN plenary meeting. So far, according to the commercialized FR1 5G NR FDDs and TDD bands, it is clear that FR1 FDD bands support 15kHz SCS, FR1 TDDs support the 30kHz SCS and FR2-1 TDD bands support the 120kHz SCS. Whether inheriting the 5G sub-carrier spacing directly to 6GR if the bands are refarming from 5G NR. This logic might be valid for MRSS operation to guarantee the orthogonality between 5G NR and 6GR to ensure the spectrum efficiency otherwise wasting some guard band in the middle of 5G NR PRBs and 6GR PRBs. However if the 5G spectrum is refarmed to 6G without any legacy 5G service left according to the potential deployment roadmap from some operators. Then, considering the FDD+TDD band combinations, some benefit could be foreseen to have the same sub-carrier spacing across FDD+TDD bands to enable faster HARQ feedback.

Moreover, for the FDD band around 2GHz, e.g. 1~2.5GHz, it is FFS for the SCS in last RAN1 agreements, we are open to discuss.

For around 7GHz, usually it has wider frequency range, 200Hz/400MHz should be supported from the commercial requirement perspective. Considering the experienced 6% relative channel bandwidth of transmitter PA capability, around 7GHz frequency can achieve 400MHz maximum channel bandwidth. For ADC/DAC design, it already support 400MHz for NR FR2-1. We think it would also be feasible to be used around 7GHz. Thus, 400MHz should be considered since it has higher resource efficiency. Otherwise, to achieve wider bandwidth, intra-band CA is usually used. However, comparing with single carrier, the spectrum utilization will be reduced and the implement complexity would become higher for intra-band CA approach.

Regarding the supported SCS around 7GHz, there are two options in last RAN1 agreements, which are 30kHz and 60kHz. Comparing with 60kHz, 30 kHz provides a longer symbol duration and cyclic prefix, making it more robust against delay spread in large cells, thus it has better coverage capability and has better capability to resist delay spread which can alleviate ISI interference. Thus it would be a good choice for coverage. Conversely, 60 kHz offers a shorter symbol time than that of 30kHz which means lower latency for 60kHz, Also 60kHz is more robust to Doppler shift, the ICI interference would be less than that of 30kHz, making it the preferred choice for high-speed mobility scenarios. In our understanding, coverage would be more important in 6GR. From this perspective, we think 30kHz should be prioritized. Nevertheless, we are also open to discuss 60kHz SCS for around 7GHz, and RAN1 is also discussing this issue. Thus whether or not support 60kHz is FFS which depends on the RAN1’s decision.

With the above consideration, the relationship among SCS, FFT size, Sampling Rate and Max.CBW can be found in Table 1.

Table 2.2-1. SCS, FFT size, Sampling Rate and Max.CBW

| Frequency range | SCS (kHz) | FFT size | Sampling Rate (Msps) | Max. CBW (MHz) |
| --- | --- | --- | --- | --- |
| Sub-6GHz (FDD) | 15/([30]) | 8k (8192) | 122.88 | 100 |
| Sub-6GHz (TDD) | 30 | 8k (8192) | 245.76 | 200 |
| around 7GHz | 30 | 16k (16384) | 491.52 | 400 |
| 24.25GHz - 52.6GHz | 120 | 8k (8192) | 983.04 | 800 |

With the above discussion, we propose 200MHz maximum channel bandwidth with 15kHz and 30kHz SCS for sub 6GHz, and 400MHz with 30kHz SCS for around 7GHz. For the range between 24.25GHz - 52.6GHz, we propose maximum 800MHz channel bandwidth in the case of the agreed 120kHz SCS.

With the above discussions, we have the following proposal:

Proposal 3: For the maximum channel bandwidth, FFT and SCS, it is proposed to adopt the following table:

| Frequency range | SCS (kHz) | FFT size | Max. CBW (MHz) |
| --- | --- | --- | --- |
| Sub-6GHz (FDD) | 15/([30]) | 8k (8192) | 100 |
| Sub-6GHz (TDD) | 30 | 8k (8192) | 200 |
| around 7GHz | 30 | 16k (16384) | 400 |
| 24.25GHz - 52.6GHz | 120 | 8k (8192) | 800 |

## 2.3	Device types

The device types were discussed in last RAN1 meeting and RAN plenary meeting, the agreements in [2] are:

Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement. Proposal 4: In terms of diverse device types, study further: - Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device type

There are many UE types in 5G NR, such as handheld UE, FWA former factor, aerial UE, Redcap UE and etc, and RAN4 defined some different RF requirements such as MPR/ΔRIB values for different UE types although most RF requirements are the same for all the UE types and for some UE types, the supported features such as CA, UL MIMO are the same. However, there are no clear metric to differentiate them which had already cause some unnecessary discussions in RAN4. We think the issue should be discussed in 6GR.

6GR is aimed to support the both existing and new services, e.g., immersive communication, HRLLC and massive connection, along with the consideration on scalable and forward compatible design for diverse device types. However, regarding device types, more factors, e.g., hardware restriction, market scale/fragment and impacts on technical design (e.g., initial access and common channel) should be well considered.

Then, it would be challenging to define all the types with finer granularity. It’s preferred to focus on some typical categories with balanced consideration. For example, since eMBB service is still with a high priority, normal UEs e.g. smartphone UEs should be definitely considered as one direction. Additionally, in robotics applications such as industrial automation, autonomous systems and collaborative robotics, the requirements for data rate, latency and reliability become increasingly higher than normal UEs. Furthermore, CPE and FWA are essential components in communication networks, primarily used to provide wireless broadband connectivity. In these cases, an advanced UE type (other than normal UE) should be considered to meet more advanced requirements e.g. joint requirements of high data rate and low latency. For this advanced device type, the form factor is not a limiting factor, allowing for inclusion of advanced features such higher modulation orders, more Tx/Rx chains to enhance performance. The motivation of defining an advanced UE type is to setup a common set of advanced features to create a good size of economy scale. For other cases with lower targets, e.g., IoT, Redcap-like application, offering lower complexity and cost the one with scalable design is considered.

Further, how the gap between each UE types could be further discussed. From our view, a normal 6G UE (handheld UE) should have higher capabilities than 5G handheld UEs

Proposal 4 For 6GR, at least the following three types of devices are recommended as the target.

- Scalable UE: Designed for the reduced capability scenarios, offering lower complexity and cost.

- Normal UE: Providing improved performance compared to 5G eMBB

- Advanced UE: Designed for the demand of high data rate, low latency scenario, with relaxed restriction of form factor and higher capability than normal UE.

Proposal 5: A normal 6G UE (handheld UE) should have higher capabilities than 5G handheld UEs

In table 2.3-1, we give some examples for the above three device types.

Table 2.3-1 Some examples for the above three device types

| Device types | Examples |
| --- | --- |
| Normal UE (Handheld UE, XR/VR, Vehicle UE, and etc.) | ... |
| Advanced UE (CPE, humanoid robot, and etc.) | ... |
| Scalable UE (surveillance camera, smart watch, sensor, and etc.) | ... |

## 2.4	Tx and Rx number

In 5G NR, there are 1Tx, 2Tx and 4Tx number supported, in which 1Tx and 2Tx can be supported for handheld UE while 4Tx is only supported for FWA form factor. For Rx number, there are 2Rx, 4Rx, 6Rx and 8Rx numbers where 2Rx, 4Rx and 6Rx/4L can be supported for handheld UE, while 6Rx/6L and 8Rx are only supported for FWA form factor.

It can be seen that 4Tx and 8Rx are supported for FWA form factor in 5G NR and we think it can also be applied to 6GR.

For 4Rx, it is mandatory supported for the frequency range larger than 2.6GHz, and optional supported for the frequency range lower than 2.6GHz. For 6Rx, it is optional supported for all the band. The supported Tx/Rx for 5G NR handheld UE can be found in table 2.4.1.

Table 2.4.1 Supported Tx/Rx number for 5G NR handheld UE

|  | Sub 1GHz | 1~3 GHz | 3~6GHz |
| --- | --- | --- | --- |
| 5G handheld UE | 1Tx/2Tx, 2Rx/4Rx/6Rx | 1Tx/2Tx, 2Rx/4Rx/6Rx | 1Tx/2Tx, 4Rx/6Rx |

So far, the number of Tx/Rx chain within the smartphone is somehow mainly physically limited by the available antenna implementation (e.g. FPC (Flexible Printed Circuit) or LDS (Laser Direct Structuring) antenna) at the edge of smartphone or at the back of smartphone close to the camera areas and the RFIC/baseband processing capability. However in the recent years, the transparent antenna [3][4][5] was developed to further expand the areas to place antenna on the smartphone/glasses/vehicle which is called Antenna-on-Display (AoD) technique. For the 6G device, the more antenna elements with AoD techniques as shown in Figure 2.4-1/2/3 are expected to further boost the capacity compared with legacy 5G device.

Figure 2.4-1: Illustration of the potential antenna implementation for 6GR smartphone

Figure 2.4-2: Illustration of the potential antenna implementation for 6GR AI+AR glasses

Figure 2.4-3: Illustration of the potential antenna implementation for Vehicle

As mentioned before, we think a normal 6G UE should have higher capabilities than 5G UEs to achieve higher peak date rate. For example, to mandate support the 4Rx at least for the frequency range larger than 1GHz. We propose to adopt up to 4Tx for transmission and up to 8Rx, depending on the frequency range which is shown in Table 2.4.2.

Table 2.4.2. Baseline Tx and Rx number for 6GR normal UE (Handheld UE)

|  | Sub 1GHz | 1~3GHz | 3~6GHz | Around 7GHz |
| --- | --- | --- | --- | --- |
| 6GR Normal UE (Handheld UE) | 1Tx/2Rx | 2Tx/4Rx | 2Tx/ 6Rx | 4Tx/8Rx |

As exception in the 6G normal UE, for vehicle UE, 8Rx has already been supported in 5G NR. Thus we think it is reasonable to support 8Rx for vehicle UE in 6GR. For XR UE, only 2Rx is supported in 5G NR, and only middle higher bands such as n7/n38/n41/n77/n78/n79 supporting XR. To further enhance the performance and improve use experience but still fit into the form factor, we propose to consider exception cases of supporting a lower number of Rx (e.g. 4Rx in some bands) in 6GR.

For advanced UE such as FWA/CPE, and robotics applications such as industrial automation, autonomous systems and collaborative robotics, we propose 8Tx/8Rx as baseline capability for 6GR, and also in order to achieve higher data rate and stringent latency requirement than 4Tx which has already been supported in 5G NR.

For scalable UE such as IoT device targeting 10Mbps peak data rate, 1Rx configuration with up to QPSK/16-QAM is sufficient.

With above consideration, we have the following proposals:

Proposal 6: For 6GR Normal UE (handheld UE) in 6GR, we propose:

|  | Sub 1GHz | 1~3GHz | 3~6GHz | Around 7GHz |
| --- | --- | --- | --- | --- |
| 6GR Normal UE (Handheld UE) | 1Tx/2Rx | 2Tx/4Rx | 2Tx/ 6Rx | 4Tx/8Rx |

- As exception, it is proposed to support 8Rx for vehicle UE and a lower number of Rx for XR UE in 6GR.

Proposal 7: For 6GR advanced UE, it is proposed to use 8Tx/8Rx as baseline capability.

Proposal 8: For 6GR scalable UE, it is proposed to use 1Tx/1Rx as baseline capability.

## 2.5 Sync raster and channel raster

For channel raster, 100 kHz and SCS based channel raster was introduced in the initial phase of 5G NR. Specifically, 100kHz channel raster was defined for LTE refarming bands, which ensures coexistence between NR and LTE, and SCS based channel raster ensures that there is no need to reserve guard bands between the aggregated carriers during carrier aggregation, thereby improving spectrum efficiency.

In Rel-18, an enhanced channel raster with 10kHz was introduced to solve PRB misalignment when configuring a narrower UE CBW within a wider BS CBW. This PRB misalignment becomes particularly noticeable when PRB number of the UE CBW and BS CBW is one odd and the other even, as illustrated in Figure 2.5-1.

Figure 2.5-1: Configure a narrower UE CBW with odd PRBs within a wider BS CBW with even PRBs

Even when PRB number of narrower UE CBW and wider BS CBW are both even or both odd, new issue may still arise in certain scenarios. If minimum guard band for narrower UE CBW is larger than that of wider BS CBW, the edge of UE CBW will exceed the lower bound of the frequency band when the first PRB of UE CBW and BS CBW keeps aligned. A typical example of theis issue is the scenario where a 30MHz UE CBW is configured within a 40MHz BS CBW, which is shown in Figure 2.5-2. In such cases, some UEs will perform self-check to determine whether the UE carrier edge exceeds the lower bound of the band. Once an overshoot is detected, the UE will stop accessing the network with RRC configuration failure. Therefore, a finer granularity channel raster is needed. In addition, enhanced channel raster also improves deployment flexibility.

Figure 2.5-2: Illustration of RRC reconfiguration failure issue

In 6GR, enhanced channel raster is also required to address PRB misalignment between UE and BS CBW with one odd and one even PRBs for the potential unequal BS BW and UE BW in the filed. It can be easily achieved through a combined implementation of Phase-Locked Loop (PLL) and digital Numerically Controlled Oscillator (NCO) implementation: PLL performs a rough frequency tuning and digital NCO conducts a fine frequency adjustments.

Meanwhile, to allow efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS) and enable coexistence between systems, SCS based channel raster shall be defined for NR refarming band supporting SCS based channel raster in 6GR.

In conclusion, 10kHz channel raster shall be supported for sub-3GHz bands, and SCS based channel raster shall be supported for FR1 high bands, FR2-1 bands.

Proposal 9: In 6GR, both 10kHz channel raster (sub-3GHz bands) and SCS based channel raster shall be supported.

Sync raster is defined for initial system acquisition, which is essential for UE to access the network. In 5G NR, the definition of sync raster considers many aspects including reducing the number of sync entries to shorten the search time for initial access than LTE, to guarantee one SSB placement/BW per channel raster for the flexible carrier placement, floating sync raster and so on. In Rel-18, a new sync raster design was introduced for 3MHz channel bandwidth, which is non-overlapping with legacy sync raster points for above 3MHz channel bandwidth to facilitate UE to differentiate.

In 6GR, in order to achieve the better UE power saving and also some potential satellite beam hopping for the larger coverage area, then the default SSB periodicity might be extended from 20ms to 160ms. To attain superior performance compared to 5G NR, particularly in reducing initial access latency and optimizing UE power efficiency, sync raster needs to be redesigned. In terms of principles to design a new sync raster, we need to consider the following aspects.

- The balance between the step size of sync raster, longer SSB periodicity and deployment flexibility to achieve better performance than 5G NR.

- Floating sync raster

- Non-overlapping with 5G sync raster

6GR SSB design may be different from that in 5G NR, and accordingly, 6GR sync raster entries should avoid overlapping with 5G to simplify UE detection.

- Reserving sufficient frequency separation between 5G and 6GR sync raster

Considering NTN Doppler shift, the frequency separation between 5G and 6GR should be at least 100kHz for sub-3GHz frequency range, 1000kHz for frequency range from 3GHz to 24.25GHz and 1500kHz for above 24.25GHz frequency range.

Proposal 10: Consider the following principles to design 6GR sync raster:

- The balance between the step size of sync raster, longer SSB periodicity and deployment flexibility to achieve better performance than 5G NR

- Floating sync raster

- Non-overlapping with 5G sync raster

- Reserving sufficient frequency separation between 5G and 6GR sync raster

With the above consideration, Figure 2.5-3 gives an example to illustrate the 5G and 6GR sync raster.

Figure 2.5-3: Illustration of 5G and 6GR sync raster

For detailed 6GR sync raster design, it needs to wait for RAN1’s conclusion about SSB design.

Proposal 11: For detailed 6GR sync raster design, it needs to wait for RAN1 conclusion about SSB design.

## 2.6	Spectrum utilization

The spectrum utilization and the relevant minimum guard band for 5G NR each channel bandwidth are summarized in table table 2.6-1.

Table 2.6-1. Spectrum utilization and the relevant minimum guard band

(a) FR1

| CBW(MHz) SCS(kHz) | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N.A | N.A | N.A | N.A | N.A |
| SU | 90.00% | 93.60% | 94.80% | 95.40% | 95.76% | 96.00% | 96.69% | 97.20% | 96.80% | 97.20% |  |  |  |  |  |
| Total GB (MHz) | 0.5 | 0.64 | 0.78 | 0.92 | 1.06 | 1.2 | 1.16 | 1.12 | 1.44 | 1.4 |  |  |  |  |  |
| Minimum GB (kHz) | 242.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 | 712.5 | 692.5 |  |  |  |  |  |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 118 | 133 | 162 | 189 | 217 | 245 | 273 |
| SU | 79.20% | 86.40% | 91.20% | 91.80% | 93.60% | 93.60% | 94.63% | 95.40% | 94.40% | 95.76% | 97.20% | 97.20% | 97.65% | 98.00% | 98.28% |
| Total GB (MHz) | 1.04 | 1.36 | 1.32 | 1.64 | 1.6 | 1.92 | 1.88 | 1.84 | 2.52 | 2.12 | 1.68 | 1.96 | 1.88 | 1.8 | 1.72 |
| Minimum GB (kHz) | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 905 | 1245 | 1045 | 825 | 965 | 925 | 885 | 845 |
| 60 | N.A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |
| SU |  | 79.20% | 86.40% | 86.40% | 89.28% | 91.20% | 90.51% | 91.80% | 92.80% | 93.60% | 94.80% | 95.66% | 96.30% | 96.80% | 97.20% |
| Total GB (MHz) |  | 2.08 | 2.04 | 2.72 | 2.68 | 2.64 | 3.32 | 3.28 | 3.24 | 3.2 | 3.12 | 3.04 | 2.96 | 2.88 | 2.8 |
| Minimum GB |  | 1010 | 990 | 1330 | 1310 | 1290 | 1630 | 1610 | 1590 | 1570 | 1530 | 1490 | 1450 | 1410 | 1370 |

(b) NR FR2-1

| CBW(MHz) SCS（kHz） | 50 | 100 | 200 | 400 |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N.A |
| % | 95.0% | 95.0% | 95.0% |  |
| Total GB(MHz) | 2.48 | 4.96 | 9.92 |  |
| Minimum GB | 1210 | 2450 | 4930 |  |
| 120 | 32 | 66 | 132 | 264 |
| % | 92.2% | 95.0% | 95.0% | 95.0% |
| Total GB(MHz) | 3.92 | 4.96 | 9.92 | 19.84 |
| Minimum GB | 1900 | 2420 | 4900 | 9860 |

Compared with E-UTRA, the spectrum utilization for 5G NR is increased, which contributes to increase the spectrum efficiency gains. It based on implementations including both filtering and windowing solutions for spectrum confinement. These implementation solutions for UE are transparent to NW.

The spectrum confinement techniques are subjected to limitation of Tx/Rx RF requirements since BS/UE is expected to always meet all Tx/Rx RF requirements. From Tx side, EVM, out-of-band emission requirements (SEM and ACLR) and spurious requirements are the main factors and from Rx side, receiver performance such as ACS/blocking should also be considered.

Higher spectrum utilization means more NRB can be transmitted while less minimum guard band which makes more difficult to achieve the Tx/Rx requirements.

It can be observed from the table 2.6-1 that the spectrum utilization is highest for 15kHz SCS for FR1 and 60kHz for FR2-1 for the same channel bandwidth, and spectrum utilization for all channel bandwidths with 15kHz and >=15MHz channel bandwidths with 30kHz SCS are higher than 90%.

Observation 5: In 5G NR, for the same channel bandwidth, higher SCS means lower spectrum utilization, lower SCS means higher spectrum utilization.

For 6GR, the above principle should also be applied. Moreover, As we discussed in section 2.4, some new channel bandwidths should be considered such as maximum 100MHz for 15kHz SCS, the spectrum utilization shall be discussed. For 15kHz, the spectrum utilization should be higher than the spectrum utilization of 30kHz for the same channel bandwidth.

6GR aims to support improved spectrum utilization and operations over one or more carriers/bands, compared to 5G NR, and also 6GR aims to support flexible utilization of spectrum resources for DL and UL over different carriers/bands.

For the existing channel bandwidth, the spectrum utilization shall not be smaller than the 5G NR considering the performance for 6GR shall surpass 5G NR.

For the guard band, in 5G NR, there exists the case that guard band for small channel bandwidth is larger than that of large channel bandwidth. For example, 30MHz@15kHz has 592.5kHz minimum guard band while 40MHz@15kHz has 552.5kHz minimum guard band. In this case, when a UE supports 30MHz channel bandwidth access to 40MHz network, then some problems would happen in the filed for band n28 after UE’s self-check of the minimum guard band RF requirements since UE frequency would overflow the BS frequency edges. RAN4 discussed how to solve this issue and consequently, RAN4 agreed to introduce the exceptional raster point for n28 40MHz BS channel bandwidth to ensure the guard band with 30MHz UE channel bandwidth. In 6NR, we think we should avoid this issue happen since there are some new larger channel bandwidths introduced. In the other work, the guard band for small channel bandwidth should be less than that of large channel bandwidth.

With the experience of 5G NR, the spectrum utilization should be discussed with other aspects like waveform, Tx/Rx requirements, PA models and so on as these factors will large impact how many NRB should be defined.

Proposal 12: The spectrum utilization for 6GR shall not be smaller than the 5G NR.

Proposal 13: For the same channel bandwidth, the spectrum utilization of lower SCS shall be higher than the spectrum utilization of higher SCS.

Proposal 14: The guard band for small channel bandwidth should be less than that of large channel bandwidth.

Proposal 15: The spectrum utilization should be discussed with other aspects like waveform, Tx/Rx RF requirements, PA models and so on.

## 2.7	Irregular channel bandwidth

Irregular channel bandwidth was discussed in Rel-17. The identified irregular channel bandwidths from the operators are listed in the following Table 2.7-1.

Table 2.7-1: Summary of operators’ input for irregular channel bandwidth (From RP-243323)

| Band (s) | Channel Bandwidth(s) |
| --- | --- |
| n5 | 7, 11 MHz |
| n12, n85 | 6, 12 MHz |
| n26 | 7 MHz |
| n28 | 13 MHz |
| n29 | 6, 11 MHz |

During Rel-17 discussion, two types of schemes were proposed to solve this issue. One is larger channel bandwidth than licensed bandwidth, and the other is overlapping UE channel bandwidths,which includes three schemes: overlapping UE CBWs from NW perspective, combined UE CBW (one cell) and overlapping CA (two cells).

For larger CBW scheme, the premise idea is to configure a larger regular CBW in SIB1 and blanking PRBs within the larger BW but outside the irregular BW. From our understanding, this should be the simplest solution we could foresee for the fast deployment in filed. However during the discussion for Rel-19 7MHz WID, the concerns as shown in Figure 2.7-1 was also raised. For example, the 10MHz digital filter within digital front-end (DFE) cannot provide the appropriate/sufficient emission attenuation within the blanked PRB which is designated for other incumbent service. In addition, if there are any high blocking signals within the blanked PRBs from the incumbent system, then 10MHz digital filter cannot also provide the corresponding attenuation to alleviate the interference towards the irregular channel bandwidth part. The corresponding signalling configuration procedure is shown in Figure 2.7-2.

Figure 2.7-1. Illustration of the larger network CBW solution

Figure 2.7-2. Illustration of signalling configuration procedure

Regarding overlapping UE CBWs from NW perspective, the key is BS using the whole irregular CBW and multiple UEs using a smaller standard CBW to cover the irregular CBW through overlapping. For instance, as illustrated in Figure 2.7-3, two overlapping 10MHz carriers can be used to cover a 13MHz channel bandwidth. This scheme has two challenges: one is overlapping UE carriers should keep PRB alignment to facilitate BS to perform same FFT and schedule resources in the overlapping region, and the other is that the network may need to configure two SSBs for two UE carriers if the overlapping region is smaller than CORESET0 bandwidth. In this case, it may create challenge of aligning SSBs in the time and frequency domain so that they do not overlap.

Figure 2.7-3: Illustration of overlapping CA solution

As for combined UE CBW (one cell), irregular CBW can be covered by “main RF carrier” and “additional RF carrier”, in which the former contains SSB. The scheme requires UE to aggregate two RF channels in baseband.

Overlapping CA (two cells) scheme is that BS side supports intra-band CA, while UE supports either single CC with regular CBW or intra-band CA for the combination. This scheme needs to allow the configuration that two carriers can be partially overlapped by adjusting the channel spacing.

In the 6G phase, if the set of irregular channel bandwidth is limited to certain number, it is fine to us to standardize these irregular channel bandwidth directly in the specification similar as Rel-19 7MHz and Rel-20 6MHz definition. As shown in the following Figure 2.7-4, for the new irregular channel bandwidth, the biggest implementation changes are coming from the additional dedicated digital filter for the corresponding bandwidth, however these are done in digital way in the baseband by software presetting configuration, if the overall number of the irregular channel bandwidth is limited, then the implementation complexity is also manageable, therefore we also think that to standardize the irregular channel width as other regular bandwidth in the specification is fine for us under the condition of the reasonable number of irregular channel bandwidth.

Figure 2.7-4. the illustration of the general DFE implementation

Proposal 16: In 6GR, if the demand for irregular CBW is limited, it is proposed to standardize the irregular channel width as other regular bandwidth in the specification.

However, there may be many non-integer irregular channel bandwidth not listed in Rel-17 SID. Given the large number of irregular channel bandwidths, it is not feasible to standardize all of them. In such cases, it’s better to develop a universal solution that can address all irregular channel bandwidths.

Proposal 17: In 6GR, if the number of irregular CBW is very large, it is not feasible to standardize all of them, and it’s better to develop a universal solution that can address all irregular channel bandwidths.

## 2.8	Modulation

The agreements about modulation in last RAN1#122 meeting are captured as below:

Agreement - For 6GR DL, 5G NR uniform QPSK, 16QAM, 64QAM, 256QAM and 1024QAM are supported as basis for study for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for CP-OFDM for data channel - FFS: Enhancements and other modulation schemes - For 6GR UL, 5G NR pi/2 BPSK, uniform QPSK, 16QAM, 64QAM, and 256QAM are supported as basis for study for DFT-s-OFDM for data channel - FFS: Enhancements and other modulation schemes

The above agreements shows the existing 5G NR modulations are supported as basis for 6GR. Meanwhile, other modulations like Shaping modulation(Geometry shaping/Probabilistic shaping), Low PAPR modulation with FDSS waveform shaping are under the discussion in RAN1.

Observation 6. For 6GR, the existing 5G NR modulations are supported as basis, other modulations are not precluded and are under discussion in RAN1.

From the specific modulation design perspective, it is up to the RAN1 work. From the RAN4 perspective, we propose to have the more through discussions how to define more appropriate EVM requirement for the agreed basis modulation order than the legacy minimum requirements to cater for more realistic configuration in field instead of focusing on SU-MIMO with up to 2/4 MIMO layers. In addition, the impacts of waveform, sub carrier spacing, phase noise, transmitter/receiver unlinearity, receiver’s decoding complexity and also operating frequency on the EVM requirements should be also taken into account.

Proposal 18: It is proposed to use the 5G EVM requirement as starting point and further discuss how to define more appropriate EVM requirement for the RAN1 agreed basis modulation order to cater for the realistic deployment instead of considering SU-MIMO with up to 2/4 MIMO layers in the corresponding BS RF and UE RF agenda.

# 3. Conclusion

In this contribution, we give some initial views on the 6GR system parameters, the conclusions are summarized below:

Waveform

Observation 1. For 6GR, CP-OFDM and DFT-s-OFDM waveforms are the basis for UL and DFT-s-OFDM waveform is the basis for DL, Other OFDM based waveforms are not precluded and under discussing in RAN1.

Observation 2. For 6GR, waveform design aims to improve spectrum efficiency, power efficiency and coverage. It should consider both DL and UL, and support multiple scenarios and needs such as ISAC as well.

Proposal 1: From RAN4 perspective, 6GR waveform design should consider the following metrics:

- EVM

- SEM/ACLR/Spurious emission

- Complexity

Proposal 2: Postpone the waveform discussion in RAN4 to 2026 Q1 when initial RAN1 progress is made.

Channel bandwidth, FFT size and numerology

Observation 3. 60kHz SCS is precluded for between 24.25GHz - 52.6GHz, and 15kHz SCS is precluded for around 7GHz.

Observation 4: Based on the simulation results, 30kHz SCS has a better performance than 60kHz SCS with about 1 dB gain.

Proposal 3: For the maximum channel bandwidth, FFT and SCS, it is proposed to adopt the following table:

| Frequency range | SCS (kHz) | FFT size | Max. CBW (MHz) |
| --- | --- | --- | --- |
| Sub-6GHz (FDD) | 15/([30]) | 8k (8192) | 100 |
| Sub-6GHz (TDD) | 30 | 8k (8192) | 200 |
| around 7GHz | 30 | 16k (16384) | 400 |
| 24.25GHz - 52.6GHz | 120 | 8k (8192) | 800 |

Device type

Proposal 4 For 6GR, at least the following three types of devices are recommended as the target.

- Scalable UE: Designed for the reduced capability scenarios, offering lower complexity and cost.

- Normal UE: Providing improved performance compared to 5G eMBB

- Advanced UE: Designed for the demand of high data rate, low latency scenario, with relaxed restriction of form factor and higher capability than normal UE.

Proposal 5: A normal 6G UE (handheld UE) should have higher capabilities than 5G handheld UEs

Tx and Rx number

Proposal 6: For 6GR Normal UE (handheld UE) in 6GR, we propose:

|  | Sub 1GHz | 1~3GHz | 3~6GHz | Around 7GHz |
| --- | --- | --- | --- | --- |
| 6GR Normal UE (Handheld UE) | 1Tx/2Rx | 2Tx/4Rx | 2Tx/ 6Rx | 4Tx/8Rx |

- As exception, it is proposed to support 8Rx for vehicle UE and a lower number of Rx for XR UE in 6GR.

Proposal 7: For 6GR advanced UE, it is proposed to use 8Tx/8Rx as baseline capability.

Proposal 8: For 6GR scalable UE, it is proposed to use 1Tx/1Rx as baseline capability.

Sync raster and channel raster

Proposal 9: In 6GR, both 10kHz channel raster (sub-3GHz bands) and SCS based channel raster shall be supported.

Proposal 10: Consider the following principles to design 6GR sync raster:

- The balance between the step size of sync raster, longer SSB periodicity and deployment flexibility to achieve better performance than 5G NR

- Non-overlapping with 5G sync raster

- Reserving sufficient frequency separation between 5G and 6GR sync raster

Proposal 11: For detailed 6GR sync raster design, it needs to wait for RAN1 conclusion about SSB design.

Spectrum utilization

Observation 5: In 5G NR, for the same channel bandwidth, higher SCS means lower spectrum utilization, lower SCS means higher spectrum utilization.

Proposal 12: The spectrum utilization for 6GR shall not be smaller than the 5G NR.

Proposal 13: For the same channel bandwidth, the spectrum utilization of lower SCS shall be higher than the spectrum utilization of higher SCS.

Proposal 14: The guard band for small channel bandwidth should be less than that of large channel bandwidth.

Proposal 15: The spectrum utilization should be discussed with other aspects like waveform, Tx/Rx RF requirements, PA models and so on.

Irregular channel bandwidth

Proposal 16: In 6GR, if the demand for irregular CBW is limited, it is proposed to standardize the irregular channel width as other regular bandwidth in the specification.

Proposal 17: In 6GR, if the number of irregular CBW is very large, it is not feasible to standardize all of them, and it’s better to develop a universal solution that can address all irregular channel bandwidths.

Modulation

Observation 6. For 6GR, the existing 5G NR modulations are supported as basis, other modulations are not precluded and are under discussion in RAN1.

Proposal 18: It is proposed to use the 5G EVM requirement as starting point and further discuss how to define more appropriate EVM requirement for the RAN1 agreed basis modulation order to cater for the realistic deployment instead of considering SU-MIMO with up to 2/4 MIMO layers in the corresponding BS RF and UE RF agenda.

# 4	Reference

[1]RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator)

[2] RP-252873, Moderator's summary on 6G diverse device types (incl. minimum UE bandwidth), Apple (moderator)

[3] Tightly Embedded Modular Antenna-in-Display (MAiD) Into the Panel Edge of Display With Dual-Polarization for 5G Smartphones,IEEE Transactions on Antennas and Propagation

[4] Broadband Antenna-on-Display Applicable for WiFi, 2021 IEEE International Symposium on Antennas and Propagation and USNC-URSI Radio Science Meeting (APS/URSI)

[5] https://www.dwchem.co.kr/main.do?lang=eng