# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2513277

# Prague, CZ, 13-17 Oct, 2025

Agenda Item:	8.4

Source: 	Xiaomi

Title: 	Overview for 6GR UE RF and general RF aspects

Document for:	Discussion

- Introduction

In the endorsed 6G RAN4 work arrangement from RAN4 chair (R4-2511652), RAN4 divides 6G study into 11 separate topic areas in initial stage. Spectrum is one of the major area with following candidate scope:

In this contribution, our preliminary view on 6G UE RF and general RF topics with following area

- Non-spectrum and non-AI UE RF

- Spectrum aggregation framework

- Discussion

  - Non-spectrum and non-AI UE RF (Huiping)

2.1.1 Revisit MOP definition and capability report

Because MOP capabilities are the fundamental to many RF requirements, how to define the MOP capabilities have big impact to the RF requirement definition structure. Currently UE MOP is based on power class. In TS 38.101-1, the following power classes are defined.

| Power class (PC) | Maximum output power (MOP, dBm) | Tolerance (dB) | MOP + Tolerance (dBm) |
| --- | --- | --- | --- |
| PC1 | 31 | +2/-3 | 33/28 |
| PC1.5 | 29 | +2/-3 | 31/26 |
| PC2 | 26 | +2/-3 | 28/23 |
| PC3 | 23 | +2/-2 | 25/21 |

The step size between PC3/PC2/PC1.5 is 3 dB. As higher output power brings benefit to UL coverage, thus can save the network deployment cost and improve user experience, PA vendors are always trying to increase PA performance to make the UE MOP bigger. However, some of the capabilities are between the two power classes. For example, if a UE can transmit 28 dBm, it only can report PC2, the extra MOP capability is wasted. It’s necessary to have some new methodology to make that kind of capabilities can work in the field.

Observation 1: If UE MOP capability is between two power classes, UE can only report the lower power class.

Proposal 1: RAN4 to revisit if the power class-based MOP definition methodology is still used in 6G.

When revisiting the methodology to define the UE MOP capability, the impact to UE RF requirements and the potential signaling structure should be considered at the same time. A good methodology should target to make the UE RF requirements and the signaling at a relatively minimum level, not bring complicated and unnecessary efforts to both the standard and the implementation.

The power class/MOP capability report structure should be revisited too. For 5G, power classes for different features are reported to NW and the signaling defined in TS 38.331 is summarized in following table.

| Parameter | Description | Applicability |
| --- | --- | --- |
| ue-PowerClass, ue-PowerClass-v1610, ue-PowerClass-v1700 | UE power class | Per band, UL MIMO also uses this capability TxD also uses this capability |
| powerClass, powerClass-v1610 | Indicates power class the UE supports when operating according to this band combination. | Per band combination |
| ue-PowerClassPerBandPerBC-r17, ue-PowerClassPerBandPerBC-v1820 | Indicates the UE power class per band per band combination. | Per band per band combination |

The problem of the above power class signal structure was raised many times in different history discussions. The issues can be summarized roughly to the following several aspects,

- ue-PowerClass is named as the power class for UE, but sometimes the related requirements can be reused by a carrier in the C/NC CA. If the name of “ue-PowerClass” is still used, it brings confusion.

- powerclass per band combination caps the CA MOP when per band power class is different for the bands. Although this is solved by higherPowerLimit-r17 signaling, it can be fixed in the beginning of 6G without introduce extra signaling.

- 1 layer and two layers UL MIMO have the same power class in current signaling. Although there’s a ULFPTx mode which allows different PA capabilities for different Tx chain, but it seems a rather complicated solution with RAN1 TPMI involved.

- Transparent TxD power class reuses ue-PowerClass but the implementation seems similar with ULFPTx fullpowerMode1. It’s also confusing if the ue-PowerClass can be different for non-TxD mode and TxD mode.

All of the above issues can be taken into account when 6G MOP capability report/definition structure is considered.

Proposal 2: RAN4 to discuss the whole UE power class/MOP capability report structure for 6G.

In the output power requirement clause in TS 38.101-1, there’re two parts which are lengthy, complicated and hard to be understood. First is the description related to UL duty cyle, especially when several power classes are introduced, it’s very difficult to understand the logic of fallback relationship. It’s doubtful how many UE vendors implemented this function in 4G/5G. We propose to discuss the possibility to remove the UL duty cycle based solution in spec.

Proposal 3: RAN4 to discuss the possibility to remove the UL duty cycle based solution in spec.

The other complicated part is the Pcmax part. The equation seems not long, but the explanation is very long which occupies 5 pages now. We received many questions from engineering team that it seems not just the wording but the Pcmax equation may also worth to be revisited.

Proposal 4: RAN4 to discuss if there’s a better way to optimize the Pcmax equation and the whole Pcmax clause.

Maximum output power requirement is the most important requirement for UE Tx, almost all of the other UE Tx requirements are defined based on UE is transmitting MOP. The current approach is that if UE supports several power classes, the Tx requirements are separated to several sets for different power classes. When PC3 was introduced in 4G, this power class is almost used in the whole 4G stage. Although PC2 was introduced in the 4G later stage, but it was limited to mostly TDD bands. And it’s not widely supported by the market. The power class definition and the requirements based on power class methodology didn’t bring much trouble for 4G. However, in 5G phase, more and more power classes are defined, the power class based requirements brings much trouble to the requirements discussion and definition. For example, each power class introduction relates to ACLR, MPR, AMPR and MSD requirements. Every requirement need be discussed, simulated and measured. For the test, it’s also complicated that each power class related requirements need be tested if there’s no specific agreement. This methodology brings too much standard discussion efforts and UE debug/test cost. Whether this approach is beneficial to the whole system performance is doubtful. In the beginning of 6G, it’s worthwhile to discuss if it’s possible to merge/omit some of the requirements for different power classes/MOP capabilities.

Proposal 5: RAN4 to discuss the possibility to merge/omit some of the Tx requirements for different power classes/MOP capabilities.

2.1.2 Simplification of MPR/AMPR requirements

The discussion of MPR/AMPR requirements is one of the most time-consuming discussions. Because the PA model can’t be that accurate and different PA modules may have different performance for the back-off operating point, usually the safe way to define the requirements needs much measurement efforts. Currently, separate MPR/AMPR requirements are defined for different MOP capabilities, and also 1PA/2PA, 1Tx/2Tx architectures uses different MPR/AMPR tables. Especially, the difference between 1Tx/2Tx is not that large, sometimes only 0.5 dB difference is found. The small difference like half dB makes no sense to UE implementation, test and NW scheduling. Simplifying the MPR/AMPR requirements can make the spec shorter, clearer and brings benefit to UE cost.

Proposal 6: RAN4 to discuss if common MPR/AMPR requirements can apply to different MOP capabilities.

Proposal 7: RAN4 to discuss if it’s necessary to separate MPR/AMPR requirements for one PA/two PA and 1Tx/2Tx architecture.

For the detail MPR/AMPR requirements, they’re very complicated now. As a RF spec to the engineering team, we’re not sure if this complication brings any benefit to the network. At least, it brings much trouble to the engineering team both from reading and debugging. Some of the aspects we think can be improved are too many RB allocation cases, too many “if, else”, necessity of non-contiguous RB allocation cases, too small step size of the values. We propose to revisit these aspects for 5G spec to reach a guideline for 6G requirements.

Proposal 8: Revisit the complicated MPR/AMPR requirements at least for the following aspects

  - The necessity of non-contiguous RB allocation.

  - Too many “if”, “else” in the clauses.

  - Too many RB allocation cases.

  - Too small step size of MPR/AMPR values.

There’s another issue for the MPR/AMPR requirements that companies are more and more interested to improve the values. We’re not supportive because when the requirements are discussed, it’s based on the sufficient evaluation and the art of the state is already considered. It is not foreseen that much improvement can be achieved in one or two years. The modified MPR bahavior brings much trouble to the spec and understanding, we propose to set a very high bar to change MPR/AMPR requirements in later releases in 6G phase unless there’s a regulation driven motivation.

Proposal 9: Set a very high bar to change MPR/AMPR requirements in later releases unless there’s a regulation driven motivation.

2.1.3 Revisit CA MSD methodology

CA MSD requirement is another complicated discussion but may not benefit the UE implementation and system performance. In 6G, the whole methodology of CA MSD requirement can be revisited. The intention of define MSD requirements and how to use it can be clarified from both operator and NW side. If the main purpose of the MSD requirement is to make the decision of deployment of carrier aggregation, then the discussion can be easy. The evaluation and discussion can be rough and the MSD requirement even doesn’t to be tested. Technically, most of the device life time works in the non-CA mode because of the limited spectrum resource. The MSD requirements are defined for specific worst-case scenarios, it is unclear whether investing significant effort in discussion, debugging, and testing them is worthwhile. Even there’s a value to define the MSD requirement, the methodology can also be revisited. Currently, HPUE MSD rules based on PC3 MSD are studied and agreed, thanks for the great efforts from CA experts. It saves much discussion work for CA. However, for the implementation and test, the MSD for each power class still are there.

Considering how to define the CA MSD in 6G, three options can be considered. First option is to reuse current approach and all NR existing requirements will be transmitted to 6G. We don’t prefer this option because current MSD requirements are not defined in a good way, some of them are defined with an accuracy of one decimal place which is worthless, some of MSD is very large that it can’t guide the deployment and implementation at all. There’s another option is to make a minor improvement to the current methodology but MSD requirement is defined in a more coarse step, for example [3] dB step size to save the discussion efforts. And very large MSD CA will not have exact requirement, for example > 15 dB，or the CA request can’t be approved. A more aggressive improvement is to remove the MSD requirement and test, the interference/desense capability for specific configuration is reported by UE. This may need more discussion that how NW use this information. It seems none of the three options are perfect, we welcome company’s views and hope it can be improved in 6G.

Proposal 10: RAN4 to revisit the CA MSD requirement methodology considering the value of MSD requirement and saving the efforts for both standard discussion and UE implementation/test.

2.1.4 RF related signaling introduction

In 38.101-1 spec, some RF related signaling capabilities are only for the purpose of test, for example, Txdiversity, dual PA, etc. These signalings are not used by NW, they can be declared by UE like BS. So we propose UE uses the BS declare approach not signaling for the cases only for test purpose. A high bar should be set to introduce RF related signaling if there’s no benefit to the system performance.

Proposal 11: UE uses the BS declare approach not signaling for the cases only for test purpose.

Proposal 12: A high bar should be set to introduce RF related signaling if there’s no benefit to the system performance.

  - Spectrum aggregation

On 6G spectrum aggregation, RAN4 needs to be deeply involved from RF/RRM feasibility persetcive to investigate potentail solutions on fully usage of operators’ spectrum with affordable cost from UE and gNB. Several directions can be considered in 6GR intial stage on flexcible spectrum usage:

- DL/UL spectrum decoupling

- Fragmented carrier

- CA via swicthing and uplink Tx swicthing

DL/UL decoupling

6GR will further extend spectrum to 7GHz or even 10GHz, how to efficient use spectrum to guranteen both coverage and high data rate request. By natually UE can utlize low bands spectrum for coverage (UL) and  high bands spectrum for DL data tranmission.  Decouple DL and UL spectrum is one potential research area for flexicble spectrum integration.

RAN4 needs to evaluate the operation restrcition and potentail reference architecture from both RF and RRM perspective, also the difference compared SUL operation for flexcible spectrum paring across low frequency to high frequency in different band group.

Fragmented spectrum

Operators’ spectrum maybe fragemented over same band or across different adjacent bands.

In Rel-19 RAN4 estiablished a study item to study using shared RF chain to receive multi non-contiguous carriers within same band.

The benefits of shared chain to receiver multi fragmented carriers as following:

- Save cost to receive multi-carriers with reduced RF/BB chain

- Support high order band combinations with more carriers with limited Tx/Rx chains

- Support high order of MIMO layers for same band combination

In Rel-19 fragmented carrier study, the restriction and performance impact were also anayzed with shared RF chain with two restrction realized:

- With shared RF chain, UE performance in in-gap interference rejection degraded as no sub-block analogue filter

- REFSENS degradation due Tx leakage with reduced attenuation on UL carrier frequency with wideband LPF under “single Rx chain mode”.

Based on RAN4 analyzed in Rel-19, some restriction maybe applied for single Rx mode:

- Power imbalance between wanted carriers

- In gap interference (power imbalance between wanted carrier and in-gap interference)

- Maximum frequency separation between carriers

Potential methods are discussed for capability signaling, CA fallback and architecture switching, and Triggering condition to enable ”One Rx RF chain” and summarized in TR 38.755.RAN4 also discussed related single Rx chain mode switching on/off mechanism with in-gap interference monitoring which may require potential RRM work.

For 6G, the fragmented carrier scenario can be further extended to inter-band cases as following:

Other WGs also discussed potential enhancement of spectrum aggregation e.g., single cell multi carrier or single cell multi-bands. RAN4 still needs to be deeply involved to study RF/RRM feasibility study incl. reference architecture, related scheduling conditions and capability design and the benefits compared to traditional CA framework.

UL Tx switching/ CA via switching

NR introduced UL Tx switching and CA via switching to dynamic/semi-static switching Tx/Rx chains over different bands/carriers to efficient use the available Tx/Rx chain and flexible spectrum.

For UL Tx switching, different cases and scenarios were introduced under different release including CA UL switching between 1Tx-2Tx across 2 bands, 2Tx-2Tx UL switching between 2 bands, 1/2Tx-1/2Tx switching between 3/4 bands including SUL bands. In Rel-19 RAN4 maintenance, RAN4 also discussed 3Tx switching and NUL-SUL switching with non-zero switching time.

Under Rel-19, RAN4 also discussed and introduced LB CA via switching for SDL and FDD carrier due to RF front-end limitation and the candidate bands as show in below table.

Rel-19 LB ca via switching only focus on FDD-SDL carrier with switching 2 carriers and Rel-20 will support LB CA via switching for FDD-SDL carrier with dual DL.

There is commonality between UL tx switching and carrier switching in terms of switching time, switching mechanism and potential RF/RRM impact basically the ideal was to share Tx/RF chain across closed operating frequency ranges within band group to achieve MIMO or frequency diversity gain.  It’s worth to study unified framework and switching mechanism for all above mention cases and new cases which on discussion in Rel-20/Rel-19 maintenance.

Proposal 13: RAN4 shall be deeply involved in day 1 for flexible spectrum usage/spectrum aggregation study

Proposal 14: RAN4 work shall focus on RF/RRM feasibility study incl. reference architecture, related scheduling conditions and capability design and the benefits compared to traditional CA framework

Proposal 15: RAN4 initial study can be focused on following area:

- DL/UL decoupling:  Band definition, RF architecture, and RRM aspects

- Fragmented spectrum (single cell multi carriers/BWPs)

  - Reference RF and BB architecture

  - RF/RRM feasibility and operation schemes between different operation modes

  - Leverage Rel-19 RAN4 fragmented carrier SI with extension to inter-band case

- CA via switching and Uplink Tx switching with unified framework and mechanism

  - Leverage NR design on LB CA via switching and uplink Tx switching with extension to more cases

- Conclusion

In this contribution, our preliminary view on 6G UE RF and general RF topics with following area

- Non-spectrum and non-AI UE RF

- Spectrum aggregation framework

For Non-spectum UE RF topic

For the direction of revisiting MOP definition and capability report,

Observation 1: If UE MOP capability is between two power classes, UE can only report the lower power class.

Proposal 1-5 : RAN4 to discuss the following issues for MOP definition and capability report

- Revisit if the power class-based MOP definition methodology is still used in 6G.

- The whole UE power class/MOP capability report structure for 6G.

- The possibility to remove the UL duty cycle based solution in spec.

- If there’s a better way to optimize the Pcmax equation and the whole Pcmax clause.

- The possibility to merge/omit some of the Tx requirements for different power classes/MOP capabilities.

For the direction of simplification of MPR/AMPR requirements

Proposal 6: RAN4 to discuss if common MPR/AMPR requirements can apply to different MOP capabilities.

Proposal 7: RAN4 to discuss if it’s necessary to separate MPR/AMPR requirements for one PA/two PA and 1Tx/2Tx architecture.

Proposal 8: Revisit the complicated MPR/AMPR requirements at least for the following aspects

  - The necessity of non-contiguous RB allocation.

  - Too many “if”, “else” in the clauses.

  - Too many RB allocation cases.

  - Too small step size of MPR/AMPR values.

Proposal 9: Set a very high bar to change MPR/AMPR requirements in later releases unless there’s a regulation driven motivation.

For the direction of revisiting CA MSD methodology,

Proposal 10: RAN4 to revisit the CA MSD requirement methodology considering the value of MSD requirement and saving the efforts for both standard discussion and UE implementation/test.

For the direction of RF related signaling introduction

Proposal 11: UE uses the BS declare approach not signaling for the cases only for test purpose.

Proposal 12: A high bar should be set to introduce RF related signaling if there’s no benefit to the system performance.

On spectrum aggregation framework

Proposal 13: RAN4 shall be deeply involved in day 1 for flexible spectrum usage/spectrum aggregation study

Proposal 14: RAN4 work shall focus on RF/RRM feasibility study incl. reference architecture, related scheduling conditions and capability design and the benefits compared to traditional CA framework

Proposal 15: RAN4 initial study can be focused on following area:

- DL/UL decoupling:  Band definition, RF architecture, and RRM aspects

- Fragmented spectrum (single cell multi carriers/BWPs)

  - Reference RF and BB architecture

  - RF/RRM feasibility and operation schemes between different operation modes

  - Leverage Rel-19 RAN4 fragmented carrier SI with extension to inter-band case

- CA via switching and Uplink Tx switching with unified framework and mechanism

  - Leverage NR design on LB CA via switching and uplink Tx switching with extension to more cases

Reference

[1] RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] RAN1#122 meeting report