3GPP TSG-RAN WG4 Meeting #116bis		    R4-2513320
Prague, Czech Republic, 13 – 17 Oct 2025

Title:	On 6GR testability and OTA

Source:	OPPO

Agenda item:	8.12

Document for:	Approval

Introduction

In the RAN plenary (RAN#108), a study item on ‘6G RAN’ was approved [1]. This contribution discusses tstability and OTA as per RAN4 guideline of 6GR discussion.

Discussion

As from the 6GR SID above, there are three aspects on testability and OTA for 6GR as BS requirement and testing framework, aspects related to the testability. In this paper, we give further discussion on the aspects related to the RAN4 test framework, non-AI testability aspects and AI testability aspects.

Conductive/OTA test framework

The RAN4 RF requirements has been separated into conductive and OTA two aspects due to the large difference of the test methods. Traditionally, conductive test is defined which link the RF connector to the test equipment. All the test are accomplished by the connection of cable which makes the test simple and fast.

From LTE, the TRP/TRS as well as the MIMO OTA tests have been studied and introduced. The reason of introducing such different test types is people begin to be willing to know the exact UE RF performance on the air interface. The TRP and TRS test mainly try to figure out the UE RF performance in both TX and RX direction. The MIMO OTA test is further to test the MIMO performance by introducing different channel models to simulate a real air environment.

From LTE, the conductive plus TRP/TRS and MIMO OTA framework has been established.

In 5G NR, the frequency range FR1 has been defined and the legacy LTE conductive/OTA test framework has been reused. The full set of UE RF conductive requirements are defined while the TRP/TRS as well as MIMO OTA tests are used. Besides, as the frequency range FR2 has been introduced, due to the beamforming of FR2 antenna pannel, the conductive test is not feasible to test the performance anymore. In such case, the FR2 has only used OTA tests.

For 5G NR, FR1 reuse similar framework of LTE as conductive test plus TRP/TRS	and MIMO OTA.

For 5G NR, FR2 only used OTA tests.

From the above observations, the very important reason is that the RF connector feasibility of a UE. In 6GR, the frequency range will be even larger than 5G. The FR1 and FR2 5G NR bands will be re-farmed step by step for 6G usage. For these bands, the legacy test framework can be reused. For FR3 as currently defined as 7.125GHz to 24.25GHz, sub-range is further proposed and the test framework can be discussed separately. The frequency range is divided into 3 sub-ranges as shown below.

Figure 2.1-1 Sub-range division of FR3

Currently the 6GR new bands are discussed while the potential new bands might be defined as around 7GHz and around 15GHz. For the sub-range 1 as around 7GHz, the range is quite similar to FR1 and we believe similar test framework can be reused. For the sub-range 3 who is very near to FR2 while no potential bands is observed at this stage, it is still reasonable to define test framework as FR2.

For 6GR, FR1 like sub-range 1 can reuse 5G FR1 test framework and FR2 like sub-range 3 can reuse 5G FR2 test framework,

While for the sub-range 2 as around 15GHz, it is hard to agree on what kind of test framework should be applied. In the 5G NR study of 7-24GHz, the RF connector feasibility has been studied and the results are captured as below table:

Table 2.1-1: RF connector and probe capability

| Supplier | Connector dimensions (mm) | Frequency range (GHz) | Insertion loss (dB) | VSWR |
| --- | --- | --- | --- | --- |
| Supplier A | < 2x2 | 0 – 12 | < 2 | < 1.6 |
| Supplier B | < 3x3 | 0 – 11 | < 2 | < 1.5 |
| Supplier C | < 3x3 | 0 – 11 | < 2 | < 1.5 |
| Supplier D | < 2x2 | 0 – 15 (NOTE 1) | < 2 | < 1.6 |
| Supplier E | < 4x4 | 0 – 26.5 | < 0.5 | < 1.25 |
| Supplier F | < 2x2 | 0 – 11 | < 0.3 | < 1.4 |
| Supplier G | < 5x2 | 0 – 26.5 | < 0.4 | < 1.4 |
| NOTE: Operation up to 12 GHz per spec sheet; operation up to 15 GHz feasible based on experimental data. |  |  |  |  |

It can be observed that at least for 11GHz multiple of connector suppliers can support while for 15GHGz at least one supplier can support.

The 15GHz connector with feasible insertion loss is on the market.

From test simplify perspective, the conductive test is always the choice. Hence it is proposed to use conductive test framework to around 15GHz bands for 6GR.

For 6GR around 15GHz band, conductive test framework is preferred.

General Testability aspects

New frequency range

The test methods evolution in 5G NR starting from the TR 38.810. Currently the test methods are all captured in the TR and applies to FR1 and FR2 as defined in 5G. While for FR3, the applicability should be further investigated. The especially to be further discussed is the test distance, measurement uncertainty and quite zone.

The test distance decides the chamber size and also the quite zone size which is cost sensitive of the test system. Also it directly depend on the test method is far field or near field. A simple calculation of near field/far field boundary for different frequencies and antenna sizes for a traditional far field anechoic chamber is calculated in table 2.2-1 and as shown below especially for FR3. As the D stands for the diameter of the smallest sphere that encloses the radiating parts of the DUT, it can be observed that the distance can be very large. Hence the study of FR3 is needed.

Table 2.2-1 Near/Far boundary for FR3 typical frequency

| D (cm) | F (GHz) | Near/Far boundary (cm) | D (cm) | F (GHz) | Near/Far boundary (cm) |
| --- | --- | --- | --- | --- | --- |
| 5 | 7 | 12 | 5 | 15 | 25 |
| 10 | 7 | 47 | 10 | 15 | 100 |
| 15 | 7 | 105 | 15 | 15 | 225 |
| 20 | 7 | 187 | 20 | 15 | 400 |
| 25 | 7 | 292 | 25 | 15 | 625 |
| 30 | 7 | 420 | 30 | 15 | 900 |

Test distance for different test method is needed.

For the measurement uncertainty, a table 2.2-2 is shown as below captured from TS 38.151 of a FR1 MPAC system. As can be seen that the example value and std uncertainty are all listed for different contributions with frequency range of 410MHz to 3GHz and 3GHz to 7.125GHz.

The MU contributors are all frequency dependent.

Table 2.2-2: Preliminary measurement uncertainty budget for FR1 MPAC system

| UID | Description of uncertainty contribution | Example value (410MHz<f≤3GHz) | Example value (3GHz <f≤7.125GHz) | Distribution of the probability | Std Uncertainty (410MHz<f≤3GHz) [dB] | Std Uncertainty (3GHz <f≤7.125GHz) [dB] |
| --- | --- | --- | --- | --- | --- | --- |
| Stage 2: DUT measurement |  |  |  |  |  |  |
| 1 | Mismatch for measurement process | 0 | 0 | U-Shaped | 0 | 0 |
| 2 | Measure distance uncertainty | 0 | 0 | Normal | 0 | 0 |
| 3 | Quality of quiet zone | 0.6 | 0.6 | Actual | 0.6 | 0.6 |
| 4 | Base Station simulator | 1.5dB | 2dB | Rectangular | 0.87 | 1.15 |
| 5 | Channel Emulator - absolute output power - output signal stability - output stability with temperature | 1.5dB 0.5dB 0.4dB | 1.5dB 0.5dB 0.4dB | Actual (normal- power; rect-stability) | 0.84 | 0.84 |
| 6 | Amplifier uncertainties | 0.7dB | 0.7dB | Rectangular | 0.4 | 0.4 |
| 7 | Random uncertainty | 0.2dB | 0.2dB | Normal | 0.12 | 0.12 |
| 8 | Throughput measurement: output level step resolution | 0.25dB | 0.25dB | Rectangular | 0.14 | 0.14 |
| 9 | DUT sensitivity drift | 0.2 | 0.2 | Rectangular | 0.12 | 0.12 |
| 10 | Signal flatness | 0 | 0 | Normal | 0 | 0 |
| Stage 1: Calibration measurement |  |  |  |  |  |  |
| 11 | Mismatch for calibration process - loopback cable path - system input path - reference antenna | 0.2 | 0.2 | U-Shaped | 0.14 | 0.14 |
| 12 | Reference antenna positioning misalignment | 0 | 0 | Normal | 0 | 0 |
| 13 | Quality of quiet zone | 0.6 | 0.6 | [Rectangular] | 0.35 | 0.35 |
| 14 | Total uncertainty of the Network Analyzer | 0.5 | 0.5 | Rectangular | 0.29 | 0.29 |
| 15 | Uncertainty of an absolute gain of the calibration antenna | 1 | 1 | Normal | 0.5 | 0.5 |
| 16 | Offset of the Phase Center of the Reference Antenna | 0 | 0 | Normal | 0 | 0 |
| Total Expanded Uncertainty, U, with 95% Confidence Interval |  |  |  |  | 3.03 | 3.38 |

The Quiet Zone (QZ) in Over-the-Air (OTA) testing refers to a controlled volume within an anechoic chamber where electromagnetic reflections are minimized to ensure accurate measurements. It is essential for evaluating the radiated performance of antennas and wireless devices, particularly in 5G FR2 (mmWave) testing, where conducted measurements are impractical.

The quality of the QZ is determined by analyzing the reflectivity level, typically using the Free Space VSWR (Voltage Standing Wave Ratio) method. Measurements are taken in a 15° grid across elevation and azimuth, yielding 264 standard deviation values for both polarizations.

The quite zone is also frequency dependent.

Figure 2.2.1-1 illustration of Quite Zone

For FR3, further study the test distance, measurement uncertainty and quite zone in new frequency range.

Device type and form factors

The 5G has introduced many different form factors such as hand-held UE, wearable device, XR and also NTN. Different form factors have introduced different test problems and they are listed as below table 2.2.2-1.

Table 2.2.2-1 Different form factors in 5G

| Form factor | Key issue |
| --- | --- |
| Hand-held UE | Baseline to define test methods |
| Wearable devices | Battery problem and antenna design complexity |
| XR | Partial sphere/Weighted test points based on the wear method |
| NTN | Partial sphere and circular polarization |

In 6GR, these form factors will be kept. However, it is suggested to study the possible device types first and possible form factors at the beginning of the release. Also, the influence of different form factor should also be considered at the beginning.

To study device type definition of 6GR on form factors and corresponding OTA test impacts.

Multiple Uplink and CA

During the 5G OTA testing, the 2TX UL MIMO has been discussed. The TPMI method has been fully discussed. Finally, it was agreed that fixed TPMI for non-coherent UE and average of different TPMI for coherent UE were used for 2TX UL test. The dynamic switching of TPMI has been dropped which although simulate the on-field scenario but hard to produce such test environment in test chamber.

The fixed and average TPMI method has been agreed in 5G for 2TX.

Further evolution of 5G has already introduced 3TX for UE and this is foreseen to be an important feature in 6GR to achieve higher output power. Besides, the CA of OTA test is also a question.

The 3TX is foreseen to be an important feature in 6GR.

To study the 2TX/3TX UL test method.

AI testability aspects

Potential 6GR OTA test method for AI BM

In 5G-A, the OTA test is expected to be utilized for AI BM in FR2. Multi AoA based RRM testing setup is considered as baseline, and single AoA tests are nor precluded. Meanwhile, companies are invited to bring analysis on what simplified spatial channel (multiple clusters coming from multiple directions) model can be emulated in this test setup. However, less progress has been made, by the end of 5G-A, RAN4 still does not have a clear understanding on how to handle this issue and have a test setup in chamber.

To consider the muli-AoA test system which can better simulate the real channel model, the 3D MPAC test system is shown below and has been proposed to be the test system for AI-BM tests while currently there is no agreement yet. The channel model emulator in the test system can help to simulate the OTA channel model and the multi-probe system can simulate different Base station beams.

Figure 2.3-1 5G NR FR2 3D MPAC test system

The 5G FR2 3D MPAC system can be one solution for AI-BM test.

However, although the FR2 3D MPAC system has been defined, it has not been globally used due to the lack of deployment of FR2 in 5G. Furthermore, the high expense of the test system and the difficulty of appropriately modeling the channel also leads to current FR2 MIMO OTA not broadly used.

The 5G FR2 MIMO OTA system has not been broadly used.

It is hard to correctly model the FR2 multi-AoA channel model and re-establish in chamber environment.

For AI-BM test in 6GR, to further study the commercial benefit to consider the FR2 deployment, test system complexity and the difficulty to model the channel.

Decouple of signal acquisition and model performance test

In 5G AI OTA testability, a lot of effort has been spent on generating signals in a chamber environment that can serve as input for AI use case models. However, key questions still lack clear answers, e.g., how to effectively simulate beam-sweeping scenarios in a chamber with limited probes, how to model channels, and whether testing model performance under simplified CDL UMi conditions is truly meaningful. From our understanding, in current 5G BM FR2 test discussions, the over-simplified OTA assumptions being used may offer little practical guidance for field deployment.

The over-simplified OTA assumptions being used may offer little practical guidance for field deployment.

In 6G, RAN4 may need to specify more reference models for different use cases and potentially for different scenarios/conditions. Besides, new LCM features like model selection, model updates, online training/tuning may also kick off new needs for model tests. Furthermore, the post-deployment issue needs to be dealt with at this time, e.g., treat as parts of LCM and identify details for testing methods. So, this time if we still need to do a lot of work for OTA setup discussion and introduce complex test procedures for the model test, it will be a big challenge.

With new LCM features, new test setup will always need to be specified which results in endless OTA test setup discussion.

In 6G, another approach for AI tests (especially for model test) could be RAN4 may explore whether/how to decouple the signal acquisition testing(e.g., or model inputs) from model performance evaluation(e.g., for model performance) for AI use cases.

-	Signal acquisition could continue to be tested via cable or OTA methods, e.g., evaluating BM Set B measurement accuracy.

-	Model performance could be assessed using standardized datasets defined by RAN4 (RAN4 specified datasets), enabling a virtual testing environment (maybe somehow like a Digital Twin setup). For instance, RAN4 study and specify test dataset for models under test. TE would provide the DUT with a predefined test dataset. The DUT would then return inference results to the TE for performance evaluation. This approach would reduce the need for complex OTA tests, accelerate model validation, and bring testing closer to real-world conditions depending on the quality of the test dataset.

For 6G, RAN4 could explore whether/how to decouple the signal acquisition testing(e.g., or model inputs) from model performance evaluation (e.g., for model performance) for AI use cases.

Signal acquisition could continue to be tested via cable or OTA methods, e.g., evaluating BM Set B measurement accuracy.

Model performance could be assessed using standardized datasets defined by RAN4 (RAN4 specified datasets), enabling a virtual testing environment.

Conclusion

This paper discussed the 6GR testability and OTA, and got below observations and proposals.

Conductive/OTA test framework

Observation 1: 	From LTE, the conductive plus TRP/TRS and MIMO OTA framework has been established.

Observation 2: 	For 5G NR, FR1 reuse similar framework of LTE as conductive test plus TRP/TRS	and MIMO OTA.

Observation 3: 	For 5G NR, FR2 only used OTA tests.

Proposal 1: 	For 6GR, FR1 like sub-range 1 can reuse 5G FR1 test framework and FR2 like sub-range 3 can reuse 5G FR2 test framework,

Observation 4: 	The 15GHz connector with feasible insertion loss is on the market.

Proposal 2: 	For 6GR around 15GHz band, conductive test framework is preferred.

New frequency range

Observation 5: 	Test distance for different test method is needed.

Observation 6: 	The MU contributors are all frequency dependent.

Observation 7: 	The quite zone is also frequency dependent.

Proposal 3: 	For FR3, further study the test distance, measurement uncertainty and quite zone in new frequency range.

Device type and form factors

Proposal 4: 	To study device type definition of 6GR on form factors and corresponding OTA test impacts.

Multiple Uplink and CA

Observation 8: 	The fixed and average TPMI method has been agreed in 5G for 2TX.

Observation 9: 	The 3TX is foreseen to be an important feature in 6GR.

Proposal 5: 	To study the 2TX/3TX UL test method.

Potential 6GR OTA test method for AI BM

Observation 10: 	The 5G FR2 3D MPAC system can be one solution for AI-BM test.

Observation 11: 	The 5G FR2 MIMO OTA system has not been broadly used.

Observation 12: 	It is hard to correctly model the FR2 multi-AoA channel model and re-establish in chamber environment.

Proposal 6: 	For AI-BM test in 6GR, to further study the commercial benefit to consider the FR2 deployment, test system complexity and the difficulty to model the channel.

Decouple of signal acquisition and model performance test

Observation 13: 	The over-simplified OTA assumptions being used may offer little practical guidance for field deployment.

Observation 14: 	With new LCM features, new test setup will always need to be specified which results in endless OTA test setup discussion.

Proposal 7: 	For 6G, RAN4 could explore whether/how to decouple the signal acquisition testing(e.g., or model inputs) from model performance evaluation (e.g., for model performance) for AI use cases.

Proposal 8: 	Signal acquisition could continue to be tested via cable or OTA methods, e.g., evaluating BM Set B measurement accuracy.

Proposal 9: 	Model performance could be assessed using standardized datasets defined by RAN4 (RAN4 specified datasets), enabling a virtual testing environment.

Reference

RP-251881	New SID: Study on 6G Radio, NTT DOCOMO (Moderator)