3GPP TSG-RAN WG4 Meeting #116	R4- 2513340

Prague, CZ, October 13th – October 17th, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on 6GR RRM requirements

Agenda item:	8.7

Document for:	Discussion

Introduction

In the last RAN4 meeting, the chair presented the RAN4 6G topic planning based on the 6G SID [R4-2511652]. One of the 6G topics to be discussed relates to RRM and included following aspects:

RAN4 driven non-AI RRM topics, MIMO operation, mobility, initial access, spectrum aggregation, multi-TRP, RRM related coverage and energy efficiency, other PHY signal/channel/procedure related RRM

According to 6G WID, the 6G design should minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting while providing energy efficiency and energy saving for both network and device side. This in mind, RAN4 should start to study how the requirements can be adapted to support these goals.

In this paper we give our view on aspects which we see should be discussed during the SI phase of 6G from RAN4 RRM point of view, including some of the items above.

General Discussion

6G ‘Baseline’

As an initial aspect, it would be good to understand in RAN4 what each company sees as being the assumed baseline UE features to be used for the 6G discussions.

The intention here is only to consider NR WIs and improvements led by RAN4 RRM (as other enhancements led by other WGs will be decided by the relevant WG). Having this in mind, RAN4 has worked on many useful RAN4 RRM enhancements during many releases. For example:

- Inter-band CA requirement for FR2 UE measurement capability of independent Rx beam.

- Inter-frequency measurement requirement without MG.

- NR support for high speed train.

- Measurement gap Enhancements:

  - Pre-configured MG pattern.

  - Multiple concurrent and independent MG patterns.

  - Network Controlled Small Gap (NCSG).

- UE power savings by relaxing RLM/BFD.

- RRM Enhancement solutions.

RAN4 need to consider which of the features defined from Rel-15 to Rel-20 in RAN4 RRM are assumed included in the 6G ‘baseline’.

UE minimum Requirements and UEs exceeding minimum requirements

RAN4 will define minimum requirements that allow various implementation options. In the legacy the approach has been that the UE may outperform the requirements. In the practical field deployments, it can be a challenge to account the wide range of UE implementations in practise. One way in legacy has been trying to make different UE implementations visible to the network is via UE capabilities and feature groups. However, sometimes the capability framework is too coarse to the finer implementation options of RAN4, and the requirements end up supporting only the minimum scenario.  This means that the UEs implementing features exceeding minimum requirements may not get the full benefit in performance and user experience. In 6G RAN4 should study how to define requirements in such a way that UEs who perform better than the minimum requirements can also benefit from this better performance. Such approach has system benefit and can benefit both network and UEs.

Study how to define RRM requirements that allow UE implementation based on minimum requirements but also allow UEs that can outperform the minimum requirements the benefits from such better performance.

Foundation of RAN4 requirement work

RAN1 is currently working on the high-level aspects of 6GR synchronization signal structure and associated periodicities, which will eventually impact RAN4 measurements requirements in the form of  cell detection requirements, measurement requirements, various activation requirements and other procedure(s) relying on using the synchronization signal(s).

When compared to 4G, 5G NR was able to enhance network energy efficiency by decreasing PSS/SSS periodicity four times from 5ms to 20ms and replacing CRS transmitted in every subframe with periodical TRS transmitted more sparsely in time domain. For 6G it is expected that time domain footprint of the always-on initial access related signals and channels can be further reduced. Reduced periodicity of the always-on signals, such as SS/PBCH, is correlated to required complexity UE searcher in initial cell selection. However, this is a discussion for RAN1.

Cell search is a generic procedure by which a UE acquires time and frequency synchronization with a cell and detects the PCI of that cell. In 5G, cell search involves UE acquiring synchronization to a cell by acquiring the broadcast PSS and SSS. Acquiring PSS grants the UE half-frame synchronization, while acquiring SSS grants the UE frame-level synchronization. The following figure illustrates the legacy SSB related components of cell search, which are visible in RAN4 requirements in both connected mode (Tidentify) and in idle mode (Tdetect). In NR SSB is used for detection, PCI, DMRS and PBCH decoding for MIB. It is up to RAN1 whether this design will change, but if it changes it will have impact to RAN4.

Figure 1:

RAN4 to study any changes to the synchronization signal design and other related reference signals and the impact on cell detection and measurement requirements.

Such impact analysis can be done once RAN1 has made sufficient progress.

Seamless integration of NTN features.

NTN was added in Rel-17 as a non-native feature in NR. There were several aspects that were specific to NTN that set it apart from the existing TN specifications (especially a very large propagation delay and a fast varying propagation delay and doppler offset). In order to handle such differences, only parts of the specifications were then adapted to NTN. In RAN4, in order to capture the specificities of NTN work, a new set of subclauses (using suffix “C”) was added to the specification. This creates significant maintenance problems, as modifications on the clauses that set the general measurement framework are not automatically propagated to NTN. It is not uncommon in maintenance work to observe maintenance CRs that modifies parts of the general measurement framework being duplicated to also create modifications in NTN clauses.

There are significant maintenance issues and overhead in NTN clauses because they are set apart from the general clauses (separated by suffix “C”). NTN usually does not leverage on newly introduced enhancements on the general clauses, as they are not propagated to NTN clauses. Similar problems are also observed with ATG clauses.

Another problem with setting NTN apart is that it causes re-work, as some NTN work items present objectives that sometimes are just replicating the work done previously for TN (CA, less than 5 MHz, RedCap, etc.).

NTN should be considered as a day-1 feature in NR and not be relegated to separate subclauses. This includes the measurement framework.

UE transmit timing

RAN4 has defined the UE UL transmit timing requirements for the different scenarios supported in the RAT. Hence, UL transmit timing requirement needs to cover UL transmit timing requirements for a UE using 1 UL transmission. Additionally, if the UE supports more than 1 TAG, the relevant UL transmit timing requirements would need to be defined for this scenario, including UE requirements if or when the minimum requirements may be exceeded. Additionally, for a UE supporting CA or DC, RAN4 has defined the UE MRTD and MTTD. However, if the time difference the UE is at least required to handle, is exceeded, the UE requirements are not clear.

Above situations should not happen, but as deployments get more challenging it can be difficult to fully avoid. Hence, although we see that the current 5G requirements would be good base also in 6G, UE requirements in case of timing being exceeded, should be defined in 6G. This would align the 6G requirements with 4G.

In 6G the UE transmit timing requirements would also need to account any aspects regarding mTRP.

Another issue related to UE transmit timing is related to how large timing errors are in relation to the CP length. UL timing error is directly related to the network ability to schedule multiple UEs on adjacent symbols or adjacent PRBs. When the UL timing error is larger than a fraction of CP, like 25%, the chance for causing inter-symbol interference is significantly increased. That is because the network needs to place FFT window for demodulate UL signals that are considering the UL timing of all UEs in that cell. Additionally, the inter-symbol interference caused by UL errors is made worst the larger is the delay spread of the channel. Examples of features with large UL timing error specified in 5G NR are larger subcarrier-spacings used in FR2-2 (480 kHz and 960 kHz) and for FR2-NTN. A detailed analysis for those features is found at [3]. Therefore, for those features the final requirements were designed taking into account only the UE implementation limitations. However, to make 6G features to work it need to work for both UE and network, and the network feasibility must be taken into account as well – obviously.

Large UL timing error has direct impact on UL demodulation performance.

In 6G, the UE transmit timing requirements would have to address practical aspects from deployments and define UE requirements for these.

Measurement framework

NR introduced measurement procedures and related measurement requirements for FR1 and FR2 back in release 15. Baseline was using 4G L3 measurement framework with the addition of covering the aspects needed for accounting FR2 spatial enhancements (DL and UL beam forming). Different from 4G, 5G was the first CA native system designed by 3GPP.

RAN4 defined SSB based NR measurement requirements for idle/inactive and connected mode for FR1 and FR2. Idle mode measurements in NR are always L3-based SSB measurements, and they cover intra-frequency, inter-frequency and inter-RAT cell reselection measurements. Additionally, idle/inactive mode CA/DC measurements.

Connected mode measurements are SSB and CSI-RS based L3 and L1 measurements for FR1 and FR2. Requirements are defined for intra-frequency, inter-frequency and inter-RAT measurements. Requirements for Connected mode are developed for measurements performed without gap-assistance and measurements performed using measurements gaps.

Intra-NR measurement requirements cover idle, inactive and connected mode measurements for FR1 and FR2.

- Idle mode measurements are SSB-based L3 intra-frequency and inter-frequency for cell reselection (mobility) and CA/DC purposes.

- Connected mode measurements are SSB and CSI-RS based L3 and L1 intra-frequency and inter-frequency.

While some aspects of NR measurement, such as reference signal type, design etc. and the supported frequency ranges directly come from the design of the physical layer and reference signals done in RAN1, and the signalling support is done by RAN2, some aspects are defined by RAN4. Such RAN4-led aspects are for example the categorization of measurement requirements into intra-frequency and inter-frequency measurements as well as the need for measurement gaps and the UE need for causing interruptions e.g. due to measurements.

Some aspects of measurement requirements are defined by RAN4 including for example the categorization of measurement requirements to intra-frequency and inter-frequency measurements as well as the need for measurement gaps and the UE need for causing interruptions e.g. to perform measurements.

In the following we will focus the discussion on intra-6G measurements.

Regarding the measurements and the related requirements in 6G, RAN4 naturally needs to follow the reference signal design, measurement and reporting configuration and the supported frequency ranges that are to be defined in coordination with other working groups. However, it is very important that RAN4 take into account the learnings from 5G to identify any issues related to the 6G measurement design as early as possible and shares this knowledge with RAN1/RAN2. This way RAN4 can help influencing the design and address problematic issues already during the 6G design.

To give structure to the 6G study regarding aspects of measurements that depend on RAN4, we have identified three high level aspects where we see potential to achieve improvements compared to NR.

These aspects are:

- Considering the purpose of requested measurements when defining the measurement requirements for connected mode and idle/inactive mode.

- Considering fluent transitions of measurements at RRC state changes and cell changes.

- Considering simplifying the measurement gap design.

In the following sections, we discuss each of these three aspects separately.

Purpose-based measurement requirements

Mobility vs. CA measurements

When developing the FR1 measurement requirements for 5G, the UE measurement requirements mostly followed the requirement structure and requirements from LTE. Similarly, many of the measurement requirement aspects for LTE followed what was defined in 3G. The basic assumption, when developing 3G and 4G measurements and requirements, was a single carrier capable UE. The assumption was also that UE is operating on a single serving cell and having one serving carrier. Cell search and measurements on neighbour cells were performed purely to enable UE mobility (including load balancing etc.).

In Rel-10 (LTE), 3GPP introduced carrier aggregation feature. The introduction of carrier aggregation did not lead to changes related to how the measurements were configured or the related UE measurement requirements. Hence, to detect, measure and report possible SCells, existing inter-frequency measurement framework was applied. This approach was kept throughout LTE, and the principle was copied directly to NR. Although NR did introduce measurement requirements to adopt the aspects of beamforming for FR2, there has been no major changes to the way measurements are configured and performed in NR. This means that large part of RAN4 measurement requirements originate from 3G and 4G, where the basic design was not originally accounting CA, and therefore not optimally considering measurements for facilitating CA. Hence, many of the current requirements are adapted to support multi-carrier operation on top of a single-carrier baseline which causes complexity in the specification and possibly in UE/network operation.

Rel-16 (and even 4G) introduced the EMR framework, where the network can configure the UE with idle/inactive mode measurements to be reported at connection setup for the purpose of fast CA/DC setup. This addition split idle/inactive mode measurements into cell reselection and idle/inactive mode CA/DC measurements. However, for connected mode, measurements on inter-frequency carriers are generic regardless of whether they are for mobility purpose or for the purpose of configuring CA. Inter-frequency measurements are handled under the same requirements.

In NR, connected mode measurement requirements are defined originally for single carrier operation, and inter-frequency are generic with no differentiation based on the purpose of the inter/frequency measurement: mobility or CA (data).

In NR, idle/inactive mode measurement requirements are defined separately for the purpose of mobility (cell reselection) and data (idle/inactive mode CA/DC measurements).

In 6G, we think categorizing the connected mode inter-frequency measurements based on the purpose of the measurements - mobility or data - should be studied. This categorization should be studied at least for L3 inter-frequency measurements due to the following differences between these two measurement categories and their purposes.

Measurements for Mobility: L3 measurements for providing mobility are important and essential to provide an uninterrupted and high quality communication between the user and the network under different UE mobility conditions. Therefore, measurements to facilitate mobility need to be performed continuously to ensure timely HO. Otherwise, there is a risk that the link between the UE and network cannot be guaranteed, and UE may lose the connection (RLF). Rel-18 introduced also L1-based mobility measurements for LTM, which can be considered as complementary mobility measurements. These are more targeted and configured closer to the cell change. Rel-20 will also introduce dynamic activation/deactivation of these measurements.

L3 mobility measurements need to be continuous to ensure uninterrupted connection between the UE and the network including facilitating timely HO’s.

In NR, L1 measurements for LTM mobility may be configured closer to cell change and may be turned on/off dynamically (Rel-20) based on network configuration.

Measurements for Carrier Aggregation: To facilitate CA, network often need UE assistance measurements. Such measurements would by nature be on-need basis due to being dependent on the actual amount of data to be transmitted and the need for CA. While such measurements are important in order to facilitate carrier aggregation and enhanced user experience, they are not essential for maintaining the link between the UE and network. The lack of measurements enabling CA can lead to a temporary reduced data rate or reduced user experience etc. Measurements for enabling CA can therefore be considered only needed when there is a need for carrier aggregation or dual connectivity. Otherwise, such measurements can be considered less necessary.

Measurements for CA are essentially only needed when there is a need for configuring the UE with carrier aggregation or dual connectivity.

To take the nature of different types of measurements into account, RAN4 should study how to enable the most efficient measurement requirements, by taking the purpose of the configured measurement into account in the study. This study should apply to both idle mode, inactive mode and connected mode.

For Connected mode, Idle mode, and Inactive mode, RAN4 to study defining measurement requirements depending on purpose of the configured measurement: mobility or data (CA).

Measurements for Beam management (L1 measurements)

L1 beam management measurements can be considered as the third type of measurements. Beam management measurements in NR are connected mode serving cell SSB- or CSI-RS based L1 measurements that are configured to ensure the UE measures the serving beam and candidate beam quality to enable timely beam switch to a better beam when needed. Rel-17 also introduced inter-cell beam management measurements for beam management of cell with different PCI. Proper beam management is essential for ensuring UE is always connected to the most optimal beam.

Beam management measurements are L1 SSB or CSI-RS based measurements. The purpose of beam management measurements is to ensure the UE is always connected to the most optimal beam.

Focusing on serving cell beam management measurements, what is important for 6G is to ensure that the measurements and measurement reporting efficiently can serve the purpose of beam management. This must be essential criteria both for the UE is in single-TRP and multi-TRP operation. One challenge was observed during Rel-18 multi-Rx work item where group-based beam reporting (GBBR), which is used to report beam groups/pairs for enabling simultaneous multi-TRP transmission, does not work. The problem identified was that the UE may or may not be able to report as many beam pairs as the network requests the UE to report. This leads to a situation where UE must report something (anything) to follow the reporting configuration. This leads unreliable reporting from the UE which for the network means useless measurement results in the reports.

Another issue related to measurements for beam management is the mismatch between measurement delay expectations between different working groups. In many cases, RAN1 designs features such as CSI framework and TCI framework, assuming very short measurement and reporting delays.  However, when specifying the UE requirements, RAN4 often concludes, that in order to obtain sufficient measurement accuracy of the reported measurement results, there is a need for averaging several measurement samples, or that beam sweeping is needed in FR2. Such delay mismatch often impacts the gain and functionality of a feature, as such additional delay may have a direct impact on the UE reporting delay and hence time for completing certain L1 procedures. For example, a feature like activation of a TCI state.

To avoid similar problems in 6G, RAN4 must be involved early in the RAN1 discussions related to any L1 beam management measurements and reporting. A starting point would be to identify problems in NR and inform RAN1/2 about these problems at early stages of the study item.

To enable robust 6G beam management measurements and reporting, RAN4 to identify any issues related to NR beam management at early stages of 6G study item and inform RAN1 about such issues.

NTN measurements

The problems discussed in the clauses above are also valid for NTN measurements. In NTN, the time period to complete measurement is enlarged compared completing the same measurements for non-NTN due to collisions between multiple SMTCs configured in the same frequency layer (up to 4) and the sharing of a single SMTC between multiple satellites to be measured. The factor K_multi_SMTC was introduced as “the scaling factor for measurement of multiple SMTCs or multiple satellites”. The problem is even worsened when there is collision between multiple SMTCs.

In NTN, especially in NGSO scenarios, the multiple configurable SMTCs and the multiple configurable satellites to be measured per SMTC increases the time to complete the measurements significantly, while the dwell time in the cell tends to be smaller.

RAN4 to study how to simplify the measurement requirements for NTN. RAN4 to inform RAN1/2 about the conclusions of the study.

Fluent reuse of measurements

Efficient Measurements across state transitions and cell changes

In later releases of both LTE and NR, 3GPP defined a feature introducing that when a UE changes its state from connected to idle-mode, the UE can be requested to perform measurements for aiding CA, and to maintain detected CA candidate cells as detected for a certain period of time – across state transitions. Correspondingly, when the UE transitions to connected mode, the UE may report some of the measurements to the network.

So far there has been no general requirements related to cell detection or measurements across states. That being state changes between Connected and Idle and Inactive modes. From an overall system point of view, including both UE and network, it is beneficial to have clear requirements related to both cell detection status and measurement performance during state transitions. Such benefits relates both to reduced delays and UE power savings which in general leads to increased user experience. Improved network power savings is also one benefit.

RAN4 should study when UE transitions between 6GR defined states and how to define measurement requirements during such transitions, for example from Idle or Inactive mode to Connected mode and from Connected mode to Idle or Inactive mode.

Similar to studying UE requirements at state transitions, we also suggest studying measurement requirement or assumptions when UE is moving in Connected mode between cells. Fluent handling of measurements during transition between the cells is important.

In NR, there are at least three types of measurements in Idle/Inactive mode: Measurements for mobility (i.e., reselection measurements), measurements for CA (EMR) and serving cell measurements performed by the low power receiver.

One of the important study areas in Idle and Inactive mode measurements and measurement requirements is how to create a common unified measurement framework that utilises the best parts of NR, allowing UE to save power in Idle and Inactive modes while bridging to enabling fast wideband connection setup upon UE or network request.

Study unified UE measurement requirements, including cell detection status and measurements, across state transitions and cell changes.

Measurement gaps and gap-less measurements

In 3G, LTE and NR 3GPP have multiple features that needs UE measurements either with gaps or without gaps.

In essence a measurement gap is a configured periodic time window, where the UE is not scheduled by the network and where the UE can perform measurements. In general, this is needed for inter-frequency measurements when the UE does not have spare RF chains that can be used for performing measurements in that band.

The use of gaps can cause considerable throughput reduction. From UE perspective, a 6 ms long gap with 40 ms repetition results in a reduction of 15 % of the available occasions where the UE can be scheduled. From system performance perspective, this can be mitigated by configuring different gap offsets for different UEs. However, since the SSB on different carriers need to be synchronized due to allocation of measurement gaps and typical SSB repetition is every 20 ms (for coverage layers) allocation of different gap pattern among different UEs, is limited. Therefore, any throughput reduction is more significant when multiple UEs have synchronized gap configurations.

Measurements without gaps, on the other hand, can be used from Rel-15 for intra-frequency measurements, and Rel-16 and Rel-18 extended gap-less measurements to inter-RAT and inter-frequency measurements. However, even though gaps are not needed, there are only few exceptions where scheduling restrictions and interruptions would not apply.

The advantage of having measurements without gaps is reduced when such measurements introduce scheduling restrictions and interruptions. Scheduling restrictions (typically in FR2) are present during the SMTC where the UE may perform measurements and impacts the network scheduler similar to that of a gap (difference being that network is not aware when there are scheduling restrictions). Additionally, in the cases when the interruption requirements are not clearly defined in 5G NR (which is the typical scenario), interruptions can have severe negative impact. In all, measurements without gaps in NR is not seen as a reliable solution in NR but can have severe negative system impact.

Scheduling restrictions and interruptions hinder the advantages of measurements without gaps.

During NR, RAN4 had a long discussion related to UE performing measurements without gaps. It became clear that assumption is that some scenarios UE can perform gap-less measurements without causing interruptions while in other scenario gap-less measurements will cause interruptions. The driving force during the NR discussions seemed to be what the UE is requested to measure and the availability of a spare UE receiver chain.

Measurements performed on carrier frequencies which is part of a band combo with the serving cell(s) will as baseline does not need measurements gaps.

Investigate in which scenarios the UE can perform measurements without gaps in 6G.

Additionally, 5G NR has specified multiple measurement gap patterns of which only a fraction is mandatory. This approach makes the whole measurement gap framework quite complex.

In Rel-17, concurrent gaps were introduced. One justification of concurrent gaps was to use it for mitigating conflicts arising from the overlap of different gap patterns. However, apart from the positioning use case, those conflicts can be avoided by appropriate configuration from the network.

In order to mitigate the measurement gap length and address interruptions RAN4 introduced the NCSG gaps (known also from 4G). The NCSG gap pattern replaces a gap occasion of, say 6 ms, by two small gap occasions before and after the SMTC occasion to be measured. The results is a reduction of the measurement gap overhead. However, the retuning time used for NCSG is still quite large. RAN4 should identify whether UE implementations in 6G can reduce the retuning time, if RAN4 specify NCSG in 6G.

Preconfigured gaps were introduced in 3GPP to solve the problem that gaps may or may not be needed for measuring certain frequencies depending on the serving carriers and BWP used. As part of the Pre-MG specification, several rules were specified regarding the activation of Pre-MG patterns. The complexity of Pre-MG has increased in the latest releases, since different types of measurement gaps that can be configured in combination with Pre-MGs has increased.

Considering the current amount of different gap types and patterns, it can be noticed that there is a significant of options defined, all adding to the overall complexity of the measurement gap framework. Given that different gap types can be configured simultaneously, and all the possible combinations, complex rules regarding overlapping gaps has been defined, which can also be dynamically changing. Therefore, in 6G we should aim at simplification of the whole measurement gap framework, with focus on the features that have larger performance gains and commercial relevance.

Measurement gap framework is excessively complex due to possible combination of several types of gaps.

As discussed in this clause, the problem caused by scheduling restrictions mitigates any advantage that would come from gap-less measurements. NTN measurements have introduced new cases of scheduling interruptions related to inter-satellite measurements. In NTN, even intra-frequency measurements can cause measurement interruptions if performed in a different satellite. With the introduction of a new mobility mechanism (soft-satellite switching without PCI change) the problem has been enhanced, as this type of transition rely heavily on several scheduling restrictions (during soft-satellite switching). The problem is further worsened in NTN, as uncertainties potentially increase the number of slots affected by the scheduling restriction (due to lack of awareness by the network of the actual propagation delay difference). Such considerations should be present when discussing the new measurement framework.

RAN4 to study how to simplify the measurement gap framework in 6G including NTN use-cases and inter-satellite measurements.

Connected mode mobility

NR connected mode mobility requirements cover the delay and interruption requirements for different handover types as well as RRC connection mobility control requirements (RRC re-establishment, random access etc.). These procedures are to be defined by RAN1/2 for 6G, but RAN4 should be involved in the study once RAN1/2 make agreements on the design. In the following, we discuss some aspects of handover with more details.

Handover

Up to Rel-17, L3-measurement based handover has been used as the way to enable mobility. L3 (RRC) based handover has used as the baseline handover principle since early days including GSM, including both intra-frequency, inter-frequency and inter-RAT HO.

In later releases of NR (Rel-16 onwards), the baseline L3 handover was enhanced by enabling DAPS handover, dual connectivity across handover (HO with PSCell addition), and conditional handover (CHO), as well as combinations of these.

L3/RRC-based handover is the baseline for NR and legacy in general. It was later enhanced by introducing e.g. DAPS handover, dual connectivity enhancements, and conditional handover.

NR Rel-18 introduced L1/L2 layer triggered mobility (LTM), where the handover i.e. cell switch decision was moved to the MAC layer, and L1 measurements may be used for triggering the cell switch. With LTM, early candidate cell configuration as well as early DL and UL synchronization were introduced as means to reduce the handover/cell switch interruption. Rel-19 enhanced LTM by introducing LTM L1 measurement enhancements (event-triggered L1 measurement reporting and LTM candidate cell CSI-RS measurements), as well as conditional LTM (CLTM) cell switch.

L1/L2 triggered mobility (LTM) was introduced in Rel-18 to enable mobility measurements and triggering of handover/cell switch on lower layers.

6G handover design will be up to RAN2 to study and define. However, some aspects of handover delay requirements can already be studied on RAN4 side. From RAN4 perspective, independent of what type of handover is to be introduced as the baseline for 6G, the handover requirements cover handover delay, which consists of multiple components covering e.g. cell search, processing delays, DL synchronization, and RACH delay.

Based on the recent development of 5G advanced, it can be expected that handover interruption reduction continues to be a big topic for discussion in 6G. RAN4 should therefore discuss whether some improvement for handover delay/interruption will be possible already on RAN4 side.

Depending on the handover design, RAN4 needs to discuss which components the handover delay consists of. While some of the components and/or their definitions will likely be carried from NR to 6G, RAN4 should study whether there is room for improvement on some of the components that will likely still be needed. Good example is the UE processing delay (e.g. Tprocessing), which is part of every NR handover delay and has been carried to NR from earlier generations. RAN4 should study whether this processing delay is still relevant for 6G, or can some improvements be expected from a 6G capable UE.

Interruption reduction can be expected to be one important aspect of the 6G handover study. As the handover delay/interruption requirement is defined by RAN4, RAN4 can study ways to achieve some improvement.

Furthermore, NR L3 handover is supported towards a known and unknown target cell, while LTM is only supported towards a known target cell. Additionally, the definition of known target cell is different for L3 handover and LTM. RAN4 should study and consider whether a more unified and accurate approach than the split between known and unknown target cell requirements could be found.

NR handover delay requirements are defined for handover to known and unknown target cell. LTM cell switch requirements are defined only for known cells. The definition of known target cell is different for L3 HO and LTM.

Last, depending on RAN1/RAN2 design, 6G baseline handover may utilize some of the aspects of enhanced handover types. Such aspects may be for example early DL synchronization and early UL synchronization, for example as defined in LTM. If such aspects are to be studied and/or defined by RAN1/2, RAN4 should be involved in the study. This is necessary to ensure that the baseline design works in a reasonable and robust manner.

RAN4 to study at least the following aspects of handover delay/interruption:

  - HO interruption reduction directly through RAN4-defined components

  - HO requirements based on target cell status (known, unknown, or other status)

  - Feasibility of any RAN1/2 created HO features (e.g. early DL or UL sync)

Beam Management and MIMO

Several features were defined in 5G NR related to MIMO, which will be considered and used in 6G.

TCI switching requirements

In general, MIMO features in 5G NR have deep connection to the beam management and TCI switching requirements. One issue experienced in 5G systems is that the TCI switching requirements include several delay components which, in certain scenarios, result in a switching delay that is very large for L1 types of procedures. In some cases, the UE is allowed for additional time for L1-RSRP measurement round, PL-RS monitoring, time/frequency synchronization. In such scenarios those delays can be as large as the delay needed for performing some L3 procedures on known target cells, which in our perspective is not reasonable. For example, considering the requirements defined for the delay within which the UE shall be able to complete a MAC-CE based TCI state switch, specified in Rel-17 in clause 8.15.3 for DL and clause 8.16.3 for UL in TS 38.133. Considering the following realistic assumptions:

- NR_slot_length = 125 μs (corresponding to a SCS of 120 kHz),

- THARQ = 8 slots,

- Nslotsubframe,µ = 8 slots,

- Tfirst-SSB, Tfirst_target-PL-RS , Ttarget_PL-RS = 20ms (corresponding to a SSB periodicity of 20 ms),

- TL1-RSPR_Measurement_Period_SSB = 160 ms,

we obtain the following delays to complete the MAC-CE based TCI state activation:

|  | Target TCI state | PL-RS | Time to complete the TCI state switch [ms] |
| --- | --- | --- | --- |
| DL | Known | - | 4 |
|  | Unknown | - | 164 |
| UL | Known | Maintained | 4 |
|  | Known | Not maintained | 106 |
|  | Unknown | - | 266 |

Note that ~100/200 ms is a significant delay for a TCI state switch, as such delay is comparable to what can be assumed for handover delays in several scenarios.

Therefore, it can be considered how beam management can be performed in a more efficient way, e.g., avoiding some of the delay steps that are considered in 5G NR. For example, RAN4 can verify which of those measurement steps can be performed in parallel. Additionally, it is not always clear to the network how long the TCI switching will take. The UEs have implementation freedom to conclude TCI switching faster than the requirements, however, there is no advantage in doing so if the network is unaware that the switching is concluded. Furthermore, some of the conditions for longer TCI switching delay are known only to the UE.

Delays related to beam management can result in TCI switching procedure delays that are as slow as some L3 procedures.

RAN4 to study the steps involved in TCI switching and identify potential reductions in TCI switching delay for 6G.

Multi TRP aspects

Additional problems with TCI switching arrive when multi-TRP operation is considered. As one example, RAN4 has specified requirements for multi-Rx in Rel-18 under the assumption that group-based beam reporting (GBBR) is used to indicate potential target beams for multi-Rx operations. However, it was also pointed out from several companies that in order for a beam pair to be a suitable candidate for simultaneous reception a number of conditions must be met, e.g., regarding angle of arrival separation and receive time difference between beams.

However, RAN4 requirements do not specify that the beams reported using GBBR must satisfy those conditions. Additionally, the RAN1 GBBR procedure is defined such that the UE must report a configured number of beam pairs even if the number of suitable beam pairs is smaller than the configured value. As a result, it is not guaranteed, even if a UE reports a ‘beam pair’ using GBBR that the UE will be effectively able to receive using that reported ‘beam pair’. Another problem related to multi-TRP operation is regarding UL with more than 1 TRP. With simultaneous transmission (STxMP) in Rel-18 there was no clear method to determine suitable UL beam pairs.

GBBR measurement and reporting requirements for multi-Rx in Rel-18 are unclear as to whether the UE can receive simultaneously with the reported beam pair.

Finally, RAN4 has defined RRM requirements regarding to 2 TA’s for multi TRP UL transmissions. Those requirements are developed in both Rel-18 for multi-DCI operation and Rel-19 for single-DCI operation and UL-only TRP. 6G is expected to define mTRP operations already in the first 6G release, so a method for controlling timing advance similar to what specified in Rel-18 and Rel-19 would need to be specified. However, in 6G the limitations introduced in NR would need to be revisited. As an example, MTTD for UL-only TRP in Rel-19 only consider timing difference levels that are close to the CP length. That limits the considerably the deployment options, since the MTTD limit is directly related to the maximum distance between TRPs for MIMO.

Small MRTD/MTTD for mTRP limits the distance between TRPs for MIMO.

RAN4 to study how to enable robust multi TRP operation considering both beam pairs and deployments

Energy efficiency

Energy efficiency, as a metric for environmental sustainability and operational cost savings, has been one of the key targets in radio system design. In LTE, it has been focused mainly on UE side hence lots of UE power saving mechanisms e.g. DRX in connected mode was introduced. In 5G NR, network energy saving is further taken into account as a basic feature from the beginning. Instead of using continuous always-on synchronization and reference signals e.g. PSS/SSS and CRS in LTE, SSBs are being transmitted in a less frequent way which improves network energy efficiency. For 6G, we aim at bringing energy efficiency to a new level to play an even more important role given the diverse network deployments, scalable design, applications etc. 6G system must be designed from day-1 considering both network and UE energy efficiency. From RAN4 point of view, the 6G requirements must be defined to support energy saving features including both network and UE sides.

While network energy saving and UE energy saving solutions are discussed separately, they are mutually influenced and impact the overall performance. As an example, network may benefit from less SSB/SIB1 transmission, but it is at the risk of longer time period for cell identification and longer measurement delay. Network may improve the measurement performance by triggering some OD-SSB, but it may increase the UE power consumption. Therefore, RAN4 should discuss the expected Network/UE behaviour considering the trade-off between network and UE energy savings and define the RRM requirements with joint network and UE optimization.

RAN4 to define the 6G UE requirements from Day-1 to enable support of energy saving features with joint network and UE optimization.

Network energy saving solutions

In 5G, network energy saving has been considered since Release 15. For example, network may not transmit SSBs in some intra-band contiguous SCells i.e. SSB-less SCells in order to save power. In Rel-18, a dedicated study item was further approved which identified several beneficial NES mechanisms categorized by time, frequency, spatial and power domains. In the following, we will start from these NES solutions and discuss the 6G potential for network energy saving from RAN4 point of view.

Flexible SSB transmission periodicity

In 5G system design, SSB is the very basic reference signal used for cell identification, RRM measurement etc. Since SSBs are being transmitted periodically in a cell, one category of NES solutions is to minimize the SSB transmission. The following mechanisms have been specified:

- SSB-less SCell operation:

  - In R15 intra-band contiguous CA scenario, if a UE is not configured with SSB or SMTC configuration, the SCell is considered as an SSB-less SCell. Given the collocation assumption, UE is expected to reuse the timing and power information from contiguous cells on the same band, which allows network to not transmit SSBs on such SCell for network energy saving.

  - In R18, the SSB-less operation is further extended to inter-band collocated SCell in FR1. Although UE may need additional efforts to activate the inter-band SSB-less SCell compared to R15, this provides more opportunity or scenarios for network to skip SSB transmission hence further improve the energy efficiency from network side.

In our view, there is clear NES benefit from SSB-less SCell operation by skipping the regular SSB transmission. However, as SSB is mandatorily used for cell search in 5G, this solution is configurable for SCell only with assistance from some other reference cell, and it is considered feasible only in specific scenarios e.g. collocation deployment.

There is clear NES gain from SSB-less operation by skipping regular SSB transmission, but it is applicable only in specific scenarios.

- On-demand SSB (OD-SSB):

  - In R19, On-demand SSB is further specified by dynamically activating and deactivating the SSB transmission in an SCell. Instead of no SSB transmission or regular SSB transmission, network is allowed to transmit SSB on a need basis. Although the NES gain is reduced compared to fully SSB-less operation, this solution can provide a generic solution in a system design keeping network energy saving in mind while minimizing UE impact. With proper network management of OD-SSB transmissions, the NES gain can be maximized without negative impact on UEs.

OD-SSB provides a generic design for a NES solution. With proper network management of OD-SSB transmission, the NES gain can be maximized without negative impact on UEs.

- SSB adaptation:

  - In addition to OD-SSB, SSB adaptation is also discussed in R19 by allowing flexible change of SSB periodicity in an SCell. SSB adaptation can be considered as a special case of the generic OD-SSB by adapting the SSB periodicity to infinity. However, adaptive SSB also gives a good balance between NES gain and minimizing the impact to the UEs. UE power savings may be lower for the adaptive SSB compared to OD-SSB.

Adaptive SSB can be considered as a special case of OD-SSB by adapting the SSB periodicity to “infinity”, but it gives a good balance between NES gain and minimizing the impact to legacy UEs. UE power savings may be lower compared to OD-SSB.

With above observations, we believe designs which allow flexible SSB transmission periodicity, including SSB-less, OD-SSB and SSB adaptation, is the most important and efficient NES design to be considered in Day-1 6G system design. RAN4 shall discuss the impact from flexible SSB transmission periodicity on RRM procedures and related UE requirements keeping a balance between maximizing the NES gain and minimizing the negative impacts on UE.

RAN4 shall discuss the impact from flexible SSB transmission periodicity on RRM procedures and related UE requirements keeping a balance between maximizing the NES gain and minimizing the negative impacts on UE.

Adaptive transmission of other signals

Besides adaptive SSB transmission, some other NES mechanisms are identified in Rel-18 NES study item. At least the following are also considered as promising candidate NES solutions for 6G Day-1 design:

- On-demand SIB1: In Rel-19, a solution is introduced triggering the SIB1 transmission upon UE requests in RRC idle and inactive mode. Network can benefit by eliminating the need for regular SIB1 transmission and thereby increasing energy saving. This solution requires careful signaling design from RAN1/2 but the impact on RRM requirements is a bit fuzzy as the idle and inactive mode requirements are not well testable currently in RAN4.

- Cell DTX/DRX: In Rel-18, Cell DTX/DRX operation is introduced by activating or deactivating the data transmission/reception at network side from time to time. Network is hence allowed to switch off its transmitters/receivers when it is not in use – enabling network energy saving. However, the solution was designed considering backward compatibility and therefore some legacy signals e.g. SSB transmission, CSI report needed to be maintained which has significant negative impact on the possible NES gain.

There are still many other NES solutions being considered in 6G study in RAN1. While 6G Day-1 design has no restrictions on backward compatibility, these solutions can be reevaluated and redesigned based on the new 6G framework. For these RAN1/2-led NES solutions, RAN4 can wait for the RAN1/2 progress before studying the impact on RRM requirements.

For the RAN1/2-led NES solutions for 6G, RAN4 need to wait for the RAN1/2 progress before studying the impact on RRM requirements.

However, we believe RAN4 can initiate discussions related to under which conditions the UE would need e.g. SSB and under which conditions the SSB would not be necessary. As 6G is a new system which aims at integrating network power savings from Day-1, it is important to ensure that also UEs are designed in a manner that can cope with such NES solutions.

RAN4 to discuss conditions where UE would need additional synchronization signal assistance prior to e.g. data reception and under which conditions this would not be necessary.

UE Energy saving

UE Power saving framework

In rel-18 LP-WUS / WUR was studied and in rel-19 LP-WUR requirements were specified for FR1 and FR2 and extended to RedCap UEs. The requirements are only specified for idle-/inactive mode and under the assumption that the WUR is operating on the same carrier frequency as MR.

The LP-WUS/WUR creates a framework for UE power saving that allows UE to offload / relax measurements while monitoring the serving cell with LP-WUR. The main components specified are listed below:

- LP-WUS monitoring – The signal UE monitors and wakes up according to configured thresholds

- LP-WUR measurements – Receiver monitoring the serving cell (either OOK based or OFDM based receiver)

- Main radio relaxation according to configured thresholds, main radio may be either relaxed or off.

In 6G we expect that only OFDM based LP-WUR will be supported, hence there is no need to consider LP-SS based requirements.  In 5G the LP-WUS coverage assumption in RAN1 has been that it is used only in the cell centre. In 6GR, the LP-WUS signal should support full cell coverage. Additionally, WUS should support inter-frequency operations. RAN4 study should be progressed alongside RAN1 work to ensure aligned understanding.

Based on RAN1 work, study how to define requirements for WUS for both idle- and connected mode in 6G framework.

The measurements defined in LP-WUS operation should be integrated as part of idle-/inactive mode measurement framework in such way that the requirements are clear and realistic for UEs to implement and for network to support. Additionally, they need to provide decent power saving gain that allows real life power saving improvements.

For the main radio relaxations, there are multiple power saving features that are added over time. For instance, MR relaxation, RedCap relaxations and various flavours of eDRX. All of the evolutionary relaxations should be pulled together under a same framework which provides UE power saving when needed and allows UE to wake up and report relevant measurements to the network when needed.

Study how to harmonise various UE power saving measurement relaxations under a single, clear framework providing real world power saving gains. Consider power saving features at least from R15 to R19, RedCap and LP-WUS/WUR and consider idle-/inactive mode and connected mode.

Requirements in idle-/inactive mode

Most requirements seems to work as intended, while in some examples, for instance cell reselection, the delays related to idle-/inactive mode procedure are sometimes so long that it is unclear if the requirements have any practical stance.

For example, in Idle mode UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria at least every 11.52s (assuming FR1 and 320ms DRX). In idle mode in FR2 an additional scaling factor N1 applies (N1=8). Most of the latency comes from cell detection.

When adding idle-mode relaxations, which can add a scaling factor of up to 16 times to normal non-relaxed requirements the delays become extremely long. For example, the time it takes for UE to perform cell detection and evaluate whether it meets the reselection criteria is over 3 minutes (in FR1 Idle mode assuming 320ms DRX). Such long delays are of course good for optimized UE power savings, but they are only realistically usable in very slow moving or static UE conditions (non-moving). However, in idle mode the network can only rely on using broadcast information and hence allowing long delay generally (using broadcast) can be very difficult and can have severe negative impacts. Hence, although UE power saving is an important goal in 6G it should be designed such that 6G is robust – both in terms of UE receiving paging and accessing from idle mode as well as maintaining the link between UE and network (both in idle and connected mode).

6GR is aimed at supporting a scalable design where for example, power saving opportunities need to be provided to all UEs including for example low complexity devices and high-end UEs. In 6GR the design of requirements for UEs applying power saving, also in Idle mode, need to follow such scalable design as well.

Study how to define generic scalable idle-mode requirements supporting a wide range of devices including from low-power UEs to high-end UEs.

Procedure related RRM

Cell activation

In LTE and 5G, cell activation including SCell activation and PSCell additional/activation has been a very hot topic in RAN4 RRM enhancement discussion. Based on the R15 framework for SCell activation delay, several solutions are specified since R17 to reduce the activation delay in different scenarios. The following are listed as an example:

- R17 Fast SCell activation: This allows UE to use aperiodic TRS (A-TRS) instead of SSBs when activating a SCell. But this applies only to known SCell and the benefit of using A-TRS is questionable.

- R18 SCell activation triggered L3 reporting: This targets FR2 unknown SCell activation. UE is allowed to send L3-RSRP measurement reporting (if present) immediately after reception of SCell activation command so that the SCell activation can continue as if it is known.

- R19 SCell activation delay reduction with EMR reporting: In this solution, EMR report is further leveraged to determine the beam information for activating the SCell so that the SCell activation delay is reduced.

- R20 Enabling data scheduling during SCell activation: While all the above SCell activation enhancements target SCell activation delay reduction, in R20 the focus is how to enable the early data scheduling before UE sends the valid CSI report. From network side, the key issue is when the SCell can be used for data transmission.

It is clear from above that SCell is expected to be activated as fast as possible. However, if data transmission can start during SCell activation procedure, the delay requirement determined based on valid CSI report becomes an artificial metric.

Considering 6G, we believe SCell activation with small activation delay is still one of the goals in CA operation. Without restriction on backward compatibility, we shall further discuss what are the necessary delay components to be counted when specifying the SCell activation delay requirements. This can be a clean-slate approach taking into account all possible conditions to minimize the SCell activation delay, including at least the following:

- To consider the measurements and reporting across different RRC states to minimize the cell activation delay.

- To leverage the measurements from other relevant cells based on network deployments to facilitate the cell activation procedure.

For cell activation in 6G, RAN4 to discuss what are the necessary delay components to be counted when specifying the SCell activation relay requirements. At least the following conditions needs to be taken into account to minimize the SCell activation delay:

  - To consider the measurements and reporting across different RRC states

  - To leverage the measurements from other relevant cells based on network deployments

Interruptions

5G NR has specified requirements for several cases where the UE is allowed to cause interruptions. These include interruptions while performing measurements, e.g. measurements without gaps, or interruptions while performing RRM procedures, e.g. in SCell activation.

Not all interruptions are not exactly defined in specification text, therefore we provide one explanation. Interruptions may be understood as small time intervals where the UE is unavailable for scheduling. The exact time where this happens is often not specified. The impact of the interruptions is that the UE is not capable of receiving or transmitting data during this time. Examples of interruptions in the RAN4 specifications for 5G NR in 38.133 include durations from 1 slot to few slots larger than the SMTC duration [1].

Even though RAN4 spent so much time developing requirements for interruptions, there are still cases where the interruption behaviour is unclear. One such example is the discussion regarding interruptions on Rel-16 measurements without gaps [2]. During this discussion, some companies identified that Rel-16 measurements without gaps didn’t have clear requirements. The final outcome of this discussion was that a UE may or may not cause interruptions when performing measurements according to Rel-16 measurements without gaps.

Figure 2 – Example of BWP switching error caused by interruptions

One example where error cases arise due to interruptions is shown in Figure 2. In this example, the UE is being scheduled by the network initially using BWP 1, and the network sends a DCI message with BWP switch command to the UE. If the UE causes interruption overlapping with this DCI command, the UE will will miss the DCI BWP switch command and continue to monitor PDCCH for scheduling on BWP 1. However, as the network requested a BWP switch the network expects that the UE is monitoring PDCCH using the search space of BWP 2. This is illustrated in this figure with orange boxes that indicated where the UE is monitoring PDCCH. This error situation can cause severe misalignment between UE and network and make the UE unavailable for a period of time (since the network have to identify the BWP switch error and perform corrective actions). This type of error case has been observed in practical deployments.

Interruptions in 5G NR has direct system impact.

In NR, RAN4 introduced multiple scenarios when UE is allowed to interrupt – which causes problems for multiple RAN1 procedures. RAN1 have specified several features in 5G NR assuming that PDCCH and DCI is very robust. That is true looking at link level results, but when combined with UE autonomous interruptions, the reliability of those procedures get compromised.

As a result, we can conclude that interruptions cause several severe system impacts when they are not clearly specified. Those impacts are listed below:

- Impact to procedures that depend on DCI commands

- Interruptions lead to throughput reduction.

- Dropped packets due to UE autonomous interruptions cannot be distinguished by the network from dropped packets due to link problems and this can have significant negative impact on throughput and network KPIs.

- KPI and throughput degradation impact may lead to unnecessary corrective actions in real deployments.

  - TP loss also has direct impact on the UE and user experience.

Interruption unknown to network has negative system performance impact.

RAN4 has broken DCI-based features by allowing random UE autonomous interruptions.

Therefore, considering the negative impacts of random, UE autonomous interruptions, RAN4 should aim at removing these. In previous releases, alternatives to interruptions have already been developed, which proves that interruptions can be replaced by deterministic small gaps or scheduling restrictions, e.g. NCSG.

RAN4 to aim at removing all UE autonomous interruptions in 6G.

Conclusion

In this paper we discuss our view on potential areas which can be worked on regarding RAN4 RRM to address some short comings in 5G which we see can be improved 6GR. In the paper we make a number of important observations while we in the following list only the Proposals:

General 6G Aspects:

- Study how to define RRM requirements that allow UE implementation based on minimum requirements but also allow UEs that can outperform the minimum requirements the benefits from such better performance.

RAN4 to study any changes to the synchronization signal design and other related reference signals and the impact on cell detection and measurement requirements.

NTN should be considered as a day-1 feature in NR and not be relegated to separate subclauses. This includes the measurement framework.

UE Transmit Timing:

In 6G, the UE transmit timing requirements would have to address practical aspects from deployments and define UE requirements for these.

Measurement Framework:

For Connected mode, Idle mode, and Inactive mode, RAN4 to study defining measurement requirements depending on purpose of the configured measurement: mobility or data (CA).

To enable robust 6G beam management measurements and reporting, RAN4 to identify any issues related to NR beam management at early stages of 6G study item and inform RAN1 about such issues.

RAN4 to study how to simplify the measurement requirements for NTN. RAN4 to inform RAN1/2 about the conclusions of the study.

Study unified UE measurement requirements, including cell detection status and measurements, across state transitions and cell changes.

Investigate in which scenarios the UE can perform measurements without gaps in 6G.

RAN4 to study how to simplify the measurement gap framework in 6G including NTN use-cases and inter-satellite measurements.

Connected Mode Mobility:

RAN4 to study at least the following aspects of handover delay/interruption:

  - HO interruption reduction directly through RAN4-defined components

  - HO requirements based on target cell status (known, unknown, or other status)

  - Feasibility of any RAN1/2 created HO features (e.g. early DL or UL sync)

Beam Management and MIMO:

RAN4 to study the steps involved in TCI switching and identify potential reductions in TCI switching delay for 6G.

RAN4 to study how to enable robust multi TRP operation considering both beam pairs and deployments

Energy Efficiency:

RAN4 to define the 6G UE requirements from Day-1 to enable support of energy saving features with joint network and UE optimization.

RAN4 shall discuss the impact from flexible SSB transmission periodicity on RRM procedures and related UE requirements keeping a balance between maximizing the NES gain and minimizing the negative impacts on UE.

RAN4 to discuss conditions where UE would need additional synchronization signal assistance prior to e.g. data reception and under which conditions this would not be necessary.

Study how to harmonise various UE power saving measurement relaxations under a single, clear framework providing real world power saving gains. Consider power saving features at least from R15 to R19, RedCap and LP-WUS/WUR and consider idle-/inactive mode and connected mode.

Study how to define generic scalable idle-mode requirements supporting a wide range of devices including from low-power UEs to high-end UEs.

Procedure Related RRM:

For cell activation in 6G, RAN4 to discuss what are the necessary delay components to be counted when specifying the SCell activation relay requirements. At least the following conditions needs to be taken into account to minimize the SCell activation delay:

  - To consider the measurements and reporting across different RRC states

  - To leverage the measurements from other relevant cells based on network deployments

Interruptions:

RAN4 to aim at removing all UE autonomous interruptions in 6G.

- References

- 38.133

- R4-2402429, Discussion on Rel-16 no-gap reporting, Nokia, Nokia Shanghai Bell, BT Plc., Vodafone, Charter Communications Inc., Deutsche Telekom, TELECOM ITALIA S.p.A. Verizon

- R4-2118959, Discussion on RRM timing requirements for extension to 71 GHz, Nokia, Nokia Shanghai Bell