3GPP TSG-RAN WG4 Meeting #116bis            	R4-2513402

Prague, CZ, 13th-17th, Oct, 2025

Agenda item:	6.6.3.3

Source:	Samsung

Title:	Discussion on demodulation requirement for NTN NGSO testing

Document for:	Discussion

- Introduction

In RAN#104 meeting, the revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN has been approved in [1]. As one of objective is NTN_testing_NGSO_channel_model with following

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel mode

In the last RAN4#116 meeting, the demodulation requirement for NTN testing was under discussion, the related agreements are captured in the WF [2] as following

- RAN4 requirements - AoA (Ө) value setting for fading channel simulation - Use cos(Ө) = 0.7 (Same as Rel-17 NTN and Rel-18 IoT-NTN UE demodulation simulation assumptions) to run simulations when using time varying Doppler shift and propagation delay model for NTN LOS channel. - NR NTN demodulation requirement with applying time-varying channel model - Introduce an additional margin of 0.5 dB to the Rel-17 NTN UE demodulation performance requirements - IoT NTN demodulation requirement with applying time-varying channel model - RAN4 can take the residual frequency offset as 200Hz and time offset error as to check the demodulation requirement impact with applying time-varying doppler shift and propagation delay channel model for IoT NTN - For Category M1 CE Mode; Te_NTN_M1_. A; 41*TS (=1.33 us). Note 1: TS is the basic timing unit defined in TS 36.211 4.1. Note 2: This requirement applies regardless of the downlink carrier bandwidth.. - For NB-IoT Downlink Bandwidth (MHz); Te_. 0.18; 97*TS (=3.16us). Note 1: TS is the basic timing unit defined in TS 36.211 4.1. - Further discuss whether to add additional margin based on simulation results in the next meeting - Sampling frequency offset impact for testing - Interesting companies including UE and TE vendors are encouraged to provide analysis and evaluation for the impact of sampling frequency offset in related to both the Doppler frequency and its rate of change (i.e., Doppler drift) caused by the time-varying Doppler shift model for NGSO. - FFS how and whether to capture the impact of sampling frequency offset if identified - Test applicability of requirement with applying time-varying channel model - Option 1: - The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration. - Option 2: - For Rel-19 UE, if UE can pass new NGSO requirements applying the new channel model, then it can skip all NGSO requirements defined in Rel-17/Rel-18 - For Rel-17/Rel-18 UE, if UE can pass new NGSO requirements applying the new channel model, then it can skip all NGSO requirements defined in Rel-17/Rel-18. - Other options are not precluded

In this contribution, the view on the demodulation requirement impact for NR NTN and IoT NTN based on TE-emulated channel model with time-varying Doppler and delay shifts was provided. Meanwhile, the initial results with applying the time-varying Doppler shift and propagation delay model were also provided to check the impact

# 2	Discussion

Fig. 1 test environment for NGSO NTN UE testing

As agreed in previous meeting, the test environment for NGSO NTN UE testing is illustrated in Fig. 1.

Additional margin for IoT NTN

According to the test framework, UE should estimate the doppler shift and propagation delay based on the satellite position, velocity and UE position information. The estimation and pre-compensation should be verified and processed before baseband. After pre-compensation, only residual frequency offset error or time offset can be handled by reference signal. To check whether there is a demodulation requirement impact with applying the time-varying doppler shift and propagation delay channel model, companies agreed to add some residual frequency offset and time offset with considering the estimation and pre-compensation error at the baseband. For NR NTN, it was agreed that 0.5dB additional margin will considered on existing requirement. For IoT NTN, companies agreed to launch simulation campaigns to assess the impact on demodulation requirement with the given residual frequency offset and time offset error.

As agreed in the last meeting, RAN4 can take the residual frequency offset and time error to check the impact with applying time-varying doppler shift and propagation delay channel model for IoT NTN with considering the worst cases.

- Frequency offset: 200Hz

- Timing error:

  - Cat M: 41*TS (=1.33 us)

  - NB-IoT: 97*TS (=3.16us)

As agreed in [3], UE will apply the Keplerian model and equation of motion models as baseline for RAN4 requirement, although there is no limitation for UE implementation to estimate the doppler shift and propagation delay based on the signalled ephemeris information. Therefore, UE can estimate the satellite position and velocity based on this model to calculate the Doppler shift and propagation delay. Then, the residual error will be small. In current evaluation, we consider the 20% of worse cases with corresponding frequency offset and time offset error 
to check the requirement

Based on our evaluation, around 1.5 dB may need to be considered as additional margin for Cat M NTN demodulation requirement. And around 0.5dB can be considered as additional margin for NB-IoT NTN demodulation requirement.

Proposal 1: Introduce an additional margin of 1.5 dB to Rel-17/18 Cat M NTN demodulation requirement for specifying the Cat M NTN demodulation requirement with applying the time-varying doppler shift and propagation delay channel model in Rel-19

Proposal 2: Introduce an additional margin of 0.5 dB to Rel-17/18 NB-IoT NTN demodulation requirement for specifying the NB-IoT NTN demodulation requirement with applying the time-varying doppler shift and propagation delay channel model in Rel-19

Test applicability rule

In Rel-17/18, the UE demodulation requirement was specified under the zero Doppler and constant propagation delay condition which may be not realistic.  For Rel-19, the time-varying doppler shift and propagation delay was considered and modeled to specify the UE demodulation requirement.

In the previous meeting, based on WID, RAN4 has already agreed and selected the test cases for NR NTN and IoT NTN UE demodulation requirement with applying the time-varying doppler shift and propagation delay channel model, considering to minimize the number of demodulation performance requirements. Therefore, only subset of previous test cases was selected to replace the existing test cases for Rel-19 UEs testing.  Regarding the Rel-17/18 UEs, as mentioned, this channel model can provide more realistic condition compared with zero Doppler and constant propagation delay condition, then, if Rel-17/18 UE can declare to meet the requirements with timing-varying channel model specified in Rel-19, we think the corresponding test cases for Rel-17/18 UE can be skipped for testing. Other cases are still be tested pending on UE capability for NR NTN and IoT NTN with the assumption of zero doppler and constant propagation delay condition.

Proposal 3: For Rel-19 UE, the requirement for selected test cases with applying time-varying doppler shift and propagation delay channel model should be meet.

Proposal 4: For Rel-17/18 UEs, if the UE can declare to meet the requirements with time-varying doppler shift and propagation delay, the UE can skip the same test case specified in Rel-17/18. Other cases are still be tested pending on UE capability for NR NTN and IoT NTN with the assumption of zero doppler and constant propagation delay condition.

# 3	Simulation

In this section, we evaluate the performance impact for selected test cases with applying the time-varying doppler shift and propagation delay channel model for IoT NTN

Similar as NR NTN, only limited cases were selected for Cat1 M and NB-IoT NTN feature.

Test cases for eMTC NTN

For evaluation, we consider the 20% of worse cases with corresponding frequency offset (200Hz) and time offset error 
(41*TS (=1.33 us)) to check the requirement

Table 1 Minimum performance for single antenna port (FRC), cat M1, PDSCH

| Test number | Bandwidth and MCS | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Frequency offset | Time offset | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 1.4MHz 16QAM 1/2 | R.1 FDD | OP.1 FDD | NTN-TDLC5-30 | 1x1 | 0Hz | 0 us | 70 | 5.09 | M1 |
| 2 | 1.4MHz QPSK 1/3 | R.2 FDD | OP.1 FDD | NTN-TDLA100-200 | 1x1 | 0Hz | 0 us | 70 | -10.57 | M1 |
| 3 | 1.4MHz 16QAM 1/2 | R.1 FDD | OP.1 FDD | NTN-TDLC5-30 | 1x1 | 2Hz | 0.05us | 70 | 5.1 | M1 |
| 4 | 1.4MHz QPSK 1/3 | R.2 FDD | OP.1 FDD | NTN-TDLA100-200 | 1x1 | 2Hz | 0.05us | 70 | -10.56 | M1 |
| 5 | 1.4MHz 16QAM 1/2 | R.1 FDD | OP.1 FDD | NTN-TDLC5-30 | 1x1 | 40Hz | 0.2660 us (8Ts) | 70 | 6.6 | M1 |
| 6 | 1.4MHz QPSK 1/3 | R.2 FDD | OP.1 FDD | NTN-TDLA100-200 | 1x1 | 40Hz | 0.2660 us (8Ts) | 70 | -8.93 | M1 |

Test cases for NB-IoT NTN

For evaluation, we consider the 20% of worse cases with corresponding frequency offset (200Hz) and time offset error 
(97*TS (=3.16 us)) to check the requirement

Table 2 Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Frequency offset | Time offset | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.1 FDD | 32 | NTN-TDLC5-200 | 1 | 1x1 | 0Hz | 0 | 70% | -6.22 | NB1, NB2 |
| 2 | 200kHz | Anchor | R.NB.1 FDD | 32 | NTN-TDLC5-200 | 1 | 1x1 | 2Hz | 2Ts | 70% | -6.21 | NB1, NB2 |
| 3 | 200kHz | Anchor | R.NB.1 FDD | 32 | NTN-TDLC5-200 | 1 | 1x1 | 40Hz | 0.632 us (20Ts) | 70% | -6.09 | NB1, NB2 |

Based on our evaluation, around 1.5 dB may need to be considered as additional margin for Cat M NTN demodulation requirement. And around 0.5dB can be considered as additional margin for NB-IoT NTN demodulation requirement.

Observation 1: Around 1.5dB performance degradation compared with Rel-17/18 Cat M NTN demodulation requirement

Observation 2: Around 0.5dB performance degradation compared with Rel-17/18 NB-IoT NTN demodulation requirement

# 4	Conclusion

In this contribution, the view on the demodulation requirement impact with time-varying Doppler shift and propagation delay is provided. And the simulation results are provided to check the performance impact with applying the time-varying Doppler shift and propagation delay for IoT NTN

Additional margin for IoT NTN

Proposal 1: Introduce an additional margin of 1.5 dB to Rel-17/18 Cat M NTN demodulation requirement for specifying the Cat M NTN demodulation requirement with applying the time-varying doppler shift and propagation delay channel model in Rel-19

Proposal 2: Introduce an additional margin of 0.5 dB to Rel-17/18 NB-IoT NTN demodulation requirement for specifying the NB-IoT NTN demodulation requirement with applying the time-varying doppler shift and propagation delay channel model in Rel-19

Test applicability rule

Proposal 3: For Rel-19 UE, the requirement for selected test cases with applying time-varying doppler shift and propagation delay channel model should be meet.

Proposal 4: For Rel-17/18 UEs, if the UE can declare to meet the requirements with time-varying doppler shift and propagation delay, the UE can skip the same test case specified in Rel-17/18. Other cases are still be tested pending on UE capability for NR NTN and IoT NTN with the assumption of zero doppler and constant propagation delay condition.

Observation 1: Around 1.5dB performance degradation compared with Rel-17/18 Cat M NTN demodulation requirement

Observation 2: Around 0.5dB performance degradation compared with Rel-17/18 NB-IoT NTN demodulation requirement

# References

R4-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

R4- 2512525, Way Forward for [116][324] NTN_testing_NGSO_channel_model_and_demod